# Natural Resources Wales permitting decisions

# Bespoke Variation

We have decided to issue the variation for Beacon Foods Limited operated by Beacon Foods Limited.

The application number is PAN-002963

The permit number is AN0359601

The permit has been varied to increase the maximum daily discharge volume of secondary treated process effluent from 45m3/day to 90m3/day as a result of an application received by the operator. A total phosphorus limit of 3.5mg/l as an annual mean has been added to Table S3.1 as a result of the increased maximum daily discharge volume. A flow monitoring point has also been added to Table S3.3 and the site map in Schedule 7 amended accordingly.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Key issues

- Annex 1 the decision checklist

# Key issues of the decision

We have included a ‘Total Phosphorus as P’ limit of 3.5mg/l as an annual mean as the discharge volume has increased from <50m3/day to >50m3/day and this increase in volume has instigated phosphorus modelling as part of the determination process. The WFD monitoring point for the Afon Tarrell which is downstream of the discharge point implies the waterbody is Good class for phosphorus, whereas it is High class for Ammonia and BOD. Under the Habitats Directive the River Usk SAC is failing to meet the phosphorus target that has been set.

Modelling showed the a limit of 3.5mg/l of phosphorus as an annual mean

will be required in order to meet the WFD no deterioration policy of 10% deterioration as a maximum reduction in quality.

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. |  |
| ### Responses to consultation and web publicising | There were no consultation or web publicising responses. The decision was taken in accordance with our guidance. |  |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. |  |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. |  |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. |  |
| Biodiversity, Heritage, Landscape and Nature Conservation and a designated site under the CRoW Act. | The application is within the relevant distance criteria of a site of heritage, landscape or nature conservation, and/or protected species or habitat . A full assessment of the application and its potential to affect the sites has been carried out as part of the permitting process. We consider that the application will not affect the features of the sites. Formal consultation has been carried out with NRW’s Natural Resource Management Officers. The consultation responses were taken into account in the permitting decision. |  |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | In light of the conclusions of an appropriate assessment, and taking account of the advice received from protected sites advisors, it has been established that the project will not adversely affect the integrity of any Natura 2000/Ramsar site, taking into account any conditions or restrictions as applicable, either alone or in-combination with other plans and projects. As documented in section 4 of OGN 200 form 1. |  |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have carried out a risk assessment on behalf of the operator. The operator considers this risk assessment is satisfactory. The assessment shows that, applying the conservative criteria in our guidance on Environmental Risk Assessment, all emissions may be categorised as environmentally insignificant. |  |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN and we consider them to represent appropriate techniques for the facility. |  |
| The permit conditions |  |  |  |
| Emission limits | We have decided that emission limits should be set for the parameters listed in the permit. It is considered that the numeric limits described below will prevent significant deterioration of receiving waters. We have imposed numeric limits because either a relevant environmental quality or operational standard requires this. The current limits of; 15mg/l Ammonia, 40mg/l BOD and 60mg/l Suspended Solids will remain on the permit. We have also included a total Phosphorus as P limit of 3.5mg/l as an annual mean. |  |
| Monitoring | We have decided that monitoring should be carried out for the parameters listed in the permit, using the methods and to the frequencies specified. These monitoring requirements have been imposed in order to ensure the volume of effluent being discharged from the site is compliant with the limit set in Table S3.1 We made these decisions in accordance with TGN 7.01. Based on the information in the application we are satisfied that the operator’s techniques, personnel and equipment have either MCERTS certification or MCERTS accreditation as appropriate. |  |
| Reporting | We have specified reporting in the permit. The operator must submit flow monitoring data when requested to do so by NRW. We made these decisions in accordance with TGN 7.01 |  |
| Considerations of foul sewer | We agree with the operators justification for not connecting to foul sewer. |  |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. |  |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” |  |