Marine pollution contingency/emergency response plan

| Site Name & Address | AGC Site Compound, Rhos Promenade, Rhos-On-Sea Colwyn Bay LL28 4EP |
| --- | --- |
| Revision No. | A |
| Completed by: | Gwilym Dickson |
| Preparation Date: | 28/04/22 |
| Construction Phase Start Date: | 23/05/22 |

CONTENTS

1.0		INTRODUCTION

2.0		PROJECT INFORMATION

3.0		DISTRIBUTION & REVIEW

4.0		POTENTIAL POLLUTANTS FROM THIS SITE

5.0	POTENTIAL RECEPTORS THAT MAY BE AFFECTED BY POLLUTION FROM THIS SITE

- POLLUTION PREVENTION MEASURES

7.0		POLLUTION RESPONSE MEASURES

8.0		MONITORING OF POTENTIAL POLLUTANTS

1.0	INTRODUCTION

This Pollution Control Plan has been developed to enable the site team to recognise the potential pollutants that may be present on this site and to prevent them from contaminating the identified receptors.

The Plan includes:

- Consideration of all the potential pollutants present on this site

- An understanding of the receptors that may be affected by the potential pollutants described in Section 4.0

- The pollution prevention measures that will be implemented to minimise the risk of polluting any of the receptors

- The response measures that are available to minimise the impact of any pollution incident.

All works will be planned in accordance with Alun Griffiths procedure EN.S.04 – Water Management Standard along with task specific MS/RA’s to cover the specific risks and controls associated with those activities and the pollution risks associated.

2.0	PROJECT INFORMATION

The works include the following key activities:

- Reclamation of rock from existing revetments on site, and construction of modified groyne structure with reclaimed material.

- Construction of outfall extensions where existing outfalls would be blocked by the imported sand material

- Support work to enable sand importation contractor to deliver the re-nourished beach on site.

- Demolition of existing kiosks and shelters

- Construction of new surfaces, including highway surfacing, paving, high quality exposed aggregate coloured concrete, and novel surface treatments.

- Structural design and construction of new shelters

- Provision of a number of artwork features

- Public realm and street furniture improvements

3.0 	DISTRIBUTION & REVIEW

| Revision No | Date | Brief Details of Amendments | Revised By |
| --- | --- | --- | --- |

4.0		POTENTIAL POLLUTANTS FROM THIS SITE

4.1	Pollutants

It is anticipated that the following have the potential to cause pollution from construction operations on this Project:

- Plant fuels and lubricating oils

- Detergents and Degreasers

- Various construction chemicals (e.g. sealants, curing agents etc)

- Cementitious materials

- Silty water (rainwater run-off or pumping out of excavations)

- Sewage

- Contaminants existing on site within contaminated land

- Wash water from vehicle/wheel cleaning

- Emissions to the atmosphere of fumes and dust

- Noise (from construction operations)

- Light

- Dust

- Aggregate

- Construction waste – Plastic, wrapping, litter etc

Under the Control of Substances Hazardous to Health Regulations 1994 assessments will be carried out on all substances which are intended for use on the project.

All new substances brought onto site will be accompanied by a manufacturer’s Material Safety Data Sheet (M.S.D.S.).

4.2	Contaminated Areas

There are no areas of contaminated identified in the Pre-Construction Information. All excavations will be completed with vigilance and observance for potential contaminants.

Any contaminated soils must be chemically analysed by a UKAS accredited laboratory prior to excavation, to sufficiently characterise the soils.  On site treatment must be considered before disposing off site and the CL:AIRE  Code of Practice may be a way of achieving this (Contact HSEQ Dept.).

5.0		POTENTIAL RECEPTORS THAT MAY BE AFFECTED BY POLLUTION FROM THIS SITE

The potential receptors on this project are:

- Watercourse

- Drains

- Sea

6.0		POLLUTION PREVENTION MEASURES

Oil will be stored and handled in line with the AGC guidance documents

HS.S.05 – Control of oil & fuel standard

EN.S.08 – Oil, fuel & chemical management standard

The following tables summarise the measures taken for each pollutant to prevent contaminating the receptors above.

| 6.1 Fuel Oils |  |  |
| --- | --- | --- |
| Location | Site operation | Prevention Measures |
| Whole site | Refuelling plant | - Operatives trained in refuelling procedures - Suitable method for transporting fuels - Daily check of re-fuelling equipment - Spill kits available - On site transport of fuels to be kept to a minimum - Drip trays or plant nappies used under plant during refuelling - Refuelling limited to designated areas. Located at the top of the access ramp onto the beach. As far away as possible from the sea. - Weather conditions to be considered daily prior to travelling with fuel. - Daily check of re-fuelling equipment - See also ‘Table 6.3 – Refuelling arrangements’ within Environmental Risk Assessment documents. |

| 6.2 Hydraulic & Engine Oils |  |  |
| --- | --- | --- |
| Location | Site operation | Prevention Measures |
| Whole site Plant maintenance areas | Plant operation Plant maintenance | - Properly maintained plant with daily checks on the condition of hydraulic pipes and joints - All items of plant to have spill kit available - Strategically placed large spill kits positioned throughout the site. - All plant operators trained in the use and disposal of oil absorbent spill kits. - When working in or near water, a specific risk assessment to be undertaken. See environmental method statement and risk assessment. - Plant will not be parked on the beach at the end of the tidal shift - Emergency towing arrangements will be in place for the recovery of potential broken down plant on the beach - Plant Fitters to be trained in pollution prevention through toolbox talks, environmental alerts, SEATS and spill kit training - All plant fitters to carry large spill kit. All maintenance work with a potential to cause a fuel or oil spill to be notified to management prior to works being undertaken. - Wherever possible, strategically positioned plant maintenance areas to be constructed. |

| Hazards | Controls |
| --- | --- |
| Chemicals | To be kept in the COSHH store and not used on the beach or near watercourse. Mop up any spills with spill kit and absorbing granules. Report all spills. |
| Detergents and Degreasers | To be kept in the COSHH store and not used on the beach or near watercourse. Mop up any spills with spill kit and absorbing granules. Report all spills. |
| Freshly mixed concrete | Use precast concrete whenever possible. Ensure formwork is properly sealed to prevent grout loss. Concrete at low tides. Use underwater mixture when possible. |
| Silty water (rainwater run-off or pumping out of excavations) | Use silt busters and silt socks for the outlet where required, |
| Sewage | Empty site septic tank on a weekly basis to avoid it becoming too full. No work is being carried out on the existing foul networks and therefore no risks present in that respect. |
| Contaminants existing on site within contaminated land | No contaminated land identified on site. See Bio Security plan for details. |
| Wash water from vehicle/wheel cleaning | Sweeper will visit site as and when required. Wheel wash system is not being used. Lorries will be traveling on tarmac roads when enter/exit site, so dirt on wheels will be minimal. |
| Emissions to the atmosphere of fumes and dust | All loose material lorries delivering to and from the site will be fully and properly sheeted at all times. A suitable dust-suppression system must be adopted where the generation of dust would otherwise occur. |
| Noise and vibration (from construction operations) | All plant will be shut down when not in use, avoid unnecessary revving and idling. All static plant shall be acoustically screened wherever practicable. All compressors and their associated tools will be sound reduced models, the covers shall be kept closed whenever they are in use. Pneumatic tools shall be fitted with dampers to reduce the effects of vibration on the user. All diesel engine plant will be fitted with air intake silencers. The site induction for all workers includes specific reference to noise and vibration. Drop heights to be minimised. Selection of appropriate pilling technique and energy input, look at pre-augering. |
| Light | To minimise light pollution we will limit night time working, where required directional lighting will used, such as LED tower lights |
| Aggregate | Use clean material wherever possible, i.e. no dust within the aggregate. Limit the amount of aggregate being imported onto site. Cap off revetment rip rap with larger rock armour as soon as possible to stop it being washed away |
| Construction waste | Site will be kept clean and tidy at all times, this will be responsibility of all on site. Bins and skips will be provided so that individuals can dispose of litter and rubbish. A cleaner will be employed to clean the site offices. |

7.0		POLLUTION RESPONSE MEASURES

Detailed in the Spill Response Procedure document.

Contacts

|  | Address Details | Contact Details |  |
| --- | --- | --- | --- |
| Principal Contractor Operations Director | AGC North Wales Office, Penamser Business Park, Penamser Road, Porthmadog, Gwynedd, LL49 9GB | Name: Tel: Email: | Owain Thomas 0 7966 318 184 Owain.Thomas@alungriffiths.co.uk |
| Principal Contractor Site Agent | AGC North Wales Office, Penamser Business Park, Penamser Road, Porthmadog, Gwynedd, LL49 9GB | Name: Tel: Email: | Gwilym Dickson 07966390280 Gwilym.Dickson@alunriffiths.co.uk |
| Principal Contractor Works Manager | AGC North Wales Office, Penamser Business Park, Penamser Road, Porthmadog, Gwynedd, LL49 9GB | Name: Tel: Email: | Darren Hughes 07966 387 831 Darren.hughes@alungriffiths.co.uk |
| Principal Contractor Senior Engineer | AGC North Wales Office, Penamser Business Park, Penamser Road, Porthmadog, Gwynedd, LL49 9GB | Name: Tel: Email: | TBC |
| Principal Contractor Health & Safety Manager | AGC Head Office, Waterways House, Merthyr Road, Llanfoist, Abergavenny, NP7 9PE | Name: Tel: Email: | Bleddyn Escott Bleddyn.escott@alungriffths.co.uk |
| Principal Contractor Health & Safety Advisor | AGC North Wales Office, Penamser Business Park, Penamser Road, Porthmadog, Gwynedd, LL49 9GB | Name: Tel: Email: | Steve Burkhill 07811 717 274 Steve.burkhill@alungriffiths.co.uk |

This plan covers oil, fuel, chemical and sewage spills If safe to do so: STOP IT CONTAIN IT REPORT IT CLEAN IT Steps - STOP Stop work immediately. Stop the leak or eliminate the source of the spill. Eliminate ignition sources. - CONTAIN Use pollution control equipment (e.g. spill kits, drip trays, bunds of earth and sand) to contain the spill. Check the spill has not reached any drains, water courses or other sensitive areas. Cover all drains / manholes to prevent the spill from entering the drainage system. - REPORT Once the spill has been contained report according to HS.S.40 Incident Reporting Standard - CLEAN-UP Attempt to soak up the spill using absorbent material. Always follow your Duty of Care for Waste when disposing of contaminated materials including spill kit/equipment.

Pollution Response

7.1 ACCREDITED OIL SPILL RESPONSE CONTRACTOR

To ensure that any spillage incidents within Alun Griffiths are managed in a timely and

effective manner, we have teamed up with OHES Environmental to provide 24/7 spill management.

Alun Griffiths Spill Management Suppliers

0333 333 1123 Quoting account number 16-1013

7.2 LOCATION OF SPILL KITS

| Spill Kit | Location |
| --- | --- |
| 30 litres Grab Pack | All site vehicles and operated plant |
| 500 litres Spill Bins | Refuelling Points |
| Emergency Spill Booms | Near Watercourses |
| More locations TBC on site |  |

7.3 RESPONSE MEASURES FOR INDIVIDUAL POLLLUTANTS

Mitigation measures are detailed within the Environmental Method and Risk assessment.

| Hazards | Response |
| --- | --- |
| Chemicals | Implement spillage response plan |
| Detergents and Degreasers | Implement spillage response plan |
| Freshly mixed concrete | Implement spillage response plan |
| Silty water (rainwater run-off or pumping out of excavations) | Implement spillage response plan |
| Sewage | Implement spillage response plan |
| Contaminants existing on site within contaminated land | Cease works. Contain identified contamination where possible. Raise issue with site management. Site management to contact specialist contaminated land expert |
| Wash water from vehicle/wheel cleaning | Implement spillage response plan |
| Emissions to the atmosphere of fumes and dust | Immediately stop at source and site team to review control measures to ensure noise pollution is minimised below acceptable limits |
| Noise and vibration (from construction operations) | Immediately stop at source and site team to review control measures to ensure noise pollution is minimised below acceptable limits. |
| Light | Immediately stop at source and site team to review control measures to ensure noise pollution is minimised below acceptable limits |
| Aggregate | Retrieve waste/material accidentally released |
| Construction waste | Retrieve waste/material accidentally released. |

Dust

8.0		MONITORING OF POTENTIAL POLLUTANTS

As per AGC-F107 ENVIRONMENTAL MONITORING PLAN