| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-017486 |
| Permit Reference Number: | EPR-GP3792SK |
| Permit Holder: | ByProduct Recovery Limited |
| Site Address: | Bod Elith Farm, Bethal, Bala, LL23 7LA |
| Grid Reference: | Farm location: 298782, 339111 Fields: 1 298925 339213 2 298672 338983 3 298859 338881 4 299028 338803 5 299322 339336 6 299768 339279 Storage: 1 298570 339020 2 298860 338770 3 299170 338990 4 299190 339100 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: IndustryRegulation&Waste.NW@cyfoethnaturiolcymru.gov.uk Environment team: TimAmgylcheddDwyforacMeirionnydd@cyfoethnaturiolcymru.gov.uk NLO: Ewan.Marshall-Harries@environment-agency.gov.uk |
| Date due: | 06/04/2022 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating B- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y |  |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and Charging Scheme page 52 |
| Fee on MyNRW | Y | Amount: £798 Date cleared on MyNRW: 25/03/2022 Transaction number: 2099190 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | Y |  |
| Site Specific Risk Assessment | n/a | High risk location only |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | 53/228/0071 given by applicant for all fields, confirmed with MyMap. |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y |  |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y |  |
| ## Has consent been given to carry out the activity? | Y |  |
| Has all of the previous land treatment table been completed? | Y | PAN-012976 Expired 24/03/22 |
| ## Which risk band does the activity fall within? | Medium (1) |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) of High risk deployment has a SSRA been provided? | n/a |  |
| ## What type of risk assessment has been submitted? | benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | N/A |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y |  |
| Has all of the waste types table been completed? | Y |  |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | N | More information needed on “Heatrick”. All other waste streams are fine. |
| Has all of waste storage table been completed? | Y | Some storage locations repeated from last deployment – RFI. |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | N | Not specified which wastes are being stored to know how much can be stored. RFI. |
| Has checklist been completed? | Y |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| If land was previously deployed on, and waste is stored in temporary field heaps; are the storage locations in the same or different locations? | Same | Some same some different, RFI. |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y |  |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | N |  |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | Y | Unbuffered issues - RFI |
| location of all storage sites for the waste to be spread | Y | Two need moving - RFI |
| any spring(or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | Y | Unbuffered issues - RFI |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | Y | Unbuffered issues - RFI |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | N |  |
| location of any public rights of way (shown on map) | Y |  |
| any buildings or houses within 250m (shown on map) | Y |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Nitrate Vulnerable Zone (NVZ) - Groundwater Safeguard Zone - Hedges - Ditches | Y | Purple moor grass and rush pastures within field boundaries. 3 sections of Ancient Woodlands outside of field boundaries. 6 SINCS within screening distance. None of the above on the site plan provided by the applicant – RFI. |
| What is the slope of the fields? | Areas of flat, 7-12 degrees and 12+ degrees across all fields. No buffers or mitigation for spreading across steep and very steep fields. RFI |  |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | n/a |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y | Aluminium, iron for 19 09 02. |
| - If yes; have they been tested for? | Y |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y | Spring, after cutting of silage, and prior to summer grazing. RFI ref “spring” – not specific enough. |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | N | Liming noted in calculator, not mentioned in ABS |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | N | Excessive liming only. Not mentioned. RFI. |
| - Included the nutrient level in their environmental risk assessment | N |  |
| - Stated the source of information that they used to set the nutrient level. | N |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y | Over-liming not mentioned |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | n/a |  |
| - Is it only for operational purposes? | n/a |  |
| - Have individual application rates for each waste been given? | n/a |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | n/a |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | N | RFI ref “spring” |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients supplied match NRW calculations? | N | Believed error regarding units for K, RFI. Also, arsenic values removed from 4 waste streams, RFI. |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y |  |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | Y | See PAN-012976 |
| - The nutrients supplied by the material? | Y |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | n/a |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | n/a |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | n/a |  |
| - Pathways | n/a |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | n/a |  |
| - Measures that will minimise or mitigate the impact will be put in place | n/a |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | N |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | n/a |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | N |

Comments

Consultation responses: Agri officer: location falls under “Good” WFD status. No info on quantities of material spread by farmer. High indice fields to be omitted or reduced application rates. Adhere to COGAP. Biodiversity officer: concern for section 7 priority habitats. Conservation officer: recommended HRA. Outside of our screening distance so cannot be taken into account. WFD officer: fields in high and medium-high groundwater vulnerability areas. Buffers must be appropriate and operator must ensure no contamination. Application must not deteriorate phosphate conditions of connecting freshwater. Applicant must check for pollution and adhere to COGAP.

MyMap and location assessment: Steep slopes not taken into account on spreading plan, some issues not marked, SINCS not marked, ancient woodlands not marked.

Protection of habitats and ecosystems Nearby areas of protected habitats outside of the deployment – no concern. One area of purple moor grass and rush pastures within field boundaries – RFI to buffer this area.

Agricultural benefit calculations: Four waste streams have had arsenic values removed from the waste analyses. Units believed to be incorrect for K values in all waste analyses.

Request for information: Request for Information 1 – 22/04/22 - Confirm who “Heatrick” are as a waste source. Waste produced from recycled plasterboard in Northern Ireland. - Two field heaps are in the same location as last year’s deployment. Moved. - Clarification required on total on-site storage intended to be used. Clarified – only 3,000 tonnes of stackable waste on site at any one time. No liquids will be stored. - Confirm if waste analyses are recorded on a “dry matter” or “sample as received” basis. Dry matter. - Amend spreading map to include ancient woodlands, some unaddressed issues, SINCs, protected habitats and slopes, buffering the issues, protected habitat within the proposed spreading area and the steep areas of slopes, as well as providing mitigation for slightly sloping ground. Included woodlands, habitats and some slopes. Issues, some slopes and SINCs still need to be buffered, and mitigation provided for the less steeply sloping areas of land. Request for Information 2 – 03/05/22 - Mark un-buffered issues on spreading map. Buffered. - Mark SINCs on spreading map. Missing section of one SINC. Corrected map provided on 16/05/22. - Buffer steep slopes correctly. Correctly buffered. - Update key for SSSI to match how it is marked on the map. Updated. - Provide mitigation for spreading on slopes marked 7-12 degrees. Provided. - Address overapplication of lime from Heatrick waste. Very slight liming value but gypsum is being applied for its use as a soil conditioner to improve the structure of the soil. No concern. - Provide arsenic values missing from four of the waste streams. Provided, no issues. - Confirm if all %K (dry weight) values are as stated (in mg/kg) or if this is a typo and it should be given as a %. Confirmed, no issues. - Provide specific information on when and how many times they intend to spread on these fields. Provided.

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 16/05/2022 |
| ## Effective date | 16/05/2022 |
| ## Permitting Officer Date | Rhiannon Smith 16/05/2022 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 16/05/2022 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 560 |
| Sign off | 10 |
| Issuing | 30 |
| Total | 660 |