	Fugitives Emissions Review
	
	
[image: image2.jpg]




	Reporting 4.1.4
	
	

	Llay Feed Mill
	
	

	EPR/AP3337HF 
	
	

	15 March 2017
	
	


Objective

The annual review of Fugitive Emissions produced at 2 Agriculture - Llay Mill is conducted to ensure that the controls and procedures that are in place are working effectively to minimise the emissions.

Potential Fugitive emissions


Dust

	
	Controls in Place

	Intake Area
	The intake of cereals is conducted within a purpose built tipping booth fitted with local exhaust ventilation. Further LEV’s are fitted to all pneumatic raw material bins, grinders, both top hoppers of the mixers and all the supplement bins on the intake of supplements.

	Despatch Area
	Product despatch is conducted within a dedicated enclosed loading area. Despatch chutes are fitted with dust socks which are cleaned or renewed at least once a year.

	Mill production
	Dust produced during the production process is controlled by the Packed Water Scrubber. The Mill process is an enclosed system which prevents dust escaping into the outer atmosphere. This works in conjunction with the Preventative Maintenance Schedule. The scrubber is fully cleaned twice a year by an outside contractor, and the plastic filters inside the scrubber are cleaned and replaced as often as is necessary (by our yard staff and Engineers) to ensure that the emissions stay well below the permitted limits. There are two cyclones on the breeder line and one on each of the broiler lines. The cyclones take away excessive dust from the pellets. This excessive dust is then ‘recycled’ back to the meal bin to continue the production process. Regular cleaning of the Mill takes place to prevent dust build up on Mill surfaces and all doors are kept closed to ensure that no dust escapes the Mill.

	General Waste Storage
	All general site waste is stored in covered skips. Staff are instructed to keep skips closed whilst not in use to prevent birds and rodents from entering.

	Feed Spills
	There are systems in place for reporting and prompt cleaning of any spills that occur in the yard area. There are spill kit wheelie bins situated in the Mill and also by the bunded areas. Bunds are cleaned as required. The yard is regularly cleaned to prevent the build-up of dusty residues. 



Liquid
	
	Controls in Place

	Oil Tanks
	Bund walls are checked regularly for any deviations, leaks or spillages. Bunds are cleaned as required. Spill kits are also available in bunded areas and these are checked regularly to ensure they are fully equipped.

	Diesel Tanks
	Bund walls are checked regularly for any deviations, leaks or spillages. Bunds are cleaned as required. Spill kits are also available in bunded areas and these are checked regularly to ensure they are fully equipped.

	Maint. Oil Drums
	Drums of oil are stored on a bunded pallet within the warehouse. Spill kits are also available in bunded areas and these are checked regularly to ensure they are fully equipped.


All the above controls are documented within the sites Environmental Management System.

Summary

There has been no changes to the controls that are used to prevent or minimise emissions, therefore no improvements/changes are required at this time. 

Review Date: March 2018
[image: image1]