[image: image2.png]Cyfoeth
Naturiol
Cymru

Natural
Resources
Wales



 
Permit with introductory note

The Environmental Permitting (England & Wales) Regulations 2010
Mr Paul Billington
Burton Meadows Farm
Burton
Nr Rossett

Wrexham
LL12 0AS
Permit number
EPR/AB3098ZL
Burton Meadows Farm
Permit number EPR/AB3098ZL
Introductory note

This introductory note does not form a part of the permit

The main features of the permit are as follows.  
Burton Meadows Farm is situated approximately 1.2 kilometres North East from the village of Burton, near Wrexham.  It is approximateley centred on National Grid Reference SJ 35810 58920. 
The installation is to be operated by Mr Paul Billington and will house a maximum of 4079 pigs. The pigs will be brought to the site at approximately 35Kg and will be raised until they reach full weight before being removed.
The pigs will be housed in existing buildings and will be fed a liquid feed. The buildings will be ventilated naturally. Slurry will be collected in underground storage tanks which will be emptied on a frequent basis, with the slurry being removed from the site.
The status log of the permit sets out the permitting history, including any changes to the permit reference number.
	Status log of the permit

	Description
	Date
	Comments

	Application EPR/GP3533VL/A001
	Duly made 09/08/15
	Application for an intensive farming pig installation permit for 4079 Pigs

	Schedule 5 request for more information
	30/10/15
	Schedule 5 request for more information

	Information received
	03/12/15
	Information received: Information required by Schedule 5 Notice.

	Information received
	17/2/16
	Information received: Updated site drainage plan

	Permit EPR/AB3098ZL determined
	08/04/16
	Permit issued


End of introductory note

	Permit


The Environmental Permitting (England and Wales) Regulations 2010
Permit number 

EPR/AB3098ZL
The Natural Resources Body for Wales (“Natural Resources Wales”)  authorises, under regulation 13 of the Environmental Permitting (England and Wales) Regulations 2010 
Mr Paul Billington (“the operator”),
of
Burton Meadows Farm

Burton

Nr Rossett

Wrexham

LL12 0AS
to operate an installation at

Burton Meadows Farm

Burton

Nr Rossett

Wrexham

LL12 0AS
to the extent authorised by and subject to the conditions of this permit.

	Name
	Date

	[image: image1.jpg]



	8 April 2016


Kevin Ashcroft – Senior Permitting Officer

Authorised on behalf of Natural Resources Wales
Conditions

1 Management

1.1 General management

1.1.1 The operator shall manage and operate the activities:
(a) in accordance with a written management system that identifies and minimises risks of pollution, including those arising from operations, maintenance, accidents, incidents, non-conformances, closure and those drawn to the attention of the operator as a result of complaints; and
(b) using sufficient competent persons and resources.
1.1.2 Records demonstrating compliance with condition 1.1.1 shall be maintained. 
1.1.3 Any person having duties that are or may be affected by the matters set out in this permit shall have convenient access to a copy of it kept at or near the place where those duties are carried out.
1.2
Energy efficiency
1.2.1
The operator shall:
(a)
take appropriate measures to ensure that energy is used efficiently in the activities;

 (b)
review and record at least every four years whether there are suitable opportunities to improve the energy efficiency of the activities; and  

(c)
take any further appropriate measures identified by a review. 

1.3
Efficient use of raw materials
1.3.1
 The operator shall: 

(a)
take appropriate measures to ensure that raw materials and water are used efficiently in the activities; 

(b)
maintain records of raw materials and water used in the activities;

(c)
review and record at least every four years whether there are suitable alternative materials that could reduce environmental impact or opportunities to improve the efficiency of raw material and water use; and

(d)
take any further appropriate measures identified by a review.

1.4
Avoidance, recovery and disposal of wastes produced by the activities

1.4.1 The operator shall take appropriate measures to ensure that:

(a)
the waste hierarchy referred to in Article 4 of the Waste Framework Directive is applied  to the generation of  waste by the activities; and

(b)
any waste generated by the activities is treated in accordance with the waste hierarchy referred to in Article 4 of the Waste Framework Directive; and

(c)
where disposal is necessary, this is undertaken in a manner which minimises its impact on the environment.

1.4.2 The operator shall review and record at least every four years whether changes to those measures should be made and take any further appropriate measures identified by a review.

2
Operations
2.1
Permitted activities

2.1.1
The operator is only authorised to carry out the activities specified in schedule 1 table S1.1 (the “activities”). 

2.2
The site 

2.2.1
The activities shall not extend beyond the site, being the land shown edged in green on the site plan at schedule 7 to this permit.

2.3
Operating techniques

2.3.1
(a) 
The activities shall, subject to the conditions of this permit, be operated using the techniques and in the manner described in the documentation specified in schedule 1, table S1.2, unless otherwise agreed in writing by Natural Resources Wales.

(b)
If notified by Natural Resources Wales that the activities are giving rise to pollution, the operator shall submit to Natural Resources Wales for approval within the period specified, a revision of any plan specified in schedule 1, table S1.2 or otherwise required under this permit which identifies and minimises the risks of pollution relevant to that plan, and shall implement the approved revised plan in place of the original from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.
2.3.2
The operator shall maintain and implement a system to record the number of animal places and animal movements.
2.3.3
The operator shall maintain and implement a system to record the quantities of solid manure or slurry exported from the installation, as defined on the site plan in schedule 7.  The record shall include as a minimum:

· the date of export from the site

· the quantity exported 

· details of the receiving site
2.3.4
The operator shall ensure that where waste produced by the activities is sent to a relevant waste operation, that operation is provided with the following information, prior to the receipt of the waste:


(a)
the nature of the process producing the waste;


(b)
the composition of the waste;


(c)
the handling requirements of the waste;


(d)
the hazardous property associated with the waste, if applicable; and


(e)
the waste code of the waste.

2.3.5
The operator shall ensure that where waste produced by the activities is sent to a landfill site, it meets the waste acceptance criteria for that landfill.

2.4
Improvement programme

2.4.1 
The operator shall complete the improvements specified in schedule 1 table S1.3 by the date specified in that table unless otherwise agreed in writing by Natural Resources Wales.

2.4.2
Except in the case of an improvement which consists only of a submission to Natural Resources Wales, the operator shall notify Natural Resources Wales within 14 days of completion of each improvement.

3
Emissions and monitoring
3.1
Emissions to water, air or land

3.1.1
There shall be no point source emissions to water, air or land except from the sources and emission points listed in schedule 3 tables S3.1 and S3.2.  

3.2 Emissions of substances not controlled by emission limits

3.2.1 Emissions of substances not controlled by emission limits (excluding odour) shall not cause pollution. The operator shall not be taken to have breached this condition if appropriate measures, including, but not limited to, those specified in any approved emissions management plan, have been taken to prevent or where that is not practicable, to minimise, those emissions.

3.2.2
The operator shall:

(a)
if notified by Natural Resources Wales that the activities are giving rise to pollution, submit to Natural Resources Wales for approval within the period specified, an emissions management plan which identifies and minimises the risks of pollution from emissions of substances not controlled by emission limits;

(b)
implement the approved emissions management plan, from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

3.2.3
All liquids in containers, whose emission to water or land could cause pollution, shall be 


provided with secondary containment, unless the operator has used other appropriate measures 

to prevent or where that is not practicable, to minimise, leakage and spillage from the primary 

container.
3.2.4 Periodic monitoring shall be carried out at least once every 5 years for groundwater and 10 years for soil, unless such monitoring is based on a systematic appraisal of the risk of contamination.
3.3
Odour
3.3.1
Emissions from the activities shall be free from odour at levels likely to cause pollution outside the site, as perceived by an authorised officer of Natural Resources Wales, unless the operator has used appropriate measures, including, but not limited to, those specified in any approved odour management plan, to prevent or where that is not practicable to minimise the odour. 
3.4
Noise and vibration
3.4.1
Emissions from the activities shall be free from noise and vibration at levels likely to cause pollution outside the site, as perceived by an authorised officer of Natural Resources Wales, unless the operator has used appropriate measures, including, but not limited to, those specified in any approved noise and vibration management plan to prevent or where that is not practicable to minimise the noise and vibration.
3.5
Monitoring 

3.5.1
The operator shall, unless otherwise agreed in writing by Natural Resources Wales, undertake the monitoring specified in the following tables in schedule 3 to this permit:

(a) point source emissions specified in tables S3.1 and S3.2;

3.6
Pests

3.6.1 
The activities shall not give rise to the presence of pests which are likely to cause pollution, hazard or annoyance outside the boundary of the site. The operator shall not be taken to have breached this condition if appropriate measures, including, but not limited to, those specified in any approved pests management plan, have been taken to prevent or where that is not practicable, to minimise the presence of pests on the site.  

3.6.2 
The operator shall:


(a)          if notified by Natural Resources Wales, submit to Natural Resources Wales for approval within the period specified, a pests management plan which identifies and minimises risks of pollution, hazard and annoyance from pests;

(b)          implement the pests management plan, from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.
4
Information

4.1
Records
4.1.1 All records required to be made by this permit shall:

(a) be legible;

(b) be made as soon as reasonably practicable;

(c) if amended, be amended in such a way that the original and any subsequent amendments remain legible, or are capable of retrieval; and

(d) be retained, unless otherwise agreed in writing by Natural Resources Wales, for at least 6 years from the date when the records were made, or in the case of the following records until permit surrender:

(i) off-site environmental effects; and

(ii) matters which affect the condition of the land and groundwater.
4.1.2 The operator shall keep on site all records, plans and the management system required to be maintained by this permit, unless otherwise agreed in writing by Natural Resources Wales. 
4.2
Reporting

4.2.1
The operator shall send all reports and notifications required by the permit to Natural Resources Wales using the contact details supplied in writing by Natural Resources Wales.
4.2.4
The operator shall, unless notice under this condition has been served within the preceding four years, submit to Natural Resources Wales, within six months of receipt of a written notice, a report assessing whether there are other appropriate measures that could be taken to prevent, or where that is not practicable, to minimise pollution.

4.3
Notifications
4.3.1
(a) 
In the event that the operation of the activities gives rise to an incident or accident which significantly affects or may significantly affect the environment, the operator  must immediately:


(i) 
inform Natural Resources Wales; 



(ii)
take the measures necessary to limit the environmental consequences of such an incident or accident; and



(iii) 
take the measures necessary to prevent further possible incidents or 
accidents.

(b)
 In the event of a breach of any permit condition the operator must immediately:


(i) 
inform Natural Resources Wales; and



(ii) 
take the measures necessary to ensure that compliance is restored within the 
shortest possible time.

(c) 
In the event of a breach of permit condition which poses an immediate danger to human health or threatens to cause an immediate significant adverse effect on the environment, the operator must immediately suspend the operation of the activities or the relevant part of it until compliance with the permit conditions has been restored.

4.3.2       Any information provided under condition 4.3.1 (a)(i), or 4.3.1 (b)(i) shall be confirmed by sending the information listed in schedule 5 to this permit within the time period specified in that schedule.
4.3.3
Where Natural Resources Wales has requested in writing that it shall be notified when the operator is to undertake monitoring and/or spot sampling, the operator shall inform Natural Resources Wales when the relevant monitoring and/or spot sampling is to take place. The operator shall provide this information to Natural Resources Wales at least 14 days before the date the monitoring is to be undertaken.

4.3.4
Natural Resources Wales shall be notified within 14 days of the occurrence of the following matters, except where such disclosure is prohibited by Stock Exchange rules: 

Where the operator is a registered company:

(a) any change in the operator’s trading name, registered name or registered office address; and

(b)
any steps taken with a view to the operator going into administration, entering into a company voluntary arrangement or being wound up.

Where the operator is a corporate body other than a registered company:
(a) 
any change in the operator’s name or address; and

(b) any steps taken with a view to the dissolution of the operator.


In any other case: 

(a) the death of any of the named operators (where the operator consists of more than one named individual);

(b)        any change in the operator’s name(s) or address(es); and

(c) any steps taken with a view to the operator, or any one of them, going into bankruptcy, entering into a composition or arrangement with creditors, or, in the case of them being in a partnership, dissolving the partnership.

4.3.5
Where the operator proposes to make a change in the nature or functioning, or an extension of the activities, which may have consequences for the environment and the change is not otherwise the subject of an application for approval under the Regulations or this permit:

(a) Natural Resources Wales shall be notified at least 14 days before making the change; and

(b) the notification shall contain a description of the proposed change in operation.

4.3.6
Natural Resources Wales shall be given at least 14 days notice before implementation of any part of the site closure plan.

 4.4
Interpretation
4.4.1
In this permit the expressions listed in schedule 6 shall have the meaning given in that schedule.
4.4.2
In this permit references to reports and notifications mean written reports and notifications, except where reference is made to notification being made “without delay”, in which case it may be provided by telephone
Schedule 1 - Operations 

	Table S1.1 activities
	
	

	Activity listed in Schedule 1 of the EP Regulations 
	Description of specified activity
	Limits of specified activity 

	Section 6.9 A(1)(a)(ii) Rearing of pigs intensively in an installation with more than 2000 places for production pigs (over 30kg)
	The rearing of pigs in an installation with a capacity for 4079 production pig places (over 30kg).
	From receipt of production pigs (over 30kg), raw materials and fuels onto the site to removal of pigs and associated wastes from site.

	Directly Associated Activity

	Drainage collection tank
	Collection of dirty yard water/wash down water in underground storage tank
	Storage pending removal from site

	Storage of pig feeds including liquid feeds.
	Above ground feed silos (all contained within bunded areas), and temporary storage of incoming IBC’s of pig feed within an on site building.
	Storage of pig feeds in 9 storage vessels with a combined capacity of 212m3.

	Slurry storage.
	Storage of slurry in under slatted floor storage tanks
	Storage in 5 under floor storage tanks with a combined capacity of 909m3.


	Table S1.2 Operating techniques

	Description
	Parts
	Date Received

	Application EPR/GP3533VL/A001
	Responses to Part B3 section 3 of the application form and referenced supporting documentation.

	03/08/15

	Application EPR/GP3533VL/A001
	Odour Management Plan
	03/08/15

	Application EPR/GP3533VL/A001
	Noise Management Plan
	03/08/15

	Application EPR/GP3533VL/A001
	Response to Schedule 5 notice
	03/12/15

	Application EPR/GP3533VL/A001
	Email describing dirty water storage tank operations
	17/02/16


	Table S1.3 Improvement programme requirements

	Reference
	Requirement
	Date

	IC1
	The Operator shall submit a report to Natural Resources Wales detailing the full extent of the on site drainage infrastructure. This report will include the outcome of all necessary site survey and investigation.
	1 November 2016


Schedule 2 - Waste types, raw materials and fuels

	Table S2.2 Permitted waste types and quantities

	Waste code
	Description

	-
	-


Schedule 3 – Emissions and monitoring

	Table S3.1  Point source emissions to air – emission limits and monitoring requirements

	Emission point ref. & location
	Parameter  
	Source
	Limit (incl. unit) 
	Reference period
	Monitoring frequency
	Monitoring standard or method

	vent from diesel  tank as shown on the site plan received with application on 3/8/15.
	None set
	Diesel Tank
	None set
	---
	---
	---

	exhaust from generator as shown on the site plan received with application on 3/8/15.
	None set
	Generator
	None set
	---
	---
	---


	Table S3.2 Point Source emissions to water (other than sewer) – emission limits and monitoring requirements

	Emission point ref. & location
	Parameter  
	Source
	Limit 

(incl. Unit)
	Reference period
	Monitoring frequency
	Monitoring standard or method

	Outlet from clean surface water drain discharging to off-site ditch at SJ 35827 58970
	None set 
	Rainwater draining from the roofs and clean yard areas only 
	None set 
	---
	---
	---


Schedule 4 - Reporting

There is no reporting under this schedule.

Schedule 5 - Notification 

These pages outline the information that the operator must provide. 

Units of measurement used in information supplied under Part A and B requirements shall be appropriate to the circumstances of the emission. Where appropriate, a comparison should be made of actual emissions and authorised emission limits.

If any information is considered commercially confidential, it should be separated from non-confidential information, supplied on a separate sheet and accompanied by an application for commercial confidentiality under the provisions of the EP Regulations.

Part A 
	Permit Number
	

	Name of operator
	

	Location of Facility
	

	Time and date of the detection 
	


	(a) Notification requirements for any malfunction, breakdown or failure of equipment or techniques, accident, or emission of a substance not controlled by an emission limit which has caused, is causing or may cause significant pollution

	To be notified immediately

	Date and time of the event
	

	Reference or description of the location of the event 
	

	Description of where any release into the environment took place
	

	Substances(s) potentially released
	

	Best estimate of the quantity or rate of  release of substances
	

	Measures taken, or intended to be taken, to stop any emission
	

	Description of the failure or accident.
	


	(b) Notification requirements for the breach of a limit

	To be notified immediately

	Emission point reference/ source
	

	Parameter(s)
	

	Limit
	

	Measured value and uncertainty
	

	Date and time of monitoring
	

	Measures taken, or intended to be taken, to stop the emission
	


	Time periods for notification following detection of a breach of a limit

	Parameter
	Notification period

	
	

	
	

	
	


	(c) Notification requirements for the detection of any significant adverse environmental effect

	To be notified within 24 hours of detection

	Description of where the effect on the environment was detected
	

	Substances(s) detected
	

	Concentrations of substances detected
	

	Date of monitoring/sampling
	


Part B - to be submitted as soon as practicable
	Any more accurate information on the matters for notification under Part A.
	

	Measures taken, or intended to be taken, to prevent a recurrence of the incident
	

	Measures taken, or intended to be taken, to rectify, limit or prevent any pollution of the environment which has been or may be caused by the emission
	

	The dates of any unauthorised emissions from the facility in the preceding 24 months.
	


	Name*
	

	Post
	

	Signature
	

	Date
	


* authorised to sign on behalf of the operator

Schedule 6 - Interpretation 

“Accident” means an accident that may result in pollution.

 “Application” means the application for this permit, together with any additional information supplied by the operator as part of the application and any response to a notice served under Schedule 5 to the EP Regulations.

“Authorised officer” means any person authorised by Natural Resources Wales under section 108(1) of The Environment Act 1995 to exercise, in accordance with the terms of any such authorisation, any power specified in section 108(4) of that Act.

 “Building” means a construction that has the objective of providing sheltering cover and minimising emissions of noise, particulate matter, odour and litter.

 “Emissions to land” includes emissions to groundwater.

“Emissions of substances not controlled by emission limits” means emissions of substances to air, water or land from the activities, either from the emission points specified in schedule 3 or from other localised or diffuse sources, which are not controlled by an emission limit..
“EP Regulations” means The Environmental Permitting (England and Wales) Regulations SI 2010 No.675 and words and expressions used in this permit which are also used in the Regulations have the same meanings as in those Regulations.

“Groundwater” means all water, which is below the surface of the ground in the saturation zone and in direct contact with the ground or subsoil.

“Industrial Emissions Directive” means DIRECTIVE 2010/75/EU OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL of 24 November 2010 on industrial emissions

“Manure and slurry” have the following meaning:

· Manures may be either slurries or solid manures.

· Slurries consist of excreta produced by livestock whilst in a yard or building mixed with rainwater and wash water and, in some cases, waste bedding and feed. Slurries can be pumped or discharged by gravity. 

· Slurry includes duck effluent, seepage from manure and wash water.

· Solid manures include farmyard manure (FYM) and comprise material from straw-based housing systems, excreta with lots of straw/sawdust/woodchips in it, or solids from mechanical separators. 

· Most poultry systems produce solid manure (litter).

· Solid manure can generally be stacked.

“Manure management plan” means the requirements described in Section 2.3 of SGN 6.09 How to Comply – Intensive Farming.  

 “SGN How to comply – Intensive Farming” The EPR Sector Guidance Note 6.09 for intensive pig and poultry farmers, version 2 published January 2010. 

 “Year” means calendar year ending 31 December.

Schedule 7 - Site plan

Installation boundary plan
[image: image3.png]Labumum
Cottage:




(Crown Copyright. All rights reserved. Natural Resources Wales, 100030994, 2016.
END OF PERMIT
PAGE  
Permit Number   EPR/AB3098ZL
Page  ii
Burton Meadows Farm

