# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for Trevayne Farm Caravan and Camping operated by Mr Richard Reed.

The application number is PAN-016320.

The permit number is EPR/CB3594HZ.

The permit regulates a pre-existing discharge of 43m3/day of domestic sewage effluent from 44 serviced static caravans and a communal shower and toilet block serving a maximum of 60 touring caravans and 80 tents, open seasonally from the end of March to the end of October. The discharge is passed via a pumping station to be treated by septic tank B before it discharges to ground via a new infiltration system built to British Standard BS6297.

There is a permit: BP0371501 that serves septic tank A. There is also another septic tank: D is registered under exemption: NRW-WQE049018. Septic tank B was installed in 2007/08 and there was another Septic tank C which is to be converted to a pumping station. Currently the discharges from tanks B and C are unregulated and it is unknown whether either infiltration systems were built to British Standard.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. |  |
| ### Responses to web publicising | No responses were received from web publicising. |  |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. |  |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. |  |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. |  |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site Note: From OGN 200 - no conceivable impact on any Natura 2000 site, by virtue of the scale or location or nature of the project. The closest Natura 200 site falls outside our screening distance of 250m. I can therefore conclude that there is no impact pathway to the site by virtue of scale. |  |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have carried out a risk assessment on behalf of the operator. The operator considers this risk assessment is satisfactory. The assessment shows that, applying the conservative criteria in our guidance on Environmental Risk Assessment, all emissions may be categorised as environmentally insignificant. |  |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN 7.01 and we consider them to represent appropriate techniques for the facility. |  |
| The permit conditions |  |  |  |
| Improvement conditions | Based on the information on the application, we consider that we need to impose improvement conditions. We have imposed improvement conditions to ensure that: the applicant constructs an infiltration system built to BS6297 of the correct size within a fixed timescale ensuring compliance with the permit. | ü |
| Emission limits | We have decided that emission limits will not be needed for the parameters listed in the permit. Although the discharge is large it is relatively low risk. The discharge is to ground and is pre-existing with no recorded of pollution. In addition, given the location of the site, adjacent to the coast, isolated, and on an elevated ridge on the headland where there will be a significant thickness of unsaturated zone, there is no significant risk to the groundwater. As the discharge is of low risk and the volume is between 20 and 50 m3/day it is appropriate to permit this activity as a descriptive permit as per OI 01_13. | ü |
| Considerations of foul sewer | We agree with the operators justification for not connecting to foul sewer. |  |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. |  |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” |  |