Permit with introductory note

The Environmental Permitting (England & Wales) Regulations 2016

Tata Steel UK Limited

Trostre Works

Trostre

Llanelli

Carmarthenshire

SA14 9SD

Permit number

EPR/BX9471IU

Trostre Works

Permit number EPR/BX9471IU

Introductory note

This introductory note does not form a part of the permit

The main features of the permit are as follows:

This permit is a consolidation of two permits held by Tata Steel UK Limited for Trostre Works in Llanelli (the permit for the Tinplate Production Plant and a previously separate permit for the Tin Anode Plant). The main purpose of the activity at the installation is the surface treatment of metals to provide cold rolled and coated steel to various customers. The main works area of the site covers approximately 80 hectares, and is located on a wide, flat coastal plain.

The raw material for the Works consists of rolled steel coils, received by road or rail into the Pickle Line receipt bay. The coils are pickled with sulphuric acid to remove all scale, in preparation for further processing at the cold reduction mill (CRM). The CRM reduces the thickness of the coil to a given size. After cold reduction, lubricant from the CRM process is removed using an electrolytic cleaning method, using caustic solution. The single reducted coil is then sent for batch or continuous annealing, depending on the product being made. After annealing, the material is temper rolled to give the customer the required surface finish and mechanical properties. The installation also processes double-reduced coil, which is sent through the Double Reducation Mill (DRM). The major volume product of the work is tinplate. There are two tin coating lines at Trostre. The tinplating process involves cleaning, pickling, and electrolytic deposition of tin to the required coating weight.

Effluent arising at the installation is treated at a designated effluent treatment plant, then discharged to the Loughor estuary.

The status log of the permit sets out the permitting history, including any changes to the permit reference number.

| Status log of permit EPR/FP3730ZM |  |  |
| --- | --- | --- |
| Description | Date | Comments |
| Application BT6365 for a new permit received | 01/11/2002 |  |
| Request for further information (RFI) notice issued | 23/12/2002 | Further site report information required |
| Request from applicant to extend response period for RFI received | 17/01/2003 | Extension agreed to 31/03/2003 |
| RFI notice issued | 14/03/2003 | Further information on the process, its emissions, noise issues and emissions monitoring required |
| Response to first RFI received | 01/04/2003 | Response to notice incomplete |
| Response to second RFI received | 03/04/2003 | Response to notice incomplete |
| RFI notice issued | 16/03/2003 | Further information on noise issues and site report required |
| Response to third RFI received | 17/04/2003 | Satisfactory responses now received to all notices, except for some minor issues on site report |
| Request for determination extension (RDE) issued | 13/05/2003 |  |
| Response to RDE received | 22/05/2003 | Extension to determination period agreed |
| Letter from the Agency to the applicant issued | 23/05/2003 | Letter to clarify deficiencies in site report |
| Response to letter received | 17/06/2003 | Satisfactory |
| RDE issued | 04/072003 |  |
| Response to RDE received | 08/07/2003 | Extension to determination period agreed |
| Application BT6365 determined and permit issued to Short Bros (Plant) Limited | 16/07/2003 |  |
| Variation application BV9250 received | Letter requesting extension to Improvement Condition (IC) 9.1 deadline received 17/09/2003 | Deadline for completion 19/09/2003 |
| Variation application BV9250 determined and variation notice issued to Short Bros (Plant) Limited | 26/09/2003 | Extension to IC 9.1 deadline to 17th October 2003 |
| Transfer application SP3235MM determined and transfer notice and consolidated permit issued to Multiserv Group Limited | 27/10/2006 | Transfer of previous permit BT6365 from Short Bros (Plant) Limited to Multiserv Group Limited |
| Transfer application EPR/FP3730ZM/T001 received | Duly made 21/12/2012 |  |
| Transfer application EPR/FP3730ZM/T001 determined and transfer notice issued to Tata Steel UK Limited | 24/12/2012 | Transfer of previous permit SP3235MM from Harsco Metals Group Limited (previously Multiserv Group Limited) to Tata Steel UK Limited |
| Variation application EPR/FP3730ZM/V002 received | Duly made 24/12/2014 | Variation to remove the regenerative thermal oxidiser |
| Variation application EPR/FP3730ZM/V002 determined and variation notice issued to Tata Steel UK Limited | 07/04/2015 |  |
| Variation application EPR/BX9471IU/V006 received | Duly made 20/09/2016 |  |
| RFI notice issued | 15/12/2016 |  |
| Response to RFI received | 19/12/2016 |  |
| Variation application EPR/BX9471IU/V006 determined and variation notice and consolidated permit issued to Tata Steel UK Limited | 11/01/2017 | Variation consolidates permit EPR/FP3730ZM with permit EPR/BX9471IU |

| Status log of the permit EPR/BX9471IU |  |  |
| --- | --- | --- |
| Description | Date | Comments |
| Application BX9471IU for a new permit received | Duly made 29/07/2004 |  |
| RFI notice issued | 22/10/2004 | Amended site report required |
| Response to RFI received | 28/01/2005 |  |
| RFI notice issued | 16/02/2005 | Site plans required |
| Response to RFI received | 17/02/2005 |  |
| Application BX9471IU determined and permit issued to Corus UK Limited | 24/03/2005 |  |
| Variation application CP3433LF received | Duly made 12/10/2005 | Proposal to change NOx emission limit |
| Variation application CP3433LF determined and variation notice issued to Corus UK Limited | 03/04/2006 |  |
| Partial surrender application RP3539LV received | Duly made 12/01/2006 |  |
| Variation application KP3232MN received | 09/06/2006 | Request to incorporate changes to composite sampling of effluent, receipt of oily waste from Pontardulais Coated Metals and excess pickling acid from Port Talbot Works |
| Partial surrender application RP3539LV determined and partial surrender notice issued to Corus UK Limited | 13/12/2006 |  |
| Variation application KP3232MN determined and variation notice issued to Corus UK Limited | 13/12/2006 | Changes to Improvement Programme and discharges to surface water |
| Variation application EPR/BX9471IU/V004 received | 12/05/2011 |  |
| RFI notice issued | 08/06/2011 | Extra information on PSA testing, status of continuous annealing process (ceased), and iron testing procedure required |
| Response to RFI received | 21/06/2011 |  |
| Variation application EPR/BX9471IU/V004 determined and variation notice issued to Tata Steel UK Limited (name change from Corus UK Limited) | 24/06/2011 |  |
| Variation application EPR/BX9471IU/V005 received | 26/09/2011 |  |
| Variation application EPR/BX9471IU/V005 determined and variation notice issued to Tata Steel UK Limited | 24/06/2011 |  |
| Variation application EPR/BX9471IU/V006 received | Duly made 20/09/2016 |  |
| RFI notice issued | 15/12/2016 |  |
| Response to RFI received | 19/12/2016 |  |
| Variation application EPR/BX9471IU/V006 determined and variation notice and consolidated permit issued to Tata Steel UK Limited | 11/01/2017 | Variation consolidates permit EPR/FP3730ZM with permit EPR/BX9471IU |

End of introductory note

Permit

The Environmental Permitting (England and Wales) Regulations 2016

Permit number

EPR/BX9471IU

The Natural Resources Body for Wales (“Natural Resources Wales”)  authorises, under regulation 13 of the Environmental Permitting (England and Wales) Regulations 2016

Tata Steel UK Limited (“the operator”),

whose registered office is

30 Millbank

London

SW1P 4WY

company registration number 2280000

to operate a regulated facility at

Trostre Works

Trostre

Llanelli

Carmarthensire

SA14 9SD

to the extent authorised by and subject to the conditions of this permit.

| Signed | Date |
| --- | --- |
|  | 11th January 2017 |

Victoria Seller

Authorised on behalf of Natural Resources Wales

Conditions

# Management

## General management

### The operator shall manage and operate the activities:

#### in accordance with a written management system that identifies and minimises risks of pollution, including those arising from operations, maintenance, accidents, incidents, non-conformances, closure and those drawn to the attention of the operator as a result of complaints; and

#### using sufficient competent persons and resources.

### Records demonstrating compliance with condition 1.1.1 shall be maintained.

### Any person having duties that are or may be affected by the matters set out in this permit shall have convenient access to a copy of it kept at or near the place where those duties are carried out.

## 1.2	Energy efficiency

### 1.2.1	The operator shall:

(a)	take appropriate measures to ensure that energy is used efficiently in the activities;

(b)	review and record at least every four years whether there are suitable opportunities to improve the energy efficiency of the activities; and

(c)	take any further appropriate measures identified by a review.

## 1.3	Efficient use of raw materials

### 1.3.1	The operator shall:

(a)	take appropriate measures to ensure that raw materials and water are used efficiently in the activities;

(b)	maintain records of raw materials and water used in the activities;

(c)	review and record at least every four years whether there are suitable alternative materials that could reduce environmental impact or opportunities to improve the efficiency of raw material and water use; and

(d)	take any further appropriate measures identified by a review.

## 1.4	Avoidance, recovery and disposal of wastes produced by the activities

### The operator shall take appropriate measures to ensure that:

### (a)	the waste hierarchy referred to in Article 4 of the Waste Framework Directive is applied  to the generation of  waste by the activities; and

### (b)	any waste generated by the activities is treated in accordance with the waste hierarchy referred to in Article 4 of the Waste Framework Directive; and

### (c)	where disposal is necessary, this is undertaken in a manner which minimises its impact on the environment.

### The operator shall review and record at least every four years whether changes to those measures should be made and take any further appropriate measures identified by a review.

# 2	Operations

2.1	Permitted activities

### 2.1.1	The operator is only authorised to carry out the activities specified in schedule 1 table S1.1 (the “activities”).

## 2.2	The site

2.2.1	The activities shall not extend beyond the site, being the land shown edged in green on the second site plan at schedule 7 to this permit.

## 2.3	Operating techniques

#### 2.3.1	(a) 	The activities shall, subject to the conditions of this permit, be operated using the

#### techniques and in the manner described in the documentation specified in 	schedule 1, table S1.2, unless otherwise agreed in writing by Natural Resources Wales.

#### (b)	If notified by Natural Resources Wales that the activities are giving rise to pollution, the operator shall submit to Natural Resources Wales for approval within the period specified, a revision of any plan or other documentation (“plan”) specified in schedule 1, table S1.2 or otherwise required under this permit which identifies and minimises the risks of pollution relevant to that plan , and shall implement the approved revised plan in place of the original from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

2.3.2	Any raw materials or fuels listed in schedule 2 table S2.1 shall conform to the specifications set out in that table.

# 3	Emissions and monitoring

## 3.1	Emissions to water, air or land

### 3.1.1	There shall be no point source emissions to water, air or land except from the sources and emission points listed in schedule 3 tables S3.1 and S3.2.

### 3.1.2	The limits given in schedule 3 shall not be exceeded.

    - Total annual emissions from the emission point(s) set out in tables schedule 3 S3.1 and S3.2 of a substance listed in schedule 3 table S3.3 shall not exceed the relevant limit in table S3.3.

    - Periodic monitoring shall be carried out at least once every 5 years for groundwater and 10 years for soil, unless such monitoring is based on a systematic appraisal of the risk of contamination.

## Emissions of substances not controlled by emission limits

### Emissions of substances not controlled by emission limits (excluding odour) shall not cause pollution. The operator shall not be taken to have breached this condition if appropriate measures, including, but not limited to, those specified in any approved emissions management plan, have been taken to prevent or where that is not practicable, to minimise, those emissions.

3.2.2	The operator shall:

#### (a)	if notified by Natural Resources Wales that the activities are giving rise to pollution, submit to Natural Resources Wales for approval within the period specified, an emissions management plan which identifies and minimises the risks of pollution from emissions of substances not controlled by emission limits;

#### (b)	implement the approved emissions management plan, from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

3.2.3	All liquids in containers, whose emission to water or land could cause pollution, shall be provided with secondary containment, unless the operator has used other appropriate measures to prevent or where that is not practicable, to minimise, leakage and spillage from the primary container.

## 3.3	Monitoring

### 3.3.1	The operator shall, unless otherwise agreed in writing by Natural Resources Wales, undertake the monitoring specified in the following tables in schedule 3 to this permit:

#### point source emissions specified in tables S3.1 and S3.2

3.3.2	The operator shall maintain records of all monitoring required by this permit including records of the taking and analysis of samples, instrument measurements (periodic and continual), calibrations, examinations, tests and surveys and any assessment or evaluation made on the basis of such data.

3.3.3	Monitoring equipment, techniques, personnel and organisations employed for the emissions monitoring programme and the environmental or other monitoring specified in condition 3.3.1 shall have either MCERTS certification or MCERTS accreditation (as appropriate), where available, unless otherwise agreed in writing by Natural Resources Wales.

3.3.4	Permanent means of access shall be provided to enable sampling/monitoring to be carried out in relation to the emission points specified in schedule 3 tables S3.1 and S3.2 unless otherwise agreed in writing by Natural Resources Wales.

## 3.4	Odour

#### 3.4.1	Emissions from the activities shall be free from odour at levels likely to cause pollution outside the site, as perceived by an authorised officer of Natural Resources Wales, unless the operator has used appropriate measures, including, but not limited to, those specified in any approved odour management plan, to prevent or where that is not practicable to minimise the odour.

3.4.2	The operator shall:

#### (a)	if notified by Natural Resources Wales that the activities are giving rise to pollution outside the site due to odour, submit to Natural Resources Wales for approval within the period specified, an odour management plan which identifies and minimises the  risks of pollution from odour;

#### (b)	implement the approved odour management plan, from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

## 3.5	Noise and vibration

#### 3.5.1	Emissions from the activities shall be free from noise and vibration at levels likely to cause pollution outside the site, as perceived by an authorised officer of Natural Resources Wales, unless the operator has used appropriate measures, including, but not limited to, those specified in any approved noise and vibration management plan to prevent or where that is not practicable to minimise the noise and vibration.

3.5.2	The operator shall:

#### (a)	if notified by Natural Resources Wales that the activities are giving rise to pollution outside the site due to noise and vibration, submit to Natural Resources Wales for approval within the period specified, a noise and vibration management plan which identifies and minimises the risks of pollution from noise and vibration;

#### (b)	implement the approved noise and vibration management plan, from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

# 4	Information

## 4.1	Records

### All records required to be made by this permit shall:

### be legible;

### be made as soon as reasonably practicable;

### if amended, be amended in such a way that the original and any subsequent amendments remain legible, or are capable of retrieval; and

### be retained, unless otherwise agreed in writing by Natural Resources Wales, for at least 6 years from the date when the records were made, or in the case of the following records until permit surrender:

- off-site environmental effects; and

- matters which affect the condition of the land and groundwater.

4.1.2	The operator shall keep on site all records, plans and the management system required to be maintained by this permit, unless otherwise agreed in writing by Natural Resources Wales.

## 4.2	Reporting

4.2.1	The operator shall send all reports and notifications required by the permit to Natural Resources Wales using the contact details supplied in writing by Natural Resources Wales.

### 4.2.2	A report or reports on the performance of the activities over the previous year shall be submitted to Natural Resources Wales by 31 January (or other date agreed in writing by Natural Resources Wales) each year.  The report(s) shall include as a minimum:

#### (a)	a review of the results of the monitoring and assessment carried out in accordance with the permit including an interpretive review of that data;

#### (b)	the annual production /treatment data set out in schedule 4 table S4.2; and

#### (c)	the performance parameters set out in schedule 4 table S4.3 using the forms specified in table S4.4 of that schedule.

### 4.2.3	Within 28 days of the end of the reporting period the operator shall, unless otherwise agreed in writing by Natural Resources Wales, submit reports of the monitoring and assessment carried out in accordance with the conditions of this permit, as follows:

### (a)	in respect of the parameters and emission points specified in schedule 4 table S4.1;

### (b)	for the reporting periods specified in schedule 4 table S4.1 and using the forms specified in schedule 4 table S4.4 ; and

### (c)	giving the information from such results and assessments as may be required by the forms specified in those tables.

### 4.2.4	The operator shall, unless notice under this condition has been served within the preceding four years, submit to Natural Resources Wales, within six months of receipt of a written notice, a report assessing whether there are other appropriate measures that could be taken to prevent, or where that is not practicable, to minimise pollution.

### 4.2.5	The operator shall submit an annual solvent management plan in order to demonstrate compliance with the requirements of the Industrial Emissions Directive, by 31 January each year in respect of the previous year.

## 4.3	Notifications

4.3.1	(a)	In the event that the operation of the activities gives rise to an incident or accident which significantly affects or may significantly affect the environment, the operator must immediately—

(i)	inform Natural Resources Wales,

(ii)	take the measures necessary to limit the environmental consequences of such an incident or accident, and

(iii)	take the measures necessary to prevent further possible incidents or accidents;

(b)	in the event of a breach of any permit condition the operator must immediately—

(i)	inform Natural Resources Wales, and

(ii)	take the measures necessary to ensure that compliance is restored within the shortest possible time;

(c)	in the event of a breach of permit condition which poses an immediate danger to human health or threatens to cause an immediate significant adverse effect on the environment, the operator must immediately suspend the operation of the activities or the relevant part of it until compliance with the permit conditions has been restored.

### 4.3.2	Any information provided under condition 4.3.1 shall be confirmed by sending the information listed in schedule 5 to this permit within the time period specified in that  schedule.

### 4.3.3	Where Natural Resources Wales has requested in writing that it shall be notified when the operator is to undertake monitoring and/or spot sampling, the operator shall inform Natural Resources Wales when the relevant monitoring and/or spot sampling is to take place. The operator shall provide this information to Natural Resources Wales at least 14 days before the date the monitoring is to be undertaken.

### 4.3.4	Natural Resources Wales shall be notified within 14 days of the occurrence of the following matters, except where such disclosure is prohibited by Stock Exchange rules:

Where the operator is a registered company:

#### any change in the operator’s trading name, registered name or registered office address; and

#### (b)	any steps taken with a view to the operator going into administration, entering into a company voluntary arrangement or being wound up.

Where the operator is a corporate body other than a registered company:

#### any change in the operator’s name or address; and

#### any steps taken with a view to the dissolution of the operator.

#### In any other case:

#### the death of any of the named operators (where the operator consists of more than one named individual);

#### any change in the operator’s name(s) or address(es); and

#### any steps taken with a view to the operator, or any one of them, going into bankruptcy, entering into a composition or arrangement with creditors, or, in the case of them being in a partnership, dissolving the partnership.

### 4.3.5	Where the operator proposes to make a change in the nature or functioning, or an extension of the activities, which may have consequences for the environment and the change is not otherwise the subject of an application for approval under the Regulations or this permit:

### Natural Resources Wales shall be notified at least 14 days before making the change; and

### the notification shall contain a description of the proposed change in operation.

### 4.3.6	Natural Resources Wales shall be given at least 14 days notice before implementation of any part of the site closure plan.

### 4.3.7	Where the operator has entered into a climate change agreement with the Government, Natural Resources Wales shall be notified within one month of:

- (a)	a decision by the Secretary of State not to re-certify the agreement;

- (b)	a decision by either the operator or the Secretary of State to terminate the agreement; and

- (c)	any subsequent decision by the Secretary of State to re-certify such an agreement.

## 4.4	Interpretation

### 4.4.1	In this permit the expressions listed in schedule 6 shall have the meaning given in that schedule.

### 4.4.2	In this permit references to reports and notifications mean written reports and notifications, except where reference is made to notification being made “immediately”, in which case it may be provided by telephone.

# Schedule 1 - Operations

| Table S1.1 Activities |  |  |
| --- | --- | --- |
| Activity listed in Schedule 1 of the EP Regulations | Description of specified activity | Limits of specified activity |
| Section 2.3 Part A(1)(a) – Surface treatment of metals and plastics exceeding 30m3 | Pickling of metal strip and cleaning of metal strip on the Pickling Line, Continuous Annealing Process Line, Cleaning Line, No.4 Electrolytic Chromium Coated Steel Line and the No.5 and 6 Electrolytic Tinning Lines | Pickling Line, the Continuous Annealing Process Line, the Cleaning Line, the No.4 Electrolytic Chromium Coated Steel Line and the No.5 and No. 6 Electrolytic Tinning Line tanks only |
| Section 1.1 Part A(1)(a) – Burning any fuel in an appliance with a rated thermal input of 50 Megawatts or more | Natural gas combustion in the Continuous Annealing Process Line, the Batch Annealing Line and the tin anode furnaces | Continuous Annealing Process Line furnaces, Batch Annealing furnaces and tin anode furnaces only |
| Section 2.2 Part A(1)(b) – Melting, including making alloys, of non-ferrous metals, including recovered products and the operation of non-ferrous metal foundries where: (i) the plant has a melting capacity of more than 4 tonnes per day for lead or cadmium or 20 tonnes per day for all other metals; and (ii) any furnace (other than a vacuum furnace), bath or other holding vessel used in the plant for the melting has a design holding capacity of 5 or more tonnes | Annealing processes in the Continuous Annealing Process Line and the Batch Annealing Line Manufacture of tin anodes at the tin anode plant | The Continuous Annealing Process Line furnaces and the Batch Annealing Line furnaces only Melting tin in natural gas-fired furnaces for the manufacture of tin anodes |
| Directly Associated Activity |  |  |
| Directly Associated Activity | Treatment and disposal of non-hazardous wastes by physico-chemical methods | Effluent Treatment Plant only. Activity may receive, store and use excess pickling acids from Tata Steel UK Limited, Port Talbot Works for use as a raw material in the effluent treatment plant |
| Directly Associated Activity | Treatment and disposal of hazardous wastes by physico-chemical methods | Effluent Treatment Plant only. Only used QWERL rolling oil, only from CSP Pontardulais or rolling oils used at the installation |
| Directly Associated Activity | Cold rolling of steel strip | Cold rolling / reversing mills only |
| Directly Associated Activity | Discharge of treated effluent | Discharge to Loughor Estuary from point W1 only |
| Directly Associated Activity | Collection, segregation, and temporary storage pending off-site disposal of waste | Dedicated waste handling, storage and disposal areas within installation boundary |
| Raw materials handling | Receipt, storage and preparation of raw materials | Operations and storage prior to melting process |
| Products, and waste products handling | Product and by-product processing and waste handling | Operations (including casting) and storage following tin melting process |

| Table S1.2 Operating techniques |  |  |
| --- | --- | --- |
| Description | Parts | Date Received |
| Application for permit BX9471IU | Proposed activities and abatement Sections 2.1.1 to 2.2.5 in application document and associated diagrams and plans | 29/07/2004 |
|  | Management techniques Sections 2.3.1 to 2.3.17 in application document |  |
|  | Monitoring Sections 2.10.1 to 2.10.7 in application document |  |
| Application for permit BT6365 (Tin Anode Plant) | Sections 2.1 to 2.11 in the application document | 01/11/2002 |
| Additional information in response to RFI issued on 14/03/2003 (application for permit BT6365) (Tin Anode Plant) | Variation determined EPR/BX9471IU | 03/04/2003 |
| Additional information in response to RFI issued on 16/03/2003 (application for permit BT6365) (Tin Anode Plant) | Noise and vibration The responses given to question 8 onf the RFI notice issued on 16/03/2003 | 17/04/2003 |
| IC1 Submissions | Corus submissions | 26/09/2005 and 07/08/2006 |
| IC 3 Submissions | Corus submission dated 31 March 2006 - study into use of Ion exchange technology for Lines 4, 5 and 6 | 02/04/2006 |
| IC4 Submission | Corus submission - assessment into the recycling or recovery of spent pickling liquors | 30/09/2005 |
| IC5 Submission | Corus submission - assess compliance with the Oil Storage Regulations | 30/09/2005 |
| IC7 Submissions | Corus submission - assessment to establish the efficiency of the Effluent Treatment Plant Corus submission - additional information | 04/01/2006 and 16/01/2006 |
| IC9 Submission | Corus submission including Appendix A - minimising water usage | 16/01/2006 |
| IC10 Submission | Corus submission - noise monitoring | 30/06/2006 |
| IC11 and IC12 Submission | Corus submission - MCERTS flow at W1 - dated 30 June 2005 | 06/07/2006 |
| IC13 Submission | Corus submission - feasibility study of installing a real-time detector for the analysis of chromium levels in the incoming flume to the Effluent Treatment Plant | 30/09/2005 |
| IC15 Submission | Corus submission | 04/01/2006 |
| IC16 Submission | Corus submission, dated 14th February 2007 | 16/02/2007 |
| IC17 Submission | Corus submission - WTP Efficiency | 29/01/2007 |
| IC18 Submission | Corus submission - inspection programme for subsurface structures, dated 2nd May 2007 | 04/05/2007 |
| IC19 Submission | Corus submission - feasibility study to reduce the temperature of the final effluent discharged | 03/05/2007 |
| IC20 Submission | Corus submission - BOD investigation into effluent, dated 27th April 2007 | 02/05/2007 |
| IC21 Submission | Corus submission - study to monitor the effluent discharged from WI for Copper, Nickel and Zinc, dated 1st February 2007 | 05/02/2007 |
| IC22 Submission | Corus submission - feasibility assessment of the options available for improving effluent discharge, dated 2nd January 2008 | 02/01/2008 |
| IC23 Submission | Tata submission | 29/10/2012 |
| Variation application EPR/BX9471IU/V006 | Sections 2.1.1, 3.2, 6.1 and 6.3 in ETL5 REACH permit variation.doc, dated 05/09/2016 | 20/09/2016 |

Schedule 2 - Waste types, raw materials and fuels

| Table S2.1 Raw materials and fuels |  |
| --- | --- |
| Raw materials and fuel description | Specification |
| - | - |

Schedule 3 – Emissions and monitoring

| Table S3.1 Point source emissions to air – emission limits and monitoring requirements |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Emission point ref. & location | Source | Parameter | Limit (including unit) [1] | Reference period | Monitoring frequency | Monitoring standard or method [2, 3] |  |
| A1 Tinplate Works | Scalemaster stack | - | - | - | - | - |  |
| A2 Tinplate Works | Acid fume stack | - | - | - | - | - |  |
| A3 | Main fume exhaust, 5 Stand CRM | - | - | - | - | - |  |
| A4 | Auxiliary fume exhaust, 5 Stand CRM | - | - | - | - | - |  |
| A5 | Caustic fume exhaust, Continuous Annealing Product Line | - | - | - | - | - |  |
| A6 | Furnace stack, Continuous Annealing Product Line | Oxides of Nitrogen as NO2 | 350 mg/m3 | Hourly average | Quarterly | ISO 10849 |  |
| A8 | Batch annealing stack [East] | - | - | - | - | - |  |
| A9 | Batch annealing stack [West] | - | - | - | - | - |  |
| A10 | Temper Mill stack | - | - | - | - | - |  |
| A11 | Main fume exhaust, Double Reduction Mill | - | - | - | - | - |  |
| A12 | ETL5 process stack | - | - | - | - | - |  |
| A13 | ECCS 4 process stack | - | - | - | - | - |  |
| A15 | Cleaning Line stack | - | - | - | - | - |  |
| A18 | Cooling Tower | - | - | - | - | - |  |
| A19 | ETL6 process stack | - | - | - | - | - |  |
| A20 | ETL6 pre-treatment process stack | - | - | - | - | - |  |
| A21 | ECCS North Wall Vent Point | - | - | - | - | - |  |
| A22 | ETL6 Chem-treat process stack/ECCS 4 chromium stack | - | - | - | - | - |  |
| A23 | ECCS 4/pickler recirculation stack | - | - | - | - | - |  |
| A1 Tin Anode Plant [Rear of building, height 9.5m, minimum 3m above edge of roof] | Gas-fired furnace flue from Furnace (melting pot) 1 | Oxides of nitrogen (expressed as carbon) | 100 mg/m3 | Minimum 4 hour sample period | Annually | - |  |
| A2 Tin Anode Plant [Rear of building, height 9.5m, minimum 3m above edge of roof] | Gas-fired furnace flue from Furnace (melting pot) 2 | Oxides of nitrogen (expressed as carbon) | 100 mg/m3 | Minimum 4 hour sample period | Annually | - |  |

Note 1: See Schedule 6 for reference conditions

Note 2: Or alternative standard or method agreed in writing by Natural Resources Wales

Note 3: Access points for sampling may be fixed rather than permanent if they comply with the requirements of Natural Resources Wales’ Technical Guidance Note M1

| Table S3.2 Point Source emissions to water (other than sewer) and land – emission limits and monitoring requirements |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Emission point ref. & location | Source | Parameter | Limit (incl. unit) | Reference Period | Monitoring frequency | Monitoring standard or method [2] |  |
| W1 - receiving water is Loughor Estuary at NGR SS5237 9827 | Process effluent discharge from effluent treatment plant | Flow | 190 ltitres/ second | Daily average | Continuous | As stated in response to IC11 |  |
|  |  | Temperature | 350C | - | Continuous | [3] |  |
|  |  | pH | 5.0 to 10.0 | - | Continuous | BS60682.50:1995 ISO 10523:1884 |  |
|  |  | Suspended solids | 60 mg/l | - | Daily composite sample | BS EN 872:1996 BS6068-2.54:1996 |  |
|  |  | Oil and grease | 12 mg/l | - | Daily composite sample | SCA Blue book 77 ISBN 0117517283 |  |
|  |  | Dissolved iron | 40 mg/l | - | Daily composite sample | BS ISO 17294-2 :2003 BS6068-2.89:2003 |  |
|  |  | Total chromium | 0.4 mg/l | - | Daily composite sample | BS ISO 17294-2 :2003 BS6068-2.89:2003 |  |
|  |  | Mercury | 0.005 mg/l | - | Monthly | Compliance based on mass balance calculation[4] |  |
|  |  | Cadmium | 0.01 mg/l | - | Monthly | Compliance based on mass balance calculation[4] |  |
|  |  | Chemical Oxygen Demand (COD) 2 hrs | 300 mg/l [1] | - | Daily spot sample | SCA Blue book 97 ISBN 0117519154 |  |
| W2 - receiving water is River Loughor at NGR SS5266 9905 | Uncontaminated surface water, East storm water drain | - | - | - | - | - |  |
| W3 - receiving water is River Loughor at NGR SS5262 9906 | Uncontaminated surface water, West storm water drain | - | - | - | - | - |  |

Note 1: Or as agreed in writing by Natual Resources Wales.

Note 2: EN BS ISO or SCA Blue book or other methods may be used alternatively if agreed in writing by Natural Resources Wales

Note 3: As method agreed in writing by Natural Resources Wales

Note 4: See Schedule 6 for interpretation

| Table S3.3 Annual limits |  |  |
| --- | --- | --- |
| Substance | Medium | Limit (kg) (Compliance based on mass balance calculation) |
| Mercury | Water | 40 in a year |
| Cadmium | Water | 40 in a year |

Schedule 4 - Reporting

Parameters, for which reports shall be made, in accordance with conditions of this permit, are listed below.

| Table S4.1 Reporting of monitoring data |  |  |  |
| --- | --- | --- | --- |
| Parameter | Emission or monitoring point/reference | Reporting period | Period begins |
| Oxides of nitrogen | A6 | Quarterly | 01/04/2005 |
| Oxides of nitrogen | A1 and A2 Tin Anode Plant | Annually | 01/07/2003 |
| Flow | W1 | Quarterly | 01/04/2005 |
| pH | W1 | Quarterly | 01/04/2005 |
| Temperature | W1 | Quarterly | 01/04/2005 |
| Suspended solids | W1 | Quarterly | 01/04/2005 |
| Oil and grease | W1 | Quarterly | 01/04/2005 |
| Dissolved iron | W1 | Quarterly | 01/04/2005 |
| Total chromium | W1 | Quarterly | 01/04/2005 |
| Mercury | W1 | Annual | 01/04/2005 |
| Cadmium | W1 | Annual | 01/04/2005 |
| COD | W1 | Quarterly | 01/04/2007 |
| Waste disposal and/or recovery, tonnes | Installation | Annually | 01/04/2005 |
| Water usage, m3 | Installation | Annually | 01/04/2005 |
| Energy usage, MW | Installation | Annually | 01/04/2005 |

| Table S4.2: Annual production/treatment |  |
| --- | --- |
| Parameter | Units |
| Tin-plated steel coil | tonnes |
| Electrically coated chrome oil | tonnes |

| Table S4.3 Performance parameters |  |  |
| --- | --- | --- |
| Parameter | Frequency of assessment | Units |
| Potable water use | Annually | m3 used per tonne of product produced |
| Energy usage | Annually | KWh used per tonne of product produced |
| Waste disposed | Annually | Tonnes used per tonne of product produced |

| Table S4.4 Reporting forms |  |  |
| --- | --- | --- |
| Media/parameter | Reporting format | Date of form |
| Air | Form Air1 | 13/12/2006 |
| Water (excluding sewer) | Form Water1 | 13/12/2006 |
| Energy | Form Energy1 | 24/03/2005 |
| Waste | Form R1 | 24/03/2005 |
| Water usage | Form WaterUsage1 | 24/03/2005 |
| Mass release to water | Form MassRelease1 | 24/03/2005 |
| Performance indicators | Form Performance1 | 24/03/2005 |

# Schedule 5 - Notification

These pages outline the information that the operator must provide.

Units of measurement used in information supplied under Part A and B requirements shall be appropriate to the circumstances of the emission. Where appropriate, a comparison should be made of actual emissions and authorised emission limits.

#### If any information is considered commercially confidential, it should be separated from non-confidential information, supplied on a separate sheet and accompanied by an application for commercial confidentiality under the provisions of the EP Regulations.

Part A

| Permit Number |  |
| --- | --- |
| Name of operator |  |
| Location of Facility |  |
| Time and date of the detection |  |

| (a) Notification requirements for any activity that gives rise to an incident or accident which significantly affects or may significantly affect the environment |  |
| --- | --- |
| To be notified within 24 hours |  |
| Date and time of the event |  |
| Reference or description of the location of the event |  |
| Description of where any release into the environment took place |  |
| Substances(s) potentially released |  |
| Best estimate of the quantity or rate of release of substances |  |
| Measures taken, or intended to be taken, to stop any emission |  |
| Description of the failure or accident. |  |

| (b) Notification requirements for the breach of a permit condition |  |
| --- | --- |
| To be notified within 24 hours |  |
| Emission point reference/ source |  |
| Parameter(s) |  |
| Limit |  |
| Measured value and uncertainty |  |
| Date and time of monitoring |  |
| Measures taken, or intended to be taken, to stop the emission |  |

| Time periods for notification following detection of a breach of a limit |  |
| --- | --- |
| Parameter | Notification period |

| (c) In the event of a breach of permit condition which poses an immediate danger to human health or threatens to cause an immediate significant adverse effect on the environment: |  |
| --- | --- |
| To be notified within 24 hours |  |
| Description of where the effect on the environment was detected |  |
| Substances(s) detected |  |
| Concentrations of substances detected |  |
| Date of monitoring/sampling |  |

Part B - to be submitted as soon as practicable

| Any more accurate information on the matters for notification under Part A. |  |
| --- | --- |
| Measures taken, or intended to be taken, to prevent a recurrence of the incident |  |
| Measures taken, or intended to be taken, to rectify, limit or prevent any pollution of the environment which has been or may be caused by the emission |  |
| The dates of any unauthorised emissions from the facility in the preceding 24 months. |  |

| Name* |  |
| --- | --- |
| Post |  |
| Signature |  |
| Date |  |

* authorised to sign on behalf of the operator

Schedule 6 - Interpretation

“annually” means once every year

“accident” means an accident that may result in pollution.

“application” means the application for this permit, together with any additional information supplied by the operator as part of the application and any response to a notice served under Schedule 5 to the EP Regulations.

“authorised officer” means any person authorised by Natural Resources Wales under section 108(1) of The Environment Act 1995 to exercise, in accordance with the terms of any such authorisation, any power specified in section 108(4) of that Act.

“Compliance based on Mass Balance Calculation” means that for the purposes of demonstrating compliance or non-compliance with a specified limit the release shall be calculated. Annual mass releases for mercury and cadmium shall be calculated from the maximum potential concentration of the metal present as contamination multiplied by the volume of the chemicals used on site during the Year. An allowance may be deducted for any proportion of the chemicals used that can be demonstrated not to have reached the emission point. The concentration of mercury and cadmium shall be calculated from the annual mass release and the volume of effluent discharged during the Year.

“emissions of substances not controlled by emission limits” means emissions of substances to air, water or land from the activities, either from the emission points specified in schedule 3 or from other localised or diffuse sources, which are not controlled by an emission limit..

“EP Regulations” means The Environmental Permitting (England and Wales) Regulations SI 2016 No.675 and words and expressions used in this permit which are also used in the Regulations have the same meanings as in those Regulations.

“groundwater” means all water, which is below the surface of the ground in the saturation zone and in direct contact with the ground or subsoil.

“Industrial Emissions Directive” means DIRECTIVE 2010/75/EU OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL of 24 November 2010 on industrial emissions

“MCERTS” means the Environment Agency’s Monitoring Certification Scheme.

“quarter” means a calendar year quarter commencing on 1 January, 1 April, 1 July or 1 October.

“year” means calendar year ending 31 December.

Where a minimum limit is set for any emission parameter, for example pH, reference to exceeding the limit shall mean that the parameter shall not be less than that limit.

Unless otherwise stated, any references in this permit to concentrations of substances in emissions into air means:

- in relation to emissions from combustion processes, the concentration in dry air at a temperature of 273K, at a pressure of 101.3 kPa and with an oxygen content of 3% dry for liquid and gaseous fuels, 6% dry for solid fuels; and/or

- in relation to emissions from non-combustion sources, the concentration at a temperature of 273K and at a pressure of 101.3 kPa, with no correction for water vapour content.

Where a minimum limit is set for any emission parameter, for example pH, reference to exceeding the limit shall mean that the parameter shall not be less than that limit.

Unless otherwise stated, any references in this permit to concentrations of substances in emissions into air means:

in relation to emissions from combustion processes, the concentration in dry air at a temperature of 273K, at a pressure of 101.3 kPa and with an oxygen content of 3% dry for liquid and gaseous fuels, 6% dry for solid fuels; and/or

in relation to emissions from non-combustion sources, the concentration at a temperature of 273K and at a pressure of 101.3 kPa, with no correction for water vapour content

Schedule 7 - Site plan

© Crown Copyright and database right [2017]. Ordnance Survey licence number 100019741.

END OF PERMIT