	[image: image1.png]Powys




	Environmental management system
Waste Storage
	EMS.S4.05

	
	
	Version: 
	1 

	
	
	Effective:      
	December 2020

	Authored by: WRAP Cymru WRAWRWWWWRAPWRAPfsdfdsfsdffsdfasdfdsfSLRSLREunomaiEunomiaEunomia
	Reviewed by: DYP
	Approved by: 

	Date: December 2020 XX2019201920192019FFenssdfdfsdfsdfgdfgdfMarFebruary 20182018
	 
	 
	



1.0 PRINCIPLE

This section outlines the general procedures for the storage of waste at North Powys Bulking Facility. The purpose of this procedure is to ensure that any site operative working on site is aware of the procedures for the correct storage of waste. 

2.0 SCOPE

This procedure covers the storage of all waste and raw materials on site. 

3.0 RESPONSIBILITY

All site operatives are responsible for carrying out the procedure as detailed within this document. Any changes required are the responsibility of the Site Manager or other designated person to update and re-issue the amended procedure. 

4.0 PROCEDURE

4.1 General Storage Requirements 

As set out in Schedule 3 of the Environmental Permit (Number: XXX , issued XXXX, the total maximum storage capacity is 425 tonnes at any one time. The following requirements on site are for the storage of all wastes:
· All areas of storage will be clearly marked and signed; 

· Storage areas will be located in appropriate areas to minimise or eliminate the double handling of wastes at the site; 

· Waste will be stored in containers/bays either externally or within the confines of the Waste Operations Building (shed) as per the sites permitting constrictions;
· Food waste will be stored in dedicated bay and sealed skips within a building, the skips will have the lids shut when loading is not taking place; 
· Tanks and containment of diesel or oils will be constructed to make sure that any leaks/spillages will be contained. Pipework will be routed within bunded areas with no penetration of contained surface. Tanker connection points will be within the bund; 

· There will be constant access available to all of the storage areas to ensure regular inspections can take place.

Storage areas will be regularly inspected and maintained, including, where appropriate, drums, vessels, surfacing / pavements and bunds. Inspections will pay particular attention to any signs of damage deterioration and leakage. If any defects are found the following procedures will be followed:
· the containment bund capacity will be checked and the integrity of the bund will be inspected to ensure no further leakage occurs;

· any storage containers that are damaged will be removed to the quarantine area to be re packaged into more suitable storage containers; 

· records of actions to rectify any damages will be recorded in the site diary;

· faults will be repaired as soon as is practicable.

5.0 Health and safety

All site operatives will work with due regard to all relevant Health and Safety Regulations currently in existence relevant to operations on site. All site operatives have a duty of care to report anyone who does not comply with the relevant Health and Safety Regulations to the Site Manager.

END OF DOCUMENT


1                                                                                      



[image: image1.png]