Permit Reference Number: BL1096IB/V012                							Operator: Castle Cement Ltd

Installation: Padeswood Works                                          							Form Number: S4 / A1

Six-monthly Reporting of Emissions to Air from Emission Point A8 for the period 1st July 2015 to 31st December 2015

| Substance / Parameter | Emission Limit Level / Unit | Result (1) |  | Test Method2 | Sample Date & Times(3) | Accreditation/ Certification (4) | Uncertainty (5) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | First 6 Months | Second 6 Months |  |  |  |  |
| Hydrogen Fluoride | 1mg/m3 over minimum 1 hour period / mg/Nm3 | 0.027 | <0.020 | ISO 15713 | 16-Nov-15 13:20 - 15:20 | MCERTS | 37 |
| Cadmium & thallium and their compounds (total) | 0.05mg/m3 over minimum 30 min, max 8 hr period / mg/Nm3 | 0.0012 | 0.00080 | BS EN 14385 | 11-Nov-15 15:15 - 19:19 | MCERTS | 16 |
| Mercury and its compounds | 0.05mg/m3 over minimum 30 min, max 8 hr period / mg/Nm3 | 0.011 | 0.011 | BS EN 13211 | 11-Nov-15 15:15 - 19:19 | MCERTS | 14 |
| Sb, As, Pb, Cr, Co, Cu, Mn, Ni, and V and their compounds (total) | 0.5mg/m3 over minimum 30 min, max 8 hr period / mg/Nm3 | 0.0051 | 0.0067 | BS EN 14385 | 15:15 - 19:19 11-Nov-15 | MCERTS | 13 |
| Dioxins / Furans (I-TEQ)6 | 0.1ng/m3 over minimum 6hr, max 8 hr period / ng/Nm3 | 0.011/0.011 | 0.022/0.022 | BS EN 1948 | 17-Nov-15 10:10 - 16:14 | MCERTS | 13 |
| Dioxins / Furans (WHO – TEQ Humans/ Mammals)6 | No Limit / ng/Nm3 | 0.0096/0.0098 | 0.020/0.020 | BS EN 1948 | 17-Nov-15 10:10 - 16:14 | MCERTS | 13 |
| Dioxins / Furans (WHO – TEQ Fish)6 | No Limit / ng/Nm3 | 0.011/0.011 | 0.023/0.023 | BS EN 1948 | 17-Nov-15 10:10 - 16:14 | MCERTS | 13 |
| Dioxins / Furans (WHO – TEQ Birds)6 | No Limit / ng/Nm3 | 0.027/0.027 | 0.057/0.057 | BS EN 1948 | 17-Nov-15 10:10 - 16:14 | MCERTS | 13 |
| Dioxin – like PCBs (WHO – TEQ Humans/ Mammals)6 | No Limit / ng/Nm3 | 0.011/0.011 | 0.0015/0.0015 | BS EN 1948 | 17-Nov-15 10:10 - 16:14 | MCERTS | 18 |
| Dioxin – like PCBs (WHO – TEQ Fish)6 | No Limit / ng/Nm3 | 0.000061 / 0.000061 | 0.000091 / 0.000091 | BS EN 1948 | 17-Nov-15 10:10 - 16:14 | MCERTS | 18 |
| Dioxin – like PCBs (WHO – TEQ Birds)6 | No Limit / ng/Nm3 | 0.0038/0.0038 | 0.0076 / 0.0076 | BS EN 1948 | 17-Nov-15 10:10 - 16:14 | MCERTS | 18 |
| Poly-cyclic aromatic hydrocarbons Total | No Limit / mg/Nm3 | 0.043 | 0.084 | BS 1SO 11338-1 | 17-Nov-15 10:10 - 16:14 | MCERTS | 49 |
| Anthanthrene | No Limit / mg/Nm3 | <0.000064 | <0.000072 | BS 1SO 11338-1 | 17-Nov-15 10:10 - 16:14 | MCERTS | 49 |
| Benzo(a)anthracene | No Limit / mg/Nm3 | <0.000064 | <0.000072 | BS 1SO 11338-1 | 17-Nov-15 10:10 - 16:14 | MCERTS | 49 |
| Benzo(b)fluoranthene | No Limit / mg/Nm3 | <0.000064 | <0.000072 | BS 1SO 11338-1 | 17-Nov-15 10:10 - 16:14 | MCERTS | 49 |
| Benzo(k)fluoranthene | No Limit / mg/Nm3 | <0.000064 | <0.000072 | BS 1SO 11338-1 | 17-Nov-15 10:10 - 16:14 | MCERTS | 49 |
| Benzo(b)napth(2,1-d)thiophene | No Limit / mg/Nm3 | <0.000064 | <0.000072 | BS 1SO 11338-1 | 17-Nov-15 10:10 - 16:14 | MCERTS | 49 |
| Benzo(c)phenathrene | No Limit / mg/Nm3 | <0.000064 | <0.000072 | BS 1SO 11338-1 | 17-Nov-15 10:10 - 16:14 | MCERTS | 49 |
| Benzo(g,h,i)perylene | No Limit / mg/Nm3 | <0.000064 | <0.000072 | BS 1SO 11338-1 | 17-Nov-15 10:10 - 16:14 | MCERTS | 49 |
| Benzo(a)pyrene | No Limit / mg/Nm3 | <0.000064 | <0.000072 | BS 1SO 11338-1 | 17-Nov-15 10:10 - 16:14 | MCERTS | 49 |
| Cholanthrene | No Limit / mg/Nm3 | <0.000064 | <0.000072 | BS 1SO 11338-1 | 17-Nov-15 10:10 - 16:14 | MCERTS | 49 |
| Chrysene | No Limit / mg/Nm3 | <0.000064 | <0.000072 | BS 1SO 11338-1 | 17-Nov-15 10:10 - 16:14 | MCERTS | 49 |
| Cyclopenta(c,d)pyrene | No Limit / mg/Nm3 | <0.000064 | <0.000072 | BS 1SO 11338-1 | 17-Nov-15 10:10 - 16:14 | MCERTS | 49 |
| Dibenzo(a,h)anthracene | No Limit / mg/Nm3 | <0.000064 | <0.000072 | BS 1SO 11338-1 | 17-Nov-15 10:10 - 16:14 | MCERTS | 49 |
| Dibenzo(a,i)pyrene | No Limit / mg/Nm3 | <0.000064 | <0.000072 | BS 1SO 11338-1 | 17-Nov-15 10:10 - 16:14 | MCERTS | 49 |
| Fluoranthene | No Limit / mg/Nm3 | <0.000064 | <0.000072 | BS 1SO 11338-1 | 17-Nov-15 10:10 - 16:14 | MCERTS | 49 |
| Indo(1,2,3-cd)pyrene | No Limit / mg/Nm3 | <0.000064 | <0.000072 | BS 1SO 11338-1 | 17-Nov-15 10:10 - 16:14 | MCERTS | 49 |
| Naphthalene | No Limit / mg/Nm3 | 0.042 | 0.083 | BS 1SO 11338-1 | 17-Nov-15 10:10 - 16:14 | MCERTS | 49 |
| Zinc and its compounds | No Limit / mg/Nm3 | 0.0031 | 0.0028 | BS EN 14385 | 11-Nov-15 15:15 - 19:19 | MCERTS | 12 |
| Benzene | No Limit / mg/Nm3 | 1.8/2.3 | 1.0 / 1.8 | BS EN 13649 | 12-Nov-15 14:35 - 14:45 / 15:05 - 15:15 | MCERTS | 19 |
| 1,3-butadiene | No Limit / mg/Nm3 | 0.50 | <0.19 / 0.39 | BS EN 13649 | 12-Nov-15 15:20 - 15:30 / 15:35 - 15:45 | MCERTS | 16 |

- The result given is the maximum value (or the minimum value in the case of a limit that is expressed as a minimum) obtained during the reported period, expressed in the same terms as the emission limit value. Where the emission limit is expressed as a range, the result given as the minimum – maximum’ measured values.

- Where an internationally recognised standard test method is used the reference number is given. Where another method that has been formally agreed with the agency is used then the appropriate identifier is given. In other cases the principal technique is stated e.g. gas chromatography

- For non-continuous measurements the date and time of the sample that produced the result is given. For continuous measurements the percentage of the process operating time covered by the result is given.

- The accreditation status of the equipment and/or the monitoring organisation, as appropriate, for the methods used for both sampling and analysis.

- The uncertainty associated with the quoted result at the 95% confidence interval, unless otherwise stated.

- The result to be reported as a range based on: All congeners less than the detection limit assumed to be zero as a minimum, and all congeners less than the detection limit assumed to be at the detection limit as a maximum.

- Due to a contaminated batch of filter papers the Zinc result is un-accredited and the result reported is a worse case result.

Signed 	   									Date   07/01/15

Victoria Smith, Quality and Environment Manager

(Authorised to sign as representative of Castle Cement Ltd)