# Schedule 5 - Notification

These pages outline the information that the operator must provide.

Units of measurement used in information supplied under Part A and B requirements shall be appropriate to the circumstances of the emission. Where appropriate, a comparison should be made of actual emissions and authorised emission limits.

#### If any information is considered commercially confidential, it should be separated from non-confidential information, supplied on a separate sheet and accompanied by an application for commercial confidentiality under the provisions of the PPC Regulations.

Part A

| Permit Number | EPR/BL1096IB/V012 |
| --- | --- |
| Name of operator | Hanson Cement Limited |
| Location of Installation | Padeswood Works, Padeswood, Mold. |
| Time and date of the detection | 12:30 07/12/15 |

| (a) Notification requirements for any malfunction, breakdown or failure of equipment or techniques, accident, or fugitive emission which has caused, is causing or may cause significant pollution |  |
| --- | --- |
| To be notified within 24 hours of detection |  |
| Date and time of the event | 12:38 07/12/15 |
| Reference or description of the location of the event | Pre Heater Tower, Level 6 |
| Description of where any release into the environment took place | As above, through the cladding, west side of tower, blowing north towards Buckley. |
| Substances(s) potentially released | Fumes and fine particles of Kiln feed. |
| Best estimate of the quantity or rate of release of substances | Release occurred over 10 minutes but was mostly visible due to fumes. Dust release estimated 50kg |
| Measures taken, or intended to be taken, to stop any emission | Increased suction on the tower until the emission subsided. |
| Description of the failure or accident. | Worn refractory on inlet to cyclone 5 allowed fumes to pass when positive pressure occurred through the PHT following a planned kiln stop. This area had been inspected 24hrs previously and had not shown signs of deterioration. |

| (b) Notification requirements for the breach of a limit |  |  |
| --- | --- | --- |
| To be notified within 24 hours of detection unless otherwise specified below |  |  |
| Emission point reference/ source |  |  |
| Parameter(s) |  |  |
| Limit |  |  |
| Measured value and uncertainty |  |  |
| Date and time of monitoring |  |  |
| Measures taken, or intended to be taken, to stop the emission |  |  |
| Time periods for notification following detection of a breach of a limit |  |  |  |
| Parameter |  |  | Notification period |

| (c) Notification requirements for the detection of any significant adverse environmental effect |  |
| --- | --- |
| To be notified within 24 hours of detection |  |
| Description of where the effect on the environment was detected |  |
| Substances(s) detected |  |
| Concentrations of substances detected |  |
| Date of monitoring/sampling |  |

Part B - to be submitted as soon as practicable

| Any more accurate information on the matters for notification under Part A. | The inlet to Cyclone 5, part of the system within the preheater tower, on level 6, had earlier in the year been found worn and an external plate repair was applied to this area to prevent the chance of future emissions and to control process issues. However despite checks in this area the repair failed unexpectedly creating a small void through which mostly fumes were emitted. These were very visible on the day due to cooler outside temperature |
| --- | --- |
| Measures taken, or intended to be taken, to prevent a recurrence of the incident | A review of the original repair was carried out, and the area had additional supports and welding carried out. Checks were then undertaken in this area to monitor the repair until major works in this area could be carried out. Corrective works in this area undertaken during the shutdown included removing the previous box repair, inserting new steel works and installing new refractory. |
| Measures taken, or intended to be taken, to rectify, limit or prevent any pollution of the environment which has been or may be caused by the emission | The emission was mainly fumes made more visible by the cooler atmospheric conditions. The dust portion dissipated rapidly and there were no off-site complaints linked to this incident. |
| The dates of any unauthorised emissions from the installation in the preceding 24 months. | 13/04/14 06/02/15 17/07/15 02/09/14 01/04/15 23/08/15 06/09/14 06/07/15 10/09/15 21/01/15 15/07/15 15/10/15 |

| Name* | Victoria Smith |
| --- | --- |
| Post | Quality and Environment Manager |
| Signature |  |
| Date | 14/03/16 |

* authorised to sign on behalf of Castle Cement Limited