| SITE LOCATION AND REFERENCE NUMBER: | DISCHARGE AT BLAENWAUN [NGR; SN 3670 4637] |
| --- | --- |
| RISK ASSESSMENT CARRIED OUT BY: | ROGER BENFIELD |
| DATE COMPLETED: | FEBRUARY 2017 |

Judgement							Action

| RECEPTOR | SOURCE | HARM | PATHWAY | PROBABILITY | CONSEQUENCE | MAGNITUDE | JUSTIFICATION OF MAGNITUDE | RISK MANAGEMENT | RESIDUAL RISK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSSi | NONE | NONE | NONE | NONE | NONE | NONE | NONE | NONE | NONE |
| Surface Water | Nutrient and organic matter within waste | Eutrophication or de-oxygenation of surface waters | Contamination of water | Low | Low | Low | Proximity of ditches / streams but of low ecological importance. Site not in NVZ. | Quality monitoring of the discharged water to ensure compliance | Very Low |

| Surface Water | Nutrient and organic matter within waste | Eutrophication or de-oxygenation of surface waters | Contamination of discharge water | Low | Low | Low | Proximity of ditches but of low ecological importance. Flow rate predictable and controlled | Flow and quality monitoring to ensure compliance | Low |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ground Water | Nutrients within waste | Deterioration in groundwater quality | Percolation through soil or stream-bed profile | Low | Low | Low | Sites are not located within Protection Zone 1 or 2. Site not in NVZ. Water is purified prior to discharge | Quality monitoring of the discharged water to ensure compliance | Very Low |
| Soil | Nutrients and organic matter | Excessive increases in nutrient /OM content | Discharge of excessive quantities of purified water | Very Low | Low | Low | Soil type is identified as Silty Clay Loam and unlikely to be affected by low level increases in nutrient | Quality monitoring of the discharged water to ensure compliance | Very Low |

| Crops on adjacent land | Water escaping to adjacent land | Excess flow causing flooding | Discharge of excessive quantities of purified water | Very Low | Very Low | Very Low | Agricultural land within close proximity to discharge point. | Flow restricted to a maximum of 0.6 litres/second | Low |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Humans & Livestock | Aerosols | Respiratory or digestive illness | Inhalation or ingestion | Very Low Water is at ambient and unlikely to vaporise | Very Low | Very Low | No livestock on site. Only authorised personnel to access discharge point | Flow control will eliminate excessive flow | Very Low |
| Highway | Discharge water will not be close to any highways | NA | Material escaping via pipework | Very Low | Very Low | Very Low | NA | Pipes will be regularly inspected and maintained | Very Low |

NOTES

- Discharge of purified water from the purification plant will be through a dedicated, sealed pipework system.

- The flow meter [to MCERTS standards] will be located close to the EXIT from the purification plant.

- The pipes will lead directly from the plant to the discharge point.

- A regular inspection and monitoring programme is planned.

- Flow meter and analysis records will be maintained and available for inspection on-site