Tomlinson’s Dairies Ltd


           Drainage & Bund Checks ENV 002
· A current site drainage plan will be drawn up and displayed in the dairy office for information. The standard colour coding of blue for surface water drainage that discharge to the environment and red for sealed or foul drainage systems will be employed.

· The level of oil/water/dairy product mix in the foul water interceptor and surface water drainage systems will be checked by visual inspection.

· The interceptors will be maintained and emptied when necessary to ensure that they operate effectively at all times. As a minimum, oil will be removed, the drainage system de-sludged and the structural integrity of the drainage system and chambers will be carried out annually. The daily checks, annual inspection and any maintenance and waste removal will be recorded in the site diary.

· Sealed gullies, sumps & interceptors will be inspected daily and after any periods of heavy rain by the operator. Where sludge/dairy product levels within the interceptor are high liquids will be gulped out and transferred to an authorized waste collection agency. 
· The interceptors will be emptied before they become full to maximum capacity. 
· The integrity of  impermeable surfaces, the interceptors and storage areas will be visually inspected by the operator on a monthly basis. To achieve this it will be necessary to sweep/ wash clean the impermeable surface that form the permitted  area and clear any debris that has collected in the drain gulleys around the dairy building. 
	Area
	When Checked 
	Date
	Comments

	Yard
	weekly
	20/12/16
	Clean no gulping required

	Bunds 
	weekly
	
	

	Hardstanding
	6 monthly
	
	

	Oil tanks
	weekly
	
	

	Foul water interceptor
	weekly
	
	

	Gulleys
	weekly
	
	

	Pumps
	weekly
	
	

	Milk silo’s
	monthly
	
	

	Milk Delivery yard
	Daily
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	


