Non Technical Summary 

Tomlinsons Dairy - PPN-00061
Tomlinsons Dairy plan to expand their milk processing facility at Five Crosses Industrial Estate, Coedpoeth, Minera LL11 3RD.     
By July 2017 the dairy will be processing 300,000,000 Litres of milk annually.

Raw milk is delivered to the site by road tankers (approx 540,000L/day) which approximates to around 24 tanker deliveries coming onto site daily by July 2017. Milk is delivered to silos under supervision of the driver, hoses are blow dry after the delivery and silo’s have high level alarms to prevent overfilling. Delivery Pipes are emptied using gravity.
Milk is then sent for heat treatment (pasteurization) and separation (homogenization) before chilling and bottling in plastic poly bottles. Chilled milk is taken off-site in various sized lorries (approx 38 vehicles daily) and artics to retailers and distribution centres.

There is no effluent plant on-site and most trade effluent is discharged under a Welsh Water trade effluent discharge consent to the foul sewer on-site. Surface water from car park areas containing clean run off is collected in surface water lagoons where solids are settled before discharging to a surface water system.

The site boundary encompasses an area of approximately 4 acres including the dairy, car parking, surface water lagoons and external storage areas such as oil tanks and silos.
[image: image1.emf]
The site is not bordered by SSSI’s or other protected areas. Two Sites of Special Scientific Interest (SSSIs) are within 2000m of the site. These both relate to the Ruabon/Llantysilio mountains and Minera 1225 and 1657m west of the site. There are also three Special Areas of Conservation (SACs) beyond 1300m from the site and located in the Berwyn a Mynyddoedd De Clwyd / Berwyn and

South Clwyd Mountains south west and west.
[image: image2.emf]
Tomlinsons dairy has two boilers for raising steam/heat and these are both rated at 2.5Mw which is below LA and EA Regulatory thresholds. Air emissions from this process are deemed to be negligible. 
A block diagram showing the new dairy layout is represented below.


[image: image3.emf]
