Bespoke Permit for Water Discharge

|  | Water Management Plan |
| --- | --- |
| Date | 18/10/2021 |

Revision	 B	13/12/2021

Revision C	24/01/2022

Revision D	18/02/2022

Revision E	17/05/2022

|  | Prepared |
| --- | --- |
| Author | Elliot Hutchings – Environmental Advisor |
| Signature |  |
| Checker Approver | Andrew Crossley – Environmental Advisor |
| Signature |  |

REVISION TABLE

General update – all pages have been revised.

TABLE OF CONTENTS

1	Preliminaries	5

1.1	Purpose	5

1.2	Scope	5

1.3	Definitions & abbreviations	5

1.4	Responsibilities	6

2	Compliance obligations	7

2.1	Environmental Permitting Regulations 2016	7

3	background and controls	7

3.1	Background	7

3.2	Procedural Control Measures (Surface water runoff)	7

4	Pollution mitigation	10

4.1	oil use, storage and Refuelling	10

4.2	Chemical use / Storage (CoSHH)	10

4.3	Permit to Pump	11

5	Monitoring	11

5.1	Siltbuster Check sheet	12

5.2	Soneco Check Sheet	12

6	Siltbuster HB Chemical Treatment System	12

6.1	overview Siltbuster	12

6.2	Siltbuster 2-Stage Chemical Pre-Treatment System	12

7	Soneco 3 stage Treatment system	14

7.1	Water Quality for Discharge	15

8	Passive Dewatering	16

9	Discharge Overviews	17

9.1	Siltbuster	17

9.2	Soneco	17

9.3	Passive Dewatering	18

10	Water Catchment on Average Rainfall	20

11	operational Arrangements	21

12	appendix 1	22

12.1	Aquatreat	22

12.2	Brenntag PAC	24

12.3	Caustic Soda Liquor	31

13	Appendix 2 – Siltbuster check sheet	39

14	Appendix 3 Soneco Check Sheet	40

15	Appendix 4	41

15.1	Rutpen A9	41

15.2	Water Lynx 360	43

15.3	Water Lynx 395	45

16	Appendix 5 – Floc mat Release Rate	48

# Preliminaries

## Purpose

The purpose of this Water Management Plan (WMP) is to identify the FVC / FCC works that may require a water discharge activity permit and control measures required to prevent such contamination of water courses by, interaction with water infrastructure or generated water effluent (Trade Effluent), and detail the management and monitoring in compliance with regulatory guidelines and advice.

- Dewatering of Excavation(s)

- Surface Water Runoff

## Scope

FVC / FCC are constructing a 17.7km / 11 mile road known as the A465 Heads of the Valleys section 5 and 6. The project will extend between Hirwaun and Dowlais Top. It will leave the current A465 between the Hirwaun and Trewaun junctions for about a mile (1.4km) and between the Taf Fechan Viaduct and the Gurnos Farm underpass for about half a mile (0.8km).

## Definitions & abbreviations

| FVC | Future Valleys Construction |
| --- | --- |
| FCC | FCC Construction |
| NRW | Natural Resources Wales |
| TSS | Total Suspended Solids |
| AGC | Alun Griffiths Contractors |
| RB | Road Bridge |
| RPS | Regulatory Position Statement |
| NTU | Nephelometric Turbidity Units |
| TV | Thames Valley |

## Responsibilities

Responsibilities to prepare and maintain this document:

### FVC / FCC

FVC / FCC as the principle contractor are responsible for driving Good Industry Practice to minimise the impacts of works on the water environment, by ensuring contractors undertake all measures to protect identified receptors of water courses by pathway means.

- Ensuring resources are available to contractors for mitigation measures;

- Updating and approving amendments to all relevant contractor documentation;

- Protecting water management infrastructure in their areas of work scope;

- Undertaking inspections, assessments and audits;

- Evaluating compliance on an ongoing basis, ensuring contractors are compliant;

- Monitoring;

- Supporting incident investigations in a timely manner;

### FVC / FCC Environmental Manager/Advisor

They will be responsible for updating this management plan and carrying out duties such as monitoring to ensure compliance and control measures are in place

All the above 1.4.1 shall be within this scope of works

### Construction Managers and Supervisors

They will be responsible for ensuring a request has been submitted for a permit to pump before entering a new area and that adequate water management arrangements have been made.

They will be responsible for ensuring any accidents that may have an impact on water quality have been reported to the FVC / FCC Environmental Team immediately to prevent further pollution from occurring.

They (or a nominated deputy) will be responsible for the implementation of the control measures as detailed in this Water Management Plan.  They will be responsible for the day to day checking and operation of equipment associated with water management.

# Compliance obligations

## Environmental Permitting Regulations 2016

These Regulations set out an environmental permitting and compliance regime that applies to various activities and industries. Permits set controls and emission standards to minimise pollution and allow operators of certain installations to operate.  Any operational ‘regulated facility’ requires an environmental permit; a regulated facility means a water discharge, groundwater and radioactive substances activities.

# background and controls

## Background

There are a total of 58 water features: three main rivers, five significant tributaries and a further 50 channels that are crossed or which lie within a 500 m buffer zone around the extent of the scheme, the majority of which are already crossed by the existing road. These include two large tributaries of the Afon Taf (the Cwm Taf Fechan and Afon Taf Fawr) and the Afon Cynon.

## Procedural Control Measures (Surface water runoff)

There shall be a Tier approach system to control and minimise surface water runoff. It is key to ensure that surface water is kept clear and away from excavations and earth works as best as reasonably practicable. Each Tier system shall be deployed to ensure the natural flow of water is not affect by works and diverted away, failure to carry out these requirements each tier approach shall be upped in measure. Where all systems are overwhelmed Tier 4 shall be implemented for active treatment and recognition with requirements of NRW / Local Authorities.

### Tier 1

Mechanisms for reducing the volume of water and silt available for transfer to surface waters. Primary mechanism of control will be segregation. Surface water/runoff will be segregated and/or minimised from contact with the surface to reduce, as far as is practical, the volume of sediment contaminated water (Silt).

Excavated drainage channels or stoned and lined V-drains (French Drainage).

Weathering off of exposed areas, tracked in, sealed or covered

Silt fencing, fibre rolls, geo-textiles and can be used to manage areas to prevent that of surface water runoff (Fig Below jute matting)

### Tier 2

Mechanisms for slowing the flow of sediment rich water to help sediment dropout and treatment. This primarily involves increasing the retention of water as it passes through the Schemes drainage channels, allowing sediment to settle out naturally. Examples of potential mitigation methods include:

Discharge to Vegetation

Straw Bales

Weirs

Pea Shingle Beds

Filter Fabrics

Erosion Berms (Fig below)

### Tier 3

Mechanisms offer solutions for retaining sediment rich water, to allow further settlement prior to discharge or further treatment. This requires the inclusion of systems that allow sediment to settle out naturally, examples of potential mitigation methods include settlement lagoons or settlement systems.

Attenuation / Settlement Lagoons – Sizing of such shall be determined on a site-by-site basis due to the  site constraints, dynamics of excavation, sites shall vary in size across the scheme.

### Tier 4

Active settlement and polishing of waters from excavations with use of flocculant-based materials. Where settlement can be achieved within lagoons added measure shall be used to help with NTU / TSS to reach a more acceptable discharge. Products shall be used at inlets of ponds and outlet drainage channels to ensure cleanliness remains to outfall point. See appendix for safety data sheets

#### Floc Mat

The floc mat is designed to treat and capture fine silts and suspended particles. Its main function is to flocculate fine sediments enabling settling and separation from water. Sediment is trapped within the matrix of the matting; a single matting can hold up to 50kg of sediment.

Material – Coir (fibres, netting, rope),

Dimensions – 2x1 x 0.1m

WaterLynx Gel Flocculant 360

WaterLynx Gel Flocculant 395

- See appendix 14 for release rate based on 5, 10, 15 and 20 mat deployment

#### Rutpen Flocculant Block A9

The A9 shall be used in aid with settlement lagoons should the settlement not be sufficient. Visual checks will be carried out to confirm clarity of water to confirm requirement for use of A9. A9 block shall be suspended at inlet of ponds to allow water to flow over block which will actively promote settlement. The product is a synthetic anionic flocculant in the form of a solid block, designed to slowly dissolve and flocculate suspended solids. The A9 block comes in 3kgs. It is estimated on average 1 A9 block of 3kg shall last approximately 3 months.

| - Representative Example 1 Matt |  |
| --- | --- |
| - Flow rate at inlet | - 40m3 (Pump) |
| - Dosage | - 1 x A9 flocculant 3kg |
| - Rinse of Polish | Pump 40m3/h – 36,000m6/90days (3 months) A9 Block 3kg – 3,000,000mg 3,000,000 / 3,600,000ltrs A9 – 0.8mg/ltrs over 90 days if run at full ratio |

Note – where active siltbuster treatment is ongoing, the above shall not be used in conjunction

### Tier 5

Active Treatment Subject to site constraints, flow rates, elevated levels of sediment load, effectiveness of passive measures, etc. it may be necessary to implement active treatment techniques to mitigate sediment release.

Treatment options can include:

Conditioning through use of coagulants i.e. ‘floc-blocks’

Lamella clarifiers

Hydro-cyclones

Silt-buster settlement tanks, etc.

# Pollution mitigation

All temporary and permanent site compounds shall be at least 15 m away from the top of the bank of main rivers or 8 m away from ordinary watercourses. No single skin storage containers containing hydrocarbons, only double skin mobile bowsers will be allowed. All storage of any hazardous substance will be within impervious or internal, bunded, secure areas.

## oil use, storage and Refuelling

All oil storage above ground containing more than 200ltrs shall follow the guidelines of the Oil Storage Regulations (OSR) Wales 2016 and GPP2.

Tanks shall be of sufficient strength and structural integrity to ensure that it won’t burst or leak in ordinary use

There shall be no single skinned tanks present on site, where there are these shall be placed into a secondary containment system

Double skinned tanks must be of the design that allows for 110% capacity of the volume of the tank

All secondary containment must hold 110% of the volume

All singular items of oils shall be store on secondary containment either of 110% capacity or 25% of the total capacity whichever is greater

Secondary containment shall be covered from excess dependent rainfall

Mobile bowsers with pipe shall be locked by padlock when not in use, parked away from any watercourse on impermeable grounds

All refuelling of plant on site shall be carried out at platform designated refuelling point. Plant working near watercourse shall track back to point for refuelling.

Regular daily maintenance of containers / tanks / delivery hoses shall be carried out to ensure integrity

## Chemical use / Storage (CoSHH)

All secondary containment must hold 110% of the volume

All singular items of oils shall be store on secondary containment either of 110% capacity or 25% of the total capacity whichever is greater

All chemicals must be accompanied with a valid Safety Data Sheet, and reference to section 6

All chemicals shall be store in a designated area, away from access / egress ports utilised by plant and vehicles

Only chemicals required for days work shall be out for use on platforms, all other chemicals shall be placed in storage as above

All products brought onto site shall be of water based quality / WRAS certification

Storage of chemical within containment tor secondary bunding shall be placed on impermeable ground or compacted hard stand ground at site

## Permit to Pump

Before any works for dewatering commences all Contractors must submit a ‘Permit to Pump’ or similar works authorisation will be sought from the Environmental Team.

Therefore, AGC / RB / TV shall:

Submit a Permit to Pump request to the FVC / FCC Environmental Team.

- The following details will be included as a minimum:

The location that the permit to pump will cover;

The purpose of the permit to pump (i.e. draining of surface water, dewatering of excavations);

A sketch plan showing the planned area for the permit to pump to cover including drainage arrangements.

Proposal on mitigation measures in place to prevent pollution.

All permit requests will be completed by those preparing for the work e.g. section engineers or project managers, with the support of the FVC / FCC Environment Team.

# Monitoring

Daily / Weekly physical checks will be completed in line with the requirements of each permit.  For monitoring purposes and consistency, these checks will be completed at the location of the upstream (Lagoon[s]) and downstream (outlet[s]) as identified in the plans submitted. The checks will include the following:

pH test

TSS / NTU

Turbidity

Dissolved Oxygen

Check for visible oil or grease (i.e. ‘rainbow’ sheen);

Check that the pumping set up appears to be functioning as designed.

Check for the build-up of solids; and

Check the pollution prevention controls are in place.

Monitoring shall be carried out with the use of a calibrated Aquaread AP 800.

pH4.01 Buffer

ORP Cal Solution

Zero Oxygen Solution

1000NTU Calibration Solution

The Monitoring Locations are listed below. Please note the NGRs provided in this list are approximate, as the exact location will vary according to waterway conditions; Additionally in some locations, access is restricted.

| Discharge location | Monitoring location Upstream | Monitoring Location Downstream |
| --- | --- | --- |
| Outlet 3: Taf Fawr East Discharge | SO 02761 07963 | SO 02822 07900 |
| Outlet 2: Dowlais West Discharge | Excavation monitoring | SO 07637 08202 |
| Outlet 1: Dowlais East Discharge | ST 91000 68000 | SO 08531 08635 |
| Outlet 13: Section 2 (Nant Melyn) | SN 97898 06245 | SN 97927 06214 |
| Outlet 10: Section 2 (Nant Hir Bridge) | Excavation monitoring | SN 98955 06987 |
| Outlet 15: Afon Cynon Discharge | SN 96771 05321 | SN 96830 05320 |
| Outlet 14: Section 2 (Croesbychan) | SN 97571 05692 | SN 97766 05570 |
| Outlet 12: Section 2 (Nant Melyn North) | SN 97898 06245 | SN 97927 06214 |
| Outlet 11: Section 2 (Culvert 5969) | NA permanent drainage design | SN 98450 06607 |
| Outlet 9: Section 2 (Nant Hir East) | Excavation monitoring | SN 98955 06987 |
| Outlet 8: Section 2 (Baverstock South) | SN 99814 07116 | SN 99764 06993 |
| Outlet 7: Section 2 (Swansea Road) | SO 00825 07356 | SO 01454 07311 |
| Outlet 4: Taf Fawr West | SO 02761 07963 | SO 02822 07900 |

## Siltbuster Check sheet

Preoperational check sheets shall be used to ensure the unit(s) full system is operational before any water discharge activities begin. Such checks shall cover the following processes…

General (Unusual sounds, pipework and connections, alarms and fault errors, cables)

Mechanical (Motors running correctly, Vents, flowmeters, dosing pumps, sludge pumps)

Consumables (Level of chemicals, damage occurred, storage containment of chemicals)

Clarifier (Visual checks clear water outlet, solid build up, hoppers i.e. de-sludge)

Reaction Tank (pH level of water 6-9, lines for blockages, visual check solids aggregating, cleaning of pH prob)

- Note – see appendix for template

## Soneco Check Sheet

Preoperational check sheets shall be used to ensure the unit(s) full system is operational before any water discharge activities begin. Such checks shall cover the following processes…

Aluminium Dose mg/l

Plate Life (Lamella)

Mix tank 1 and 2 (Slurry build up)

Water conductivity

Treated Water TSS gage / set up

# Siltbuster HB Chemical Treatment System

## overview Siltbuster

The project has adopted the 2-stage chemical pre-treatment system (as and when required pH correct with Sodium Hydroxide).

## Siltbuster 2-Stage Chemical Pre-Treatment System

Many of the Siltbusters are being used to treat surface run off following rain dependent fall and as such with regularly dry up upon cease of downfall. The 2-stage chemical pre-treatment system will comprise of the Silt-buster HB50. This unit is subject to treatment quantities of 5-60m3/h – slow settling solids. Set up system shall include the following process…

Inlet electromagnetic flow meter to record the volume of water treated and allow flow proportional dosing of coagulant and polymer.

Mix Tank to allow the controlled mixing of the treatment chemicals ensuring flocculation of the solids (size dependant on flow).

Flow proportional control system for coagulant and flocculant polymer dosing. The use of flow proportional dosing system minimises the risk associated with the overdosing of the treatment chemicals.

Coagulant dosing pump and associated pipework to allow the automatic flow proportional addition of the coagulant.

Flocculant make-up system complete with 1000 litre make up tank, mixer bridge 110 V (32 Amp) mixer complete with paddle (potable water will be required on site for polymer dilution) and flow proportional dosing pump (peristaltic or similar).

1 No IBC spill stand/containment bunds for the temporary storage of chemicals.

Siltbuster HB50 Gravity Operated Settlement Units (Recovery of Suspended Solids)

2 No. Siltbuster HB50 Lamella Clarifiers (dependent on flow rate) to separate the suspended solids from the treated water.

| - Site Setup as provided by Silt-buster |  |
| --- | --- |

### Treatment and dosing

Improved settling characteristics shall be achieved using two stage chemical pre-treatment in which…

A coagulant (PAC) is added to the water to cause the fine particulates to aggregate together into larger particles.

A flocculant (anionic polymer) is added to bind the particles together and increase the particle size, enhancing the settlement rate. The addition of the polymer shall enable near complete removal of the suspended solids.

### Estimated Coagulant Consumption

At a dose rate of 12.5mg/l of PAC at a flow rate of 60m3/hr, it is estimated that 10.25 litres of coagulant will be consumed per hour. If consistently operated over a 24-hour period, we would expect 246.1 litres of coagulant be consumed per day.

### Estimated Flocculant Consumption

At a dose rate of 2mg/l of EM533 at a flow rate of 60m3/hr, at a working strength of 0.2% it is estimated that 60 litres of flocculant as a working strength solution (water + flocculant) will be consumed per hour. This would equate to 59.6 litres of water only per hour, and 0.4 litres of flocculant only per hour. If consistently operated over a 24-hour period, expect 4.8 litres of neat flocculant to be consumed per day, and 1430.4 litres of water only to be consumed per day.

### Sodium Hydroxide Consumption

Due to the nature of the predominantly upland landscape dominated by acid grassland and peatland habitats sections of the works generate acidic water. Where such waters are treated via siltbuster, the pH will be adjusted accordingly i.e. either to match the upstream pH or adjusted if it was to be discharged to existing vegetation, where that vegetation may be adversely affected by differences in pH.

NOTE – the above values are in place to give a direct concentration should operation be required around clock. It is not the intention to run such treatment for prolonged period of times.

### Water Quality for Discharge

Direct water quality from the treated facility shall be free from solids as best a practicably possible due to Siltbuster operational usage. The aim is to remove solids (TSS) from the water and discharge water back into the receiving environment to which it has come from. Water shall be discharge direct back to surrounding waters as per upstream values indication. An example can be seen below for achievable measures.

| Lab Result | Feed Water (From Lagoon) | Treated Outlet from Silt-buster |
| --- | --- | --- |
| pH | 7.82 | 7.05 |
| Total Suspended Solids (TTS) | 4294mg/l | 11mg/l |
| Evidence |  |  |

# Soneco 3 stage Treatment system

The Soneco electro-generates Aluminium from a solid metal anode to condition water that has become contaminated during construction activities, (rainwater run-off). Conditioning the water allows the solids to be consolidated and removed in the lamella settling tank, and the treated water to be discharged as appropriate.

Three-stage treatment system

1. Soneco Process with Aluminium Electrodes

2. 2 Stage Flocculation Tank (no additional chemicals, this is a mix tank for settlement)

3. Lamella Settlement Unit for Solids Liquid Separation

Collected waters are pumped to the treatment system via the feed pump. The Soneco reactor electro-generates Aluminium which encourages the solids in the water to coagulate and then flocculate in the 2-stage flocculation tank. The bound particles are then settled in a lamella tank, which allows the clean water to overflow from the top of the tank, and the accumulated solids being pumped from the bottom the tank for disposal.

A flowmeter is used on the inlet to determine flow rate through the system and to proportionally add aluminium whilst an online turbidity monitor is used to monitor discharge water quality.  Ultrasound is applied to manage passivation of the anodes.

## Water Quality for Discharge

| Sample of site waters pre testing |  |  |
| --- | --- | --- |
| Post Testing of sampled water with active treatment |  |  |
| Dose rate of 2.3mg/l Al | Dose Rate of 5mg/l Al | Dose rate 8.3mg/l Al |

# Passive Dewatering

Passive dewatering shall be carried out across site as and where flow rates can be controlled by the Tier approach system to a successful attenuation design. Where waters shall enter designed ponds on a Tier 3 basis, there shall be added Tier 4 approach to allow for greater settlement of TSS within water contained. Waters shall not be pumped from such containment but shall be allowed to fill and passively discharge. Within the passive approach there shall be use of the A9 Flocculant block at inlet of attenuation ponds, at the outlet, only if required there shall be added measures with flocculant matting for extra polish and removal of TSS to ensure NTU is of an acceptable clarity upon onsite monitoring.

See section 3.2.4.2 and appendix for calculations of usage of flocculant material

# Discharge Overviews

## Siltbuster

| Project | A465 Section 5 and 6 Hirwaun to Dowlais Top |
| --- | --- |
| Site Office | Baverstock Compound, Dynevor Arms, Llwydcoed, Aberdare CF44 0ND |
| Discharge Location | - Taf Fawr East - SO 02853 07963 - Dowlais West Discharge – SO 07668 08201 - Dowlais East Discharge – SO 08537 08631 - Section 2 (Nant Melyn East) - SO 97928 06232 - Section 2 (Nant Hir Bridge) – SO 98964 06973 - Section 2 (Baverstock South) – SO 00036 06815 |
| Effluent Discharge | - Taf Fawr East Discharge - Dowlais West Discharge (Direct to drainage ditch, flow to Dowlais Reservoir) - Dowlais East Discharge (Surface Water Drainage, potential flow Nant Carno) - Section 2 (Nant Melyn East) - Section 2 (Nant Hir Bridge) - Section 2 (Baverstock South) |
| Nature | Dewatering of excavations, capture of surface water run off to settlement ponds, treatment of water via chemical treatment before discharge to either existing drainage, ditch or direct to watercourse as per GPS Ref |
| Volume Calculation | The element of discharge rates is designed to be at a worst-case scenario upon rainfall dependent water creating excess flows at work face. It is not envisioned that such rates of pumping shall be required on a constant basis but rather a necessity for when require. Therefore, at each location noted it is the below estimation… Siltbuster HB50 treatable of 5-60m3/h 3600sec (60secs to 1 min) – 13.8ltrs/s 10 hour working day, subject to operation over 10 hours this would conclude 300 to 600m3/Day, rainfall dependent Note – unit shall be run below maximum to ensure full efficiency |
| Process of Treatment | Lagoon settlement, Primary Settlement, Chemical Settlement |
| Outline Drawing | See associated documents Taf Fawr East Discharge Section 2 Active Treatment Discharge Dowlais Discharge |

## Soneco

| Project | A465 Section 5 and 6 Hirwaun to Dowlais Top |
| --- | --- |
| Site Office | Baverstock Compound, Dynevor Arms, Llwydcoed, Aberdare CF44 0ND |
| Discharge Location | - Afon Cynon – SN 96818 05324 |
| Effluent Discharge | - Afon Cynon Discharge (to grassland or direct to Afon Cynon) |
| Nature | Dewatering of excavations, capture of surface water run, treatment of water via mechanical treatment before discharge to either existing drainage, ditch or direct to watercourse as per GPS Ref |
| Volume Calculation | The element of discharge rates is designed to be at a worst-case scenario upon rainfall dependent water creating excess flows at work face. It is not envisioned that such rates of pumping shall be required on a constant basis but rather a necessity for when require. Therefore, at each location noted it is the below estimation… Soneco 3 stage treatment 3600sec (60secs to 1 min) – 13.8ltrs/s 10 hour working day, subject to operation over 10 hours this would conclude a maximum 100m3/Day, rainfall dependent. Unit is designed for 10m3/h Note – unit shall be run below maximum to ensure full efficiency |
| Process of Treatment | Primary Settlement at culvert out point, Mechanical Settlement |
| Outline Drawing | - See associated documentation Afon Cynon Discharge |

## Passive Dewatering

| Project | A465 Section 5 and 6 Hirwaun to Dowlais Top |
| --- | --- |
| Site Office | Baverstock Compound, Dynevor Arms, Llwydcoed, Aberdare CF44 0ND |
| Discharge Location | - Section 2 (Croesbychan) – SN 97883 05518 - Section 2 (Nant Melyn North) – SN 97920 06254 - Section 2 (Culvert 5969) – SN 98361 06591 - Section 2 (Swansea Road) – SO 01451 07307 - Taf Fawr West – SO 02703 07993 |
| Effluent Discharge | - Section 2 (Croesbychan) - Section 2 (Nant Melyn North) - Section 2 (Culvert 5969) - Section 2 (Swansea Road) - Taf Fawr West Discharge |
| Nature | Dewatering of excavations, capture of surface water run off to settlement ponds, treatment with flocculant products prior to natural flow into existing drainage ditches, land or watercourse as per GPS Ref |
| Volume Calculation | The element of discharge rates is designed to be at a worst-case scenario upon rainfall dependent water creating excess flows at work face. It is not envisioned that such rates of pumping shall be required on a constant basis but rather a necessity for when require. Therefore, at each location noted it is the below estimation… Selwood pumps for hire 20-30m3/h 3600sec (60secs to 1 min) – 13.8ltrs/s 10 hour working day, subject to operation over 10 hours this would conclude 200-300m3/Day, rainfall dependent. This is waters pumped to a primary settlement and not a direct discharge but an overflow from attenuation via, floc and floc matt polish |
| Process of Treatment | Lagoon settlement, Primary Settlement, Chemical Settlement |
| Outline Drawing | See associated document Section 2 Passive Discharge Nant Ffrwd East West Taf Fawr West Discharge |

## Siltbuster Waste Arisings

Waste arisings shall be processed by means of drying out to allow moisture content to demise. There shall be no off-site removal of material from this process as the slurry or silt shall be tested as per client materials management plan criteria to ascertain if material is good for landscape fill process.

# Water Catchment on Average Rainfall

The below table is an estimated summary of each site inclusive of its catchment area along with the average rainfall prediction as per Natural Resource Wales measuring station situated at Hirwain 2022. The information below is a guideline for a month period, as this cannot be assessed as each monthly rainfall event can change.

The calculation used to determine such runoff is concluded (Cumulative actual 301mm recorded February 2022) …

Average Rainfall (m) x Catchment Area (m2) x Surface Coefficient 0.1

| Untreated Ground Catchments |  |
| --- | --- |
| Soil on slopes less 10% | 0.0 – 0.3 |
| Rocky Natural Catchments | 0.2 – 0.5 |

Reference: Pacey, Arnold and Cullis, Adrian 1989, Rainwater Harvesting: The collection of rainfall and runoff in rural areas, Intermediate Technology Publications, London

| Site Name | Catchment Area m2 | Surface run off estimate on average highest rainfall per 1 month (m3) | Attenuation Pond (m3) |
| --- | --- | --- | --- |
| Outlet 3: Taf Fawr East Discharge | 7,700 | 231 | 509 |
| Outlet 2: Dowlais West Discharge | 24,891 | 747 | 2000 |
| Outlet 1: Dowlais East Discharge | 95,730 | 2,872 | 1500+350 |
| Outlet 14: Section 2 (Nant Melyn East) | 8,040 | 241 | 675 |
| Outlet 10: Section 2 (Nant Hir Bridge) | 9,670 | 290 | 800 |
| Outlet 15: Afon Cynon Discharge | 13,708 | 411 | 1744 + 798 |
| Section 2 (Croesbychan) | 110,000 | 3300 | 1318 |
| Outlet 13: Section 2 (Nant Melyn North) | 5,774 | 173 | 675 |
| Outlet 11: Section 2 (Culvert 5969) | 432 | 13 | 1683 |
| Outlet 9: Section 2 (Nant Hir East) | 19,997 | 599 | 800 |
| Outlet 8: Section 2 (Baverstock South) | 128,779 | 3863 | 1992 |
| Outlet 7: Section 2 (Swansea Road) | 42,275 | 1268 | 1714 |
| Outlet 4: Taf Fawr West | 6,142 | 184 | 100 |

# Operational Arrangements

FVC shall operate the Silt-buster unit at location on a daily basis. The operational outfit shall follow the daily pattern of works, Monday to Friday 0700hrs to 1700hrs. Only at which there is consistent heavy down pour causing sub sequential flooding of excavations shall the unit be operated at night. This shall be monitored with weather patterns on daily basis and notice given for dedicated night / weekend shift team when required. Provision is catered for 24/7 hour support to Siltbuster operation.

# appendix 1

## Aquatreat

## Brenntag PAC

## Caustic Soda Liquor

# Appendix 2 – Siltbuster check sheet

| Silt Buster Daily Check Sheet |  |  |  |
| --- | --- | --- | --- |
| Description | Check | Outcome (tick all good) | Issues / Action |
| General | Any unusual sounds, vibrations or smells |  |  |
|  | Pipework checks, leaks, frozen, wear and tear |  |  |
|  | Control panel check no faults or alarms |  |  |
|  | Power cables in working order, Visual |  |  |
| Mechanical | Motors running, no signs of sticking, bearing noise |  |  |
|  | Motor vents clear and free from debris |  |  |
|  | Flowmeter registering presence of water |  |  |
|  | Chemical dosing pump head in auto mode |  |  |
|  | Sludge pump activities in auto / manual mode |  |  |
| Consumables | Check level of Coagulant |  |  |
|  | Check level of Flocculant |  |  |
|  | Bunding of chemicals not compromised |  |  |
| Clarifier | Visual check of clear water at outlet of clarifier |  |  |
|  | Lamella plates free from solid build up |  |  |
|  | Hoppers de-sludge |  |  |
| Reaction Tank | Check pH level on controls 6-9 |  |  |
|  | Check chemical dosing line blockage, solution entering |  |  |
|  | Wipe clean pH probe |  |  |
| Check by |  | Company |  |
| pH Level |  | Further Comment |  |
| Coagulant pump rate | l/hr |  |  |
| Flocculant pump rate | l/hr |  |  |

# Appendix 3 Soneco Check Sheet

| Soneco Check Sheet |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Measurement |  | Units | Value |  | Max | Comment |
| Balance Tank Level |  | M |  |  |  |  |
| Instantaneous flow |  | M3/h |  |  |  |  |
| Feed Pump |  | RPM |  |  |  |  |
| Aluminium Dose |  | Mg/l |  |  |  |  |
| Plate Life |  | Days |  |  |  |  |
| Mix Tank 1 |  | RPM |  |  |  |  |
| Mix Tank 2 |  | RPM |  |  |  |  |
| Water Conductivity |  | µS/cm |  |  |  |  |
| Treated Water TSS |  | Mg/l |  |  |  |  |
| Check by |  |  | Flow rate discharge set |  |  | Further Comment |
| Company |  |  |  |  |  |  |

# Appendix 4

## Rutpen A9

## Water Lynx 360

## Water Lynx 395

# Appendix 5 – Floc mat Release Rate