# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for Fir View Tan Y Fridd

Holiday Home Park operated by Sheehan Holdings Limited.

The application number is PAN-016382

The permit number is EPR/CB3599FY

The discharge is for a new caravan site to discharge secondary treated sewage effluent to ground water, with a maximum volume of 33.3 cubic metres per day. Treatment is via package treatment plant and infiltration system meeting British Standards for design and construction. The discharge serves an additional section of the caravan park containing 54 static caravans, consisting of 45no. 2-bedroom caravans, and 7no. 3-bedroom caravans. The site is open 9 months of the year from February to November.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. | ü |
| ### Responses to web publicising | There were no responses made to web publicising. The decision was taken in accordance with our guidance. | ü |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. | ü |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. | ü |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. | ü |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site The proposal is not located within a catchment of a failing fresh water SAC nor is it within screening distance of any other SAC/SPA/RAMSAR. Screening tool used: WQ - GW activity bespoke permit > 2m3/d |  |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have reviewed the operator's assessment of the environmental risk from the facility. The operator’s risk assessment is satisfactory. The assessment shows that, applying the conservative criteria in our guidance on Environmental Risk Assessment all emissions may be categorised as environmentally insignificant with the exception of Ammonia. Ammonia will be controlled via limit set within the permit and table S3.1 | ü |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. Proposed measures are in compliance with that set in TGN 7.01 How to comply with your environmental permit. The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN and we consider them to represent appropriate techniques for the facility. | ü |
| The permit conditions |  |  |  |
| Emission limits | We have decided that emission limits should be set for the parameters listed in the permit. It is considered that the numeric limits described below will prevent significant deterioration of receiving waters. We have imposed numeric limits because either a relevant environmental quality or operational standard requires this. - Ammoniacal nitrogen (as N) - 3 mg/l - ATU-BOD as O2 - 20 mg/l - pH 6 – 9 The limits have been applied to ensure compliance with WFD is achieved at the surface water receptor located down gradient of the discharge. pH has been include to ensure dosing to adjust alkalinity does not have a negative impact on the infiltration system and natural surrounding ground conditions. | ü |
| Considerations of foul sewer | We agree with the operators justification for not connecting to foul sewer. | ü |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. | ü |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” | ü |
| Environment Wales Act 2016 |  |  |
| Biodiversity and resilience of ecosystems duty | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case, we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for a groundwater water discharge activity permit under EPR2016. | ü |