
Permit Number: 
EPR/FP3031CW




Operator: 

Headland Agrochemicals Ltd

Facility:

Pentre Agrochemicals Plant


Form Number: 
Air1 / 27/09/13
Reporting of emissions to air for the period from 1st October 2014 to 30th September 2015 

	Emission

Point
	Substance /

Parameter
	Emission Limit Value
	Reference Period
	Result [1]
	Test

Method [2]
	Sample

Date and Times [3]
	Uncertainty

[4]

	A001
	Particulate Matter
	20 mg/m3
	Averaged over the sampling period (at least half an hour)
	0.85mg/m3
	BS EN 13284-1
	10:23-11:23 am

31st March 2015
	0.41

	A006
	Nitric Acid Vapour
	No limit set
	Spot Sample
	22mg/m3
	USEPA M7d
	10:49-11:49am

31st March 2015
	2.6

	
	Oxides of Nitrogen (NO and NO2 expressed as NO2)
	200 mg/m3
	Averaged over the sampling period (at least half an hour)
	6.6mg/m3
	BS EN 14792
	10:49 -11:49am

31st March 2015
	4.1


[1]
The result given is the maximum value (or the minimum value in the case of a limit that is expressed as a minimum) obtained during the reporting period, expressed in the same terms as the emission limit value. Where the emission limit value is expressed as a range, the result is given as the ‘minimum – maximum’ measured values.

[2]
Where an internationally recognised standard test method is used the reference number is given. Where another method that has been formally agreed with Natural Resources Wales is used, then the appropriate identifier is given. In other cases the principal technique is stated, for example gas chromatography.

[3]
For non-continuous measurements the date and time of the sample that produced the result is given. For continuous measurements the percentage of the process operating time covered by the result is given.   

[4]
The uncertainty associated with the quoted result at the 95% confidence interval, unless otherwise stated.

Signed Carl Rowe………………………….  

Date 15th October 2015………………………..   



(Authorised to sign as representative of Operator)
