Permit Number: 
EPR/DP3137EG




Operator: 
Margam Green Energy Limited
Facility:

Margam Green Energy Plant


Form Number: Residues 1 
Date : 03/09/2019
Reporting of residue quality for the period from:          

20/06/2019 to 30/06/2019
	
	Residue Composition (TOC, Metals, Dioxins, etc.)

	
	TOC
(<3%)
	Sb mg/kg
	Cd mg/kg
	TI
mg/kg
	Hg
mg/kg
	Pb
mg/kg
	Cr
mg/kg
	Cu
mg/kg
	Mn
mg/kg
	Ni
mg/kg
	As
mg/kg
	Co
mg/kg
	V
mg/kg 
	Zn
Mg/kg
	DIOXIN /FURAN         I-TEQ
ng/kg
	DIOXIN / FURAN
	DIOXIN-LIKE PCBS

	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	WHO-TEQ ng/kg
	WHO-TEQ ng/kg

	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	Humans / mammals
	Birds
	Fish
	Humans / mammals
	Birds
	Fish

	Bottom Ash (including boiler ash)
	8.6
	16.1
	2.03
	<0.07
	<0.36
	995
	179
	1,330
	761
	47.4
	77.7
	10.4
	14.5
	3,070
	
	
	
	
	
	
	

	APC residues
	N/A
	315
	42.2
	
	4.5
	8,974
	160
	1392
	898
	32.8
	986
	8.3
	14.1
	28,640
	
	1,207
	2,632
	1,277
	1,207
	2,632
	1,277


Reporting of Ash Solubility for the period from:          

20/06/2019 to 30/06/2019
	Ash solubility (Metals)

	
	Sb mg/kg
	Cd mg/kg
	TI
mg/kg
	Hg
mg/kg
	Pb
mg/kg
	Cr
mg/kg
	Cu
mg/kg
	Mn
mg/kg
	Ni
mg/kg
	As
mg/kg
	Co
mg/kg
	V
mg/kg
	Zn
mg/kg

	Bottom ash (including boiler ash)
	16.1
	2.03
	<0.07
	<0.36
	995
	179
	1330
	761
	47.4
	77.7
	10.4
	14.5
	3070

	APC residues
	315
	42.2
	
	4.5
	8974
	160
	1392
	898
	32.8
	986
	8.3
	14.1
	28,640


Signed ………John Williams…………………………………………….  

Date…03/09/2019…..   

(Authorised to sign as representative of Margam Green Energy Limited) 
