| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-018374 |
| Permit Reference Number: | EPR/HB3503LF |
| Permit Holder: | Severn Trent Water Limited |
| Site Address: | Noyadd Farm, Rhayader, Powys, LD6 5HH Dderw, Cwmdauddwr, Rhayader, Powys, LD6 5EY GlanElan, Rhayader, Powys, LD6 5HG |
| Grid Reference: | Farm location: SN 95773 66917 SN 96155 68527 SN 96267 66445 Fields: 01 TM DF SN 95989 68431 02 TM DF SN 96432 68587 03 TM DF SN 96554 68500 04 TM DF SN 96460 68421 05 TM DF SN 96596 68364 06 TM DF SN 96452 68242 08 TM DF SN 96359 68353 Noyadd Farm 30E SN 95583 66668 6481 + 6895 SN 95666 65879 7788 SN 95771 65885 8978 SN 95895 65804 9291 SN 95924 65926 Glan-Elan 04 TM NGE SN 96528 66883 05 TM NGE SN 96566 66979 06 TM NGE SN 96604 67089 10 TM NGE SN 96502 67186 11 TM NGE SN 96527 66781 12 TM NGE SN 96341 66867 Storage: 1 SN 96341 68522 2 SN 96433 68324 3 SN 95533 66602 4 SN 95972 65891 5 SN 96602 67158 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: GwastraffaGorfodi.Canolbarth@cyfoethnaturiolcymru.gov.uk Environment team: environmentteam.northpowys@cyfoethnaturiolcymru.gov.uk NLO: TBC |
| Date due: | 06/07/2022 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating A- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | EA, live, no variations. |
| Correct application form | Y |  |
| Correct fee | N | See permit compliance multiplier above and Charging Scheme page 52 No fee info provided inc no reference |
| Fee on MyNRW | N | Paid on 22/06/22 Amount: £1018 Date cleared on MyNRW: 22/06/22 Transaction number: 2102372 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | N |  |
| Site Specific Risk Assessment | Y | High risk location only |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y | EPR/HB3503LF |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y | Mr Robert Tucker |
| ## Has the technically competent manager details been provided? | Y | Mr Lee Musgrove |
| ## Has the nominated competent person details been provided? | Y | Mr Matthew Rouse |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | Number provided: 52/258/0016. Provided further down the form: 52/258/0004 and 52/258/0006. All agree with MyMap. |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y | 38.3ha |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N | Single application |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y | Mr Tony Morgan of Noyadd Farm Mr Tony Morgan of Dderw Mr Rob lewis of Glan-Elan |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y |  |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | Y | PAN-014351. Expired. |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | High |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | Y |  |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | site‐specific risk assessment |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | N/A |  |
| If no have details of the assessment evidence to support this conclusion been provided? | N | RFI |
| - About the waste |  |  |
| Has all parts of the waste types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable of non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y | 832 tonnes total, one waste 19 09 02 |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y | Own knowledge and Google |
| - About the storage |  |  |
| Has all of waste storage table been completed? | Y |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| If land was previously deployed on, and waste is stored in temporary field heaps; are the storage locations in the same or different locations? | Different | Different to PAN-014351 |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y |  |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | n/a |  |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | Y |  |
| location of all storage sites for the waste to be spread | Y |  |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | N | Issues to north of fields 9291 and 8978 need to be marked. |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | Y |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | Y | Elenydd – Mallaen SPA. 410m from field 8978. Carn Gafallt SSSI – 410m from field 8978. Rhagnetydd Gwy Uchaf/Upper Wye Tributaries SSSI – 43m from field 9251, 21m from 30E, River Wye (Upper Wye) / Afon Gwy (Gwy Uch af) SAC 437m from field 11 TM NGE, 24m from field 02 TM DF Coetiroedd Cwm Elan / Elan Valley Woodlands SAC. 410m away from 8978. River Wye/Afon Gwy (Wales) SPA. 43m from 9291, 21m from 30E, 437m from 11 TM NGE, 24m from field 02 TM DF Cwm Cwynllyn SSSI – 381m from 01 TM DF New House Meadow SSSI – 223m from 10 TM NGE Coed y Cefyn 580m away - screens out As to increase buffers along River Wye. |
| location of any public rights of way (shown on map) | Y | One not marked |
| any buildings or houses within 250m (shown on map) | Y |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Nitrate Vulnerable Zone (NVZ) - Groundwater Safeguard Zone - Hedges - Ditches | Y | See full MyMap report. |
| What is the slope of the fields? | 0-7 degrees (transparent)- fields #01 TM DF, 08 TM DF, 04 TM DF, 30E, 10 TM NGE, 06 TM NGE, 05 TM NGE, 12 TM NGE, 04 TM NGE, 11 TM NGE, and 9291. 7 -12 degrees (yellow)- fields #02 TM DF, 03 TM DF, 05 TM DF. Buffer zones applied but wider zones to be requested due to SAC. Small area of 06 TM DF, no concern. Sections of 6481 + 6895, 7788, and 8978. 12 degrees plus (red)- Sections of 6481 + 6895, 7788, and 8978. No concern due to solid waste not being a run-off risk, and all issues are correctly buffered. Sections of #02 TM DF, 03 TM DF, 05 TM DF. Buffer zones applied but wider zones to be requested due to SAC |  |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y | Too high |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y |  |
| - If yes; have they been tested for? | Y | Iron, aluminium and sulphur. |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | Y |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y | Excepting the zinc – query this |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | N | RFI |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | n/a |  |
| - Soil physical properties (e.g. organic matter and soil structure) | n/a |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | N | Zinc |
| - Included the nutrient level in their environmental risk assessment | N | Zinc |
| - Stated the source of information that they used to set the nutrient level. | N | Zinc |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | n/a |  |
| - Is it only for operational purposes? | n/a |  |
| - Have individual application rates for each waste been given? | n/a |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | n/a |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | N | RFI |
| - Have they stated when the waste will be spread? | N | RFI |
| - Have they stated how often the waste will be spread? | N | RFI |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients supplied match NRW calculations? | Y | Slight discrepancies but likely due to rounding as discrepancies are consistent. No concern. |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y | See PAN-014351. Site officer not aware of any spreading taking place under this deployment. |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | Y |  |
| - The nutrients supplied by the material? | Y |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | Y |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | Y |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | Y |  |
| - Pathways | Y |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | Y |  |
| - Measures that will minimise or mitigate the impact will be put in place | Y |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | Y |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | N |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | Y |
| No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site. - No conceivable pathways to the protected sites. Please see Form 1 saved within this application’s folder for initial screening out information. |  |

Comments

Consultation responses: - No records of spreading under the previous deployment - Groundwater vulnerability has now been assessed as med-high.

MyMap, location and slope assessment: MyMap assessment and location assessment – see PAN-018374 MyMap Report Slope assessment: See slope assessment Field number: See slope assessment Slope of the land: See slope assessment Soil type: Medium Wastes types applied (solid, liquid, sludge etc.): Sludge (stackable) Cropping type: Mix of silage and grazing Method of application: Rear discharge spreader Weather conditions before during and after spreading: To follow CoGAP Other risk factors not mentioned above: N/A Overall conclusion – low risk due to appropriate buffers and spreading methods.

CPH details: Three provided, all correct according to our system.

Protection of habitats and ecosystems Please see Appendix 4 and Form 1 saved for information.

Agricultural benefit calculations: Zinc levels too high. Slight variations due to rounding differences between applicant and NRW, no concern other than zinc.

Request for information: Request for Information – 13/07/2022 - Payment info and payment missing. Provided and tracked on the system. - Issues need marking to the North of Noyadd Farm. Buffered. - Rights of way need to be buffered through fields aof Noyadd Farm and Glan-Elan. Buffered and storage changed. - Slopes in Noyadd Farm fields - Elan Valley Woods is a SPA not an SAC. Mistake on my behalf – is a SAC. - Missing River Wye SPA from risk ass. Also an SAC not a SPA. No issues. - Excess zinc. Our calculated total zinc addition at 22 tonnes/ha: 15.16kg/ha. Applicant’s calculation: 14.4kg.ha. - Timing of applications not given. One application, either in spring or summer, before an advanced period of grazing or following a silage cut. - EIA evidence has not been provided. Provided. Request for Information – 25/07/2022 - Further clarification required on excessive zinc application and a reduction of application rate if needed. Applcation had been judged on dry matter on ignition, not total solids. Upon correction, the values match correctly and are within the limits for

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 02/08/2022 |
| ## Effective date | 02/08/2022 |
| ## Permitting Officer Date | Rhiannon Smith 02/08/2022 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Matthew Fryer (ToP) 02/08/2022 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 70 |
| Determination | 645 |
| Sign off | 30 |
| Issuing | 60 |
| Total | 805 |