# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for Waun Mary Gunter Farm operated by Mr Roland Phillips.

The application number is PAN-017437

The permit number is EPR-CB3691ZL

The discharge is for a new surface water discharge at Waun Mary Gunter Farm, Blaenavon, Pontypool, NP4 9SJ.  The package treatment plant system will be serving 1x 3 bedroomed residential property.  The PTP conform to current British Standards BS 12566. The treated effluent is domestic in nature. The discharge is into an unnamed drain of the Afon Lwyd.  The drain is seasonally dry in warm weather, this will be addressed by the addition of a perforated pipe system.  A sewage effluent discharge of a low volume 1 m3/day has been estimated.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

- Annex 2 the consultation responses

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. | ü |
| ### Responses to consultation and web publicising | The web publicising and consultation responses (Annex 2) were taken into account in the decision. The decision was taken in accordance with our guidance. | ü |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. | ü |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. | ü |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. | ü |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site Note: From OGN 200 - no conceivable impact on any Natura 2000 site, by virtue of the scale or location or nature of the project. This proposal has been screened and is outside any Natura 2000/Ramsar sites. | ü |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have carried out a risk assessment on behalf of the operator. The operator considers this risk assessment is satisfactory. | ü |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes according to TGN 7.01 . The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN and we consider them to represent appropriate techniques for the facility. | ü |
| The permit conditions |  |  |  |
| Considerations of foul sewer | We agree with the operators justification for not connecting to foul sewer. | ü |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. | ü |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” | ü |
| Environment Wales Act 2016 |  |  |
| Biodiversity and resilience of ecosystems duty | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case, we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for a groundwater discharge activity permit under EPR2016. | ü |

# Annex 2: Consultation and web publicising responses

Response received from

TBC

Brief summary of issues raised

TBC

Summary of actions taken or show how this has been covered

TBC