	[image: image1.png]Cyfoeth
Naturiol
Cymru

Natural
Resources
Wales




	EPR Compliance Assessment Report
	Report ID: AP3433ES/0245135

 


	This form will report compliance with your permit as determined by an NRW officer

	Site
	Dowlais Slaughterhouse NRW/EPR/AP3433ES/V002
	Permit Ref
	AP3433ES

	Operator/ Permit holder
	2 Sisters Red Meat Limited
	

	Date
	04/08/2015 
	Time in
	12:00
	Out
	16:00

	What parts of the permit were assessed
	Q1,2,3,4 returns 2014, Q1 return 2015. 2014 EMS and Fugitives reports, Site closure plan.

	Assessment
	Report/data review
	EPR Activity:
	Installation
	X
	Waste Op
	
	Water Discharge
	

	Recipient’s name/position
	Rob Lavercombe, Environment Manager

	Officer’s name
	Richard Taylor
	Date issued
	04/08/2015


	Section 1 - Compliance Assessment Summary

	This is based on the requirements of the permit under the Environmental Permitting Regulations.  A detailed explanation and any action you may need to take are given in the “Detailed Assessment of Compliance” (section 3).  This summary details where we believe any non-compliance with the permit has occurred, the relevant condition and how the non-compliance has been categorised using our Compliance Classification Scheme (CCS).  CCS scores can be consolidated or suspended, where appropriate, to reflect the impact of some non-compliances more accurately.  For more details of our CCS scheme, contact your local office.

	Permit Conditions and Compliance Summary                    
	Condition(s) breached

	a) Permitted activities 
	1. Specified by permit
	N
	
	

	b) Infrastructure
	1.
Engineering for prevention & control of pollution
	N
	
	

	
	2. Closure & decommissioning
	A
	
	

	
	3.
Site drainage engineering (clean & foul)
	O
	
	

	
	4. Containment of stored materials
	N
	
	

	
	5. Plant and equipment
	N
	
	

	c) General management
	1. Staff competency/ training
	N
	
	

	
	2. Management system & operating procedures
	O
	
	

	
	3. Materials acceptance
	N
	
	

	
	4. Storage handling, labelling, segregation
	N
	
	

	d) Incident  management
	1. Site security
	N
	
	

	
	2. Accident, emergency & incident planning
	N
	
	

	e) Emissions

	1. Air
	N
	
	

	
	2. Land & Groundwater
	O
	
	

	
	3. Surface water
	C3
	
	6.4.2

	
	4. Sewer
	O
	
	

	
	5. Waste
	N
	
	

	f) Amenity
	1. Odour
	N
	
	

	
	2. Noise
	N
	
	

	
	3. Dust/fibres/particulates
	N
	
	

	
	4. Pests, birds & scavengers
	N
	
	

	
	5. Deposits on road
	N
	
	

	g) Monitoring and records, maintenance and reporting
	1. Monitoring of emissions & environment
	O
	
	

	
	2. Records of activity, site diary, journal & events
	N
	
	

	
	3. Maintenance records
	N
	
	

	
	4. Reporting & notification
	C4
	
	5.1.5

	h) Resource efficiency
	1. Efficient use of raw materials
	N
	
	

	
	2. Energy
	A
	
	

	KEY:  C1, C2, C3, C4 = CCS breach category ( * suspended scores are marked with an asterisk),
A = Assessed (no evidence of non-compliance), N = Not assessed, NA = Not Applicable, O = Ongoing non-compliance – not scored

	
	
	
	
	

	Number of breaches recorded 
	2
	Total compliance score

(see section 5 for scoring scheme)
	4.1


	If the Total No Breaches is greater than zero, then please see Section 3 for details of our proposed enforcement response 


	Section 2 – Compliance Assessment Report Detail

	This section contains a report of our findings and will usually include information on:

	· the part(s) of the permit that were assessed (e.g. maintenance, training, combustion plant, etc)

· where the type of assessment was ‘Data Review’ details of the report/results triggering the assessment

· any non-compliances identified 

· any non-compliances with directly applicable legislation 
· details of any multiple non-compliances 
	· information on the compliance score accrued inc. details of suspended or consolidated scores.

· details of advice given
· any other areas of concern 

· all actions requested
· any examples of good practice.

· a reference to photos taken

	This report should be clear, comprehensive, unambiguous and normally completed within 14 days of an assessment.




	 

The purpose of this CAR form is to record the regulatory actions taken against the site and to bring the site up to date with their submissions to NRW.
The following documents have been received and evaluated by NRW; they are listed with the date of receipt, followed by the comments and actions against each form. The actions are summed up below, plus a list of regulatory responses.
 

a). 10 Jan 14 – Annual Fugitive Emissions Report (IC9.16) for 2013. Two odour complaints listed in July and October 2013. All waste contractors named and addressed with descriptions of how odours are minimised at site. Report accepted in full.
 

b). 27 Jan 14 – EMS report for 2013. Good items of achievable objectives listed. Switching to LED lighting is a simple gain. Policy of replacing equipment with the most efficient type available in place. Bulk storage vessels purchased for ETP chemicals. New spill kits purchased. Report accepted in full.
 

c). 31 Jan 2014 – Q4 2013 returns – Received Forms A2, A4, A5, S2, R1, R2. Forms A4 and 5 are nil returns due to the incinerator remaining out of commission. A2 has CO measured at 0ppm from both stacks. NO2 were to follow. Reporting in ppm is not the required reporting nomenclature. The form also asks for results to be reported in Mg/m3 at UK standard reference conditions. The reporting of the figures in ppm rather than mgm3 is a breach of permit conditions earning a Cat 4 breach. However, this score has been listed on the CAR form with the score suspended in lieu of late action from NRW. The permit reference to the required standard is laid down in permit condition 10.1.3 b.
 

d). 3 Feb 2014 – Q4 2013 returns, Form A1, A3 (nil returns), W1 and S1. W1 shows Mineral Oil and hydrocarbons from release points W1 and W2 at 4.0 mg/l. Is this the limit of detection of the equipment? Also, a breach of permit limit of 14mg/l against a maximum allowance of 5mg/l was reported from W2. This has been recorded but not scored due to the late evaluation of this report by NRW.
 

e). 30 Apr 14 – Q1 2014 report covering Jan – Mar. Site submitted Forms A1, A3, W1, and S1. The forms used are now old, and need to be changed so that ‘St Merryn Meats Ltd’ is replaced with ‘2 Sisters Red Meat’. Action – NRW to complete. All forms accepted.
 

f). 4 Aug 2014 – Q2 report covering Apr-Jun 14. A breach of pH max was recorded at 12.6 against the parameter 6-11. See Summary of actions at the bottom. There was also a discrepancy with the testing lab where they omitted to test for the correct suit of oils and greases for April and May. This was corrected for June, and the explanation was accepted by NRW. All other recordings were within limits.
 

g). 5 Aug 2014. Schedule 1 part A and B received for an odour complaint on 25th Jul 2014. Explanation accepted, no breach of permit recorded.
 

h). 23 Oct 2014 – Q3 returns July –Sep. Nil returns again for A1, and A3. Mineral oil and suspended solids measurements all within limits, but SS for W1 taken on 25th Sept shows an approach to the limit. Form S1 shows pH measurements which do not correspond with each other. The minimum reading is 6.2, and the maximum is 7.5, but the average is given as 2.73. Action; Site to explain if this reading is an error. Likewise the measurement for the Dissolved Oxygen (DO) gives a minimum of 2.73, a max of 8.9, but an average of 2.73. Action, please explain if this is an error. 
 

i). 22 Dec 14 – IC 9.16 Fugitive emissions report. 2 complaints received by site in Jul and Sep were shown. All waste contractors were shown. Some have been checked for their environmental record. One was fined in 2012 by the Environment agency, and carries past links to the BSE crisis. Site is recommended to carry out waste acceptance audits on all waste contractors to ensure they meet the high standards of 2 sister red meat. Recommendation – carry out waste acceptance audits on all contracted waste receivers. The site has a scheduled 3 yearly integrity test on the diesel tank. This is seen as a strong proactive measure by NRW.
 

j). 28 Jan 2015 – IC 9.14 Site Closure Plan (Bi annual requirement). The plan was submitted after the due date (31 Dec 2014) but accepted without penalty. Please ensure the next due date is met. This will be 28 Jan 2017. The report covered how the site will deal with notifications, bulk gas supplies, refrigeration systems, and dismantling of buildings. NRW accepted this as a workable document.
 

k). 3 Feb 2015 – EMS report for pc 4.1.3. The report was submitted on time. It covered Utility useage LED lighting as an improvement item. But this was already mentioned in the previous year’s submission. Indeed 4 out of the five topic under this heading had already been submitted in last year’s report. Only planning for a site-wide internal electronics system (SCADA) was new, and this was only in the planning stage. Recommendation, Site to prioritise new achievable objectives for next year’s submission which set measurable goals, and report against them for the 2015 submission due Feb 2016.
 

l). 17 Feb 2015 – Annual returns and Q4 returns Oct-Dec 14. Forms A1, A2, A3, A4 and A5 were Nil returns because the incinerator they apply to is not being operated by site. Form S2 was sent with no recorded organophosphorus content, with a note that they will be sent when this work has been achieved (sent 24th Mar 15). Form R1 showed Energy and water consumption figures which included the correct conversion factors and accepted. R2 Figures were cross checked against the site Pollution Inventory report and found to be accurate. Form W1 showed values all within the accepted limits. Form S1 showed a max peak of 12.90 for pH and an average of 3.32. Both are outside the parameters of 6-11 for effluent to sewer. However, the average does not tally with other reported max-min figures. Action. Site to explain if this reported figure is erroneous. The daily and hourly flows also breached their limits of 800 m3 day, and 34m3 hr which are lumped together with the pH breach as a single Cat 3. Also the reporting of this was not first notified to NRW by schedule 1 as a requirement under pc 5.1.1.a. earning a Cat 4.
 

These are breaches in their own right but will be dealt with together for practicality. The non-issue of a schedule 1 form to NRW earns a Cat 4 breach of pc 5.1.1. The 3 other breaches should be scored as a Cat 3 each. However, in practical terms the hourly and daily flow exceedences will be dealt with as one breach, with the pH spike of 12.90 given a Cat 3. The hourly/flow returns will have a suspended score because of the doubt over how much flow is added into the sites effluent load by neighbouring sites. Site have been previously urged in CAR forms (14th June 2013 action 5) to introduce a maximum setting for the effluent treatment plant at the permit level (800 m3/day) to prevent breaches. Site is recommended to reassess this issue as a matter of urgency. 
 

Form A4 has reported figures in the incorrect nomenclature. Site reported in ppm rather than mg/m3 as stated in the permit. The permit requires this submission to be at uk standard reference conditions 273k, and an pressure of 101.3kPa10.1.3 b. The exceedences result in a 2 x Cat 3 breaches and 2 x Cat 4 breaches for permit exceedences totalling 8.2 points - suspended as pre 2015 figures.
 

m). 24 Mar 2015 – Organophosphorous pesticide lab results. These were sent to NRW as soon as available. The result of Diazinon for 5/3/15 was notably high compared to subsequent returns, but within Human health guidelines found I H1 Annex D, table 11 – environmental standards for diazinon. 
 

n). 9 Apr 2015 – Q1 returns Jan – Mar. Nil returns received for A1 and A3. W1 showed all measurements within limits. Form S1 showed a breach of minimum pH limits at 5.9 against the min 6. It also carried breaches in the maximum allowed limits for daily flow and hourly flow at 927 m3 day against a max 800m3 day and 38.6 against 34m3 hour. The three breaches were not notified via a schedule 1 form. Site must follow permit requirements as laid out in 5.1.1. 
The breaches for pollutants were seen as being minor in nature, and have been combined into a single Cat 3 breach. The non- submittal of schedule 1 earns a cat 4 scoring 0.1 points.
 

List of breaches from this CAR form and regulatory responses
It is NRW’s decision to itemise all breaches of permit conditions as listed above. However, because the invoices for annual subsistence fees have already been issued to sites, it would be unfair to score all the breaches which incurred during 2013 or 14. Inaction on our part has led to this loss of potential income from a raised OPRA score. Therefore these will be listed on the CAR form but not scored. Only breaches from 2015 onwards will be scored.
 

Breaches 
c). Cat 4, score 0.1 points (suspended). Reporting in ppm, not mg/m3 as required in pc 6.1.2.
d). Cat 3, score 4 points (suspended) Breach of mineral oil hydrocarbons limit of 5mg/l with 14mg/l.
f). Cat 3, score 4 points (suspended) pH breach. 12.6 against a max of 11.
l). Cat 3, score 4 points, plus 1 x Cat 4 0.1 points. Total 4.1.points (Suspended) (inclusive of 3x Cat 3 breaches for flow exceedence and pH, amalgamated to a single Cat 3 for overall water quality. 1x Cat 4 for non-reporting of breaches on schedule 1 forms breaching pc 5.1.1. 
n). 3 x Cat 3 breaches amalgamated to a single Cat 3 for low pH, daily flow max, and hourly flow max. 1 x Cat 4 for non issue of schedule 1 as required by pc 5.1.1.
c, d, f & l breaches have been suspended and not scored. ‘n’ has been scored and earns a total of 4.1 points.
 

Actions
1. (e) NRW to re-issue reporting forms to replace ‘St Merryns’ with ‘2 Sisters’ throughout.

2. (h) Site to explain if there is a transcription error in the reporting of average pH and DO.

3. (k) Site to set achievable targets for EMS and report measurable progress against them for the 2015 report.

4. (l) Again discrepancies in the average pH reported figures need to be addressed.

5.  

Recommendations
1. (i) Carry out audits on waste contractors.

2. (l) Re assess the approach to limiting the effluent treatment plant flow to 88m3 day.

3. (m) Be aware of Diazinon levels in offsite effluent.

END.
 




	[image: image2.png]Cyfoeth
Naturiol
Cymru

Natural
Resources
Wales




	EPR Compliance Assessment Report
	Report ID: AP3433ES/0245135


	This form will report compliance with your permit as determined by an NRW officer

	Site 
	Dowlais Slaughterhouse NRW/EPR/AP3433ES/V002
	Permit
	AP3433ES

	Operator/ Permit
	2 Sisters Red Meat Limited
	Date
	04/08/2015


	Section 3- Enforcement Response
	Only one of the boxes below should be ticked

	You must take immediate action to rectify any non-compliance and prevent repetition. 
Non-compliance with your permit conditions constitutes an offence and can result in criminal prosecutions and/or suspension or revocation of a permit.  Please read the detailed assessment in Section 2 and the steps you need to take in Section 4 below.

	Other than the provision of advice and guidance, at present we do not intend to take further enforcement action in respect of the non-compliance identified above.  This does not preclude us from taking enforcement action if further relevant information comes to light or advice isn’t followed.
	X

	In respect of the above non-compliance you have been issued with a warning. At present we do not intend to take further enforcement action. This does not preclude us from taking additional enforcement action if further relevant information comes to light or offences continue.
	

	We will now consider what enforcement action is appropriate and notify you, referencing this form.
	


	Section 4- Action(s) 

	Where non-compliance has been detected and an enforcement response has been selected above, this section summarises the steps you need to take to return to compliance and also provides timescales for this to be done.

	Criteria Ref.
	CCS Category
	Action Required/Advised 
	Due Date 

	See Section 1 above 
	
	

	E3
	C3
	See list of actions and recommendations at the foot of the dialogue box.
	N/A

	G4
	C4
	No action specified
	N/A


	Section 5 - Compliance notes for the Operator
	
	Section 6 – General Information

	To ensure you correct actual or potential non-compliance we may
( advise on corrective actions verbally or in writing 

( require you to take specific actions in writing 

( issue a notice
( require you to review your procedures or management system
( change some of the conditions of your permit

( decide to undertake a full review of your permit
	
	Data protection notice
The information on this form will be processed by the Natural Resources Wales (NRW) to fulfill its regulatory and monitoring functions and to maintain the relevant public register(s). The NRW may also use and/or disclose it in connection with:

(  offering/providing you with its literature/services relating to environmental matters

(  consulting with the public, public bodies and other organisations (e.g. Health and Safety Executive, local authorities) on environmental issues

(  carrying out statistical analysis, research and development on environmental issues

(  providing public register information to enquirers

(  investigating possible breaches of environmental law and taking any resulting action

(  preventing breaches of environmental law

(  assessing customer service satisfaction and improving its service

(  Freedom of Information Act/Environmental Information Regulations request.

The NRW may pass it on to its agents/representatives to do these things on its behalf. You should ensure that any persons named on this form are informed of the contents of this data protection notice.

	Any breach of a permit condition is an offence and we may take legal action against you.
( We will normally provide advice and guidance to assist you to come back into compliance either after an offence is committed or where we consider that an offence is likely to be committed. This is without prejudice to any other enforcement response that we consider may be required.

( Enforcement action can include the issue of a formal caution, prosecution, the service of a notice and or suspension or revocation of the permit. 

See our Enforcement and Civil Sanctions guidance for further information
	
	

	This report does not relieve the site operator of the responsibility to 

( ensure you comply with the conditions of the permit at all times and prevent pollution of the environment
( ensure you comply with other legislative provisions which may apply.
	
	

	Non-compliance scores and categories
	
	Disclosure of information

The NRW will provide a copy of this report to the public register(s).  However, if you consider that any information contained in this report should not be released to the public register(s) on the grounds of commercial confidentiality, you must write to your local area office within twenty working days of receipt of this form indicating which information it concerns and why it should not be released, giving your reasons in full.

	CCS category
	Description
	Score
	
	

	C1
	A non-compliance which could have a  major environmental effect
	    60
	
	

	C2
	A non-compliance which could have a significant environmental effect
	31
	
	

	C3
	A non-compliance which could have a  minor environmental effect
	     4
	
	Customer charter
What can I do if I disagree with this compliance assessment report?
If you are unable to resolve the issue with your site officer, you should firstly discuss the matter with the officer’s line managers.  If you wish to raise your dispute further through our official Complaints and Commendations procedure, phone our general enquiry number  0300 065 3000 (Mon to Fri 08.00–18.00) and ask for the Customer Contact team or send an email to enquiries@naturalresourceswales.gov.uk.  If you are still dissatisfied you can make a complaint to the Public Services Ombudsman for Wales.   For advice on how to complain to the Ombudsman phone their helpline on 0845 607 0987.

	C4
	A non-compliance which has no potential environmental effect
	    0.1
	
	

	Operational Risk Appraisal (Opra) - Compliance assessment findings may affect your Opra score and/or your charges. This score influences the resource we use to assess permit compliance.
	
	

	
	
	


CAR 2 V1.0 


