Mondelez UK Confectionery Production Limited Decision Document

Rev No1

# Bespoke permit

The application number is: PAN-008579

The Applicant / Operator is: Mondelez UK Confectionery Production Limited

The Facility is located at:	 Chirk Confectionery Plant, Station Avenue, Chirk, Wrexham, LL14 5LT

We have decided to grant the permit for Chirk MCP Boiler No. 2 (37/7602) operated by Mondelez UK Confectionery Production Limited.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

## Purpose of this document

This decision document:

explains how the application has been determined

provides a record of the decision-making process

shows how all relevant factors have been taken into account

justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

## Structure of this document

- Table of contents

- Key issues

## Table of Contents

## Contents

Bespoke permit	2

Purpose of this document	3

Structure of this document	4

Table of Contents	5

Contents	5

Key issues of the decision	6

1	Our decision	6

2	How we reached our decision	7

2.1 Receipt of Application	7

2.2	 Consultation on the Application	7

2.3	 Requests for Further Information	7

3	The Legal Framework	7

4	The Facility	8

4.1 Description of the Facility and related issues	8

4.2 Operation of the Facility – general issues	10

5	Minimising the Facility’s environmental impact	12

5.1	Assessment of Impact on Air Quality	13

5.2	Impact on Habitats sites, SSSIs, non-statutory conservation sites etc	14

5.3 European Sites	15

Assessment of Likely Significant Effect:	16

Appropriate assessment:	17

HRA Overall conclusion:	17

6  Setting ELVs and other Permit conditions	17

6.1	Monitoring	18

6.2	Other Permit Conditions	19

6.3	Reporting	20

7 MCPD Charges and Subsistence Fees	20

# Key issues of the decision

## Our decision

We have decided to grant a permit for Chirk MCP Boiler No. 2 (37/7602).

We consider that, in reaching that decision, we have taken into account all relevant considerations and legal requirements and that the permit will ensure that a high level of protection is provided for the environment and human health.

This Application is to operate a regulated facility which is subject principally to the Medium Combustion Plant Directive (MCPD)  regulations.

The permit contains many conditions taken from our standard Environmental Permit template including the relevant Annexes. We developed these conditions in consultation with industry, having regard to the legal requirements of the Environmental Permitting Regulations (EPR) and other relevant legislation. This document does not therefore include an explanation for these standard conditions. Where they are included in the permit, we have considered the Application and accepted the details are sufficient and satisfactory to make the standard conditions appropriate.

## How we reached our decision

### 2.1 Receipt of Application

The Application was accepted as duly made on 31/12/2019. This means we considered it was in the correct form and contained sufficient information for us to begin our determination, but not that it necessarily contained all the information we would need to complete that determination.

The Applicant made a claim for no claim for commercial confidentiality. We have not received information in relation to the Application that appears to be confidential in relation to any party. We have not accepted the Applicants claim for commercial confidentiality.

### 2.2	 Consultation on the Application

There was no requirement to carry out a consultation on the application.

### 2.3	 Requests for Further Information

There was no requirement for requests for further information.

## The Legal Framework

The Permit will be granted, under Regulation 13 of the EPR.  The Environmental Permitting regime is a legal vehicle which delivers most of the relevant legal requirements for activities falling within its scope.  In particular, the regulated facility is:

plant as described by Schedule 25A covering the Medium Combustion Plant Directive (MCPD) regulations;

subject to aspects of the Well-Being of Future Generations (Wales) Act 2015 and the Environment (Wales) Act 2016 which also have to be addressed.

We address the legal requirements directly where relevant in the body of this document.  NRW is satisfied that this decision is consistent with its general purpose of pursuing the sustainable management of natural resources (SMNR) in relation to Wales, and applying the principles of SMNR. In particular, NRW acknowledges that it is a principle of sustainable management to take action to prevent significant damage to ecosystems. We consider that, in granting the Permit a high level of protection will be delivered for the environment and human health through the operation of the Facility in accordance with the permit conditions. NRW is satisfied that this decision is compatible with its general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources.

## 4	The Facility

### 4.1 Description of the Facility and related issues

#### 4.1.1	The permitted activities

The Facility is subject to the EPR because it carries out an activity as described in Schedules 25A of the EPR:

One or more new medium combustion plant (MCP) between 1 and <50MWth but aggregated to <50MWth which will be put into operation after 20/12/2018 at a specific location.

#### 4.1.2	The Site

The Mondelez UK Confectionery Production Ltd factory at Station Avenue, in Chirk, Wrexham, North Wales, undertakes the first stages of cocoa bean processing or the production of chocolate and chocolate-based products.

An existing boiler house at the site contains three dual fuel natural gas/gasoil fired steam raising boiler plant in a shared boiler house, with independent hot gas ductwork terminating in a common stack. Boiler Number 2 has been deemed to be obsolete, having been in operation for more than 40 years. Mondelez propose to replace Boiler Number 2 with a new, low emission, high efficiency dual fuel natural gas/gasoil unit which will assume lead duty. The new plant will bring environmental improvements in terms of reduced emissions and substantial efficiency gains to the process. The two older boilers will remain on a stand-by basis only. The primary fuel will be natural gas (on an interruptible gas supply contract), with low-sulphur gasoil available as a standby fuel.

#### 4.1.3	What the Facility does

This application relates specifically to the installation of a proposed new steam raising boiler plant, to generate heat and steam for use in the production process, including pasteurisation and cocoa butter processing.

The new boiler will be known as ‘Chirk MCP Boiler No.2 (37/7602)’ (where 37/7602 is the boiler serial number) and will be supplied by Cochran; the application is submitted based on a design which encompasses the following:

- High efficiency (with economiser), and

- Low emissions (MCPD emission limit compliant, with ‘Low NOx’ burners).

The new unit will be a traditional dual fuel design, sized at seven (7) tonnes/hour steam

capacity. The primary fuel will continue to be natural gas (on an interruptible gas supply

contract), with low-sulphur gasoil available as a standby fuel. It is proposed to have a new dedicated 16 metre stack, exiting the boiler house on the opposite side of the boiler plant to the existing. The envisaged flue design is a plant mounted, twin wall stainless steel construction. The stack will be not be fitted with any cowls, rain guards or other such equipment which could impact dispersion. This new dual fuel (natural gas/gas oil) plant will assume lead duty, with natural gas the primary fuel (envisaged 100% of the time).

#### 4.1.4	Key Issues in the Determination

The key environmental and human health issues considered during the determination of this variation were:

- Air quality – Oxides of Nitrogen

This will be discussed separately in this decision document.

### 4.2 Operation of the Facility – general issues

#### 4.2.1	Administrative issues

The Applicant is the sole Operator of the Facility. We are satisfied that the Applicant is the person who will have control over the operation of the Facility if the Permit were to be granted; and that the Applicant will be able to operate the Facility so as to comply with the conditions included in the Permit, if issued.

#### Relevant Convictions

NRW’s COLINS Database has been checked to ensure that all relevant convictions have been declared.

No relevant convictions were found.

#### Financial Provision

There is no known reason to consider that the operator will not be financially able to comply with the permit.  The decision was taken in accordance with RGN 5 on Operator Competence.

#### 4.2.2	Management

We are satisfied that appropriate management systems and management structures will be in place for this Facility, and that sufficient resources are available to the Operator to ensure compliance with all the Permit conditions.

#### 4.2.3	Operating techniques

As stated above, the new boiler plant (MCP Boiler No.2 (37/7602)) will assume lead duty. It will run continuously with anestimated target annual down time of no greater than two weeks total. During periods of planned maintenance and inspection the existing boiler units will ensure continued supply of heat to the downstream processes, also utilising mains gas supply. Only in the event of an interruption to the mains gas supply to the site, would oil firing be utilised. Should such an interruption occur, the more efficient, newer plant covered by this permit would remain on lead duty.

The boiler will be operated in accordance with the instructions provided by Cochran and records will be retained to demonstrate this. A boiler log will be maintained via automated PLC1, recording hours of operation, gas use, feed water make-up and steam output. Records will be retained within the wider site management system (outlined in Section III Supporting Information, in response to question 3d3 Management systems). Any periods of abnormal operation will be recorded, and the measures taken to ensure compliance is restored as soon as practicable.

The boiler will be under a planned maintenance system and although under normal operations the plant will be unmanned, routine checks will be undertaken by staff on site. The appropriate staff will attend detailed training on the operation of the new plant and equipment. As appropriate, specialist contractors under the direction of Mondelez will be utilised for servicing and maintenance works. Further information on the site management system is provided in Section III Supporting Information, in response to question 3d3 Management systems.

## 5	Minimising the Facility’s environmental impact

For this kind of regulated activity, the principal emissions are emissions to air. There are no permit conditions for water, land, energy efficiency, odour or noise and BAT does not apply.

The next sections of this document explain how we have approached the critical issue of assessing the likely impact of air emissions from the MCP on human health and the environment and what measures we are requiring to ensure a high level of protection.

We have reviewed the operator's assessment of the environmental risk from the facility. The operator’s risk assessment is satisfactory. The assessment shows that, applying the conservative criteria in our guidance on Environmental Risk Assessment, all emissions may be categorised as environmentally insignificant.

We will discuss the operators risk assessment in more detail as follows:

### 5.1	Assessment of Impact on Air Quality

This section of the decision document deals primarily with the dispersion modelling of emissions to air from the stack and its impact on local air quality.

The Applicant has assessed the Installation’s potential emissions to air against the relevant air quality standards, and the potential impact upon human health.  These assessments predict the potential effects on local air quality from the Installation’s stack emission.

The air impact assessments, and the dispersion modelling has been based on the Installation operating continuously (8760 hours per annum) at the relevant long-term or short-term emission limit values, i.e. the maximum permitted emission rate.

We are in agreement with this approach.  The assumptions underpinning the model have been checked and are reasonably precautionary. The model outputs have been reviewed by Natural Resources Wales. The output from the model has then been used to inform further assessment of health impacts.

Initial investigations and a stack height assessment were undertaken to identify the appropriate stack height for new dedicated stack serving the new boiler. This concluded that a proposed stack height of 16 m above ground level would be taken forward for the detailed assessment to support the Environmental Permit application.

The detailed assessment indicates that the predicted NO2 concentrations at sensitive human receptors or at any location in the vicinity of the boiler do not exceed any relevant long-term or short-term EQS. The impact can be described as ‘insignificant’ for both long-term average and short-term average NO2 concentrations.

### 5.2	Impact on Habitats sites, SSSIs, non-statutory conservation sites etc

There are two SSSIs within 2km of the site:

- Chirk Castle & Parkland

- River Dee

As a result, an Appendix 4 form - CRoW Act 2000: Natural Resources Wales application for permission - Formal Notice, has been completed in conjunction with this application.

The key pollutants released from this new MCP are oxides of nitrogen (NOx as NO2). Detailed modelling has been undertaken by the applicant using the AERMOD dispersion model version 5.2.2 to predict the impacts associated with stack emissions from the plant at a 16m stack height level, on the two SSSI sites of ecological interest within 2 km, by comparing predicted ground level concentrations of NOx with relevant critical levels as well as nutrient nitrogen deposition loads. Manufacturer emission limits were assumed, and the plant was assumed to be operating continuously at full load for 8760 hours per year for long term impacts and all hours of the year for short term impacts.

The significance criteria provided by the EA states that for SACs and SSSIs the impact can be considered to be insignificant if the long-term PC (process contribution) is less than 1% of the annual average Critical Level and 10% of the 24-hour average critical level. The critical level for NO2 is 30μg/m3 annually and 75μg/m3 daily (24hours).

The results of the air quality modelling natural gas (see table below - note only the SSSIs are being assessed for this permission) firing indicate that the predicted NOx concentrations at ecological receptor SSSI sites are less than 1% of the annual mean EQS and less than 10% of the 24-hour mean EQS. Therefore, the impact at ecological receptors for normal operation using natural gas is insignificant.

In conclusion, the impact on airborne NOx concentrations, and acid and nitrogen deposition at the designated SSSI’s was determined to be insignificant.

### 5.3 European Sites

The following Natura 2000 sites are located within the relevant screening distance (10 km) of the main site emission point to air:

There are a total of three SACs and one SPA within 10km (relevant screening distance for this sort of facility) of the proposed stack location. These are:

- River Dee and Bala Lake / Afon Dyfrdwy a Llyn Tegid SAC

- Berwyn a Mynyddoedd De Clwyd / Berwyn and South Clwyd Mountains SAC

- Johnstown Newt Sites SAC

- Berwyn SPA

A FORM 1 has been completed with regards to a Habitats Regulations Assessment (HRA). This is required because there is a conceivable impact pathway to the Natura 2000 sites listed above.

Detailed air quality modelling has been carried out with regards to the potential impact from the facility. The maximum predicted ground level concentrations of NOx have been compared with relevant critical level thresholds above which damage may be sustained to sensitive plants and animals. For European sites within 10km, an assessment of deposition impacts has also been completed. Critical loads refer to the threshold beyond which deposition of pollutants to water or land results in measurable damage to vegetation and habitats. Predicted concentrations of NOx can be used to determine nutrient nitrogen deposition rates, using typical deposition velocities. The maximum predicted deposition rates have been identified with site specific critical loads obtained from APIS.

For ecological receptors, the detailed air quality modelling assessment indicates that the predicted NOx concentrations, and nitrogen and acid deposition, at European designated sites would be ‘insignificant’. Based on the available information, the potential in-combination effects with other planned developments at the European sites are not anticipated to lead to significant impacts. Based on the above assessment, it is concluded that the boiler is acceptable from an air quality perspective.

This project complies with NRW agreed criteria for ruling out significant effects without conducting a project-specific LSE test. Therefore, this project is considered not likely to have a significant effect on any Natura 2000 sites, either alone or in-combination with other plans and projects.

## Assessment of Likely Significant Effect:

The project has been screened for likelihood of significant effects and, taking account of the advice received from protected sites advisors, is considered not likely to have a significant effect on any Natura 2000/Ramsar site (As documented in section 3.2 of OGN 200 form 1, or section 5 if applicable).

## Appropriate assessment:

In light of the conclusions of an appropriate assessment, and taking account of the advice received from protected sites advisors, it has been established that the project will not adversely affect the integrity of any Natura 2000/Ramsar site, taking into account any conditions or restrictions as applicable, either alone or in-combination with other plans and projects.

## HRA Overall conclusion:

Long-term impacts and short term impacts on human receptors  are within specified criteria and therefore insignificant.

An assessment of the impact on nearby sensitive ecological sites has also been completed. The impact on airborne NOx concentrations and nutrient nitrogen deposition and acid deposition at the sensitive habitats was determined to be insignificant.

In light of the conclusions of the appropriate assessment, it has not been ascertained that the project will not adversely affect the integrity of any Natura 2000/Ramsar site, as documented in section 4 of OGN 200 form 1, and section 5 is applicable.

## 6  Setting ELVs and other Permit conditions

We have decided that emission limits should be set for the parameters listed in the permit. Emissions Limit Values (ELVs) are in line with those set out in the MCP Directive.

### 6.1	Monitoring

We have decided that monitoring should be carried out for the parameters listed in Schedule 3 of the permit using the methods and to the frequencies specified in those tables. These monitoring requirements have been imposed in order to demonstrate compliance with the emissions limits in the permit, as per the ELV and monitoring frequency requirements specified within the MCP Diective.

For a new Medium Combustion Plant, that is a boiler fuelled by natural gas, the monitoring requirements are as follows:

| Pollutant | Type of MCP | Fuel Type | Emission Limit Value (mg/Nm3) | Monitoring Required |
| --- | --- | --- | --- | --- |
| NOx | Other medium combustion plant (Boiler) | Natural Gas | 100 | Periodic – once within 4 months of permit issues, and then every 3 years |
| Carbon Monoxide | Other medium combustion plant (Boiler) | Natural Gas | No limit set | Periodic – once within 4 months of permit issues, and then every 3 years |

Monitoring requirements are defined at a temperature of 273.15 K, a pressure of 101.3 kPa and after correction for the water vapour content of the waste gases at a standardised O2 content of 15% for engines and gas turbines and 3% all other MCPs.

For emissions to air, the methods for continuous and periodic monitoring are in accordance with the Environment Agency’s Techical Guidance Note M5 for monitoring of stack gas emissions from medium combustion plants and specified generators.

Based on the information in the Application and the requirements set in the conditions of the permit we are satisfied that the monitoring techniques, personnel and equipment employed by the Operator will have either MCERTS certification or MCERTS accreditation as appropriate.

### 6.2	Other Permit Conditions

For a Medium Combustion Plant, the facility must adhere to the following operating techniques for MCP:

  - Each MCP must be operated in accordance with the manufacturer’s instructions and records must be made and retained to demonstrate this.

  - The operator must keep periods of start-up and shut-down of each MCP as short as possible.

  - There must be no persistent emission of ‘dark smoke’ as defined in section 3(1) of the Clean Air Act 1993.

  - The stack must be vertical and unimpeded by cowls or caps.

### 6.3	Reporting

We have specified the reporting requirements in Schedule 4 of the Permit to ensure data is reported to enable timely review by Natural Resources Wales to ensure compliance with permit conditions.

### 7 MCPD Charges and Subsistence Fees

The type of application regarding MCPD will have an associated charge. The MCPD application type will also form the basis for ongoing subsistence fee’s. More information on this can be found in our charging scheme on our website.