# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for Forest Wood Quarry operated by Aggregates Express Quarry Ltd.

The application number is PAN-016979

The permit number is EPR/CB3696CN

The permit is for the regulation of dewatering water (comprising a mixture of rainwater and site drainage) from a flooded quarry. The discharge will pass from the quarry to the quarry sump and will then be pumped through a series of settlement lagoons before begin discharged to a culver leading to Nant Rhydalog. Settlement in the lagoon is expected to achieve a suspended solids limit of 50 mg/l. The maximum daily discharge will be 15,120m3/day discharge, with a maximum discharge rate of 175 l/second. There was previously a discharge at the site permitted under AN0229002 which was surrendered in 2021.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

- Annex 2 the consultation responses

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. | ü |
| ### Responses to consultation and web publicising | The web publicising and consultation responses (Annex 2) were taken into account in the decision. No responses were received from web publicising. The decision was taken in accordance with our guidance. | ü |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. | ü |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. | ü |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. | ü |
| Biodiversity, Heritage, Landscape and Nature Conservation and a designated site under the Wildlife and Countryside Act. | The application is within the relevant distance criteria of a site of heritage, landscape or nature conservation, and/or protected species or habitat . A full assessment of the application and its potential to affect the site has been carried out as part of the permitting process. We consider that the application will not affect the features of the site. | ü |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site Note: From OGN 200 - no conceivable impact on any Natura 2000 site, by virtue of the scale or location or nature of the project. |  |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have carried out a risk assessment on behalf of the operator. The operator considers this risk assessment is satisfactory. The assessment shows that, applying the conservative criteria in our guidance on Environmental Risk Assessment, all emissions may be categorised as environmentally insignificant. | ü |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. Measures meet that as specified in TGN 7.01. The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN and we consider them to represent appropriate techniques for the facility. |  |
| The permit conditions |  |  |  |
| Emission limits | We have decided that emission limits should be set for the parameters listed in the permit. It is considered that the numeric limits described below will prevent significant deterioration of receiving waters. - Suspended solids (measured after drying at 105oC) – 50 mg/l - pH between 6 -9 We have imposed numeric limits because either a relevant environmental quality or operational standard requires this. The suspended solids limit has been set in line with current Water Quality Standards and to ensure good lagoon design/operation in accordance with NRW’s “Lagoon Guidance 2008”. The pH limit has been set to safeguard the receiving environment. | ü |
| Considerations of foul sewer | We agree with the operators justification for not connecting to foul sewer. | ü |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. | ü |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” | ü |
| Environment Wales Act 2016 |  |  |
| Biodiversity and resilience of ecosystems duty | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case, we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for a groundwater / stand alone water discharge activity permit under EPR2016. | ü |

# Annex 2: Consultation responses

Response received from

Rhondda Cynon Taff Local Council Ecologist

Brief summary of issues raised

The discharge is 280m upstream of the Rhydalog Local Wildlide Site (SINC). Site drainage if silty or otherwise could have detrimental impacts on the watercourse, ditch communities and wetland habitat within the SINC.

Summary of actions taken or show how this has been covered

There will be a limit of 50mg/l Suspended Solids in the permit. This is the standard limit to maintain Water Quality Standards and is tighter then the previous limit at the site of 100mg/l. This will help to reduce the amount of Suspended Solids discharged. There is also a condition in the permit that any substances not controlled by emission limits shall not cause pollution. As part of this condition it states that if the activities give rise to pollution the Operator will be required to submit and emissions management plan which will identify and minimise the risks of pollution from emissions of substances not controlled by emissions lilmits.

[