| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-018630 |
| Permit Reference Number: | EPR-AB3891CX |
| Permit Holder: | Mr Daniel & Mrs Carys James |
| Site Address: | Trewindsor Farm, Llangoedmor, Cardigan, Ceredigion, SA43 2LN. |
| Grid Reference: | Farm location: SN 22205 47060 Fields: 1 Trewindsor 1 (29/30) SN 21959 47209 2 Trewindsor 2 (31) SN 21891 47110 3 Trewindsor 3 (32) SN 21834 46975 4 Trewindsor 4 (50) SN 22221 46877 5 Trewindsor 5 (52) SN 22138 47403 6 Trewindsor 6 (53) SN 22242 47532 7 Trewindsor 7 (51) SN 21979 47600 8 Land at Tyriet 8 SN 17655 50758 9 Land at Tyriet 9 SN 17428 50710 Storage: SN 22048 46943 SN 21836 47476 SN 17956 50063 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: GwastraffaGorfodi.Canolbarth@cyfoethnaturiolcymru.gov.uk Environment team:CeredigionEnvironmentTeam@cyfoethnaturiolcymru.gov.uk NLO: Malcolm.Dines@cyfoethnaturiolcymru.gov.uk |
| Date due: | 02/08/2022 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating A- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | Welsh-PaLS |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and Charging Scheme page 52 |
| Fee on MyNRW | Y | Amount: £1018 Date cleared on MyNRW: 31/08/22 Transaction number: 2105145 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | Y |  |
| Site Specific Risk Assessment | Y | High risk location only |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | Y |  |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | Applicant provided: 55/226/0032 Mr Lynn Jones Fields T1, T2, T3, T4, T5, T6, T7, are matched to this but with the ownership associated to Tygwyn farm on MyMap 55/226/0027 Mr Morris Davies was also provided by the applicant for fields T8 and T9 which are associated with this however ownership on MyMap is given as HL & HM Lewis and son. |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y |  |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y |  |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y | Confirmed via RFi |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | Y | PAN-014268 |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | High | Within 500m of SSSI/SAC |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | Y |  |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | site‐specific risk assessment benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | n/a |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y | Evidence shows fields are of established agricultural use |
| - About the waste |  |  |
| Has all parts of the waste types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable of non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y | States all waste a low RAN, but 2 are high RAN- see RFI |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y | DMS and Google |
| - About the storage |  |  |
| Has all of waste storage table been completed? | Y |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| If land was previously deployed on, and waste is stored in temporary field heaps; are the storage locations in the same or different locations? | n/a | Land previously deployed upon, but no field heaps were used. |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y | Correctly buffered |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | n/a |  |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | n/a |  |
| location of all storage sites for the waste to be spread | Y |  |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | Y | Correctly buffered |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | n/a |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | Y | 423m from field 8 SSSI Aberath Carreg Wyland 371m from field 9 SSSI Cacau Crug Bychan. 423m away from SACs Cardigan Bay and West Wales marine |
| location of any public rights of way (shown on map) | Y | No issues |
| any buildings or houses within 250m (shown on map) | Y |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Nitrate Vulnerable Zone (NVZ) - Groundwater Safeguard Zone - Hedges - Ditches | Y | 45 areas of protected habitat, some appear in field 9, however inspection of EIA photos show that the fields are heavily cultivated and currently contain grass for silage. 1 SAM- crug bychan round barrow, no issue |
| What is the slope of the fields? | 0-7 degrees (transparent)- All fields 7 -12 degrees (yellow)- very small portions of field 9 12 degrees plus (red)- none |  |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the waste types applied for. |  |  |
| - Are there any special testing requirements? | Y | 02 05 02 pH, fats and oils, salmonella 02 07 02 pH and no high trajectory spread methods due to leaf scorch risk |
| - If yes; have they been tested for? | Y |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y | Field 1 is below 5.2 pH |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | Y |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | Y |  |
| - Is it only for operational purposes? | Y |  |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | n/a |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients supplied match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y |  |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | Y |  |
| - The nutrients supplied by the material? | Y |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | Y |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | Y |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | Y |  |
| - Pathways | Y |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | Y |  |
| - Measures that will minimise or mitigate the impact will be put in place | Y |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | Y |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | N |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | Y |
| No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site. Due to distance from spread area to SACs no credible impact pathway could be identified. (423m away from nearest spread area [field 8]), this in combination with CoGAP, low spread rates and low trajectory spread methods remove an impact pathway. Fields are mostly below or at target nutrients with majority of nutrients being applied CoGAP will be observed, giving buffer zones of 10m around watercourses. Spread rates are low and spread methods are low trajectory. Spread rates are also low. Given these factors. |  |

Comments

Consultation responses: Consultees requested clarification on if the applicant was spreading 1 waste per field, why were two waste codes assigned to field 6, 8 and 9. Clarified this via email that table 3 is used to indicate what waste may be spread in each field and the Penderyn waste stream is only suitable for field 6, 8 and 9 due to pH concerns but that only one waste stream will be used per field as the applicant has stated.

MyMap, location and slope assessment: MyMap assessment: PAN-018630 mymap assessment Location assessment: PAN-018630 RoW and slope assessment MyMap screening indicated habitat might be located in field 9 however on inspection of EIA evidence and Mymap satellite imaging showed fields is established agricultural use. Field was also previously spread under PAN-014268. Slope assessment: All fields aside from small portions of field 9 are relatively flat (0-7 degrees of slope). Field number: Small portions of Field 9 Slope of the land: 7-12 Soil type: Medium/clay Waste types applied: Liquid Cropping type: Grass silage Method of application: Dribble bar or Injection (requires RFI confirmation) Weather conditions before during and after spreading: Will only be spread in suitable conditions in accordance with CoGAP. Risk of runoff: Low. due to low trajectory/subsoil spreading methods, CoGAP and the low amounts of waste being spread.

CPH details: Applicant provided: 55/226/0032/ Mr Lynn Jones Fields T1, T2, T3, T4, T5, T6, T7, are matched to this but with the ownership associated to Tygwyn farm on MyMap 55/226/0028 Mr Morris Davies was also provided by applicant fields T8 and T9 are associated with this however ownership on MyMap is give and HL & HM Lewis and son. A question needs to be asked regarding field ownership though the CPHs are correct. RFI

Protection of habitats and ecosystems EIA evidence submitted by the applicant shows fields are of established agricultural use and do not contain semi natural habitat. Areas which were marked as potential protected habitat in field 9 were investigate and showed only grass silage growing.

Agricultural benefit calculations: Our calculations are a match with the applicants aside form rounding errors. Volac waste stream will cause an increase in phosphate on field 9 however this field is below target. All other fields will be delivered a maintenance amount of nutrients to keep fields at or around target indices. First milk and dairy partners waste streams are high RAN. Applicant has stated on LPD1 that they are not. Penderyn waste is low pH 4.35 however it may only be spread on fields 6, 7 and 8 and these fields have pHs high enough to buffer this low pH.

Request for information: SENT: - Missing payment - High RAN waste. - Spread methods/Odour management - Landownership - CPH number. Response 1: 31/08/22 1. Missing payment: Applicant confirmed payment had not been sent but had now been paid. 2. High RAN waste: Applicant confirmed the understood wastes were high RAN. 3. Spread methods/Odour management: Applicant challenged question. Question was clarified in second RFI sent 01/09/22. 4. Landownership: Applicant submitted documents confirming landownership and CPH numbers associated with those landowners 5. CPH number: as above Response 2: 06/09/22 Applicant provided definitive spread methods and a site-specific odour management plan. RFI closed

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 08/09/22 |
| ## Effective date | 08/09/22 |
| ## Permitting Officer Date | Ethan Banfield |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 08/09/2022 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 1142 |
| Sign off | 10 |
| Issuing |  |
| Total |  |