| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-018736 |
| Permit Reference Number: | EPR-BB3990ZX |
| Permit Holder: | Beacon Foods Limited |
| Site Address: | Garngaled Farm, Llanspyddid, Brecon, Powys, LD3 8PE |
| Grid Reference: | Farm location: SO 0095127879 Fields: 9408 Banky Field - SO 01931 28095 9995 5 Acre - SO 01992 27962 7785 Top Newton - SO 01770 27856 4984 Top Sheep - SO 01518 27842 4470 Top Field - SO 01431 27700 9466 Long Field - SO 00936 27667 2368 61/2 Acres Above The Lane -SO 01231 27677 0865 Holly Bush - SO 01076 27660 1079 Lane Field - SO 01098 27795 9977 Above House Field - SO 00990 27775 1515 Bottom Newton - SO 02130 28164 9060 Next to Long Field - SO 00901 27604 8459 Little Field - SO 00837 27594 7694 6 ½ Acres Across The Road - SO 00782 27659 6856 Potato Field - SO 00674 27566 5560 Cilwych - SO 00549 27582 6971 Bog Field - SO 00661 27725 Storage: N/A |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: GwastraffaGorfodi.Canolbarth@cyfoethnaturiolcymru.gov.uk Environment team: environmentteam.southpowys@cyfoethnaturiolcymru.gov.uk NLO: Liz.Park@cyfoethnaturiolcymru.gov.uk |
| Date due: | 10/08/2022 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating A- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | Welsh- DMS/PaLS, no variations |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and Charging Scheme page 52 |
| Fee on MyNRW | Y | Amount: £1018 Date cleared on MyNRW: 25/07/2022 Transaction number: 2104112 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | Y |  |
| Site Specific Risk Assessment | Y | High risk location only |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | CPH given in request for information: 52/058/0008 Mymap record all fields: 52/058/0008 DT & DSE DAVIES |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y |  |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y |  |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y |  |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | Y | Previous deployment PAN-014403 expired: 28/07/2022 |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | High |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | Y |  |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | site‐specific risk assessment |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | n/a |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y |  |
| - About the waste |  |  |
| Has all parts of the waste types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable of non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y |  |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y |  |
| - About the storage |  |  |
| Has all of waste storage table been completed? | n/a |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | n/a |  |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | n/a |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | n/a |  |
| If land was previously deployed on, and waste is stored in temporary field heaps; are the storage locations in the same or different locations? | n/a |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y |  |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | Y |  |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | n/a |  |
| location of all storage sites for the waste to be spread | n/a |  |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | n/a |  |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | n/a |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | Y |  |
| location of any public rights of way (shown on map) | Y |  |
| any buildings or houses within 250m (shown on map) | Y |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Nitrate Vulnerable Zone (NVZ) - Groundwater Safeguard Zone - Hedges - Ditches | N | See comments at the bottom of the decision document. |
| What is the slope of the fields? | 0-7 degrees (transparent)- fields 1515, 0865, 2368,4470 7 -12 degrees (yellow)- fields 5560, 6556, 6971, 7694,9977,1079,4964,7785, 9995, 12 degrees plus (red)- fields 9408 (partially) Areas of highly sloping land have been removed from the deployment and won’t be spread on. |  |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | N |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | N |  |
| - If yes; have they been tested for? | n/a |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | n/a |  |
| - Soil physical properties (e.g. organic matter and soil structure) | n/a |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | n/a |  |
| - Is it only for operational purposes? | n/a |  |
| - Have individual application rates for each waste been given? | n/a |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | n/a |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients supplied match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y |  |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | Y | Check previous deployment: PAN-014403 |
| - The nutrients supplied by the material? | N |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | Y |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | Y |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | Y |  |
| - Pathways | Y |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | Y |  |
| - Measures that will minimise or mitigate the impact will be put in place | Y |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | Y |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | N |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | Y |
| No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site. This conclusion was reached due to the following reasons: - The closest field (4984) is located 375 meters from the River Usk SAC. Due to the distance the SAC is from the proposed landspreading area there is no impact pathway as the distance from these two sites with act as a physical barrier preventing nutrients entering the SAC. There is also a main road & Llanspyddid village that runs near field 4984 before the SAC which will further buffer theoretical runoff into the drainage network. - 20m no spread buffer zones are in place in fields 5560 & 6971 which border a surface water stream. This no spread buffer zone is double the recommendation of the buffer zone stated in SR2010No4 and will effectively prevent nutrients entering the water course/ SAC. - The application rate of this waste stream (EWC 02 03 01) will be 50 Tonnes/ha spilt into two 25 tonnes/ ha dressings. This low application rate reduces the risk of surface run off and/or leachate risk as the ground is highly unlikely to become over saturated allowing the soil to effectivity absorb the waste applied. - The application method of this waste is appropriate by using low trajectory surface spreading in the spring. - Areas of highly sloping land (12o+) have been removed from the deployment application to reduce the risk of surface runoff and/or leachate. - The spreading of this waste will be undertaken in the spring/ early summer when weather conditions are dry. This means the soil will have a lower water content allowing the wastes to be effectively absorbed by the soil and therefore mitigating the risk of surface runoff and/or leachate. - All machinery to spread the waste is kept in mechanically sound condition, and calibrated. - The land spreading activity will comply with the Code of Good Agricultural Practice guidelines (COGAP). |  |

Comments

Consultation responses: Numerous comments were left on the comment record which include comments on spreading area near the River Usk SAC, Drinking Water Protection Zone & High Groundwater Vulnerability which were all resolved at determination. The officer was contacted with the decisions made at determination. There was no reply and this has been taken as no comments to add. The email is available to view on the DMS.

MyMap, location and slope assessment: MyMap assessment: - Field 6971 & 5560 border a stream. A 20 meter no spread zone is in place to prevent nutrients entering the stream. - Field 1515 is located 146 meters from a source protection zone 2 ‘Brecon’. - The spreading fields are located next to a SAC and SSSI the closest field 4984 is located 375 meters from SAC/SSSI (River Usk). - Fields 4984 7785 & 9408 border an ancient woodland the request for information has asked to buffer this area by 10 meters to prevent nutrients entering the site. - All fields are within the Beacon Beacons National Park and has been accounted for in the site specific risk assessment. Slope assessment: Field number: 5560, 6556, 6971, 7694,9977,1079,4964,7785 & 9995 Slope of the land: 7-12o Moderately sloping Soil type: Medium soils Wastes types applied: Liquid Cropping type: Grass Method of application: surface spreading via low trajectory splash plate. Weather conditions: Waste will be spread as a spring dressing in appropriate weather conditions and will not be spread on land that has been frozen for 12 hours or more, in the preceding 24 hours. The waste will also not be spread on land, which is waterlogged, frozen or snow covered, or during excessive rainfall as mentioned in the Agricultural benefit statement. The runoff risk for these fields have been downgraded from elevated to low. The applicant is now spreading the waste via low trajectory surface spreading in the spring and early summer. The runoff risk is therefore less as the soil will uptake the waste more effectively due to spring being more appropriate to spread wastes in terms of crop requirements and weather conditions. Areas of highly sloping land have been removed from the deployment and won’t be spread on. The application rate of the wastes is low at 25 T per ha per dressing (2 dressings) meaning runoff is highly unlikely. All other fields (1515, 0865, 2368 & 4470) are broadly flat to gently sloping (0-7o) and therefore do not pose a surface runoff risk.

CPH details: LPD1: 52/340/0013 Mymap record all fields: 52/058/0008 DT & DSE DAVIES

Protection of habitats and ecosystems - Field 1515 & 9408 contain a protected species: Threatened Lichen. No trees in field no action needed.

Agricultural benefit calculations: Vegetable wash waters do not appear in the latest version of RB209, so dirty water has been used as a model to calculate availability of phosphate and potassium to the crops. The phosphate additions added by this waste stream is using the 50% availability rate as outline in table 2.10 section 2 of RB209 for dirty water. The phosphate additions from this waste stream are low and are applying 8.53 kg of P per ha (available) or 17.06 kg per ha at the total phosphorus amount. This is below the crop offtake amount therefore, the phosphorus applied to the soil is likely to be accepted by the soil with little risk of phosphate leaching.

Request for information 1 31/08/2022 - Confirm that consent has be given for this proposed landspreading activity to occur for all fields/landowners under this deployment. The applicant has provided the signature of the landowner which matches the CPH record on Mymap. - Confirm the CPH number for all fields relevant to this deployment. The applicant has confirmed the CPH number to be 52 058 0008 which matches the Mymap record. - Confirm the name of the permit holder. The applicant has confirmed the correct permit holder on a querying email dated 01/09/22 available on the DMS. - Confirm how much material was spread under the previous deployment. The applicant has provided this value in the previous land treatment table. - Update the site plan to remove areas of highly sloping land from all fields from this deployment. The applicant has updated the site plan to remove highly sloping land from the deployment. - Update the ABS so that it explains the weather conditions that this proposed land spreading activity will occur on. The applicant has updated the ABS and states wastes will be spread in spring/early summer when weather conditions are appropriate /favourable. - Update the ABS to further justify the reasoning for an autumn dressing. The applicant has now decided to spread the waste in spring/early summer. - Confirm and update the ABS to show that the waste relevant for this deployment (EWC 02 03 01) will be applied by soil injection only. The applicant has decided to spread the waste in spring and not autumn. The runoff risk is therefore less as the soil will uptake the waste more effectively due to spring being more approperate to spread wastes in terms of crop requirements and weather conditions. The applicant will now also spread via low trajectory splash plate which is an approperate method to spread this waste as per TGN 8.01. - Update the site plan to show a 10 meter buffer zone around the perimeter of fields 4984, 7785 & 9408 that border the ancient woodland. The applicant has included the 10 meter no spread zone in the site plan. - Update the site plan to show the source protection zone 2 located 147 meters from the proposed land spreading activity. The applicant has acknowledged the source protection zone in the submitted site plan. - Update the site plan to clearly show the location of the river Usk SSSI & SAC (with a key) located 375 meters from the proposed land spreading activity. The applicant has acknowledged The River Usk SAC/SSSI in the submitted site plan. - Provide the soil analysis report for the fields listed above or remove these fields from the deployment application. The applicant has now provided all the soil analysis relevant to this deployment. - Retake the photographs for the soil analysis for the fields listed above so the P, K and Mg index as well as the date the analysis was undertaken can be clearly seen. The applicant has retaken the photographs to show all the information needed to assess each field. - Update the risk assessment document to refer to the ‘woodlands’ to ancient woodlands and refer to the 10 meter no spread zone that will be in place (action 9) to prevent nutrients entering the site. The applicant has updated the risk assessment. - Update the risk assessment so it mentions that all the fields in this deployment are located in the Brecon Beacons national park. The applicant has updated the risk assessment. - Provide the name for the waste stream referenced ‘247kgs/ha 22-3-13’. The applicant has clarified this waste stream is referring to artificial fertiliser. Request for Information 2 01/09/2022 - Provide photographic evidence of the EIA evidence and explain how all 17 fields in this deployment are agriculturally improved. The applicant has provided photographic evidence of all the fields relevant for this deployment and show they are agriculturally improved.

Other information to note:

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 15/09/2022 |
| ## Effective date | 15/09/2022 |
| ## Permitting Officer Date | Owen Clee 15/09/2022 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 15/09/2022 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 1,260 |
| Sign off | 10 |
| Issuing | 10 |
| Total | 1,340 |