| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-018834 |
| Permit Reference Number: | EPR-GP3792SK |
| Permit Holder: | Byproduct Recovery Limited |
| Site Address: | Abergelli Farm, Felindre, Swansea, SA5 7NN |
| Grid Reference: | Farm location: SN 65102 01649 Fields: 6201- SN 65620 01010 3907- SN 65390 02070 0415- SN 65040 02150 1163- SN 66016 00728 0173- SN 66120 00645 0455- SN 66030 00562 Storage: N/A |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: wasteregulation.swwales@cyfoethnaturiolcymru.gov.uk Environment team: swanseaenvironmentteam@cyfoethnaturiolcymru.gov.uk NLO: Ewan.Marshall-Harries@environment-agency.gov.uk |
| Date due: | 18/08/2022 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating B- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | English- NLO |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and Charging Scheme page 52 |
| Fee on MyNRW | Y | Amount:£798.00 Date cleared on MyNRW: 08/09/22 Transaction number: 2103916 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | Y |  |
| Site Specific Risk Assessment | n/a | High risk location only |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | Y |  |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | 59/159/0050 CPH number provided for fields: 0415 3907 6201 Owner WB & Same Llewllyn 55/002/0122 CPH number provided for fields: 1163 0455 0173 Owner HT & MM Thomas Provided by applicant and confirmed on MyMap. |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y |  |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y |  |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y | Consent provided in the supporting documents section |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | Y | PAN-014967 Live until 23/09/22 |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | Medium (1) |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | n/a |  |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | N |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y | Field 0415 was missing from EIA evidence- field was subsequently removed from application |
| - About the waste |  |  |
| Has all parts of the waste types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable of non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y |  |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y |  |
| - About the storage |  |  |
| Has all of waste storage table been completed? | N/a |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | n/a |  |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | n/a |  |
| If land was previously deployed on, and waste is stored in temporary field heaps; are the storage locations in the same or different locations? | n/a | No storage |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y |  |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | n/a |  |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | n/a |  |
| location of all storage sites for the waste to be spread | n/a | No storage |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | Y | 4 NALD deregulated abstractions are within 500ms of the application however they are over 250m away from spread area, no issues. |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | Y |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | n/a |  |
| location of any public rights of way (shown on map) | Y | No issues |
| any buildings or houses within 250m (shown on map) | Y | Farm buildings attached to the fields being spread. No issue |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Nitrate Vulnerable Zone (NVZ) - Groundwater Safeguard Zone - Hedges - Ditches | Y | 4 areas of woodland 37 areas of protected habitat 7 species 7 SINCs -Waun Garn Wen -Llety- morfil -Rhyd Y Pandy Valley and grassland -Rhos Fawr -lower lliw reservoir -Pant Lasau -middle llan No issues |
| What is the slope of the fields? | 0-7 degrees (transparent)- fields # all fields 7 -12 degrees (yellow)- Portion of field 1163. 12 degrees plus (red)- No fields- N/a |  |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | n/a | Not required for 19 09 02 or 19 06 06 |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y | 19 09 02 need to be tested for iron and aluminium 19 06 06- needs to be tested for plastic, glass and pathogens |
| - If yes; have they been tested for? | Y |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | Y |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y | Fields are at a high pH level (7.7-8.2) however this issue has been addressed in the agben. |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | Y | Waste may be spread in combination |
| - Is it only for operational purposes? | Y |  |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | Y |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients supplied match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y |  |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | Y |  |
| - The nutrients supplied by the material? | Y |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | n/a |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | n/a |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | n/a |  |
| - Pathways | n/a |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | n/a |  |
| - Measures that will minimise or mitigate the impact will be put in place | n/a |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | N |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | n/a |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | N |

Comments

Consultation responses: Consultee requested a 10m buffer on fields which border the SINC, assessing these fields against the spread methods and wastes this would seem reasonable. Consultee also noted that field 6201 this was not picked up in MyMap assessment so clarification has been requested of the consultee. However, where the SINCs border fields there are water courses or other physical barriers such as hedges and road which will be buffered as standard. An email reflecting this was sent to the consultee. Further consultation response confirmed they were satisfied with current mitigation in place- email is saved on the DMS.

MyMap, location and slope assessment: MyMap assessment: PAN-018834 Mymap assessment Location assessment: PAN-018834 RoW and slope map Slope assessment: Field number: central portion of 1163 Slope of the land: 7-12 degrees of slope Soil type: medium Wastes types applied (solid, liquid, sludge etc.): sludge and sludge cake Cropping type: grass silage Method of application: sludge with be applied using low trajectory splash plate, sludge cake will be applied using a rear discharge spreader. Weather conditions before during and after spreading: CoGAP will be observed, spreading will only take place in suitable conditions. Run off risk: low- the sloped area is surrounded by more flat areas, spread rates and methods should mitigate any run off risk.

CPH details: 59/159/0050 CPH number provide for fields: 0415, 3907, 6201. Owner WB & Same Llewllyn 55/002/0122 CPH number provide for fields:1163, 0455, 0173. Owner HT & MM Thomas Provided by applicant and confirmed on MyMap.

Protection of habitats and ecosystems: Evidence provided by applicant shows fields are of established agricultural use and contain no semi natural habitat, however field 0415 is missing from evidence provided.

Agricultural benefit calculations: All fields are below target indices expect for field 0455 which is below target for potash and at target for phosphate. Waste stream felindre sludge cake will provide a slight increase in phosphate, this addition is acceptable in line with guidance for RB209 for grass silage. Waste stream felindre liquid sludge provides no increase in phosphate, potash or magnesium but will provide a maintenance amount of phosphate at offtake. Anaerobic digestate will provide an increase in both phosphate and potash these increases are in line with RB209 guidance for grass silage. A concern with this waste stream is that its pH of 8.1 could raise the pH of the soils in the fields, these fields already have a high pH- pH being between 7.7 and 8.2. at higher pHs nutrients become less accessible to plants reducing the offtake of nutrients and giving a risk of nutrient leeching. However given that fields are already at this pH the low amounts that will be spread would indicate that in this case the benefits outweigh the risks.

Request for information: SENT: 16/09/22 1. Missing payment? (also checked with banking.) 2. Waste pH and field pH issues for anaerobic digestate. 3. EIA evidence field 0415 missing. 4. Previous deployment still live. First response: 23/09/22 Closed the RFI questions However 0415 field appear to contain semi natural habitat. Consulted with local conservation officers who confirmed that field did containing semi natural habitat. Second RFI sent 26/09/22: Advising applicant field 0415 needed to be removed or a screening opinion from Welsh Government needed to be applied for. Applicant responded on 30/09/22 confirming they had removed field 0415.

Other information to note:

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 04/10/22 |
| ## Effective date | 04/10/22 |
| ## Permitting Officer Date | Ethan Banfield 04/10/22 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 04/10/22 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 980 |
| Sign off | 10 |
| Issuing |  |
| Total |  |