# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for Sant Ffraed House operated by SNACO UK LTD.

The application number is PAN-016434

The permit number is EPR/CB3597HR

The discharge is for Sant Ffraed House and a new wedding venue with a maximum discharge rate of 20m3/d. The discharge is treated by a package treatment plant and additional sand filter bed, before discharging via a bespoke drainage field.  This groundwater discharge is replacing an existing permitted surface water discharge of 3.1m3/d (AN0165601).

The discharge is located within the phosphorus failing River Usk SAC catchment. Granting this permit will remove the current surface water discharge of 3.1m3/d discharging 10mg/l of phosphorus to Ffrwd Brook, a tributary of the River Usk, and replace it with a discharge of 0.05mg/l of phosphorus to the River Usk SAC catchment. The existing surface water permit (AN0165601) will be surrendered upon commission of the discharge to groundwater treatment system.

To ensure the bespoke drainage field functions as proposed, operator self-monitoring will be under taken for 12 months with samples taken bi-monthly and the results submitted to NRW.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. | ü |
| ### Responses to consultation | There where no responses received to the public consultation. The decision was taken in accordance with our guidance. | ü |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. | ü |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. | ü |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. | ü |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | In light of the conclusions of an appropriate assessment, and taking account of the advice received from protected sites advisors, it has been established that the project will not adversely affect the integrity of any Natura 2000/Ramsar site, taking into account any conditions or restrictions as applicable, either alone or in-combination with other plans and projects. (As documented in section 4 of OGN 200 form 1, and section 5 if applicable) Link to completed Form 1 and Form 2 |  |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have reviewed the operator's assessment of the environmental risk from the facility. The operator’s risk assessment is satisfactory. The assessment shows that, applying the conservative criteria in our guidance on Environmental Risk Assessment or similar methodology supplied by the operator and reviewed by ourselves, all emissions may be categorised as environmentally insignificant. | ü |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. Expectation is for an infiltration systems to meet British Standard BS 6297:2007+A1:2008. Where this is not possible it is for the applicant to evidence the system proposed is capable of achieving, at minimum, equivalent level of treatment of a British Standard system. The bespoke discharge field proposed has been assessed via appropriate recognised method for Ground Water Risk Assessment and deemed acceptable in achieving the required level treatment and safe guarding of the environment. The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN and we consider them to represent appropriate techniques for the facility. | ü |
| The permit conditions |  |  |  |
| Incorporating the application | We have specified that the applicant must operate the permit in accordance with descriptions in the application, including all additional information received as part of the determination process. These are specified in the Operating Techniques table in the permit. This ensures the bespoke drainage field is constructed as specified in the information supplied on 31/05/2022 as part repone to Schedule 5 request. | ü |
| Emission limits | We have decided that emission limits should be set for the parameters listed in the permit. It is considered that the numeric limits described below will prevent significant deterioration of receiving waters. We have imposed numeric limits because either a relevant environmental quality or operational standard requires this. - Ammoniacal nitrogen (as N) - 2 mg/l - ATU-BOD as O2 - 10 mg/l - Suspended solids (measured after drying at 105o C) - 15 mg/l - Total phosphorus as P - 6mg/l Limits set are for the protection of the water quality to ensure compliance with current WFD regulations, Habitats Regulations, The Groundwater (England and Wales) Regulations 2009 and the associated environmental water quality standards. | ü |
| Monitoring | We have decided that monitoring should be carried out for the parameters listed in the permit, using the methods and to the frequencies specified. Monitoring requirements are imposed to ensure the bespoke package treatment plant and drainage system operate to accordingly. We made these decisions in accordance with EPR 7.01 How to comply with your permit and The Groundwater (England and Wales) Regulations 2009. Based on the information in the application we are satisfied that the operator’s techniques, personnel and equipment have either MCERTS certification or MCERTS accreditation as appropriate. | ü |
| Reporting | We have specified reporting in the permit. We have ask for reporting to ensure the treatment system is working correctly after initial start up and to allow NRW to achieve proportionate regulation of the discharge. We made these decisions in accordance with EPR 7.01 How to comply with your permit and The Groundwater (England and Wales) Regulations 2009 | ü |
| Considerations of foul sewer | We agree with the operators justification for not connecting to foul sewer. | ü |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. | ü |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” | ü |
| Environment Wales Act 2016 |  |  |
| Biodiversity and resilience of ecosystems duty | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case, we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for a groundwater discharge activity permit under EPR2016. | ü |