| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-019641 |
| Permit Reference Number: | EPR-AB3891CX |
| Permit Holder: | Mr Daniel James and Mrs Carys James |
| Site Address: | Hafod Farm, Ferwig, Cardigan, Ceredigion, SA43 1PU |
| Grid Reference: | Farm location: SN 18094 50342 Fields: 5505 - SN 16551 51051 4885 - SN 16480 50858 Daries 2056 - SN 16203 50569 Tawyn 0834 - SN 16088 50345 Tawyn 5928 - SN 16598 50289 Tawyn 3370 - SN 16338 49708 Dun 2 - SN 16983 50759 Heolgwyddil 1 - SN 17767 49425 Heolgwyddil 2 - SN 17531 49496 Tawyn 3234 - SN 16327 50327 Storage: SN 17052 50301 SN 17791 49535 SN 17956 50061 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: GwastraffaGorfodi.Canolbarth@cyfoethnaturiolcymru.gov.uk Environment team: CeredigionEnvironmentTeam@cyfoethnaturiolcymru.gov.uk NLO: Malcolm.Dines@cyfoethnaturiolcymru.gov.uk |
| Date due: | 02/11/2022 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating A- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | Welsh- No variations |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and Charging Scheme page 52 |
| Fee on MyNRW | Y | Amount: £1,018.00 Date cleared on MyNRW: 17/10/2022 Transaction number: 2106773 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | Y |  |
| Site Specific Risk Assessment | Y | High risk location only |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | See comments at the bottom of the decision document. |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y |  |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y |  |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y |  |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | Y | Previous deployment: PAN-015154 Expires: 31/10/2022 |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | High |  |
| Is the risk banding correct | Y |  |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | Y |  |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | Site‐specific risk assessment |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | n/a |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y |  |
| - About the waste |  |  |
| Has all parts of the waste types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable of non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y |  |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y |  |
| - About the storage |  |  |
| Has all of waste storage table been completed? | Y |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| If land was previously deployed on, and waste is stored in temporary field heaps; are the storage locations in the same or different locations? | n/a |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y |  |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | n/a |  |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | Y |  |
| location of all storage sites for the waste to be spread | Y |  |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | Y |  |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | n/a |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | Y |  |
| location of any public rights of way (shown on map) | Y |  |
| any buildings or houses within 250m (shown on map) | Y |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Nitrate Vulnerable Zone (NVZ) - Groundwater Safeguard Zone - Hedges - Ditches | N | See comments at the bottom of the decision document. |
| What is the slope of the fields? | 0-7 degrees (transparent)- fields: 5505, 4885, Dun 2, 2056, 0834, 5928, 3370, Heol 1, Heol 2 7 -12 degrees (yellow)- field: 3324 12 degrees plus (red)- fields: n/a |  |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y | See comments at the bottom of the decision document. |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | n/a |  |
| - If yes; have they been tested for? | n/a |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | n/a |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | n/a |  |
| - Is it only for operational purposes? | n/a |  |
| - Have individual application rates for each waste been given? | n/a |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | n/a |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients supplied match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y | See previous deployment: PAN-015154 |
| - How much material was spread? | N |  |
| - At what application rate material was spread? | Y |  |
| - The nutrients supplied by the material? | N |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | Y |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | Y |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | Y |  |
| - Pathways | Y |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | Y |  |
| - Measures that will minimise or mitigate the impact will be put in place | Y |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | Y |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | N |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | Y |
| No FORM 1 completed: Cardigan Bay SAC and the West Wales Marine SAC - Field 0834 borders the two SACs and contains a 20 meter no spread buffer zone to prevent nutrients entering the SACs. - Field 2056 is located 8 meters from the two SACs and contains a 20 meter no spread buffer zone to prevent nutrients entering the SACs. - Field 5505 is located 19 meters + 10 meter no spread zone is in place in the field (total distance 29 meters from the SACs). This distance will mitigate the risk of nutrients entering the SACs. - The fields closest to the two SACs (2056 & 0834) are on flat to gently sloping land meaning the risk of runoff/leachate is low as the waste will be incorporated into the soil effectively - To mitigate the pollution risk to watercourses/ SACs the waste materials will not be applied at a rate greater than 50 t/ha in a single application. - Wastes will be spread with low trajectory trailing hose (dribble bar) applicator or be sub-surface injected, which are efficient methods to prevent odour transmission & nutrient losses. - Wastes will be spread at appropriate times when the crop will benefit from the nutrients provided i.e. spring/summer. - The wastes will be applied following the Code of Good Agricultural Practice. - Regular servicing of all machinery is conducted and spreading equipment is annually calibrated. To prevent waste being held in faulty machinery replacement spreading equipment will be available. - Spreading machinery will travel over the field in a direction which will most easily allow the machinery to turn within the boundaries of the field. Any spreading equipment will be turned off and/or lifted out of the soil prior to turning at the end of each run. - Machinery turns will be routed to avoid rutting and wheel slip. The turns will not be executed on any buffer strips. - There will be sufficient trained staff available to ensure that the operation continues throughout operational hours. - Weather conditions will be monitored prior to spreading with wind speed and direction. |  |

Comments

Consultation responses: No comments were raised on the form.

MyMap, location and slope assessment: MyMap assessment: Fields 4885 & Dun 2 are located next to a water issue. There is a 10 meter no spread buffer zone in place along the boundary of the fields and the water issues to prevent nutrients entering the water. No action needed. There is also a pond located in Dun 2 which has been buffered by 10 meters to prevent nutrients entering the water. No action needed. Field Heolgwyddil 1 is located next to a water issue. There is a 10 meter no spread buffer zone in place along the boundary of the field and the water issue to prevent nutrients entering the water. No action needed. Field 4885 is located near a spring. The field has been buffered by 50 meters in accordance with the SRP. No action needed. The proposed spreading fields are located near The Cardigan Bay SAC, The West Wales Marine SAC, The Aberarth Carreg Wylan SSSI and Afon Teifi SSSI. See the below table for further details. The no spread buffer zones in place will mitigate the risk of nutrients entering the SACs/ SSSIs. Field; No spread buffer zone in place in field (meters); Distance from SAC (meters); Distance from SSSI (meters). Field 0834; 20; Borders both SACs; Borders both SSSIs. Field 2056; 20; 8; 8. Field 5505; 10; 19; 19. Field 3370; n/a; 61; 61 150 (Afon Teifi SSSI). Slope assessment: Field number: Field 3324 Slope of the land: 7-12o Soil type: Medium Wastes types applied: Liquid Cropping type: Mazie Method of application: Dribble bar or shallow slot injector. Weather conditions before during and after spreading: The wastes will be applied in conditioned outlined in COGAP. Wastes will be spread on field in a split applications and will not exceed 50t/ha in any one application to a field. So the risk of runoff will be mitigated. The application method of applying these wastes is also appropriate which have a lower risk of runoff (dribble bar or deep leg injector). Therefore the runoff risk for this deployment has been determined as low. Field 3324 therefore requires no further action. The rest of the fields are flat to gently sloping and do not pose a runoff risk. No action needed.

CPH details: CPH given: 55/226/0027 Fields: 0834, 3324, 5928, Heol 1, Heol 2 & 3370 55/226/0027 Mrs J C Davies & Son Fields: 2056, 5505, 4885 & Dun 2 55/226/0030 LJ Jenkins & Partners

Protection of habitats and ecosystems Fields 2056 and 0834 are located next to two protected species: Atlantic Salmon & European Eel. The 20 meter no spread buffer zone mentioned above is in place to protect these species. Fields 3370 is located within a protected habitat: Lowland dry acid grassland and Lowland calcareous grassland. On further determination it was concluded the protected habitat was an historic site as grasslands are not present in this field. The field has contained maize and winter wheat within the last 3 years. See the request for information responses on the DMS for further details.

Agricultural benefit calculations: Field Tawyn 0834 & Tawyn 3324 has a soil index value below optimum levels for phosphorus (1) and will receive additional phosphorus from waste stream Volac to raise the soil index value (SIV) closer to optimum levels. The increase comply with RB209 Section 3, pg10 for grass silage with 3 cuts at the expected yield of 47t. (An additional 110 kg/ha of P for a SIV of 1). Because fields Tawyn 0834 & is below optimum levels, the available nutrients of phosphorus (60%) was used to calculate the amount of phosphate added to the soil instead of the total phosphorus value within the nutrient calculator for waste stream Volac .This follows Guidance stated in RB209 Section 2 page 12.

Request for information: 21/11/22 - Confirm that all the county parish holding (CPH) numbers have been provided for this deployment and consent from all landowners for all fields relevant for this deployment has been granted. The applicant has confirmed that all the relevant CPH numbers relevant to this deployment have been given (55/226/0027). - Update the site plan so it shows the protected habitat located within field 3370. The applicant required further details regarding on how to obtain more information on the protected habitats which was addressed to the applicant in the request for information 2. - Update the SSRA to include the protected habitat located in field 3370 in the source, pathway & receptor format. The applicant required further details regarding on how to obtain more information on the protected habitats which was addressed to applicant in the request for information 2. - Provide a site specific odour management plan for this deployment which includes details on how odour will be managed especially in field Tawyn 3370. The applicant has detailed sufficiently how odour will be managed during the deployment. The applicant has also confirmed that field 3370 will receive wastes via shallow slot injection to reduce the risk of odour to the nearby receptors. - Update the ABS to describe the SNS index for fields Tawyn 5928, Tawyn, 3370, Dun 2, Heolgwyddil 2 as either as low, moderate or high. The applicant has explained how the SNS index for forage crops is given as a numerical value and not low, moderate or high which applies for grass. No further action was required. Request for information 2: 24/11/22 - Update the site plan so that it shows the protected habitat located within field 3370. This Action has been retracted from the request for information as the protected habitat in question is an historic habitat. The field does not contain grassland as described on MyMap (Lowland dry acid grassland and Lowland calcareous grassland). The applicant has provided satellite imagery of field 3370 showing the field currently contains Winter Wheat where previous years the field contained maize. - Update the site specific risk assessment to include the protected habitat located in field 3370 in the source, pathway & receptor format. This Action has been retracted from the request for information as the protected habitat in question is an historic habitat. The field does not contain grassland as described on MyMap (Lowland dry acid grassland and Lowland calcareous grassland). The applicant has provided satellite imagery of field 3370 showing the field currently contains Winter Wheat where previous years the field contained maize.

Other information to note: The wastes analysis reports for waste streams Dairy Partners & First Milk are close to being 12 months old. The Date the application was received by NRW was 19/10/2022 and the dates the waste analysis were reported was the 22/10/2021. No action was needed.

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 28/11/22 |
| ## Effective date | 28/11/22 |
| ## Permitting Officer Date | Owen Clee 28/11/22 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 28/11/2022 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 1120 |
| Sign off | 10 |
| Issuing | 10 |
| Total | 1,200 |