| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-019450 |
| Permit Reference Number: | EPR-GP3792SK |
| Permit Holder: | ByProduct Recovery Ltd |
| Site Address: | Bwlchmawr Farm, Brynteg, Llanybydder, Carmarthenshire, SA40 9XA |
| Grid Reference: | Farm location: 248663,243281 Fields: 2 257994 243053 4 257600 242995 6 258118 242484 7 257997 242352 8 258274 242312 9 258528 242447 10 258399 242542 12 258084 242217 13 257388 242979 0273 252985 240821 1976 253197 240774 0656 253047 240610 2149 253208 240474 8690 252843 240910 Storage: 1 258038, 243175 2 258470, 242555 3 258092, 242138 4 253167, 240582 5 255230, 243346 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: wasteregulation.swwales@cyfoethnaturiolcymru.gov.uk Environment team: Carms.EnvironmentTeam@cyfoethnaturiolcymru.gov.uk NLO: Ewan.Marshall-Harries@environment-agency.gov.uk |
| Date due: | 13/10/2022 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating B- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | English- EA public register, live, variation took place 2015 on DMS. |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and Charging Scheme page 52 |
| Fee on MyNRW | Y | Amount: £798 Date cleared on MyNRW: 23/09/2022 Transaction number: 2105848 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | Y |  |
| Site Specific Risk Assessment | n/a | High risk location only |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | See comments at the bottom of the decision document |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y |  |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y |  |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y |  |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | Y | Previous deployment: PAN-014886 Expired: 23/09/2022 |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | Medium (1) |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | n/a |  |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | Benefit statement assessment |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | n/a |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y |  |
| - About the waste |  |  |
| Has all parts of the waste types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable of non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y |  |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y |  |
| - About the storage |  |  |
| Has all of waste storage table been completed? | Y |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| If land was previously deployed on, and waste is stored in temporary field heaps; are the storage locations in the same or different locations? | Different |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | n/a |  |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | n/a |  |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | n/a |  |
| location of all storage sites for the waste to be spread | Y |  |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | n/a |  |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | n/a |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | n/a |  |
| location of any public rights of way (shown on map) | Y |  |
| any buildings or houses within 250m (shown on map) | Y |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Nitrate Vulnerable Zone (NVZ) - Groundwater Safeguard Zone - Hedges - Ditches | N | See comments at the bottom of the decision document |
| What is the slope of the fields? | 0-7 degrees (transparent)- fields 2 & 12 7 -12 degrees (yellow)- Fields 8, 9, 12, 8690, 0273, 1976, 0656 and 2194 12 degrees plus (red)- Fields 9, 13 ,1976 0273, 2194 & 0656 contain small areas of highly sloping land These areas have no spread buffer zones in place from the submitted site plan no action needed. |  |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | n/a |  |
| - If yes; have they been tested for? | n/a |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | n/a | Not significant, check comments at bottom of decision document. |
| - Soil physical properties (e.g. organic matter and soil structure) | n/a |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | Y |  |
| - Is it only for operational purposes? | Y |  |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | Y |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients supplied match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y | See previous deployment: PAN-014886 |
| - How much material was spread? | N |  |
| - At what application rate material was spread? | Y |  |
| - The nutrients supplied by the material? | N |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | n/a |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | n/a |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | n/a |  |
| - Pathways | n/a |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | n/a |  |
| - Measures that will minimise or mitigate the impact will be put in place | n/a |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | N |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | n/a |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | N |

Comments

Consultation responses: No comments were left on the form.

MyMap, location and slope assessment: The Cefn Blaenau SSSI appears to be located 477m from the proposed land spreading area which would make this deployment become a high risk deployment. The submitted site plan includes a non-spreading area located in field 12 that makes the field distance from the SSSI over the 500m (See MyMap report). Therefore this deployment can be accepted as a medium risk deployment. Slope assessment: Field number: Fields 8, 9, 12, 8690, 0273, 1976, 0656 and 2194 Slope of the land: 7-12o Soil type: Medium Wastes types applied (solid, liquid, sludge etc.): liquid and solids Cropping type: Grass Method of application: EWC 02 01 06 will be spread via low trajectory splash plate, on a tractor and tanker. Abattoir wastes (EWC 02 02 04) will be shallow injected Weather conditions before during and after spreading: Wastes will be applied when ground and weather conditions are suitable, following CoGAP. For fields 8, 9, 12, 8690, 0273, 1976, 0656 and 2194 it will be requested for liquids to be spread via low trajectory methods to reduce the risk of surface runoff and or leachate. Fields 9, 13 ,1976 0273, 2194 & 0656 contain small areas of highly sloping land at 12o+ .These areas have no spread buffer zones in place from the submitted site plan no action needed.

CPH details: CPH Given in LPD1: 55/294/0014 & 55/099/0031 MyMap record: Fields 2-13 55/294/0014 DS Davies Fields 0273, 0665, 1976 and 2149: 55/055/0031 GJ & A Jones The CPH numbers match for fields 2-13 and are a close match for fields 0273, 0665, 1976 and 2149. The names of the land owners both match with the MyMap record.

Protection of habitats and ecosystems A protected habitat: Threatened Bryophyte is located in fields 2149, 0656 0273 and 1976. As waste is being applied to field and not hedges/trees it is not likely the protected species will be effected from this landspreading activity. No action required. A protected habitat: Purple moor grass and rush pastures is located in between fields 6, 7, 8 and 12. Along the border of fields 9 and 10 there is also a purple moor grass and rush pastures & Lowland dry acid grassland. As these habitats are located outside the spreading area no action was required.

Agricultural benefit calculations: Fields 2, 4 , 7, 8, 9, 10, 12, 13 have an soil index value (SIV) of 0 for phosphorus which is below optimum and will receive additional phosphorus from waste stream Llanybydder (sludge & liquid) Lairage & Bwlchmawr slurry to raise the soil index value closer to optimum levels. The increase comply with RB209 Section 3, pg10 for grass silage with 2 cuts at the expected yield of 38t. (An additional 125 kg/ha of P for a SIV of 0). Because fields 2, 4 , 7, 8, 9, 10, 12, 13 are below optimum levels, the available nutrients of phosphorus (50%) was used to calculate the amount of phosphate added to the soil instead of the total phosphorus value within the nutrient calculator for waste stream Llanybydder Sludge & liquid. This follows Guidance stated in RB209 Section 2 page 12. Waste streams Llanybydder sludge, Llanybydder liquid and Lairage have an minimal liming effect on the soils where the waste streams add 0.10, 0.07 & 0.37 h/a of liming respectively. These small increases will not significantly effect the current pH. of the fields which ranges from 5.4-6.9.

Request for information: 15/11/22 - Reduce the storage limit for the farm store storage to 1,250T. The applicant has confirmed only 1,250 T of non-stackable wastes will be stored at any one time, across all storage locations. - Confirm non-stackable wastes will not be stored in field heaps. The applicant has confirmed non-stackable wastes will be stored at the tower hill lagoon. - Confirm how non-stackable waste streams will be stored under this deployment. The applicant has confirmed non-stackable wastes will be stored at Tower hill lagoon. - Confirm if you wish to spread on fields 3327, 6A, 0566, 9784 and A. The applicant has confirmed they do not wish to spread on the listed fields above and has retracted them from the CPH forms. - Confirm the CPH number for fields 2, 4, 6, 7, 8, 9, 10, 11, 12 and 13. The applicant has provided the CPH number for the listed fields as 55/294/0014 and matches the Mymap record. - Update the site plan so the key and map show the correct letter to represent the storage locations listed above. The applicant has updated the site plan so the key matches with the map. - Locate field heap A to a new suitable location away from sloping land. The application has moved field heap A to a new suitable location away from sloping land. - Update the ABS to confirm non-stackable wastes will be applied via low trajectory methods to reduce the risk of surface runoff and/or leachate. The applicant has confirmed waste will be spread via low trajectory methods. - Confirm that field 2149 has been named incorrectly and update the site plan with the correct field reference or provide a soils analysis for field 2149. The applicant has confirmed field 2149 was a typo and has been amended to 2194 in the site plan. - Adjust the application rate of Llanybydder DAF cake to meet the crop requirement of phosphate for the grass for fields with a soil index 2 and above. The applicant has reduced the application rate for Llanybydder DAF cake to meet the crop requirements for fields with an soil index value 2 and above for phosphorous. - Recalculate the nutrients applied for potassium Bwlchmawr Farm slurry will provide the fields or provide your working on how the figure 163kg/ha was reached. The applicant was using the available potassium content and has now amended the ABS to show the total supplied which matches the nutrient calculator. - Update the ABS to state the application method of applying EWC 02 02 04 to the ground will be undertaken by soil injection only or remove this waste stream from the deployment application. The ABS incorrectly mentions 02 02 04 can be spread via dribble bar or shallow slot injector. Where TGN 8.01 states only soil injection can be used. It will be requested in the request for information 2 to remove the mention of the dribble bar from the ABS. - Update the ABS to confirm the best practises mentioned in TGN 8.01 page 63 will be followed throughout the duration of this deployment for EWC 02 02 04. The applicant has updated the ABS stating the best practises of TGN 8.01 will be followed throughout the duration of the deployment for EWC 02 0 02 04. - Provide a site specific odour management plan for EWC 02 02 04 for this deployment. The applicant has provided an odour management plan although within the document section 3.1.3 gives vague statements on how wastes will be applied to the fields. Table 4 also states stackable wastes will be spread via rear discharge spreader which is not appropriate for EWC 02 02 04. The request for information 2 was sent out to address these issues. - Update the LPD1 supplement form so that it states Bwlchmawr slurry is high in readily available nitrogen. The applicant has updated the form to show Bwlchmawr slurry is high in readily available nitrogen. Request for information 2 30/11/22 - Update the agricultural benefit statement (ABS) to state EWC 02 02 04 will only be applied via soil injection or remove this waste stream from the deployment application. The applicant has updated the ABS and now states 02 02 04 will be shallowed injected. - Update section 3.1.3 of the Odour management plan (OMP) to state the specific application methods that will be used to apply the wastes in this deployment to the fields. The applicant has updated the OMP to state the spreading methods will be the ones listed in the ABS. - Update table 4 of the OMP to state EWC 02 02 04 will only be applied to the fields in this deployment via soil injection only or remove EWC 02 02 04 from the deployment application. The applicant has updated the OMP to state EWC 02 02 04 will be soil injected.

Other information to note: Bwlchmawr slurry has a high readily available nitrogen content at 34%. The Benefit statement complies with the closed periods that apply to this waste stream from 15th Oct 31st Jan. See The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021.

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 05/12/2022 |
| ## Effective date | 05/12/2022 |
| ## Permitting Officer Date | Owen Clee 05/12/2022 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 05/12/2022 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 1,150 |
| Sign off | 10 |
| Issuing | 10 |
| Total | 1,230 |