Rest

Permit with introductory note

The Environmental Permitting (England & Wales) Regulations 2016

Vale of Glamorgan Council

Vale of Glamorgan Resource Recovery Facility

Atlantic Trading Estate

Barry

CF63 3RF

Permit number

EPR/CB3794FE

Vale of Glamorgan Resource Recovery Facility

Permit number EPR/CB3794FE

Introductory note

This introductory note does not form a part of the permit

The main features of the permit are as follows.

This is a Tier 3 bespoke permit for a resource recovery facility station taking hazardous and non-hazardous waste from household and commercial premises collected by the Vale of Glamorgan Council and the household waste recycling centre (HWRC), adjacent to the site.

Treatment operations are limited to manual and/or mechanical sorting and/or separation, baling and bulking up of non-hazardous waste only. All treatment is for the purpose of recovery or disposal.

The maximum quantity of waste accepted and processed on site shall not exceed 74,999 tonnes per year.

Permitted waste types are collected dry recyclables (paper, card, glass, metals, and plastics), adult hygiene products (AHPs), food waste, small waste electronic and electrical equipment (WEEE), textiles, unprocessed green waste, aerosols, batteries, and residual waste. Hazardous waste will be limited to WEEE and batteries only.

Relevant storage and treatment capacity thresholds have been included in the permit to reflect the legal waste facility operational limits; above which, the site would be operating as an Industrial Emissions Directive installation. (See Table S1.1 for relevant limits.)

This permit does not allow any point source emission into surface waters or groundwater except:

- clean, rainfall dependant drainage from areas of the site not used in connection with the storage and/or treatment of waste

All areas of the site used for the storage or treatment of wastes, as well as vehicle washdown, drain to foul sewer and are discharged under consent from the relevant water undertaker.

All waste stored on site will be stored on an impermeable surface.

All emissions from the activity which are not controlled via actual emission limits, are controlled through the ‘emission of substances not controlled by emission limits condition’ and all relevant process requirements set out in Tables S1.1 and S1.2.

The status log of the permit sets out the permitting history, including any changes to the permit reference number.

| Status log of the permit |  |  |
| --- | --- | --- |
| Description | Date | Comments |
| Application EPR/CB3794FE/A0001 | Duly made 01/04/22 | Application for resource recovery facility. |
| Additional information received in response to Schedule 5 Notice | 19/07/22 | Consisting of updated: - Discharge consent - Fire fighting water storage plans - Discharge approvement in principle - Noise source files - Noise source data - Schedule 5 action log - Non-technical summary - Environmental risk assessment - Operating techniques document - Fire Prevention and Mitigation Plan - Noise assessment - Odour management plan |
| Additional information received in response to a re-issued Schedule 5 notice and second Schedule 5 notice | 30/09/22 | Consisting of updated: - Non-technical summary - Fire Prevention and Mitigation Plan - Odour Management Plan - Pest management plan - Noise assessment - Fire detection system locations |
| Additional information | 03/10/22 | Consisting of updated Fire Prevention and Mitigation Plan and moisture detector specification |
| Additional information received in response to an informal request for information on 14/11/22 | 14/11/22 | Consisting of updated: - Fire Prevention and Mitigation Plan - Environmental Risk Assessment - Odour Management Plan |
| Permit determined EPR/CB3794FE | 12/12/22 | Permit issued to Vale of Glamorgan Council. |

End of introductory note

Permit

The Environmental Permitting (England and Wales) Regulations 2016

Permit number

EPR/CB3794FE

The Natural Resources Body for Wales (“Natural Resources Wales”) authorises, under regulation 13 of the Environmental Permitting (England and Wales) Regulations 2016

Vale of Glamorgan Council (“the operator”),

of

Vale of Glamorgan Council

Civic Offices

Holton Road

Barry

CF63 4RU

to operate waste operations at

Vale of Glamorgan Resource Recovery Facility

Atlantic Trading Estate

Barry

CF63 3RF

to the extent authorised by and subject to the conditions of this permit.

| Name | Date |
| --- | --- |
| Huw Davies | 12/12/2022 |

Authorised on behalf of Natural Resources Wales

Conditions

# Management

## General management

### The operator shall manage and operate the activities:

#### in accordance with a written management system that identifies and minimises risks of pollution, including those arising from operations, maintenance, accidents, incidents, non-conformances, closure and those drawn to the attention of the operator as a result of complaints; and

#### using sufficient competent persons and resources.

### Records demonstrating compliance with condition 1.1.1 shall be maintained.

### Any person having duties that are or may be affected by the matters set out in this permit shall have convenient access to a copy of it kept at or near the place where those duties are carried out.

### The operator shall comply with the requirements of an approved competence scheme.

## Avoidance, recovery and disposal of wastes produced by the activities

### The operator shall take appropriate measures to ensure that:

### the waste hierarchy referred to in Article 4 of the Waste Framework Directive is applied to the generation of waste by the activities; and

### any waste generated by the activities is treated in accordance with the waste hierarchy referred to in Article 4 of the Waste Framework Directive; and

### where disposal is necessary, this is undertaken in a manner which minimises its impact on the environment.

### The operator shall review and record at least every four years whether changes to those measures should be made and take any further appropriate measures identified by a review.

# 2	Operations

2.1	Permitted activities

### 2.1.1	The operator is only authorised to carry out the activities specified in schedule 1 table S1.1 (the “activities”).

## 2.2	The site

2.2.1	The activities shall not extend beyond the site, being the land shown edged in green on the site plan at schedule 7 to this permit.

## 2.3	Operating techniques

#### 2.3.1	(a)	The activities shall, subject to the conditions of this permit, be operated using the techniques and in the manner described in the documentation specified in schedule 1, table S1.2, unless otherwise agreed in writing by Natural Resources Wales.

#### (b)	If notified by Natural Resources Wales that the activities are giving rise to pollution, the operator shall submit to Natural Resources Wales for approval within the period specified, a revision of any plan or other documentation (“plan”) specified in schedule 1, table S1.2 or otherwise required under this permit which identifies and minimises the risks of pollution relevant to that plan, and shall implement the approved revised plan in place of the original from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

### 2.3.2	Waste shall only be accepted if:

### (a)	it is of a type and quantity listed in schedule 2 table S2.1; and

### (b)	it conforms to the description in the documentation supplied by the producer and holder.

## 2.4	Technical requirements

WEEE treatment

### 2.4.1	The storage (including temporary storage) and treatment of WEEE shall be carried out in accordance with the technical requirements of Annex VIII of the WEEE Directive.

Hazardous waste storage and treatment

### 2.4.2	Hazardous waste shall not be mixed, either with a different category of hazardous waste or with other waste, substances or materials, unless it is authorised by schedule 1 table S1.1 and appropriate measures are taken.

## 2.5	Pre-operational conditions

2.5.1	The activities shall not be brought into operation until the measures specified in schedule 1 table S1.3 have been completed.

# 3. Emissions and monitoring

## Emissions of substances not controlled by emission limits

### Emissions of substances not controlled by emission limits (excluding odour) shall not cause pollution. The operator shall not be taken to have breached this condition if appropriate measures, including, but not limited to, those specified in any approved emissions management plan, have been taken to prevent or where that is not practicable, to minimise, those emissions.

3.1.2	The operator shall:

#### (a)	if notified by Natural Resources Wales that the activities are giving rise to pollution, submit to Natural Resources Wales for approval within the period specified, an emissions management plan which identifies and minimises the risks of pollution from emissions of substances not controlled by emission limits;

#### (b)	implement the approved emissions management plan, from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

## 3.2	Odour

#### 3.2.1	Emissions from the activities shall be free from odour at levels likely to cause pollution outside the site, as perceived by an authorised officer of Natural Resources Wales, unless the operator has used appropriate measures, including, but not limited to, those specified in any approved odour management plan, to prevent or where that is not practicable to minimise the odour.

## 3.3	Noise and vibration

#### 3.3.1	Emissions from the activities shall be free from noise and vibration at levels likely to cause pollution outside the site, as perceived by an authorised officer of Natural Resources Wales, unless the operator has used appropriate measures, including, but not limited to, those specified in any approved noise and vibration management plan to prevent or where that is not practicable to minimise the noise and vibration.

3.3.2	The operator shall:

#### (a)	if notified by Natural Resources Wales that the activities are giving rise to pollution outside the site due to noise and vibration, submit to Natural Resources Wales for approval within the period specified, a noise and vibration management plan which identifies and minimises the risks of pollution from noise and vibration;

#### (b)	implement the approved noise and vibration management plan, from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

## 3.4	Pests

3.4.1	The activities shall not give rise to the presence of pests which are likely to cause pollution, hazard or annoyance outside the boundary of the site. The operator shall not be taken to have breached this condition if appropriate measures, including, but not limited to, those specified in any approved pests management plan, have been taken to prevent or where that is not practicable, to minimise the presence of pests on the site.

## 3.5	Fire

3.5.1	The operator shall manage and operate the activities in accordance with a written fire prevention plan using the current, relevant fire prevention and mitigation plan guidance.

3.5.2	The operator shall:

(a)	if notified by Natural Resources Wales that the activities could cause a fire risk, submit to Natural Resources Wales a fire prevention and mitigation plan which identifies and minimises the risks of fire;

(b)	Operate the activity in accordance with the fire prevention and mitigation plan, from the date of submission, unless otherwise agreed in writing by Natural Resources Wales.

# 4	Information

## 4.1	Records

### All records required to be made by this permit shall:

### be legible;

### be made as soon as reasonably practicable;

### if amended, be amended in such a way that the original and any subsequent amendments remain legible, or are capable of retrieval; and

### be retained, unless otherwise agreed in writing by Natural Resources Wales, for at least 6 years from the date when the records were made, or in the case of the following records until permit surrender:

- off-site environmental effects; and

- matters which affect the condition of the land and groundwater.

4.1.2	The operator shall keep on site all records, plans and the management system required to be maintained by this permit, unless otherwise agreed in writing by Natural Resources Wales.

## 4.2	Reporting

4.2.1	The operator shall send all reports and notifications required by the permit to Natural Resources Wales using the contact details supplied in writing by Natural Resources Wales.

### 4.2.2	Within one month of the end of each quarter, the operator shall submit to Natural Resources Wales using the form made available for the purpose, the information specified on the form relating to the site and the waste accepted and removed from it during the previous quarter.

## 4.3	Notifications

### 4.3.1	Natural Resources Wales shall be notified without delay following the detection of:

### any malfunction, breakdown or failure of equipment or techniques, accident, or emission of a substance not controlled by an emission limit which has caused, is causing or may cause significant pollution;

### the breach of a limit specified in the permit; or

### any significant adverse environmental effects.

### 4.3.2	Any information provided under condition 4.3.1 shall be confirmed by sending the information listed in schedule 5 to this permit within the time period specified in that schedule.

### 4.3.3	Where Natural Resources Wales has requested in writing that it shall be notified when the operator is to undertake monitoring and/or spot sampling, the operator shall inform Natural Resources Wales when the relevant monitoring and/or spot sampling is to take place. The operator shall provide this information to Natural Resources Wales at least 14 days before the date the monitoring is to be undertaken.

### 4.3.4	Natural Resources Wales shall be notified within 14 days of the occurrence of the following matters, except where such disclosure is prohibited by Stock Exchange rules:

Where the operator is a registered company:

#### any change in the operator’s trading name, registered name or registered office address; and

#### (b)	any steps taken with a view to the operator going into administration, entering into a company voluntary arrangement or being wound up.

Where the operator is a corporate body other than a registered company:

#### any change in the operator’s name or address; and

#### any steps taken with a view to the dissolution of the operator.

#### In any other case:

#### the death of any of the named operators (where the operator consists of more than one named individual);

#### any change in the operator’s name(s) or address(es); and

#### any steps taken with a view to the operator, or any one of them, going into bankruptcy, entering into a composition or arrangement with creditors, or, in the case of them being in a partnership, dissolving the partnership.

### 4.3.5	Where the operator proposes to make a change in the nature or functioning, or an extension of the activities, which may have consequences for the environment and the change is not otherwise the subject of an application for approval under the Regulations or this permit:

### Natural Resources Wales shall be notified at least 14 days before making the change; and

### the notification shall contain a description of the proposed change in operation.

## 4.4	Interpretation

### 4.4.1	In this permit the expressions listed in schedule 6 shall have the meaning given in that schedule.

### 4.4.2	In this permit references to reports and notifications mean written reports and notifications, except where reference is made to notification being made “without delay”, in which case it may be provided by telephone.

Schedule 1 - Operations

| ### Table S1.1 activities |  |  |
| --- | --- | --- |
| ### Activity reference | ### Description of activities for waste operations | ### Limits of activities |
| ### A1 – household, commercial and industrial waste transfer station with treatment | R3: recycling/reclamation of organic substances which are not used as solvents R4: recycling/reclamation of metals and metal compounds R5: recycling/reclamation of other inorganic materials R13: storage of wastes pending any of the operations numbered R1 to R12 (excluding temporary storage, pending collection, on the site where it is produced) D9: Physico-chemical treatment not specified elsewhere in this Annex which results in final compounds or mixtures which are discarded by means of any of the operations numbered D1 to D12 (e.g. evaporation, drying, calcination, etc.) D14: repackaging prior to submission to any of the operations numbered D1 to D13 D15: storage pending any of the operations numbered D1 to D14 (excluding temporary storage, pending collection, on the site where it is produced) | All hazardous and non-hazardous waste must be stored and treated on an impermeable surface with sealed drainage. Treatment operations shall be limited to manual and/or mechanical: - sorting and/or separating - baling - bulking of permitted waste for the purpose of recovery or disposal. Sorting and/or separating and baling must take place within a building equipped with suitable containment measures. There shall be no treatment of: - waste electronic and electrical equipment - waste batteries and/or accumulators - hazardous waste other than bulking up for onward transfer. The maximum quantity of hazardous waste that can be stored at the site in total for recovery or disposal, shall not exceed 50 tonnes at any one time. Waste types as specified in Table S2.1. Table continued overleaf |

|  | Discharge of rainfall dependent run-off from the emission points indicated on “46821_2002_008 – DETAILED DRAINAGE LAYOUT – SHEET 1OF2 – Rev C1’ and ‘46821_2002_009 – DETAILED DRAINAGE LAYOUT – SHEET 2OF2 – Rev C1’ as incorporated in Table S1.2 of this permit. | Clean surface water from roofs, or from areas of the site that are not being used in connection with storing and treating waste, may be discharged via an interceptor to surface water. All other drainage from areas of the site used in connection with the storage and treatment of waste must be discharged to foul sewer subject to a consent issued by the relevant sewage undertaker. No visible oil, grease or suspended solids shall be present in the discharge to surface water drains. |
| --- | --- | --- |

| ### Table S1.2 Operating techniques |  |  |
| --- | --- | --- |
| ### Description | ### Parts | ### Date Received |
| ### Technical Guidance Document: ‘How to comply with your environmental permit’ | ### All relevant sections | ### N/A |
| ### Sector Guidance Note IPPC S5.06: Guidance for the Recovery and Disposal of Hazardous and Non Hazardous Waste | ### All relevant sections | ### N/A |
| ### Fire prevention and mitigation plan guidance - Waste Management | ### All relevant sections | ### N/A |
| ### Application | ### Drainage layouts. Document references: ‘46821_2002_008 – DETAILED DRAINAGE LAYOUT – SHEET 1OF2 – Rev C1’ and ‘46821_2002_009 – DETAILED DRAINAGE LAYOUT – SHEET 2OF2 – Rev C1’. Dated 04th February 2021. ### Dust management plan. Document reference: ‘VoG.015 Dust Management Plan’. Dated July 2021. | ### 11/10/21 |
| ### Response to Schedule 5 Notice dated 05/07/22 | ### Operating techniques document. Document reference: VoG.007 Operating Techniques Document v2.0. Dated July 2021. ### Sections: ### 3.3 ‘Waste Pre-Acceptance’ ### 3.4 ‘Waste Acceptance’. ### 3.5 ‘Waste Storage’ ### 4.1.3 ‘Containment Bunding’ ### 4.5 ‘Noise’ ### 4.7 ‘Litter’ | ### 19/07/22 |
| ### Response to Schedule 5 Notice (Re-issue) and Second Schedule 5 Notice dated 13/09/22 | ### Pest Management Plan. Document reference: ‘VoG.016 Pest Management Plan_V1.0’. Dated September 2022. | ### 30/09/22 |
| ### Response to email dated 27/10/2022 | ### Odour management plan. Document reference: VoG.014 Odour Management Plan_v5.0. Dated July 2021. ### Fire Prevention and Mitigation Plan. Document reference: VoG.009 FPMP – VoG Resource Recovery Facility v5.0. Dated November 2022. ### Noise Impact Assessment. Document reference: ‘VoG.013 Noise Assessment’. Dated September 2022. ### Section 7.1 ‘Embedded Mitigation’. | ### 14/11/22 |

| ### Table S1.3 Pre-operational measures |  |
| --- | --- |
| ### Reference | ### Pre-operational measures |
| ### PO1 | ### Completion of all four acoustic barriers, referenced in table 11, section 7.1 of the Noise Impact Assessment (VoG.913 Noise Assessment) for inspection and approval by an authorised officer on behalf of Natural Resources Wales. |

# Schedule 2 - Waste types, raw materials and fuels

| Table S2.1 Permitted waste types and quantities for waste transfer station |  |
| --- | --- |
| Maximum quantity | The maximum quantity of waste to be accepted on site shall not exceed 74,999 tonnes per year. |
| Exclusions | Notwithstanding the waste types set out in this table, wastes having any of the following characteristics shall not be accepted: - consisting solely or mainly of dusts, powders or loose fibres - sludges - odorous or odour producing |
| Waste code | Description |
| 15 | WASTE PACKAGING, ABSORBENTS, WIPING CLOTHS, FILTER MATERIALS AND PROTECTIVE CLOTHING NOT OTHERWISE SPECIFIED |
| 15 01 | packaging (including separately collected municipal packaging waste) |
| 15 01 01 | paper and cardboard packaging |
| 15 01 02 | plastic packaging |
| 15 01 04 | metallic packaging |
| 15 01 05 | composite packaging |
| 15 01 06 | mixed packaging |
| 15 01 07 | glass packaging |
| 15 02 | absorbents, filter materials, wiping cloths and protective clothing |
| 15 02 03 | absorbents, filter materials, wiping cloths and protective clothing other than those mentioned in 15 02 02 |
| 16 | WASTES NOT OTHERWISE SPECIFIED IN THE LIST |
| 16 02 | wastes from electrical and electronic equipment |
| 16 02 13* | discarded equipment containing hazardous components other than those mentioned in 16 02 09 to 16 02 12 |
| 16 02 14 | discarded equipment other than those mentioned in 16 02 09 to 16 02 13 |
| 16 02 16 | components removed from discarded equipment other than those mentioned in 16 02 15 |
| 16 06 | batteries and accumulators |
| 16 06 02* | Ni-Cd batteries |
| 16 06 03* | mercury-containing batteries |
| 16 06 04 | alkaline batteries (except 16 06 03) |
| 16 06 05 | other batteries and accumulators |
| 19 12 | wastes from the mechanical treatment of waste (for example sorting, crushing, compacting, pelletising) not otherwise specified |
| 19 12 05 | glass |
| 20 | MUNICIPAL WASTES (HOUSEHOLD WASTE AND SIMILAR COMMERCIAL, INDUSTRIAL AND INSTITUTIONAL WASTES) INCLUDING SEPARATELY COLLECTED FRACTIONS |
| 20 01 | separately collected fractions (except 15 01) |
| 20 01 01 | paper and cardboard |
| 20 01 02 | glass |
| 20 01 08 | biodegradable kitchen and canteen waste |
| 20 01 10 | clothes |
| 20 01 11 | textiles |
| 20 01 34 | batteries and accumulators other than those mentioned in 20 01 33 |
| 20 01 36 | discarded electrical and electronic equipment other than those mentioned in 20 01 21, 20 01 23 and 20 01 35 |
| 20 01 39 | plastics |
| 20 01 40 | metals |
| 20 01 99 | other fractions not otherwise specified, consisting solely of separately collected fractions of municipal wastes (comprising of adult hygiene products (AHPs) and nappies only) |
| 20 02 | garden and park wastes (including cemetery waste) |
| 20 02 01 | biodegradable waste |
| 20 03 | other municipal wastes |
| 20 03 01 | mixed municipal waste |
| 20 03 03 | street-cleaning residues |
| 20 03 07 | bulky waste |

# Schedule 3 – Emissions and monitoring

#### There are no emission limits or associated monitoring requirements.

# Schedule 4 - Reporting

#### There is no reporting under this schedule.

# Schedule 5 - Notification

### These pages outline the information that the operator must provide.

### Units of measurement used in information supplied under Part A and B requirements shall be appropriate to the circumstances of the emission. Where appropriate, a comparison should be made of actual emissions and authorised emission limits.

#### If any information is considered commercially confidential, it should be separated from non-confidential information, supplied on a separate sheet and accompanied by an application for commercial confidentiality under the provisions of the EP Regulations.

## Part A

| ## Permit Number |  |
| --- | --- |
| ## Name of operator |  |
| ## Location of Facility |  |
| ## Time and date of the detection |  |

| ## (a) Notification requirements for any malfunction, breakdown or failure of equipment or techniques, accident, or emission of a substance not controlled by an emission limit which has caused, is causing or may cause significant pollution |  |
| --- | --- |
| ## To be notified within 24 hours of detection |  |
| ## Date and time of the event |  |
| ## Reference or description of the location of the event |  |
| ## Description of where any release into the environment took place |  |
| ## Substances(s) potentially released |  |
| ## Best estimate of the quantity or rate of release of substances |  |
| ## Measures taken, or intended to be taken, to stop any emission |  |
| ## Description of the failure or accident. |  |

| ## (b) Notification requirements for the breach of a limit |  |
| --- | --- |
| ## To be notified within 24 hours of detection unless otherwise specified below |  |
| ## Emission point reference/ source |  |
| ## Parameter(s) |  |
| ## Limit |  |
| ## Measured value and uncertainty |  |
| ## Date and time of monitoring |  |
| ## Measures taken, or intended to be taken, to stop the emission |  |

| ### Time periods for notification following detection of a breach of a limit |  |
| --- | --- |
| ### Parameter | ### Notification period |

| ## (c) Notification requirements for the detection of any significant adverse environmental effect |  |
| --- | --- |
| ## To be notified within 24 hours of detection |  |
| ## Description of where the effect on the environment was detected |  |
| ## Substances(s) detected |  |
| ## Concentrations of substances detected |  |
| ## Date of monitoring/sampling |  |

## Part B - to be submitted as soon as practicable

| ### Any more accurate information on the matters for notification under Part A. |  |
| --- | --- |
| ### Measures taken, or intended to be taken, to prevent a recurrence of the incident |  |
| ### Measures taken, or intended to be taken, to rectify, limit or prevent any pollution of the environment which has been or may be caused by the emission |  |
| ### The dates of any unauthorised emissions from the facility in the preceding 24 months. |  |

| ### Name* |  |
| --- | --- |
| ### Post |  |
| ### Signature |  |
| ### Date |  |

### * authorised to sign on behalf of the operator

Schedule 6 - Interpretation

“accident” means an accident that may result in pollution.

“Annex I” means Annex I to Directive 2008/98/EC of the European Parliament and of the Council on waste.

“Annex II” means Annex II to Directive 2008/98/EC of the European Parliament and of the Council on waste.

“application” means the application for this permit, together with any additional information supplied by the operator as part of the application and any response to a notice served under Schedule 5 to the EP Regulations.

“authorised officer” means any person authorised by Natural Resources Wales under section 108(1) of The Environment Act 1995 to exercise, in accordance with the terms of any such authorisation, any power specified in section 108(4) of that Act.

“building” means a construction that has the objective of providing sheltering cover and minimising emissions of noise, particulate matter, odour and litter.

“D” means a disposal operation provided for in Annex I to Directive 2008/98/EC of the European Parliament and of the Council on waste.

“emissions to land” includes emissions to groundwater.

“emissions of substances not controlled by emission limits” means emissions of substances to air, water or land from the activities, either from the emission points specified in schedule 3 or from other localised or diffuse sources, which are not controlled by an emission or background concentration limit.

“EP Regulations” means The Environmental Permitting (England and Wales) Regulations SI 2016 No.1154 and words and expressions used in this permit which are also used in the Regulations have the same meanings as in those Regulations.

“groundwater” means all water, which is below the surface of the ground in the saturation zone and in direct contact with the ground or subsoil.

“hazardous substance” means a substance classified as hazardous as a consequence of fulfilling the criteria laid down in parts 2 to 5 of Annex I to Regulation (EC) No 1272/2008.

“hazardous waste” has the meaning given in the Hazardous Waste (Wales) Regulations 2005 (as amended).

“impermeable surface” means a surface or pavement constructed and maintained to a standard sufficient to prevent the transmission of liquids beyond the surface.“Pests” means Birds, Vermin and Insects.

“quarter” means a calendar year quarter commencing on 1 January, 1 April, 1 July or 1 October.

“R” means a recovery operation provided for in Annex II to Directive 2008/98/EC of the European Parliament and of the Council on waste.

“Waste code” means the six digit code referable to a type of waste in accordance with the list of wastes established by Commission Decision 2000/532/EC as amended from time to time (the ‘List of Wastes Decision’) and in relation to hazardous waste, includes the asterisk.

“Waste Framework Directive” or “WFD” means Waste Framework Directive 2008/98/EC of the European Parliament and of the Council on waste.

“WEEE” means waste electrical and electronic equipment.

“WEEE Directive” means Directive 2012/19/EU of the European Parliament and of the Council of 4th July 2012 on waste electrical and electronic equipment (WEEE).

“year” means calendar year ending 31 December.

Schedule 7 - Site plan

END OF PERMIT