Bespoke Permit for Water Discharge

|  | Water Management Plan |
| --- | --- |
| Date | 18/10/2021 |

Revision	 B	13/12/2021

Revision C	10/03/2022

Revision D	13/04/2022

Revision E	27/07/2022

|  | Prepared |
| --- | --- |
| Author | Elliot Hutchings – Environmental Advisor |
| Signature |  |
| Checker Approver |  |
| Signature |  |

REVISION TABLE

General update – all pages have been revised.

TABLE OF CONTENTS

1	Preliminaries	5

1.1	Purpose	5

1.2	Scope	5

1.3	Definitions & abbreviations	5

1.4	Responsibilities	6

2	Compliance obligations	7

2.1	Environmental Permitting Regulations 2016	7

3	background and controls	7

3.1	Background	7

3.2	Procedural Control Measures (Surface water runoff)	7

4	Pollution mitigation	10

4.1	oil use, storage and Refuelling	10

4.2	Chemical use / Storage (CoSHH)	10

4.3	Permit to Pump	11

5	Monitoring	11

5.1	Siltbuster Check sheet	12

6	B6 Bespoke permit for discharge Prince Charles (Water Discharge Activity)	12

6.1	overview	12

6.2	Siltbuster 2-Stage Chemical Pre-Treatment System	13

7	Gurnos Cut (Water Discharge Activity)	15

7.1	overview	15

7.2	Siltbuster 2-stage chemical Pre-Treatment System	16

8	Mostyn (Water Discharge Activity)	17

8.1	Overview	17

8.2	Siltbuster 2-stage chemical Pre-Treatment System	18

9	Water Catchment on Average Rainfall	20

10	operational Arrangements	21

11	appendix 1	21

11.1	Aquatreat	21

11.2	Brenntag PAC	24

11.3	Caustic Soda Liquor	31

12	Appendix 2 – Siltbuster check sheet	39

13	Appendix 3	40

13.1	Rutpen A9	40

13.2	Water Lynx 360	42

13.3	Water Lynx 395	44

14	Appendix 14 – Floc mat Release Rate	47

# Preliminaries

## Purpose

The purpose of this Water Management Plan (WMP) is to identify the FVC / FCC works that may require a water discharge activity permit and control measures required to prevent such contamination of water courses by, interaction with water infrastructure or generated water effluent (Trade Effluent), and detail the management and monitoring in compliance with regulatory guidelines and advice.

- Dewatering of Excavation(s)

- Surface Water Runoff

## Scope

FVC / FCC are constructing a 17.7km / 11 mile road known as the A465 Heads of the Valleys section 5 and 6. The project will extend between Hirwaun and Dowlais Top. It will leave the current A465 between the Hirwaun and Trewaun junctions for about a mile (1.4km) and between the Taf Fechan Viaduct and the Gurnos Farm underpass for about half a mile (0.8km).

## Definitions & abbreviations

| FVC | Future Valleys Construction |
| --- | --- |
| FCC | FCC Construction |
| NRW | Natural Resources Wales |
| TSS | Total Suspended Solids |
| AGC | Alun Griffiths Contractors |
| RB | Road Bridge |
| RPS | Regulatory Position Statement |
| NTU | Nephelometric Turbidity Units |

## Responsibilities

Responsibilities to prepare and maintain this document:

### FVC / FCC

FVC / FCC as the principle contractor are responsible for driving Good Industry Practice to minimise the impacts of works on the water environment, by ensuring contractors undertake all measures to protect identified receptors of water courses by pathway means.

- Ensuring resources are available to contractors for mitigation measures;

- Updating and approving amendments to all relevant contractor documentation;

- Protecting water management infrastructure in their areas of work scope;

- Undertaking inspections, assessments and audits;

- Evaluating compliance on an ongoing basis, ensuring contractors are compliant;

- Monitoring;

- Supporting incident investigations in a timely manner;

### FVC / FCC Environmental Manager/Advisor

They will be responsible for updating this management plan and carrying out duties such as monitoring to ensure compliance and control measures are in place

All the above 1.4.1 shall be within this scope of works

### Construction Managers and Supervisors

They will be responsible for ensuring a request has been submitted for a permit to pump before entering a new area and that adequate water management arrangements have been made.

They will be responsible for ensuring any accidents that may have an impact on water quality have been reported to the FVC / FCC Environmental Team immediately to prevent further pollution from occurring.

They (or a nominated deputy) will be responsible for the implementation of the control measures as detailed in this Water Management Plan.  They will be responsible for the day to day checking and operation of equipment associated with water management.

# Compliance obligations

## Environmental Permitting Regulations 2016

These Regulations set out an environmental permitting and compliance regime that applies to various activities and industries. Permits set controls and emission standards to minimise pollution and allow operators of certain installations to operate.  Any operational ‘regulated facility’ requires an environmental permit; a regulated facility means a water discharge, groundwater and radioactive substances activities.

# background and controls

## Background

There are a total of 58 water features: three main rivers, five significant tributaries and a further 50 channels that are crossed or which lie within a 500 m buffer zone around the extent of the scheme, the majority of which are already crossed by the existing road. These include two large tributaries of the Afon Taf (the Cwm Taf Fechan and Afon Taf Fawr) and the Afon Cynon.

## Procedural Control Measures (Surface water runoff)

There shall be a Tier approach system to control and minimise potential silt laden surface water runoff. It is key to ensure that surface water is kept clear and away from excavations and earth works as best as reasonably practicable. Each Tier system shall be deployed to ensure the natural flow of water is not affect by works and diverted away, failure to carry out these requirements each tier approach shall be upped in measure. Where all systems are overwhelmed Tier 4 / 5 shall be implemented for active treatment and recognition with requirements of NRW / Local Authorities.

### Tier 1

Mechanisms for reducing the volume of water and silt available for transfer to surface waters. Primary mechanism of control will be segregation. Surface water/runoff will be segregated and/or minimised from contact with the surface to reduce, as far as is practical, the volume of sediment contaminated water (Silt).

Excavated drainage channels or stoned and lined V-drains (French Drainage).

Weathering off of exposed areas, tracked in, sealed or covered

Silt fencing, fibre rolls, geo-textiles and can be used to manage areas to prevent that of surface water runoff (Fig Below jute matting)

### Tier 2

Mechanisms for slowing the flow of sediment rich water to help sediment dropout and treatment. This primarily involves increasing the retention of water as it passes through the Schemes drainage channels, allowing sediment to settle out naturally. Examples of potential mitigation methods include:

Discharge to Vegetation

Straw Bales

Weirs

Pea Shingle Beds

Filter Fabrics

Erosion Berms (Fig below)

### Tier 3

Mechanisms offer solutions for retaining sediment rich water, to allow further settlement prior to discharge or further treatment. This requires the inclusion of systems that allow sediment to settle out naturally, examples of potential mitigation methods include settlement lagoons or settlement systems.

Attenuation / Settlement Lagoons

### Tier 4

Active settlement and polishing of waters from site run off and excavations with use of flocculant-based materials. Where settlement can be achieved within lagoons added measure shall be used to help with NTU / TSS to reach a more acceptable discharge. Products shall be used at inlets of ponds and outlet drainage channels to ensure discharges remain silt free. See appendix for safety data sheets

#### Floc Mat

The floc mat is designed to treat and capture fine silts and suspended particles. Its main function is to flocculate fine sediments enabling settling and separation from water. Sediment is trapped within the matrix of the matting; a single matting can hold up to 50kg of sediment.

Material – Coir (fibres, netting, rope),

Dimensions – 2x1 x 0.1m

WaterLynx Gel Flocculant 360

WaterLynx Gel Flocculant 395

- See appendix 14 for release rate based on 5, 10, 15 and 20 mat deployment

#### Rutpen Flocculant Block A9

The A9 shall be used in aid with settlement lagoons should the settlement not be sufficient. Visual checks will be carried out to confirm clarity of water to confirm requirement for use of A9. A9 block shall be suspended at inlet of ponds to allow water to flow over block which will actively promote settlement. The product is a synthetic anionic flocculant in the form of a solid block, designed to slowly dissolve and flocculate suspended solids. The A9 block comes in 3kgs. It is estimated on average 1 A9 block of 3kg shall last approximately 3 months.

| - Representative Example 1 Matt |  |
| --- | --- |
| - Flow rate at inlet | - 40m3 (Pump) |
| - Dosage | - 1 x A9 flocculant 3kg |
| - Rinse of Polish | Pump 40m3/h – 36,000m6/90days (3 months) A9 Block 3kg – 3,000,000mg 3,000,000 / 3,600,000ltrs A9 – 0.8mg/ltrs over 90 days if run at full ratio |

Note – here active siltbuster treatment is ongoing, the above shall not be used in conjunction

### Tier 5

Active Treatment Subject to site constraints, flow rates, elevated levels of sediment load, effectiveness of passive measures, etc. it may be necessary to implement active treatment techniques to mitigate sediment release.

Treatment options can include:

Conditioning through use of coagulants i.e. ‘floc-blocks’

Lamella clarifiers

Hydro-cyclones

Silt-buster settlement tanks, etc.

# Pollution mitigation

All temporary and permanent site compounds shall be at least 15 m away from the top of the bank of main rivers or 8 m away from ordinary watercourses. No single skin storage containers containing hydrocarbons, only double skin mobile bowsers will be allowed. All storage of any hazardous substance will be within impervious or internal, bunded, secure areas.

## oil use, storage and Refuelling

All oil storage above ground containing more than 200ltrs shall follow the guidelines of the Oil Storage Regulations (OSR) Wales 2016 and GPP2.

Tanks shall be of sufficient strength and structural integrity to ensure that it won’t burst or leak in ordinary use

There shall be no single skinned tanks present on site, where there are these shall be placed into a secondary containment system

Double skinned tanks must be of the design that allows for 110% capacity of the volume of the tank

All secondary containment must hold 110% of the volume

All singular items of oils shall be store on secondary containment either of 110% capacity or 25% of the total capacity whichever is greater

Secondary containment shall be covered from excess dependent rainfall

Mobile bowsers with pipe shall be locked by padlock when not in use, parked away from any watercourse on impermeable grounds

All refuelling of plant on site shall be carried out at platform designated refuelling point. Plant working near watercourse shall track back to point for refuelling.

Regular daily maintenance of containers / tanks / delivery hoses shall be carried out to ensure integrity

## Chemical use / Storage (CoSHH)

All secondary containment must hold 110% of the volume

All singular items of oils shall be store on secondary containment either of 110% capacity or 25% of the total capacity whichever is greater

All chemicals must be accompanied with a valid Safety Data Sheet, and reference to section 6

All chemicals shall be store in a designated area, away from access / egress ports utilised by plant and vehicles

Only chemicals required for days work shall be out for use on platforms, all other chemicals shall be placed in storage as above

All products brought onto site shall be of water based quality / WRAS certification

Storage of chemical within containment tor secondary bunding shall be placed on impermeable ground or compacted hard stand ground at site

## Permit to Pump

Before any works for dewatering commences all Contractors must submit a ‘Permit to Pump’ or similar works authorisation will be sought from the Environmental Team.

Therefore, AGC / RB shall:

Submit a Permit to Pump request to the FVC / FCC Environmental Team.

- The following details will be included as a minimum:

The location that the permit to pump will cover;

The purpose of the permit to pump (i.e. draining of surface water, dewatering of excavations);

A sketch plan showing the planned area for the permit to pump to cover including drainage arrangements.

Proposal on mitigation measures in place to prevent pollution.

All permit requests will be completed by those preparing for the work e.g. section engineers or project managers, with the support of the FVC / FCC Environment Team.

# Monitoring

Daily / Weekly physical checks will be completed in line with the requirements of each permit.  For monitoring purposes and consistency, these checks will be completed at the location of the upstream (Lagoon[s]) and downstream (outlet[s]) as identified in the plans submitted. The checks will include the following:

pH test

TSS / NTU

Turbidity

Dissolved Oxygen

Check for visible oil or grease (i.e. ‘rainbow’ sheen);

Check that the pumping set up appears to be functioning as designed.

Check for the build-up of solids; and

Check the pollution prevention controls are in place.

Monitoring shall be carried out with the use of a calibrated Aquaread AP 800.

pH4.01 Buffer

ORP Cal Solution

Zero Oxygen Solution

1000NTU Calibration Solution

## Siltbuster Check sheet

Preoperational daily check sheets shall be used to ensure the unit(s) full system is operational before any water discharge activities begin. Such checks shall cover the following processes…

General (Unusual sounds, pipework and connections, alarms and fault errors, cables)

Mechanical (Motors running correctly, Vents, flowmeters, dosing pumps, sludge pumps)

Consumables (Level of chemicals, damage occurred, storage containment of chemicals)

Clarifier (Visual checks clear water outlet, solid build up, hoppers i.e. de-sludge)

Reaction Tank (pH level of water 6-9, lines for blockages, visual check solids aggregating, cleaning of pH prob)

All silt busters have contingency pumping arrangements in the event of mechanical failure or primary pump.

- Note – see appendix for template

# B6 Bespoke permit for discharge Prince Charles (Water Discharge Activity)

## overview

| Project | A465 Section 5 and 6 Hirwaun to Dowlais Top |
| --- | --- |
| Site Office | Baverstock Compound, Dynevor Arms, Llwydcoed, Aberdare CF44 0ND |
| Discharge Location | National Grid Ref – N51°46.0844 W003°23.1754 |
| Monitoring Point | Upstream – SO 05066 08683 Downstream – SO 04457 08620 |
| Effluent Discharge | Prince Charles Discharge |
| Nature | Dewatering of excavation, capture of surface water run off of works to settlement ponds, treatment of water via chemical treatment before discharge to existing drainage ditch, entry point Taff Fechan. |
| Volume Calculation | Siltbuster HB50 treatable of 50m3/h 3600sec (60secs to 1 min) – 13.8ltrs/s 10 hour working day, subject to operation over 10 hours this would conclude 450m3/Day The nominal 10 hour working day can be extended in response to server storms by running unit at 24 hours a day, with concomitant staffing provided even during nights, providing a treatment volume of up to 1,080m3 per day. |
| Process of Treatment | Lagoon settlement, Primary Settlement, Chemical Settlement |
| Waste Arisings | Siltbuster units shall require an element of “desludging”. Secondary to the dewatering ponds shall be in place “slurry ponds”. Slurry ponds shall be for TSS that have been captured within the lamella system, they shall be processed by means of drying out to allow moisture content to demise. There shall be no off-site removal of material from this process as the slurry or silt shall be tested as per client materials management plan criteria to ascertain if material is good for landscape fill process. |
| Outline Drawing |  |

## Siltbuster 2-Stage Chemical Pre-Treatment System

The 2-stage chemical pre-treatment system will comprise of the Silt-buster HB50. This unit is subject to treatment quantities of 5-50m3/h – slow settling solids. Set up system shall include the following process…

Inlet electromagnetic flow meter to record the volume of water treated and allow flow proportional dosing of coagulant and polymer.

Mix Tank to allow the controlled mixing of the treatment chemicals ensuring flocculation of the solids (size dependant on flow).

Flow proportional control system for coagulant and flocculant polymer dosing. The use of flow proportional dosing system minimises the risk associated with the overdosing of the treatment chemicals.

Coagulant dosing pump and associated pipework to allow the automatic flow proportional addition of the coagulant.

Flocculant make-up system complete with 1000 litre make up tank, mixer bridge 110 V (32 Amp) mixer complete with paddle (potable water will be required on site for polymer dilution) and flow proportional dosing pump (peristaltic or similar).

1 No IBC spill stand/containment bunds for the temporary storage of chemicals.

Siltbuster HB50 Gravity Operated Settlement Units (Recovery of Suspended Solids)

2 No. Siltbuster HB50 Lamella Clarifiers (dependent on flow rate) to separate the suspended solids from the treated water.

| - Site Setup as provided by Silt-buster |  |
| --- | --- |

### Treatment and dosing

Improved settling characteristics shall be achieved using two stage chemical pre-treatment in which…

A coagulant (PAC) is added to the water to cause the fine particulates to aggregate together into larger particles.

A flocculant (anionic polymer) is added to bind the particles together and increase the particle size, enhancing the settlement rate. The addition of the polymer shall enable near complete removal of the suspended solids.

### Estimated Coagulant Consumption (PAC Brenntag)

At a dose rate of 25mg/l of PAC at a flow rate of 50m3/hr, it is estimated that 17.09 litres of coagulant will be consumed per hour. If consistently operated over a 24-hour period, expect 410.2 litres of coagulant be consumed per day.

### Estimated Flocculant Consumption (AquaTreat)

At a dose rate of 2.25mg/l of EM533 at a flow rate of 50m3/hr, at a working strength of 0.2% it is estimated that 56.25 litres of flocculant as a working strength solution (water + flocculant) will be consumed per hour. This would equate to 55.9 litres of water only per hour, and 0.375 litres of flocculant only per hour. If consistently operated over a 24-hour period, we would expect 9 litres of neat flocculant to be consumed per day, and 237.6 litres of water only to be consumed per day

### Water Quality for Discharge

| Lab Result | Feed Water (From Lagoon) | Treated Outlet from Silt-buster |
| --- | --- | --- |
| pH | 7.82 | 7.05 |
| Total Suspended Solids (TTS) | 4294mg/l | 11mg/l |
| Evidence |  |  |

# Gurnos Cut (Water Discharge Activity)

## overview

| Project | A465 Section 5 and 6 Hirwaun to Dowlais Top |
| --- | --- |
| Site Office | Baverstock Compound, Dynevor Arms, Llwydcoed, Aberdare CF44 0ND |
| Discharge Location | National Grid Ref – SO 03697 08322 |
| Monitoring Point | Discharge as above Upstream – SO 03939 08699 Downstream – SO 03374 08148 |
| Effluent Discharge | Gurnos Cut Discharge |
| Nature | Dewatering of excavation, capture of surface water run off of works to settlement ponds, treatment of water via chemical treatment before discharge to existing ditch to Taff Fechan |
| Volume Calculation | Siltbuster HB50 treatable of 25m3/h 3600sec (60secs to 1 min) – 13.8ltrs/s 10 hour working day, subject to operation over 10 hours this would conclude 250m3/Day The nominal 10 hour working day can be extended in response to server storms by running unit at 24 hours a day, with concomitant staffing provided even during nights, providing a treatment volume of up to 600m3 per day. |
| Process of Treatment | Lagoon settlement, Primary Settlement, Chemical Settlement |
| Waste Arisings | Siltbuster units shall require an element of “desludging”. Secondary to the dewatering ponds shall be in place “slurry ponds”. Slurry ponds shall be for TSS that have been captured within the lamella system, they shall be processed by means of drying out to allow moisture content to demise. There shall be no off-site removal of material from this process as the slurry or silt shall be tested as per client materials management plan criteria to ascertain if material is good for landscape fill process. |
| Outline Drawing |  |

## Siltbuster 2-stage chemical Pre-Treatment System

The 2-stage chemical pre-treatment system will comprise of the Silt-buster HB50. This unit is subject to treatment quantities of 3-30m3/h – slow settling solids. Set up system shall include the following process…

Inlet electromagnetic flow meter to record the volume of water treated and allow flow proportional dosing of coagulant and polymer.

Mix Tank to allow the controlled mixing of the treatment chemicals ensuring flocculation of the solids (size dependant on flow).

Flow proportional control system for coagulant and flocculant polymer dosing. The use of flow proportional dosing system minimises the risk associated with the overdosing of the treatment chemicals.

Coagulant dosing pump and associated pipework to allow the automatic flow proportional addition of the coagulant.

Flocculant make-up system complete with 1000 litre make up tank, mixer bridge 110 V (32 Amp) mixer complete with paddle (potable water will be required on site for polymer dilution) and flow proportional dosing pump (peristaltic or similar).

1 No IBC spill stand/containment bunds for the temporary storage of chemicals.

Siltbuster HB50 Gravity Operated Settlement Units (Recovery of Suspended Solids)

1 No. Siltbuster HB50 Lamella Clarifiers (dependent on flow rate) to separate the suspended solids from the treated water.

| - Site Setup as provided by Silt-buster |  |
| --- | --- |

### Treatment and dosing

Improved settling characteristics shall be achieved using two stage chemical pre-treatment in which…

A coagulant (PAC) is added to the water to cause the fine particulates to aggregate together into larger particles.

A flocculant (anionic polymer) is added to bind the particles together and increase the particle size, enhancing the settlement rate. The addition of the polymer shall enable near complete removal of the suspended solids.

### Estimated Coagulant Consumption (PAC Brenntag)

At a dose rate of 15mg/l of PAC at a flow rate of 30m3/hr, it is estimated that 6.15 litres of coagulant will be consumed per hour. If consistently operated over a 24-hour period, we would expect 147.7 litres of coagulant be consumed per day.

### Estimated Flocculant Consumption (AquaTreat)

At a dose rate of 1.7mg/l of EM533 at a flow rate of 30m3/hr, at a working strength of 0.2% it is estimated that 26.25 litres of flocculant as a working strength solution (water + flocculant) will be consumed per hour. This would equate to 26.04 litres of water only per hour, and 0.175 litres of flocculant only per hour. If consistently operated over a 24-hour period, expect 4.2 litres of neat flocculant to be consumed per day, and 625.8 litres of water only to be consumed per day.

### Water Quality for Discharge

| Lab Result | Feed Water (From Lagoon) | Treated Outlet from Silt-buster |
| --- | --- | --- |
| pH | 7.76 | 6.76 |
| Total Suspended Solids (TTS) | 1926mg/l | 7mg/l |

# Mostyn (Water Discharge Activity)

## Overview

| Project | A465 Section 5 and 6 Hirwaun to Dowlais Top |
| --- | --- |
| Site Office | Future Valleys Construction Site Compound Aberdare CF44 0LX |
| Discharge Location | National Grid Ref – SO 02486 07531 |
| Monitoring Location | Discharge as above Upstream – Downstream – |
| Effluent Discharge | Mostyn Discharge |
| Nature | Capture of surface water run off of works to settlement ponds, treatment of water via chemical treatment before discharge to existing culvert. Flow outlet direct Nant Ffrwd |
| Volume Calculation | Siltbuster HB50 treatable of 60m3/h 3600sec (60secs to 1 min) – 13.8ltrs/s 10 hour working day, subject to operation over 10 hours this would conclude 550m3/Day The nominal 10 hour working day can be extended in response to server storms by running unit at 24 hours a day, with concomitant staffing provided even during nights, providing a treatment volume of up to 1,440m3 per day. |
| Process of Treatment | Lagoon settlement, Primary Settlement, Chemical Settlement |
| Waste Arisings | Siltbuster units shall require an element of “desludging”. Secondary to the dewatering ponds shall be in place “slurry ponds”. Slurry ponds shall be for TSS that have been captured within the lamella system, they shall be processed by means of drying out to allow moisture content to demise. There shall be no off-site removal of material from this process as the slurry or silt shall be tested as per client materials management plan criteria to ascertain if material is good for landscape fill process. |
| Outline Drawing | Distance of change from orginal application. |

## Siltbuster 2-stage chemical Pre-Treatment System

The 2-stage chemical pre-treatment system will comprise of the Silt-buster HB50. This unit is subject to treatment quantities of 6-60m3/h – slow settling solids. Set up system shall include the following process…

Inlet electromagnetic flow meter to record the volume of water treated and allow flow proportional dosing of coagulant and polymer.

Mix Tank to allow the controlled mixing of the treatment chemicals ensuring flocculation of the solids (size dependant on flow).

Flow proportional control system for coagulant and flocculant polymer dosing. The use of flow proportional dosing system minimises the risk associated with the overdosing of the treatment chemicals.

Coagulant dosing pump and associated pipework to allow the automatic flow proportional addition of the coagulant.

Flocculant make-up system complete with 1000 litre make up tank, mixer bridge 110 V (32 Amp) mixer complete with paddle (potable water will be required on site for polymer dilution – to be provided by others) and flow proportional dosing pump (peristaltic or similar).

1 No IBC spill stand/containment bunds for the temporary storage of chemicals.

- Siltbuster HB50 Gravity Operated Settlement Units (Recovery of Suspended Solids)

2 No. Siltbuster HB50 Lamella Clarifiers (dependent on flow rate) to separate the suspended solids from the treated water.

### Treatment and dosing

Improved settling characteristics shall be achieved using two stage chemical pre-treatment in which…

A coagulant (PAC) is added to the water to cause the fine particulates to aggregate together into larger particles.

A flocculant (anionic polymer) is added to bind the particles together and increase the particle size, enhancing the settlement rate. The addition of the polymer shall enable near complete removal of the suspended solids.

### Estimated Coagulant Consumption (PAC Brenntag)

At a dose rate of 12.5mg/l of PAC at a flow rate of 60m3/hr, it is estimated that 10.25 litres of coagulant will be consumed per hour. If consistently operated over a 24-hour period, we would expect 246.1 litres of coagulant be consumed per day.

### Estimated Flocculant Consumption (AquaTreat)

At a dose rate of 2mg/l of EM533 at a flow rate of 60m3/hr, at a working strength of 0.2% it is estimated that 60 litres of flocculant as a working strength solution (water + flocculant) will be consumed per hour. This would equate to 59.6 litres of water only per hour, and 0.4 litres of flocculant only per hour. If consistently operated over a 24-hour period, expect 4.8 litres of neat flocculant to be consumed per day, and 1430.4 litres of water only to be consumed per day.

### Water Quality for Discharge

| Lab Result | Feed Water (From Lagoon) | Treated Outlet from Silt-buster |
| --- | --- | --- |
| pH | 7.64 | 7.25 |
| Total Suspended Solids (TTS) | 3758 | 23 |

# Water Catchment on Average Rainfall

The below table is an estimated summary of each site inclusive of its catchment area along with the average rainfall prediction as per Natural Resource Wales measuring station situated at Hirwain 2022. The information below is a guideline for a month period, as this cannot be assessed as each monthly rainfall event can change.

The calculation used to determine such runoff is concluded (Cumulative actual 301mm recorded February 2022) …

Average Rainfall (m) x Catchment Area (m2) x Surface Coefficient 0.1

| Untreated Ground Catchments |  |
| --- | --- |
| Soil on slopes less 10% | 0.0 – 0.3 |
| Rocky Natural Catchments | 0.2 – 0.5 |

Reference: Pacey, Arnold and Cullis, Adrian 1989, Rainwater Harvesting: The collection of rainfall and runoff in rural areas, Intermediate Technology Publications, London

| Site Name | Catchment Area m2 | Surface run off estimate on average highest rainfall per 1 month (m3) | Attenuation Pond (m3) |
| --- | --- | --- | --- |
| Prince Charles Discharge | 7,500 | 225 | 5190 |
| Gurnos | 9,800 | 294 | 1500 |
| Mostyn | 4,100 | 123 | 1,131 |
| Based on the above calculations there is an expectance of… 6,750m3 over a monthly high rainfall event as per NRW raw data 301mm – Prince Charles 8,820m3 over a monthly high rainfall event as per NRW raw data 301mm – Gurnos 3,690m3 over a monthly high rainfall event as per NRW raw data 301mm - Mostyn |  |  |  |

# operational Arrangements

FVC / FCC / AGC / RB shall operate the Silt-buster unit at location on a daily basis. The operational outfit shall follow the daily pattern of works, Monday to Friday 0700hrs to 1700hrs. Only at which there is consistent heavy down pour causing sub sequential flooding of excavations shall the unit be operated at night. This shall be monitored with weather patterns on daily basis and notice given for dedicated night / weekend shift team if ever required.

Note - The nominal 10 hour working day can be extended in response to severe storms by running unit at 24 hours a day, with concomitant staffing provided even during nights.

# appendix 1

## Aquatreat

## Brenntag PAC

## Caustic Soda Liquor

# Appendix 2 – Siltbuster check sheet

| Silt Buster Daily Check Sheet |  |  |  |
| --- | --- | --- | --- |
| Description | Check | Outcome (tick all good) | Issues / Action |
| General | Any unusual sounds, vibrations or smells |  |  |
|  | Pipework checks, leaks, frozen, wear and tear |  |  |
|  | Control panel check no faults or alarms |  |  |
|  | Power cables in working order, Visual |  |  |
| Mechanical | Motors running, no signs of sticking, bearing noise |  |  |
|  | Motor vents clear and free from debris |  |  |
|  | Flowmeter registering presence of water |  |  |
|  | Chemical dosing pump head in auto mode |  |  |
|  | Sludge pump activities in auto / manual mode |  |  |
| Consumables | Check level of Coagulant |  |  |
|  | Check level of Flocculant |  |  |
|  | Bunding of chemicals not compromised |  |  |
| Clarifier | Visual check of clear water at outlet of clarifier |  |  |
|  | Lamella plates free from solid build up |  |  |
|  | Hoppers de-sludge |  |  |
| Reaction Tank | Check pH level on controls 6-9 |  |  |
|  | Check chemical dosing line blockage, solution entering |  |  |
|  | Wipe clean pH probe |  |  |
| Check by |  | Company |  |
| pH Level |  | Further Comment |  |
| Coagulant pump rate | l/hr |  |  |
| Flocculant pump rate | l/hr |  |  |

# Appendix 3

## Rutpen A9

## Water Lynx 360

## Water Lynx 395

# Appendix 14 – Floc mat Release Rate