| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-019532 |
| Permit Reference Number: | EPR-KP3594VE |
| Permit Holder: | Trade Effluent Services |
| Site Address: | Play Yn LLan Llangynhafal LL16 4LN |
| Grid Reference: | Farm location: SJ 13281 63393 Fields: MJ1 SJ 12844 63224 MJ2 SJ 12692 63119 MJ3 SJ 12957 63101 MJ4 SJ 12836 62927 MJ5 SJ 13049 63001 MJ6 SJ 13143 63094 MJ8 SJ 11308 61467 MJ7 SJ 13285 62933 MJ9 SJ 11424 61484 MJ10 SJ 11527 61576 MJ11 SJ 11520 61345 MJ12 SJ 11481 61184 MJ13 SJ 11406 61032 MJ7a SJ 13425 63029 Storage: 1 SJ 11401 61169 2 SJ 11321 61260 3 SJ 13198 62976 4 SJ 13343 63270 5 SJ 13078 63097 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team:WasteReg.NE@cyfoethnaturiolcymru.gov.uk Environment team: DenbighshireEnvironmentTeam@cyfoethnaturiolcymru.gov.uk NLO: Amy.Henderson@cyfoethnaturiolcymru.gov.uk |
| Date due: | 21/10/2022 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating B- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | Welsh- PaLS |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and Charging Scheme page 52 |
| Fee on MyNRW | Y | Amount: £798 Date cleared on MyNRW: 10/10/2022 Transaction number: 2106381 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | Y |  |
| Site Specific Risk Assessment | n/a | High risk location only |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | Y |  |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | Supplied by the applicant- 56/073/0114 owner Mr Merrion Jones Found on MyMap 1,2,3,4,5,6,7,7a 53/392/0009- M. Jones A’I Gwmni 8,9,10,11,12,13- 55/272/0055- R. Huws Details supplied by applicant do not match those on system RFI Correct CPH could not be supplied however applicant provided an additional written and signed letter from the landowner confirming consent to the activity |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y |  |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y | Details of only one landowner has been provided. RFI provided a second written letter providing |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y |  |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | n/a | A table showing fields were spread with FYM last year but no deployments have ever been issued on this land. |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | Medium (1) |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | n/a |  |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | N |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y | Satellite images provided show fields are established agricultural use. |
| - About the waste |  |  |
| Has all parts of the waste types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable of non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y |  |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y | Google and DMS |
| - About the storage |  |  |
| Has all of waste storage table been completed? | Y |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| If land was previously deployed on, and waste is stored in temporary field heaps; are the storage locations in the same or different locations? | n/a |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y | Correctly buffered |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | Y | 1 Source protection zone-Plas yr Esgob-211m away from field 10 |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | n/a |  |
| location of all storage sites for the waste to be spread | Y | Storage tanks were amended to assist odour issues. |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | Y | Field 10 has a water issue |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | n/a |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | n/a |  |
| location of any public rights of way (shown on map) | Y | Not mark on spread plan- corrected via rfi |
| any buildings or houses within 250m (shown on map) | n/a |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Nitrate Vulnerable Zone (NVZ) - Groundwater Safeguard Zone - Hedges - Ditches | Y | 8 areas of protected habitat- no issue. 11 areas of protected species – no issue. 1 Source Protection Zone-Plas yr Esgols 211m from field 10 1 SAM- Rheul round barrows. 495m from west field 13 1- BRYNIAU CLWYD A DYFFRYN DYFRDWY/CLWYDIAN RANGE AND DEE VALLEY Area of Outstanding Natural Beauty under fields MJ6 and MJ7/7a |
| What is the slope of the fields? | 0-7 degrees (transparent)- fields: all fields 7 -12 degrees (yellow)- fields: n/a 12 degrees plus (red)- fields: n/a |  |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y | Secanim and Kelloggs are out of date. up to date waste analysis sent on 02/12/22 in RFI response |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y | 020204- Conductivity, Fats/oils/grease Odour 02 03 05- odour 02 07 05- pH and soil type (calcareous soils) 02 05 01- odour, sodium 02 07 05- pH |
| - If yes; have they been tested for? | Y |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y | All year including winter. Corrected Via RFI |
| - Amount of waste to be applied and application rate | Y | Secanim is providing too much nitrogen Corrected via RFI amended spread rates |
| - Soil chemical properties (e.g. liming and pH) | Y |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y | Initially there was a risk of over nourishing fields, corrected via RFI |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | Y | Waste will be mixed only if adverse conditions prevent spreading |
| - Is it only for operational purposes? | Y |  |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | N |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y | Applicant has stated they will use injection however they have said, Typically, wastes will be applied by deep-leg injector. |
| - Is the timing of application appropriate? | Y | They have stated intention to spread over autumn/winter and also given a frame that covers an entire year. Corrected via RFI |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y | Rowan Food delivers 425 kg/ha which is 175kg/ha over the acceptable amount This was corrected via RFI and reduced application rate. |
| Does the total nutrients supplied match NRW calculations? | Y | As above |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y |  |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | Y |  |
| - The nutrients supplied by the material? | Y |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | n/a |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | n/a |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | n/a |  |
| - Pathways | n/a |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | n/a |  |
| - Measures that will minimise or mitigate the impact will be put in place | n/a |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | N |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | n/a |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | N |

Comments

Consultation responses: Consultee noticed a spring (issue) at field 10, which requires a 50m buffer zone as part of the RFI. The comments also highlight a request to increase the width of the no spread zone on the Clwyd by an additional 10ms. Also noted a water discharge exemption in field 3. No further action was required by submitted comments.

MyMap, location and slope assessment: MyMap assessment: In draft on WIP Location assessment: See my map report. Slope assessment: Fields are all sloping between 0-7 degrees. No further assessment is required.

CPH details: Supplied by the applicant- 56/073/0114 owner Mr Merrion Jones Found on MyMap 1,2,3,4,5,6,7,7a 53/392/0009- M. Jones A’I Gwmni 8,9,10,11,12,13- 55/272/0055- R. Huws Details supplied by applicant do not match those on system RFI Resolved via written letter from landowner confirming they are happy.

Protection of habitats and ecosystems Images provided by applicant show fields are of established agricultural use, further my map assessment did not show any habitat present.

Agricultural benefit calculations: Secanim and Kelloggs wastes are out of date. Calculations for waste streams secanim, corda goole, croda widnes, maelor foods, encirc, beechdean, penderyn and kelloggs match the waste interpretations provided by the applicant, taking rounding errors into account. Waste stream Rowan foods however do not, the applicant’s agben has intended to spread the waste at 250 tonnes/ha. This will deliver 425kg/ha of nitrogen which is 175kg/ha over the acceptable amount. presumed applicant calculations have been completed using a different analysis Penderyn is high RAN. Beechdean (3.76pH), Kelloggs (3.94pH) and Rowan Foods (3.82pH) are low pH, these pHs in the waste analysis do not match the statement on waste pH in the agricultural benefit statement. The applicant has also stated that these wastes are slightly acidic, this does not reflect the actual nature of the waste.

Request for information: Sent-23/11/22 1. CPH numbers do not match those submitted by the applicant 2. Landowner details 3. Written Consent 4. Storage location 1 5. Storage location 2 6. Storage location 5 7. Storage lagoon 8. Storage location of waste ambiguous language 9. Source protection zone 10. Public rights of way 11. Spring in field 10 12. Timing of spreading 13. Timing of spreading language 14. Waste pH 15. Odour management 16. Waste analysis out of date 17. Rowan food nitrogen addition Reply 02/11/22 1. Applicant state details were provided by the Farmer and requested a method to prevent this question from arising in the future. 2. Applicant stated these were the details provided by the Farmer. 3. Document is the same farm order with a copy and paste of the fields. 4. Resolved 5. Resolved 6. Resolved 7. Resolved 8. Resolved 9. Resolved 10. Resolved 11. Resolved 12. Applicant still intends to spread on maize fields in January. 13.Applicant did not provide a clear agreement to inform NRW of changes. 14. Resolved 15. Resolved 16. Resolved 17. Applicant stated they updated the analysis, in fact they reduced application rate however the issue was resolved. RFI 2 09/12/22 1. Applicant has not provided a correct CPH nor justified why it is incorrect. 2. Applicant has not provided knowledge of the second landowner. 3. Amended document is not newly sign. 12. Requested all fields not be spread until February. 13. Language was ambiguous, request clear ascent to informing NRW before amending spread timings. 18. High FOGs in Secanim 19. D.wise waste added to spread plan but no waste analysis RFI Response 2 16/12/22 Resolved all outstanding questions.

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date |  |
| ## Effective date |  |
| ## Permitting Officer Date | Ethan Banfield 16/12/22 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 16/12/2022 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 1520 |
| Sign off | 10 |
| Issuing |  |
| Total |  |