| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-019774 |
| Permit Reference Number: | EPR-GP3792SK |
| Permit Holder: | ByProduct Recovery Limited |
| Site Address: | Tylebrithos, Cantref, Brecon, LD3 8LR |
| Grid Reference: | Farm location: SO 05078 25701 Fields: Abercynrig 2 - SO 07297 26904 Abercynrig 10 - SO 06919 26499 1 - SO 11043 23464 2 - SO 11169 23269 10 - SO 12318 22415 11 - SO 12064 22598 16 - SO 12325 22953 Storage: SO 04988 25616 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: GwastraffaGorfodi.Canolbarth@cyfoethnaturiolcymru.gov.uk Environment team: environmentteam.southpowys@cyfoethnaturiolcymru.gov.uk NLO: Ewan.Marshall-Harries@environment-agency.gov.uk |
| Date due: | 21/11/2022 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating B- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | English- EA public register, live, variation took place 2015 on DMS. |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and Charging Scheme page 52 |
| Fee on MyNRW | Y | Amount: £1,018 Date cleared on MyNRW: 04/11/2022 Transaction number: 2108037 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | Y |  |
| Site Specific Risk Assessment | Y | High risk location only |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | See comments at the bottom of decision document. |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y |  |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y |  |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y |  |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | Y | Previous deployment: PAN-015421 Expires: 04/01/2023 |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | High |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | Y |  |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | site‐specific risk assessment |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | n/a |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y |  |
| - About the waste |  |  |
| Has all parts of the waste types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable of non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y |  |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y |  |
| - About the storage |  |  |
| Has all of waste storage table been completed? | Y |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| If land was previously deployed on, and waste is stored in temporary field heaps; are the storage locations in the same or different locations? | n/a |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y |  |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | n/a |  |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | Y |  |
| location of all storage sites for the waste to be spread | Y |  |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | Y |  |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | n/a |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | Y |  |
| location of any public rights of way (shown on map) | Y |  |
| any buildings or houses within 250m (shown on map) | Y |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Nitrate Vulnerable Zone (NVZ) - Groundwater Safeguard Zone - Hedges - Ditches | N | See comments at the bottom of the decision document. |
| What is the slope of the fields? | 0-7 degrees (transparent)- Fields 2, 10, 11 & 16. 7 -12 degrees (yellow)- Fields 1, Abercynrig 2 (partially). 12 degrees plus (red)- Fields Abercynrig 2 (partially) |  |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | n/a |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y | Fe, Al , Mn |
| - If yes; have they been tested for? | Y |  |
| If spreading waste soil, has the analysis been carried out at an CERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | Y |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | n/a |  |
| - Soil physical properties (e.g. organic matter and soil structure) | n/a |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | Y |  |
| - Is it only for operational purposes? | Y |  |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | Y |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients supplied match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y |  |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | Y |  |
| - The nutrients supplied by the material? | N | See previous deployment: PAN-015421 |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | Y |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | Y |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | Y |  |
| - Pathways | Y |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | Y |  |
| - Measures that will minimise or mitigate the impact will be put in place | Y |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | Y |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | Y |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | Y |
| Appropriate assessment: In light of the conclusions of an appropriate assessment, and taking account of the advice received from protected sites advisors, it has been established that the project will not adversely affect the integrity of any Natura 2000/Ramsar site, taking into account any conditions or restrictions as applicable, either alone or in-combination with other plans and projects. (As documented in section 4 of OGN 200 form 1, and section 5 if applicable) |  |
| HRA Overall conclusion: In light of the conclusions of the appropriate assessment, it has not been ascertained that the project will not adversely affect the integrity of any Natura 2000/Ramsar site, as documented in section 4 of OGN 200 form 1, and section 5 is applicable. |  |

Comments

Consultation responses: The comments left on the form are as followed: - A pond was identified in field 16. In the request for information it was requested the pond be correctly buffered in compliance with SR2014N04. - River Usk (SAC/SSSI) located near spreading areas. The SAC/SSSI has been considered in the determination of this deployment were a form 1 and appendix 4 assessment has been sent for consultation. - Medium, Med-High and High Groundwater vulnerability. This has been considered during the determination of this deployment where the wastes in this deployment comply with CoGAP, and are applied appropriately using a dribble bar. This will allow the soil to directly received the nutrients from the waste. The wastes will be applied to the fields at no more than an 50t/ha in any 3-week period so will allow the soils to accept the wastes, mitigating the risk of leaching to groundwater. - Please ensure a pre-notification form is submitted at least 2 days prior to spreading. It was explained to the commenting officer Pre notifications are delt with operations. In permitting we check they have confirmed they will inform NRW when they wish to spread and is a requirement of submitting the LPD1 form. As they have done this, we are satisfied at the deployment application/determination stage. - Please ensure spreading follows CoGAP. This was confirmed in the agricultural benefit statement. - Field 2 has index 4 Phosphorus. The nutrient calculator shows 9 of the 10 waste streams are adding less phosphate than in crop offtake so the total phosphate content of the fields will not change One waste stream (whitebourne) is adding an additional 10 kg of phosphate which has been questioned in the request for information 1. The full email response is available on the DMS.

MyMap, location and slope assessment: MyMap assessment: See the MyMap report on the DMS for details of the full assessment. Slope assessment: Field number: Fields 1, 2, Abercynrig 2 Slope of the land: small areas of land are moderately sloping (7-12o) and highly sloping (12o+) Soil type: Medium soils Wastes types applied: sludges Cropping type: Grass Method of application: Dribble bar Weather conditions before during and after spreading: In Compliance with COGAP. Conclusion: The overall runoff risk has been deemed low due to the fields mostly flat to gently sloping. The application method of using the dribble bar is also an appropriate method of applying this waste while factoring in the soil type and spreading timing. The application rate of the wastes to the fields is also appropriate where no more than 50t/ha will be spread on a field in any 3-week period with a total application rate that does not exceed 250t/ha. All other fields contain land that are flat to gently sloping and do not pose a runoff risk.

CPH details: CPH given on LPD1: 52/045/0010 (fields 2 & 10) + 52/050/002 (fields 1, 2, 10, 11, 16) MyMap Report: Fields 1, 2, 10, 11, 16 = 52/053/0010 GJ OWEN & CO Field Abercynrig 2 = 52/045/0020 EXCUTORS OF JRC LLOYD & 52/053/0010 DC & SA PHILIPS & SONS Field Abercynrig 10 = 52/045/0020 EXCUTORS OF JRC LLOYD The CPH numbers are a close match with the Mymap record but the applicant has confirmed the CPH numbers provided are correct in the request for information 1 response.

Protection of habitats and ecosystems

Agricultural benefit calculations: DCWW Whitebourne sludge is adding an additional 10.41 kg of phosphorus to fields Abercynrig 2, 10 & 16 as these fields will be grazed ,the additional phosphate is required to replenish nutrients the lost from grazing.

Request for information: 13/12/2022 - Confirm all the CPH numbers relevant to this deployment. The applicant has confirmed the CPH numbers provided are correct. - Confirm permission from all landowners has been given for fields relevant for this deployment. The applicant has confirmed that consent from all landowners has been given for fields under this deployment. - Provide the total landspreading hectare size for this deployment application. The applicant has confirmed the total hectares size to be 46.9 ha. - Update the LPD1 supplement form so it mentions how much waste was spread under PAN-015421. The applicant has provided the LPD1 supplement form. - Confirm if waste streams relevant for this deployment have been measured on a dry matter or SAR basis. The applicant has confirmed the units have been measured on a dry matter basis. - Confirm Waste stream Cray refers to Crai named in the deployment documents. The applicant has confirmed ‘cray’ was a typographic error. - Update the site plan key so that it provides details of the no spread buffer zone distances. The applicant has updated the site plan to show the no spread zone distances. - Update the site plan to show the location of the mentioned storage tank. The applicant has confirmed all the storage tank locations within the ABS. - Update site plan to show protected habitat bordering field 1. The applicant has updated the site plan to show the no spread buffer zone around the field. - Update the Site specific risk assessment (SSRA) so that the protected habitat is risked assessed in the source, receptor and pathway format. The applicant has updated the SSRA to include the protected site. - Update the SSRA so the Brecon Beacons National Park is adequately risked assessed within the document. The applicant has updated the SSRA to include the national park. - Confirm If the available concentration has been used and if yes, the percentage used. The applicant has confirmed the percentage used is 20% and matches the nutrient calculator. - Justify this increase of phosphorus for the fields listed above. The applicant has stated the additional phosphorus requirement is for grazed fields. - Update the ABS so it does not state stackable waste streams will be applied via rear discharge spreader. The applicant has updated the ABS. - Confirm the storage pit also refers to the lagoon which is mentioned in the ABS. The applicant has confirmed that the lagoon will not be built in time for this deployment and therefore has removed reference to this in the ABS. The applicant had also clarified the storage pit refers to the storage tank and will comply with the is a with the requirements of TGN 8.01. - Update the agricultural benefit statement (ABS) to state the storage lagoon will comply with TGN 8.01. The applicant has confirmed that the lagoon will not be built in time for this deployment and therefore has removed reference to this in the ABS. - Update the ABS to confirm the storage lagoon will be built before waste under this deployment will be spread on the fields. The applicant has confirmed that the lagoon will not be built in time for this deployment and therefore has removed reference to this in the ABS. - Confirm that before waste is stored in the lagoon it will be clean and empty prior to waste being stored in the lagoon. The applicant has confirmed that the lagoon will not be built in time for this deployment and therefore has removed reference to this in the ABS. - Provide further justification on spreading these high Fe containing waste streams mentioned above on fields Abercynrig 10, 2, and 11 that have a field pH less than 5.5, or remove these waste streams from these fields. The applicant has removed these wastes streams from the listed fields mentioned above. - Update the ABS so that it acknowledges the potential negative impacts spreading elevated levels of Fe may have on the crops, soils and nearby water courses while explaining any mitigation measures that will be in place. The applicant has removed these wastes streams from fields with a soil pH of 5.5 or less. - Confirm that you will not start the activity as described in application PAN-019774 until 05/01/2023 or when we have agreed the deployment in writing (whichever is the later). The applicant has confirmed the deployment not start until 05/01/2023 or until the deployment has been agreed in writing. Request for information 2: 14/12/2022 - Update the site plan so that the pond located in field 16 is correctly buffered. The applicant has updated the site plan.

Other information to note:

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 06/01/2023 |
| ## Effective date | 06/01/2023 |
| ## Permitting Officer Date | Owen Clee 05/01/2023 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 06/01/2023 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 1,680 |
| Sign off | 10 |
| Issuing | 10 |
| Total | 1,760 |