PAN-019864 My Map Screening Report:

Application map

Identifies Non spreading areas

Storage location

No Storage following RFI

Detailed river network:

Watercourse immediately adjacent to NW of Fields 11, 12 and N, NE of Field 12. Drain intersects Field 11. Non spreading buffers in place on application maps.

Watercourse to N of Fields 14,15. Non spreading buffer in place on application maps

Watercourse and drain to SE of Fields 15, 4, 5, 8. Watercourse to SW of Field 8. Non spreading buffer in place on application maps

Groundwater protection zones

None within range

Wells, springs or boreholes used for the supply of water for human consumption including from a private water supply or supplies

Issue and pond directly adjacent to Fields 8, 9. Ponds adjacent to Fields 9, 17. Non spreading buffer in place on application maps

Well to NE of Field 16. Non spreading buffer in place on application maps

European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m

None within range

Other protected sites:

- Protected Species-

- Protected Habitats-

- Scheduled Ancient Monument-

- Local Nature Reserve-

- National Nature Reserve-

- Ancient Woodlands-

- Local Wildlife Sites (SINC)-

- National Park-

- Nitrate Vulnerable Zone (NVZ)-

- Groundwater Safeguard Zone-

- Hedges-

- Ditches-

Purple moor grass and rush pastures to W and NE of application area and within fields 11, 13. Lowland dry acid grassland and Lowland calcareous grassland to NE of field 15 and adjacent to fields 9,10,11, 13, 17. Non spreading buffer in place on application map.

Local Wildlife site Cors Tafarn-y-Grib to NNE of field 12 at 368.5m A very wet basin mire, with predominantly aquatic vegetation, also smaller areas of sedge-rich wet meadow and willow carr. A large part of the site is dominated by water horsetail with bottle sedge and bogbean. The rare greater spearwort and common spearwort. Topography indicates land slopes towards Field 12 so risk is low. Non spreading buffer is also in place on application map.

Historic Monuments-

ST PEULAN'S CHURCH

Religious, Ritual and Funerary

Medieval

CAPEL-BACH

Religious, Ritual and Funerary

Post Medieval Chapel

Both adjacent to fields 11,12 & 13. Non spreading buffers in place on application maps

No other sites of interest within range.

Slope Map

All fields 0-7, some areas 7-12 with minimal areas >12.

Public right of way

Non spreading buffers in place on application map

END