| #### CRoW Appendix 4 (140_10_SD02) # CRoW Act 2000: Natural Resources Wales application for permission - Formal Notice |  |  |
| --- | --- | --- |
| Natural Resources Wales Formal Notice. Requirements of Section 28I of the Wildlife & Countryside Act 1981 as amended by the Countryside and Rights of Way Act (CRoW) 2000. Duty in relation to granting any consent, licence or permit for activities likely to damage Sites of Special Scientific Interest (SSSI). |  |  |
| Guide to filling in this form for Natural Resources Wales staff: To be completed by Permitting Officers for any applications for a permission which the Natural Resources Wales has considered under S28G duties to protect and enhance SSSIs. This applies to all proposed permissions within a SSSI, and to operations outside the SSSI boundary which are likely to damage its special features. Refer to OI 140_10 ‘Applying the Countryside and Rights of Way (CRoW) Act 2000 to applications for permits with potential for impact on Sites of Special Scientific Interest (SSSI)’, including the flowchart in Appendix 2. |  |  |
| Pink italic text – drafting notes, to be deleted before completion/consultation. Blue text – examples, to be replaced with permission-specific information. Ensure you have completed all sections. |  |  |
| Natural Resources Wales area/region/NPS hub: | North West Wales |  |
| Name of SSSI: | Mynydd Parys SSSI |  |
| Type of permission: | Land spreading deployment under standard rules permit SR 2010 No4 Mobile plant for land spreading. |  |
| Date for Natural Resources Wales permit determination: | 19/01/2023 |  |
| Predicted 28 day date for response from NRW conservation/ecology (under S28 I(4)): | n/a |  |
| Natural Resources Wales reference no: | PAN-020023 |  |
| National grid reference: | Farm location: SH 44000 89616 Fields: 1 SH 44324 89834 2 SH 44285 89639 3 SH 44428 89720 4 SH 44512 89622 5 SH 44631 89709 7 SH 44709 89855 8 SH 44819 89999 9 SH 45049 90018 12 SH 45346 90163 13 SH 45239 89983 |  |
| Description of proposal: | A land spreading application to spread 3 waste streams (EWC 02 05 01 & 02 05 02) produced by Glanbia Cheese Limited at Trysglwyn Isaf Farm, Anglesey. Wastes are to be spread on 10 fields which contain grass. The application rate of the wastes ranges from 140-200 tonnes per ha, with a total tonnage to spread a maximum of 10,000 tonnes. The waste will be spread over 40 ha hectares of land within the listed fields at Trysglwyn Isaf Farm. The waste spread under this deployment are composed of non-stackable sludges and will be spread on delivery and are only despatched to the field site when conditions will be suitable. Liquid sludges will be spread via trailing shoe applicator. The applications of waste to the crops are planned to be either prior to, or shortly after each of the planned the silage cuts, however this is highly dependent on crop stage, ground and weather conditions. Applications are planned to take place in: - 1st application start of March 2023 - 2nd application start of May 2023 - 3rd application start of July 2023 - 4th or any remaining limits spread after the final cut in September. Liquids will not be applied at times when the fields are too wet/cold and are not planned to be between October 2023 – January 2024 when conditions are not anticipated to be in line with per conditions. The land spreading activity will comply with the Code of Good Agricultural Practice guidelines (COGAP). Documents related to this deployment can be found on the DMS. |  |
| Is the proposed activity within (wholly or partially) the SSSI boundary? | NO Field 1 (closest field) is located 173 meters from the Mynydd Parys SSSI. Below is a table detailing the distance that fields within screening distance are from the SSSI (500m). Field; Distance field is from SSSI (meters). 1; 173. 2; 334. 3; 318. 4; 455. 5; 392. 7; 185. 8; 351. |
| Has there been any pre-application discussion or correspondence with NRW conservation/ecology | NO |  |
| What aspect(s) of the proposed permission may damage the features which are of special interest for the SSSI? The following ‘Operations Requiring Consent’ (or other activities associated with the permission) that may cause damage) are relevant to the proposed permission. 1. Cultivation, including ploughing, rotovating, harrowing and re-seeding. 4. Introduction of mowing or other methods of cutting vegetation. 5. Application of manure, fertilisers and lime. 7 Dumping, spreading or discharging of any materials. 26. Use of vehicles likely to damage lichen heath. The following SSSI features and mechanisms of impact have been considered to assess the likelihood of damage: Geological - Mineral bearing spoil-tips, mineral veins and rock exposures in disused quarries, mines and tunnels. Mynydd Parys SSSI is of national importance for the wide-range of copper- and lead-bearing minerals that are found in spoil tips and rock exposures associated with numerous disused mines. Rare minerals found on this site include Pisanite, Antlerite, Basaluminite and Anglesite, for which Mynydd Parys is the type locality. The rich variety of minerals remaining both in the mine spoil and in the underground workings provide clues to the origin of these rocks and the conditions in which they formed. Biological - Assemblage of metallophyte lichens Toxic metal tolerant lichens are only found on areas where the soil, rock or other surface on which they grow is rich in metals such as copper and zinc. 10 nationally scarce lichens have been recorded from this site, together with a nationally scarce liverwort. Habitats - These include areas of heathland and wetland. This mixture of habitats is important for wildlife, including chough which nest on site. Mynydd Parys also supports stag’s horn clubmoss at its only known location on Anglesey. |  |  |
| Decision |  |  |
| i) The proposed permission is not likely to damage any of the flora, fauna or geological or physiological features which are of special interest. - Field 1 is the closest field to the SSSI located 173 meters from the SSSI. There is a 50 meter no spread buffer zone in place in field 1 to mitigate the risk of nutrients/PTEs entering the SSSI. - Fields 2, 3, 4, 5, 7, 8 are within screening distance of the SSSI ranging from 185m – 455m. These distances are far away from the SSSI where there is no impact pathway for nutrients and/or PTEs to enter the SSSI. Therefore the SSSI is at little risk from this landspreading activity. - Fields 1, 7 & 8 contain small areas of land that are moderately sloping at 7-12o. The moderately sloping areas are contained within the fields so waste is highly unlikely to runoff. The rest of the fields contain flat to gently sloping land (0-7o) and do not pose a runoff risk. - Wastes will be spread by dribble bar application directly to the soil. Which is an appropriate application method for the waste type which are efficient methods to prevent odour transmission & nutrient losses. - As shown on the nutrient calculator for this application, all fields being spread on have a greater overall offtake of phosphate than is being added by the spreading, so the overall levels of phosphate in the soils will decrease from their current level within the 12 month period of this deployment. There will be no overall phosphate increase to the land as a result of this application. - Water courses/wells have been appropriately buffered in accordance with the stand rules permit to prevent nutrients and/or PTEs from entering the water courses/ SSSI. - All wastes streams are non-stackable liquid sludges, so dust will not be generated during the activity. and as such, no dust from this activity should enter any protected water courses. - Wastes will be spread at appropriate times when the crop will benefit from the nutrients provided i.e. spring, summer & autumn. Applications will be carried out when ground conditions are suitable in line with COGAP. - The wastes will be applied following the Code of Good Agricultural Practice. - A site-specific risk assessment has been provided by the operator to take into account any potential issues and has provided appropriate mitigation measures to protect the SSSI from harm. - All turns made by the application machinery will be gentle with the intention of preventing wheel slip and excess soil damage in the form of ruts. - All deliveries to the field will be made during the working day to limit noise/disturbance to nearby sensitive (human) receptors. - All application machinery is calibrated and regularly checked in order to ensure the stated application rates are maintained. - There are sufficiently trained staff to maintain sickness and holiday cover. Natural Resources Wales is minded to: Issue the permission |  |  |
| Name and job title of Natural Resources Wales officer: | Owen Clee Permitting Officer |  |
| Date form sent to NRW conservation/ecology | n/a - saved for information purposes only |  |
| For Natural Resources Wales use only, once NRW conservation/ecology response received |  |  |
| NRW conservation/ecology comment on assessment: | Please delete as appropriate: i) NRW conservation/ecology advise the operation can go ahead ii) NRW conservation/ecology advise the operation can go ahead with conditions iii) NRW conservation/ecology advise against permitting the operation Please ensure that the NRW conservation/ecology response is attached to this Formal Notice. |  |
| Name and job title of NRW conservation/ecology officer: |  |  |
| Date of receipt of NRW conservation/ecology response: |  |  |