Rest

Permit with introductory note

The Environmental Permitting (England & Wales) Regulations 2016

Hurt Plant Hire Ltd

Nant Newydd Quarry Inert Landfill

Brynteg

Anglesey

LL78 7JJ

Permit number

EPR/RP3337SE

Nant Newydd Quarry Inert Landfill

Permit number EPR/RP3337SE

Introductory note

This introductory note does not form a part of the permit

The main features of the permit are as follows.

This is a Tier 3 bespoke permit for an inert landfill.

Permitted activities consist of receipt, handling, storage and disposal of wastes only.

Permitted waste types are inert only.

The maximum quantity of waste accepted and processed on site shall not exceed 125,000 tonnes per year.

This permit does not allow any point source emission into surface waters or groundwater except:

- clean, rainfall dependant drainage from areas of the site not used in connection with the storage and/or treatment of waste, and

- integral discharge(s) from the permitted activity in accordance with the limits set out in Schedule 2 of the permit.

All emissions from the activity which are not controlled via actual emission limits, are controlled through the ‘emission of substances not controlled by emission limits condition’ and all relevant process requirements set out in Tables S1.1 and S1.2.

The burning of any wastes, either in the open, inside buildings or in any form of incinerator is not permitted.

The status log of the permit sets out the permitting history, including any changes to the permit reference number.

| Status log of the permit |  |  |
| --- | --- | --- |
| Description | Date | Comments |
| Waste Management Licence NOW-110-L EAWML 37144 | 19/07/96 | Original waste management licence issued to W.J. Owen & Sons |
| EAWML 37144 Modification | 22/03/99 | Waste returns conditions added |
| EAWML 37144 | 26/06/04 | Transfer in full to Messrs Hugh Selwyn Owen & Gwilym Tyler Owen |
| Permit determined EPR/RP3337SE (formally EAWML 37144) | 30/03/07 | Consolidated and modernised Pollution Prevention Control (PPC) permit issued to Mrs J.T. Owen & Mr G.T. Owen. |
| Transfer determined EPR/RP3337SE/T002 | 23/05/18 | Full transfer of permit to Clive Hurt (Plant Hire) Limited complete |
| Variation determined EPR/RP3337SE/V003 | 28/01/19 | Variation to increase the annual throughput and clarify surface water emission and monitoring points issued. |
| Change of company address | 05/05/22 | Company address changed to 11 Neptune Court Hallam Way, Whitehills Business Park, Blackpool, Lancashire, England, FY4 5LZ |
| Change of Company name | 10/06/22 | Company name changed to Hurt Plant Hire Ltd |
| Variation application EPR/RP3337SE/V004 | Duly Made 17/08/22 | Operator led variation to: - revise existing compliance limits and monitoring regime specified for groundwater (Tables S4.2 and S4.6), surface water (Tables S4.1 and S4.5) and gas (Tables S4.3 and S4.4). - discharge improvement conditions 1.3.2, 1.3.6 and 1.3.9 - update the facility name to Nant Newydd Quarry Inert Landfill - update the company name and address. The regulator has consolidated the permit. |
| Additional information received in response to Schedule 5 notice | 11/10/22 | Consisting of: - Schedule 5 Response - Landfill Gas Risk Assessment Addendum - updated Monitoring Point Locations (Rev B) |
| Variation issued EPR/RP3337SE | 19/01/23 | Varied and consolidated permit issued. |

End of introductory note

Permit

The Environmental Permitting (England and Wales) Regulations 2016

Permit number

EPR/RP3337SE

The Natural Resources Body for Wales (“Natural Resources Wales”) authorises, under regulation 13 of the Environmental Permitting (England and Wales) Regulations 2016

Hurt Plant Hire Ltd (“the operator”),

whose registered office is

11 Neptune Court

Hallam Way

Whitehills Business Park

Blackpool

Lancashire

England

FY4 5LZ

company registration number 01853066

to operate waste operations at

Nant Newydd Quarry Inert Landfill

Brynteg

Anglesey

LL78 7JJ

to the extent authorised by and subject to the conditions of this permit.

| Name | Date |
| --- | --- |
| Huw Davies | 19/01/2023 |

Authorised on behalf of Natural Resources Wales

Conditions

# Management

## General management

### The operator shall manage and operate the activities:

#### in accordance with a written management system that identifies and minimises risks of pollution, including those arising from operations, maintenance, accidents, incidents, non-conformances, closure and those drawn to the attention of the operator as a result of complaints; and

#### using sufficient competent persons and resources.

### Records demonstrating compliance with condition 1.1.1 shall be maintained.

### Any person having duties that are or may be affected by the matters set out in this permit shall have convenient access to a copy of it kept at or near the place where those duties are carried out.

### The operator shall comply with the requirements of an approved competence scheme.

## Avoidance, recovery and disposal of wastes produced by the activities

### The operator shall take appropriate measures to ensure that:

### the waste hierarchy referred to in Article 4 of the Waste Framework Directive is applied to the generation of waste by the activities; and

### any waste generated by the activities is treated in accordance with the waste hierarchy referred to in Article 4 of the Waste Framework Directive; and

### where disposal is necessary, this is undertaken in a manner which minimises its impact on the environment.

### The operator shall review and record at least every four years whether changes to those measures should be made and take any further appropriate measures identified by a review.

## 1.3	Finance

### 1.3.1	The financial provision for meeting the obligations under this permit set out in the agreement made between the operator and Natural Resources Wales dated 23/05/2018 shall be maintained by the operator throughout the subsistence of this permit and the operator shall produce evidence of such provision whenever required by Natural Resources Wales.

### 1.3.2	The operator shall ensure that the charges it makes for the disposal of waste in the landfill cover all of the following:

- the costs of setting up and operating the landfill;

- the costs of the financial provision required by condition 1.3.1; and

- the estimated costs for the closure and aftercare of the landfill.

# 2	Operations

2.1	Permitted activities

### 2.1.1	The operator is only authorised to carry out the activities specified in schedule 1 table S1.1 (the “activities”).

## 2.2	The site

2.2.1	The activities shall not extend beyond the site, being the land shown edged in green on the site plan at schedule 7 to this permit.

## 2.3	Operating techniques

#### 2.3.1	(a)	The activities shall, subject to the conditions of this permit, be operated using the techniques and in the manner described in the documentation specified in schedule 1, table S1.2, unless otherwise agreed in writing by Natural Resources Wales.

#### (b)	If notified by Natural Resources Wales that the activities are giving rise to pollution, the operator shall submit to Natural Resources Wales for approval within the period specified, a revision of any plan or other documentation (“plan”) specified in schedule 1, table S1.2 or otherwise required under this permit which identifies and minimises the risks of pollution relevant to that plan, and shall implement the approved revised plan in place of the original from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

### 2.3.2	Waste shall only be accepted if:

### (a)	it is of a type and quantity listed in schedule 2 table(s) S2.1 [, S2.2 etc.]; and

### (b)	it conforms to the description in the documentation supplied by the producer and holder.

## 2.4	Improvement programme

### 2.4.1	The operator shall complete the improvements specified in schedule 1 table S1.3 by the date specified in that table unless otherwise agreed in writing by Natural Resources Wales.

### 2.4.2	Except in the case of an improvement which consists only of a submission to Natural Resources Wales, the operator shall notify Natural Resources Wales within 14 days of completion of each improvement.

## 2.5	Pre-operational conditions

2.5.1	The activities shall not be brought into operation until the measures specified in schedule 1 table S1.4 have been completed.

## 2.6	Landfill Engineering

### 2.6.1	No construction of any new cell shall commence until the operator has submitted construction proposals and Natural Resources Wales has confirmed that it is satisfied with the construction proposals.

### 2.6.2	The construction of a new cell shall take place only in accordance with the approved construction proposals unless:

- any change to the approved construction proposals would have no impact on the performance of any element of the design; or

- a change has otherwise been agreed in writing by Natural Resources Wales.

### 2.6.3	No disposal of waste shall take place in a new cell until the operator has submitted a CQA Validation Report and Natural Resources Wales has confirmed that it is satisfied with the CQA Validation Report.

### 2.6.4	No construction of landfill infrastructure shall commence until the operator has submitted relevant construction proposals or a written request to use previous construction proposals and Natural Resources Wales has confirmed that it is satisfied with the construction proposals.

### 2.6.5	The construction of the landfill infrastructure shall take place only in accordance with the approved construction proposals unless:

- any change to the approved construction proposals would have no impact on the performance of any element of the design; or

- a change has otherwise been agreed in writing by Natural Resources Wales.

### 2.6.6	The operator shall submit a CQA Validation Report as soon as practicable following the construction of the landfill infrastructure.

### 2.6.7	Where pollution controls are immediately necessary to prevent an incident or accident, then conditions 2.6.4 and 2.6.5 do not apply and the relevant landfill infrastructure may be constructed, provided that the construction proposals are submitted to Natural Resources Wales as soon as practicable.

### 2.6.8	For the purposes of conditions 2.6.1, 2.6.3 and 2.6.4, Natural Resources Wales shall be deemed to be satisfied where it has not, within the period of four weeks from the date of receipt of the relevant construction proposals or CQA Validation Report, either:

- confirmed whether or not it is satisfied; or

- informed the operator that it requires further information.

    - Where Natural Resources Wales has required further information under condition 2.6.8(a), Natural Resources Wales shall be deemed to be satisfied where it has not, within the period of four weeks from the date of receipt of the further information, either:

- confirmed whether or not it is satisfied; or

- informed the operator that it requires further information.

## 2.7		Waste acceptance

2.7.1	Wastes shall only be accepted for disposal if:

- they are listed in schedule 2, and

- they are inert waste, and

- they are not liquid waste (including waste waters but excluding sludge), and

- all the relevant waste acceptance procedures have been completed, and

- they fulfil the relevant waste acceptance criteria, and

- they have not been diluted or mixed solely to meet the relevant waste acceptance criteria, and

- they are wastes which have been treated, except for wastes for which treatment is not technically feasible.

### 2.7.2	The operator shall visually inspect:

- without unloading it, waste that is not in an enclosed container or enclosed vehicle on arrival at the landfill; and

- waste at the point of deposit;

and shall satisfy itself that it conforms to the basic characterisation documentation submitted by the holder.

### 2.7.3	Where the operator has taken samples to establish that the waste is in conformity with the documentation submitted by the holder then the samples taken shall be retained for at least one month and results of any analysis for at least two years.

### 2.7.4	The operator on accepting each delivery of waste shall provide a receipt to the person delivering it.

### 2.7.5	The total quantity of waste that shall be deposited in the landfill shall be limited by the pre-settlement levels shown on drawing ESID4 (dated September 2005).

### 2.7.6	The quantity of waste that is deposited in the landfill in any year shall not exceed the limits in schedule 2 table S2.1.

### 2.7.7	The operator shall maintain and implement a system which ensures that a record is made of the quantity, characteristics, date of delivery and, where practicable, origin of any waste that is received for disposal or recovery and of the identity of the producer, or in the case of multiple collection vehicles, of the collector of such waste. Any information regarded by the operator as commercially confidential shall be clearly identified in the record.

## 2.8	Closure and aftercare

### 2.8.1	The operator shall maintain a closure and aftercare management plan.

# 3	Emissions and monitoring

## 3.1	Emissions to water, air or land

### 3.1.1	There shall be no point source emissions to water, air or land except from the sources and emission points listed in schedule 3 tables S3.1.

### 3.1.2 	The limits for surface water quality specified in schedule 3, table S3.1 shall not be exceeded.

3.1.3	The limits for landfill gas arising from the facility set out in schedule 3, tables S3.3 shall not be exceeded.

3.1.4 	There shall be no emission from the activities into groundwater of any hazardous substances contrary to the EP Regulations.

3.1.5 	There shall be no emission from the activities into groundwater of any non-hazardous pollutants so as to cause pollution.

3.1.6 	The trigger levels for emissions into groundwater for the parameter(s) and monitoring point(s) set out in schedule 3 table S3.2 shall not be exceeded.

### 3.1.7 	The operator shall submit to Natural Resources Wales a review of the Hydrogeological Risk Assessment:

(a)	between nine and six months prior to the sixth anniversary of the granting of the permit, and

(b)	between nine and six months prior to every subsequent six years after the sixth anniversary of the granting of the permit.

## 3.2 	Emissions of substances not controlled by emission limits

### 3.2.1 	Emissions of substances not controlled by emission limits (excluding odour) shall not cause pollution. The operator shall not be taken to have breached this condition if appropriate measures, including, but not limited to, those specified in any approved emissions management plan, have been taken to prevent or where that is not practicable, to minimise, those emissions.

3.2.2	The operator shall:

#### (a)	if notified by Natural Resources Wales that the activities are giving rise to pollution, submit to Natural Resources Wales for approval within the period specified, an emissions management plan which identifies and minimises the risks of pollution from emissions of substances not controlled by emission limits;

#### (b)	implement the approved emissions management plan, from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

3.2.2	All liquids in containers, whose emission to water or land could cause pollution, shall be provided with secondary containment, unless the operator has used other appropriate measures to prevent or where that is not practicable, to minimise, leakage and spillage from the primary container.

## 3.3	Odour

#### 3.3.1	Emissions from the activities shall be free from odour at levels likely to cause pollution outside the site, as perceived by an authorised officer of Natural Resources Wales, unless the operator has used appropriate measures, including, but not limited to, those specified in any approved odour management plan, to prevent or where that is not practicable to minimise the odour.

3.3.2	The operator shall:

#### (a)	if notified by Natural Resources Wales that the activities are giving rise to pollution outside the site due to odour, submit to Natural Resources Wales for approval within the period specified, an odour management plan which identifies and minimises the risks of pollution from odour;

#### (b)	implement the approved odour management plan, from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

## 3.4	Noise and vibration

#### 3.4.1	Emissions from the activities shall be free from noise and vibration at levels likely to cause pollution outside the site, as perceived by an authorised officer of Natural Resources Wales, unless the operator has used appropriate measures, including, but not limited to, those specified in any approved noise and vibration management plan to prevent or where that is not practicable to minimise the noise and vibration.

3.4.2	The operator shall:

#### (a)	if notified by Natural Resources Wales that the activities are giving rise to pollution outside the site due to noise and vibration, submit to Natural Resources Wales for approval within the period specified, a noise and vibration management plan which identifies and minimises the risks of pollution from noise and vibration;

#### (b)	implement the approved noise and vibration management plan, from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

## 3.5	Monitoring

### 3.5.1	The operator shall, unless otherwise agreed in writing by Natural Resources Wales, undertake the monitoring and any other actions specified in the following tables in schedule 3 to this permit

#### point source emissions specified in table S3.1;

#### groundwater specified in tables S3.2 and S3.5;

#### landfill gas specified in table S3.3;

#### surface water specified in table S3.4;

3.5.2	The operator shall maintain records of all monitoring required by this permit including records of the taking and analysis of samples, instrument measurements (periodic and continual), calibrations, examinations, tests and surveys and any assessment or evaluation made on the basis of such data.

### 3.5.3	A topographical survey of the site referenced to Ordnance Datum shall be carried out:

- annually, or prior to the disposal of waste in any new cell or new development area of the landfill whichever is the shorter period, and

- following closure of the landfill or part of the landfill.

The topographical survey shall be used to produce a plan of a scale adequate to show the surveyed features of the site.

# 4	Information

## 4.1	Records

### All records required to be made by this permit shall:

### be legible;

### be made as soon as reasonably practicable;

### if amended, be amended in such a way that the original and any subsequent amendments remain legible, or are capable of retrieval; and

### be retained, unless otherwise agreed in writing by Natural Resources Wales, for at least 6 years from the date when the records were made, or in the case of the following records until permit surrender:

- the results of groundwater monitoring;

- landfill gas monitoring;

- waste types and quantities;

- topographical surveys;

- the specification and as built drawings of the basal and sidewall engineering systems.

4.1.2	The operator shall keep on site all records, plans and the management system required to be maintained by this permit, unless otherwise agreed in writing by Natural Resources Wales.

## 4.2	Reporting

4.2.1	The operator shall send all reports and notifications required by the permit to Natural Resources Wales using the contact details supplied in writing by Natural Resources Wales.

### 4.2.2	A report or reports on the performance of the activities over the previous year shall be submitted to Natural Resources Wales by 31 January (or other date agreed in writing by Natural Resources Wales) each year. The report(s) shall include as a minimum:

- a review of the results of the monitoring and assessment carried out in accordance with this permit against the relevant assumptions, parameters and results in the risk assessments submitted in relation to this installation and any agreed amendments thereto;

- where the operator’s management system encompasses annual improvement targets, a summary report of the previous year’s progress against such targets;

- the annual production/ treatment set out in schedule 4 table S4.2.

- details of any contamination or decontamination of the site which has occurred;

- the topographical surveys required by condition 3.5.3 other than those submitted as part of a CQA validation report;

- the volumetric difference (reported in cubic metres) between the most recent topographical survey and the previous annual topographical survey i.e. the additional volume of the landfill void that is occupied by waste;

- an assessment of the settlement behavior of the landfill body based on the difference between the most recent topographical survey and previous annual topographical survey for the areas of the landfill which did not receive waste between the surveys;

- a calculation of the remaining capacity (reported in cubic metres) derived from the pre-settlement contours and the most recent topographical survey;

- a summary of the WAC compliance testing undertaken in the period.

### 4.2.3	Within 28 days of the end of the reporting period the operator shall, unless otherwise agreed in writing by Natural Resources Wales, submit reports of the monitoring and assessment carried out in accordance with the conditions of this permit, as follows:

(a)	in respect of the parameters and emission points specified in schedule 4 table S4.1;

(b)	for the reporting periods specified in schedule 4 table S4.1 and using the forms specified in schedule 4 table S4.3; and

(c)	giving the information from such results and assessments as may be required by the forms specified in those tables.

### 4.2.4	Within one month of the end of each quarter, the operator shall submit to Natural Resources Wales using the form made available for the purpose, the information specified on the form relating to the site and the waste accepted and removed from it during the previous quarter.

## 4.3	Notifications

### 4.3.1	Natural Resources Wales shall be notified without delay following the detection of:

### any malfunction, breakdown or failure of equipment or techniques, accident, or emission of a substance not controlled by an emission limit which has caused, is causing or may cause significant pollution;

### the breach of a limit specified in the permit; or

### any significant adverse environmental effects.

### 4.3.2	Any information provided under condition 4.3.1 shall be confirmed by sending the information listed in schedule 5 to this permit within the time period specified in that schedule.

### 4.3.3	Where Natural Resources Wales has requested in writing that it shall be notified when the operator is to undertake monitoring and/or spot sampling, the operator shall inform Natural Resources Wales when the relevant monitoring and/or spot sampling is to take place. The operator shall provide this information to Natural Resources Wales at least 14 days before the date the monitoring is to be undertaken.

### 4.3.4	Natural Resources Wales shall be notified within 14 days of the occurrence of the following matters, except where such disclosure is prohibited by Stock Exchange rules:

Where the operator is a registered company:

#### any change in the operator’s trading name, registered name or registered office address; and

#### (b)	any steps taken with a view to the operator going into administration, entering into a company voluntary arrangement or being wound up.

Where the operator is a corporate body other than a registered company:

#### any change in the operator’s name or address; and

#### any steps taken with a view to the dissolution of the operator.

#### In any other case:

#### the death of any of the named operators (where the operator consists of more than one named individual);

#### any change in the operator’s name(s) or address(es); and

#### any steps taken with a view to the operator, or any one of them, going into bankruptcy, entering into a composition or arrangement with creditors, or, in the case of them being in a partnership, dissolving the partnership.

### 4.3.5	Where the operator proposes to make a change in the nature or functioning, or an extension of the activities, which may have consequences for the environment and the change is not otherwise the subject of an application for approval under the Regulations or this permit:

### Natural Resources Wales shall be notified at least 14 days before making the change; and

### the notification shall contain a description of the proposed change in operation.

## 4.4	Interpretation

### 4.4.1	In this permit the expressions listed in schedule 6 shall have the meaning given in that schedule.

### 4.4.2	In this permit references to reports and notifications mean written reports and notifications, except where reference is made to notification being made “without delay”, in which case it may be provided by telephone.

Schedule 1 - Operations

| ### Table S1.1 activities |  |  |
| --- | --- | --- |
| ### Activity reference | ### Description of activities for waste operations | ### Limits of activities |
| ### A1: ### Inert Landfill | ### D5: Specially engineered landfill (e.g. placement into lined discrete cells which are capped and isolated from one another and the environment, etc.) | Receipt, handling, storage and disposal of wastes, consisting of the types and quantities specified in condition 2.7, as an integral part of landfilling. |
|  | ### Discharge of site drainage from the landfill | ### From surface water management system to point of entry to controlled waters. |
|  | ### Storage of fuel for operation of plant and equipment | ### Fuel storage tank. |

| ### Table S1.2 Operating techniques |  |  |
| --- | --- | --- |
| ### Description | ### Parts | ### Date Received |
| ### Application ### EPR/RP3337SE/A001 | ### The response to questions B2.1, B2.2, B2.3, B2.5, B2.7, B2.8, B2.9, B2.11, B3.1 of the Application excluding the answers to the following questions: - B2.2.1 to B2.2.6 - B2.2.38 - B2.2.39 - B2.2.41 - B2.2.42 - B2.2.46 - B2.2.47 - B2.2.52 - B2.2.57 - B3.1.2 | ### 18/11/05 |
| ### Response to Schedule 4 Notice dated 3rd April 2006 ### EPR/RP3337SE/A001 | ### All parts | ### 11/05/06 |
| ### Additional information submitted following analysis of existing waste types ### EPR/RP3337SE/A001 | ### All parts | ### 08/09/06 |
| ### Additional information submitted following analysis of existing waste types EPR/RP3337SE/A001 | ### All parts | ### 20/09/06 |
| ### Additional information submitted following analysis of existing waste types ### EPR/RP3337SE/A001 | ### All parts | ### 27/09/06 |
| ### Additional information submitted following analysis of existing waste types ### EPR/RP3337SE/A001 | ### All parts | ### 06/10/06 |
| ### Submission of relevant WAMITAB certificate ### EPR/RP3337SE/A001 | ### All parts | ### 20/12/06 |
| ### Application ### EPR/RP3337SE/V003 | ### Fugitive Particulate Risk Assessment and Management Plan v2 Report No. 3276R/010/02 Appendix C ### Sections: - 2.2. Sources of Fugitive Dust, Mud and Debris, and Control Measures - 2.3. Dust Monitoring and Action Plan | ### 11/09/18 |
| ### Response to Schedule 5 Notice dated 29/10/18 ### EPR/RP3337SE/V003 | ### Noise Management Plan Report No. 3276/R/012/01 ### Sections: - 4.2 Control and Management Measures - 5.0 Action Plan | ### 13/11/18 |
| ### Response to informal request for information sent 22/11/18 ### EPR/RP3337SE/V003 | ### Environmental monitoring plan Report Number: 3276/R/001/1 ### Sections: - 4.1.1 [surface water monitoring points] - 4.1.2 [description of surface water system] - 4.1.6 [surface water action plans] | ### 22/11/18 |
| ### Technical Guidance Document: ‘How to comply with your environmental permit’ | ### All relevant sections | ### N/A |
| ### Application ### EPR/RP3337SE/V004 | ### K03276-BLP-R-ENV-00015 Rev A Monitoring Review Final Version August 22 Issued NRW. Sections: - Table 10. Proposed Action Level for Carbon Dioxide- action levels - Table 11. Proposed Compliance and Action Levels for Methane- action levels | ### 17/08/22 |
| ### Response to Schedule 5 Notice dated 21/09/22 ### EPR/RP3337SE/V004 | ### 3276.4.006 Monitoring Point Locations Rev B | ### 11/10/22 |

| ### Table S1.3 Improvement programme requirements |  |  |
| --- | --- | --- |
| ### Reference | ### Requirement | ### Date |
| ### 1.3.1 | ### The operator shall submit to Natural Resources Wales for approval, revised Interim Groundwater Monitoring and Contingency Action Plans for the boreholes installed in accordance with pre-operational condition Table S1.4 – reference 3. These plans shall be based on the Environment Agency guidance LFTGN01 “Hydrological Risk Assessment for Landfills” and LFTGN02 “Guidance on Monitoring of Landfill Leachate, Groundwater and Surface Water”. ### The plans shall be implemented by the Operator from the date of approval in writing by Natural Resources Wales. | ### 01/06/07 |
| ### 1.3.2 | ### The Operator shall obtain 12 months of new groundwater monitoring data at the frequencies specified for the substances listed in tables S4.2 and S4.6 from all monitoring boreholes required under Table S1.3 reference 1.3.1 and Table S1.4 reference 3. ### A report shall be sent to Natural Resources Wales for approval that: - Fully characterises the groundwater regime at the site. This shall include all groundwater level and quality data; characterises variations in groundwater levels; determines the groundwater flow direction; provides a fully annotated, scaled contoured plan of groundwater contours within the Carboniferous Limestone aquifer across the site; provides full characterisation and interpretation of groundwater quality in the inflow and outflow zones. The report shall also include a plan/drawing indicating all groundwater and other monitoring points; - Reviews the 12 months of new groundwater monitoring data from the monitoring boreholes; - Undertakes a detailed review of the Control Levels and Trigger Limits included in this Permit. The review shall include revised post-interim Control Levels and Trigger Limits derived in accordance with Agency guidance document LFTGN01 “Hydrological Risk Assessment for Landfills”; - Considers the need for the installation of additional groundwater monitoring boreholes to demonstrate compliance with the requirements of the Groundwater Regulations and the Landfill Regulations; - Includes a revised and updated Site Conceptual Model including detailed, appropriately-scaled and fully annotated cross-section drawing(s) that incudes detailed information required under other Improvement conditions and pre-operational conditions to fully revise the understanding and interactions of the groundwater regime, the geology and wastes at the site; - Includes revised numerical modelling demonstrating that the site complies with the Groundwater Regulations. This modelling shall include containment engineering that complies with the requirements of the Landfill Regulations and fully justified modelled containment Source Terms. ### The proposals and plans in the report shall be implemented by the Operator from the date of approval in writing by Natural Resources Wales. | ### Within 15 months following installation of groundwater monitoring boreholes required under pre-operational condition Table S1.4 reference 3. ### Improvement condition completed |
| ### 1.3.3 | ### Following approval of the report submitted Improvement programme reference 1.3.2, the Operator shall submit to Natural Resources Wales for approval, revised post-interim Groundwater monitoring and Contingency Action Plans. These plans shall include all appropriate information as required under Improvement conditions under Table S1.3 reference 1.3.2 and Table S1.4 reference 3. ### The plans shall be implemented by the Operator from the date of approval in writing by Natural Resources Wales. | ### Within 2 months of the completion of improvement programme reference 1.3.2 |
| ### 1.3.4 | ### The Operator shall carry out a survey of all existing gas monitoring boreholes G1, G2 and G3 (as indicated on drawing LO6 of the Working Plan, Appendix 2, submitted with the PPC Application) to demonstrate that they are fit-for-purpose. A report on the results of this survey shall be submitted to Natural Resources Wales for approval. The survey shall: - Confirm the depth / level to which the borehole extends beneath the existing waste/ground surface; - Confirm that the installation details of these boreholes is adequate, including construction materials, slotted and plain casings, grouting materials with the depths/ elevations of these materials, and borehole security measures; - Confirm whether these boreholes are blocked, and, if blocked, provide mitigation measures; - Propose additional measures to ensure that the existing gas monitoring boreholes are secure; - If the above cannot be fully completed, the operator shall submit proposals to install new gas monitoring points to comply with the requirements of the Landfill regulations and LF TGN03 “Guidance on the Management of Landfill Gas”. Once approved by Natural Resources Wales proposed gas monitoring locations shall be installed / commissioned within the agreed time and included immediately into the agreed interim and post-interim revised Landfill Gas Management and Contingency Action plans. ### The proposals in the report shall be implemented by the Operator from the date of approval in writing by Natural Resources Wales. | ### 01/07/07 |
| ### 1.3.5 | ### The Operator shall submit to Natural Resources Wales for approval revised interim Landfill Gas Management and Contingency plans. These plans shall incorporate detailed written proposals for the design, construction and installation of gas monitoring locations and infrastructure in accordance with Natural Resources Wales guidance document LF TGN03 “Guidance on the Management of Landfill Gas”. These plans shall ensure that the representative sampling of gas concentration within the waste mass and gas concentrations beyond the site boundary from potential lateral gas migration is completed in accordance with the Landfill Regulations. ### Gas monitoring locations shall include, as a minimum an adequate number of locations: - Within the waste mass to ensure compliance with in-waste monitoring requirements; - Beyond the site boundary to ensure compliance with the requirements for the monitoring of lateral emissions. ### Once approved by Natural Resources Wales, the gas monitoring locations shall be installed / commissioned within the agreed time period, and included immediately into the agreed and revised Landfill Gas Management and Contingency Action Plans. ### These proposals shall be implemented by the Operator from the date of approval in writing by Natural Resources Wales. | ### 01/07/07 |
| ### 1.3.6 | ### The Operator shall acquire 12 months of monthly gas monitoring data for all external locations beyond the site boundary required under Improvement condition reference 1.3.5 in Table S1.3 (above). ### A written report shall be submitted to Natural Resources Wales for approval that: - Undertakes a review of the Control Levels and Compliance Limits in this Permit; - Includes, where justified, revised post-interim Control Levels and Compliance Limits for Agency approval. ### The proposals in the report shall be implemented by the Operator from the date of approval in writing by Natural Resources Wales. | ### Within 15 months of the completion of improvement programme reference 1.3.5 ### Improvement condition completed |
| ### 1.3.7 | ### Following approval of the report submitted under Improvement programme reference 1.3.6, The Operator shall submit to Natural Resources Wales for approval, revised post-interim Landfill Gas Management and Contingency Action Plans. These plans shall include all other appropriate information as required under improvement conditions under Table S1.3 references 1.3.4, 1.3.5 and 1.3.6. ### The plans shall be implemented by the Operator from the date of approval in writing by Natural Resources Wales. | ### Within 2 months of the completion of improvement programme reference 1.3.6 |
| ### 1.3.8 | ### The Operator shall submit to Natural Resources Wales for approval interim Surface Water Management and Contingency Action Plans. These plans shall be based on the Environment Agency guidance LFTGN02 “Guidance on Monitoring of Landfill Leachate, Groundwater and Surface Water”. ### These plans shall include additional measures to mitigate against the potential for pollution to the adjacent surface water regime. This includes potential pollution arising from runoff that flows through and from the existing waste materials already deposited under the Waste Management Licence, and from liquids accumulated within and pumped from eh quarry sump. The need for additional monitoring points shall be reviewed. ### The proposals shall be implemented by the Operator from the date of approval in writing by Natural Resources Wales. | ### 01/06/07 |
| ### 1.3.9 | ### The operator shall; - Acquire 12 months of new surface water monitoring data at the frequencies specified in Tables S4.1 and S4.5 for locations SW1, SW2a, SW2b, SW3, and SW4 (indicated on drawing 3276/4/006 Rev A Monitoring Point Location drawing in Table S1.2. of this permit) and using this new data: - Undertake a review of the quality determinands for surface water Control Levels and compliance limits where included in this permit; - Provide a report to Natural Resources Wales including, where justified, revised post-interim Control Levels and Compliance Limits with appropriately justified determinands at each appropriately justified compliance monitoring points for Natural Resources Wales approval. ### The proposals set out in the report shall be implemented by the Operator from the date of approval in writing by Natural Resources Wales. | ### 1st May 2020 ### Improvement condition completed |
| ### 1.3.10 | ### Following approval of the report submitted under Improvement programme reference 1.3.9, the Operator shall submit revised post-interim Surface Water Management and Contingency Action Plans. These plans shall include all other appropriate information as required under improvement condition under Table S1.3 reference 1.3.8 and 1.3.9. ### These plans shall be implemented by the Operator from the date of approval in writing by Natural Resources Wales. | ### Within 2 months of the completion of improvement programme reference 1.3.9 |
| ### 1.3.11 | ### The Operator shall submit a drawing / plan to Natural Resources Wales confirming all monitoring locations. This drawing / plan shall be scaled, and annotated with clear and consistent nomenclature for al monitoring locations. This drawing / plan shall also include local features such as haulage and other roads, field boundaries, surface water features, indicated map north, ground elevation information, buildings, services & infrastructures, completed / restored parts of the landfill and security fencing details. ### The notification requirements of condition 2.5.2 shall be deemed to have been complied with on submission of the drawing / plan, | ### 01/07/07 |
| ### 1.3.12 | ### The Operator shall submit a report to Natural Resources Wales regarding nitrate concentrations in the surface water regime. For this report, the Operator shall investigate and determine the source(s) of the observed high concentrations of nitrate in the surface water regime. The Operator shall submit the results of this investigation, and include proposals to mitigate against future potentially-polluting releases of nitrate from the site to the surface water regime. ### The proposals shall be implemented by the Operator from the date of approval in writing by Natural Resources Wales. | ### 01/10/07 |

| ### Table S1.4 Pre-operational measures |  |
| --- | --- |
| ### Reference | ### Pre-operational measures |
| ### 1 | ### The Operator shall submit to Natural Resources Wales for approval details for a Landfill Regulations-compliant engineered geological barrier to be installed onto the quarry base and full height of the sidewalls in any unfilled area where new landfilling is proposed. Where the newly constructed geological barrier is to be constructed adjacent to existing waste deposits, provision shall be made for an extension of the barrier over the lower section of the existing waste slope. The submitted details shall include a plan clearly identifying where engineered basal and sidewall geological barrier is required. ### A revised Stability Risk Assessment shall be submitted to Natural Resources Wales for approval. This Stability Risk Assessment shall adequately investigate and include details of the engineered geological barrier installed onto the quarry base and sidewalls. Account shall be taken of any required groundwater control systems and potential groundwater pressures acting behind the constructed geological barrier. ### Following Agency approval, the operator shall install the geological barrier in accordance with the agreed design. A full construction quality assurance (CQA) programme shall be completed and a CQA Validation Report provided to Natural Resources Wales following engineering installation prior to waste infilling in accordance with the requirements of section 2.7 of this permit. |
| ### 2 | ### Prior to construction of the sidewall geological barrier the Operator shall ensure that the basal and sidewall subgrade is stable and free from loose or significantly jointed/fractured rock which may displace under loading from the landfilled waste, thereby resulting in integrity failure of the barrier. Any such remedial measures shall be required and reported through the Construction Quality Assurance procedures for the lining works as detailed above in pre-operational condition reference 1. |
| ### 3 | ### The Operator shall submit to Natural Resources Wales for written proposals, including timescales, for the installation of groundwater monitoring boreholes. A minimum of four boreholes shall be included. There shall be located and installed within the Carboniferous Limestone aquifer adjacent to the site boundary and beyond areas of exiting or proposed waste infilling. The boreholes shall not be installed into the existing wastes. The boreholes shall be designed and installed so as to enable adequate monitoring of the groundwater inflow (or hydraulically-upgradient) and outflow (or downgradient) regions to demonstrate that the site fully complies with the requirements of the Groundwater Regulations and the Landfill Regulations. The proposals shall include appropriately detailed construction and Installation information, and a suitably scaled and annotated drawing / plan showing the locations and survey levels of all groundwater monitoring installations. ### The proposals shall include the location and installation details, and be designed and installed in accordance with Environment Agency documents LFTGN01 'Hydrogeological Risk Assessment for Landfills' and LFTGNO2 'Guidance on Monitoring of Landfill Leachate, Groundwater and Surface Water'. ### Once approved, the boreholes shall be installed and commissioned within the agreed time period, and included immediately into the agreed Groundwater Monitoring and Contingency Action plans required under Table S1.3 references 1.3.1 and 1.3.3. |

# Schedule 2 - Waste types, raw materials and fuels

| Table S2.1 Permitted waste types and quantities for inert landfill |  |
| --- | --- |
| Maximum quantity | The maximum quantity of waste to be accepted on site shall not exceed 125,000 tonnes per year. |
| Exclusions | Notwithstanding the waste types set out in this table, wastes having any of the following characteristics shall not be accepted: - odorous or odour producing - hazardous wastes - construction and demolition waste from constructions polluted with inorganic or organic hazardous substance e.g. because of production processes in the construction, spoil pollution, storage and use of pesticides or other hazardous substances, etc., unless it is made clear that the demolished construction was not significantly polluted - No construction and demolition waste from constructions, treated, covered or painted with materials containing hazardous substances in significant amounts |
| Waste code | Description |
| 10 | WASTES FROM THERMAL PROCESSES |
| 10 11 | wastes from manufacture of glass and glass products |
| 10 11 03 | waste glass-based fibrous materials excluding those with organic binders |
| 15 | WASTE PACKAGING, ABSORBENTS, WIPING CLOTHS, FILTER MATERIALS AND PROTECTIVE CLOTHING NOT OTHERWISE SPECIFIED |
| 15 01 | packaging (including separately collected municipal packaging waste) |
| 15 01 07 | glass packaging |
| 17 | CONSTRUCTION AND DEMOLITION WASTES (INCLUDING EXCAVATED SOIL FROM CONTAMINATED SITES) |
| 17 01 | concrete, bricks, tiles and ceramics |
| 17 01 01 | concrete |
| 17 01 02 | Bricks1 |
| 17 01 03 | tiles and ceramics1 |
| 17 01 07 | mixtures of concrete, bricks, tiles and ceramics other than those mentioned in 17 01 061 |
| 17 02 | wood, glass and plastic |
| 17 02 02 | glass |
| 17 05 | soil (including excavated soil from contaminated sites), stones and dredging spoil |
| 17 05 04 | soil and stones other than those mentioned in 17 05 03 |
| 19 | WASTES FROM WASTE MANAGEMENT FACILITIES, OFF-SITE WASTE WATER TREATMENT PLANTS AND THE PREPARATION OF WATER INTENDED FOR HUMAN CONSUMPTION AND WATER FOR INDUSTRIAL USE |
| 19 12 | wastes from the mechanical treatment of waste (for example sorting, crushing, compacting, pelletising) not otherwise specified |
| 19 12 05 | glass |
| 20 | MUNICIPAL WASTES (HOUSEHOLD WASTE AND SIMILAR COMMERCIAL, INDUSTRIAL AND INSTITUTIONAL WASTES) INCLUDING SEPARATELY COLLECTED FRACTIONS |
| 20 01 | separately collected fractions (except 15 01) |
| 20 01 02 | glass |
| 20 02 | garden and park wastes (including cemetery waste) |
| 20 02 02 | soil and stones |

# Schedule 3 – Emissions and monitoring

| ### Table S3.1 Point Source emissions to water (other than sewer) and land – emission limits and monitoring requirements |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ### Emission point ref. & location | ### Parameter | ### Source | ### Limit (incl. unit) | ### Limits – other (incl. unit) | ### Reference Period | ### Monitoring frequency | ### Monitoring standard or method |
| ### NGR: SH48081 81037 (Point SW4 as indicated on 3276.04.006 Rev B Monitoring Point Location drawing as incorporated into Table S1.2.) Or as otherwise agreed in writing by Natural Resources Wales | ### Suspended solids | ### Surface water runoff and surface water courses | ### 100 mg/l | ### 50 mg/l on any 2 consecutive occasions | ### Spot sample | ### Quarterly | ### In accordance with Monitoring of landfill leachate, groundwater and surface water: LFTGN 02, issued February 2003, unless otherwise agreed in writing with Natural Resources Wales. |
|  | ### Sulphate |  | ### 250 mgSO4/l | ### - |  |  |  |
|  | ### Phosphate (as Phosphorus) |  | ### 1.2 mgPO4-P/l | ### 0.1 mgPO4-P/l on any 2 consecutive occasions |  |  |  |
|  | ### Oil & Grease |  | ### None visible | ### - |  |  |  |

| ### Table S3.2 Trigger levels for emissions into groundwater and monitoring requirements |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ### Monitoring point reference | ### Parameter | ### Limit (incl. unit) | ### Reference Period | ### Monitoring frequency | ### Monitoring standard or method |
| ### Boreholes BH2D and BH5D as indicated on 3276.04.006 Rev B Monitoring Point Location drawing as incorporated into Table S1.2. | ### Chloride | ### 65 mg/l | ### Spot sample | ### Quarterly | ### In accordance with Monitoring of landfill leachate, groundwater and surface water: LFTGN 02, issued February 2003, unless otherwise agreed in writing with Natural Resources Wales. |
|  | ### Arsenic | ### 0.005 mg/l |  |  |  |
|  | ### Nickel | ### 0.02 mg/l |  |  |  |
|  | ### Zinc | ### 0.13 mg/l |  |  |  |
|  | ### Ammoniacal Nitrogen | ### 0.39 mgNH4-N/l |  |  |  |
| ### Borehole BH2D as indicated on 3276.04.006 Rev B Monitoring Point Location drawing as incorporated into Table S1.2. | ### Sulphate | ### 250 mgSO4/l |  |  |  |

| ### Table S3.3 Landfill gas in external monitoring boreholes- limits and monitoring requirements |  |  |  |  |
| --- | --- | --- | --- | --- |
| ### Monitoring point reference/description | ### Parameter | ### Limit (incl. unit) | ### Monitoring frequency | ### Monitoring standard or method |
| ### Boreholes BH1S and BH2S as indicated on 3276.04.006 Rev B Monitoring Point Location drawing as incorporated into Table S1.2. | ### Carbon Dioxide (CO2) | ### - | ### Quarterly | ### In accordance with Management of landfill gas: LFTGN 03, issued September 2004, unless otherwise agreed in writing with Natural Resources Wales. |
|  | ### Methane (CH4) | ### 1.1% v/v |  |  |
| ### Borehole BH4D as indicated on 3276.04.006 Rev B Monitoring Point Location drawing as incorporated into Table S1.2. | ### Carbon Dioxide (CO2) | ### - |  |  |
|  | ### Methane (CH4) | ### - |  |  |
| ### Boreholes BH1S, BH2S and BH4D as indicated on 3276.04.006 Rev B Monitoring Point Location drawing as incorporated into Table S1.2. | ### Oxygen | ### - |  | ### In accordance with the application and the requirements of Table S1.3, condition references 1.3.7, unless otherwise agreed in writing with Natural Resources Wales |
|  | ### Atmospheric pressure | ### - |  |  |
|  | ### Differential Pressure | ### - |  |  |
|  | ### Meteorological data | ### - |  |  |

| ### Table S3.4 Surface water – other monitoring requirements |  |  |  |  |
| --- | --- | --- | --- | --- |
| ### Location or description of point of measurement | ### Parameter | ### Monitoring frequency | ### Monitoring standard or method | ### Other specifications |
| ### Surface water monitoring points SW1, SW2a, SW2b, SW3, and SW4 (as indicated on 3276.04.006 Rev B Monitoring Point Location drawing as incorporated into Table S1.2). ### Or as otherwise agreed in writing by Natural Resources Wales | ### pH | ### Quarterly | ### In accordance with Monitoring of landfill leachate, groundwater and surface water: LFTGN 02, issued February 2003, unless otherwise agreed in writing with Natural Resources Wales. | ### All Hazardous Substances (formally List I substances) shall be screened to the MRV limits unless otherwise agreed in writing with Natural Resources Wales. |
|  | ### Oil & Grease |  |  |  |
|  | ### Temperature |  |  |  |
|  | ### Electrical Conductivity |  |  |  |
|  | ### Ammoniacal Nitrogen |  |  |  |
|  | ### Chloride |  |  |  |
|  | ### BOD |  |  |  |
|  | ### COD |  |  |  |
|  | ### Total Suspended Solids |  |  |  |
|  | ### TOC |  |  |  |
|  | ### Iron |  |  |  |
|  | ### Chromium |  |  |  |
|  | ### Manganese |  |  |  |
|  | ### Nickel |  |  |  |
|  | ### Lead |  |  |  |
|  | ### Phosphate as P |  |  |  |
|  | ### SO4 |  |  |  |
|  | ### Nitrate (for TON) |  |  |  |
|  | ### Nitrite (for TON) |  |  |  |
|  | ### Dissolved Oxygen |  |  |  |
|  | ### Mercury |  |  |  |
|  | ### Arsenic |  |  |  |
|  | ### PAH |  |  |  |
|  | ### Total Oxidised Nitrogen | ### Annually |  |  |
|  | ### Calcium |  |  |  |
|  | ### Cadmium |  |  |  |
|  | ### Magnesium |  |  |  |
|  | ### Sodium |  |  |  |
|  | ### Potassium |  |  |  |
|  | ### Alkalinity (CaCO3) |  |  |  |
|  | ### Copper |  |  |  |
|  | ### Zinc |  |  |  |
|  | ### TPH (including Mineral oil C8 to C40) |  |  |  |
|  | ### Mecoprop |  |  |  |
|  | ### Dibromomethane |  |  |  |
|  | ### Full Hazardous Substances (formally List I substances) screening | ### Every two years |  |  |

| ### Table S3.5 Groundwater – other monitoring requirements |  |  |  |  |
| --- | --- | --- | --- | --- |
| ### Location or description of point of measurement | ### Parameter | ### Monitoring frequency | ### Monitoring standard or method | ### Other specifications |
| ### Boreholes BH2D, BH3DA, BH4D, and BH5D as indicated on 3276.04.006 Rev B Monitoring Point Location drawing as incorporated into Table S1.2. | ### Water level | ### Quarterly | ### Spot sample | ### All Hazardous Substances (formally List I substances) shall be screened to the MRV limits unless otherwise agreed in writing with Natural Resources Wales. |
|  | ### Temperature |  |  |  |
|  | ### pH |  |  |  |
|  | ### Electrical Conductivity |  |  |  |
|  | ### Dissolved oxygen |  |  |  |
|  | ### Chloride |  |  |  |
|  | ### Sulphate |  |  |  |
|  | ### Arsenic |  |  |  |
|  | ### Nickel |  |  |  |
|  | ### Zinc |  |  |  |
|  | ### Ammoniacal Nitrogen |  |  |  |
|  | ### PAH |  |  |  |
|  | ### Alkalinity | ### Annually |  |  |
|  | ### Calcium |  |  |  |
|  | ### Chromium |  |  |  |
|  | ### COD |  |  |  |
|  | ### Copper |  |  |  |
|  | ### Magnesium |  |  |  |
|  | ### Manganese |  |  |  |
|  | ### Lead |  |  |  |
|  | ### Mercury |  |  |  |
|  | ### Nitrate (for TON) |  |  |  |
|  | ### Nitrite (for TON) |  |  |  |
|  | ### Phenols Monohydric |  |  |  |
|  | ### Phosphate as P |  |  |  |
|  | ### Potassium |  |  |  |
|  | ### Selenium |  |  |  |
|  | ### Sodium |  |  |  |
|  | ### TOC |  |  |  |
|  | ### MTBE |  |  |  |
|  | ### Iron |  |  |  |
|  | ### TPH (including Mineral oil C8 to C40) |  |  |  |
|  | ### Diazinon |  |  |  |
|  | ### Fluoride |  |  |  |
|  | ### Full Hazardous Substances (formally List I substances) screening | ### Every two years |  |  |

# Schedule 4 - Reporting

### Parameters, for which reports shall be made, in accordance with conditions of this permit, are listed below.

| ### Table S4.1 Reporting of monitoring data |  |  |  |
| --- | --- | --- | --- |
| ### Parameter | ### Emission or monitoring point/reference | ### Reporting period | ### Period begins |
| ### Emissions to water ### Parameters as required by condition 3.6.1 | ### Surface water monitoring point SW4 (as indicated on 3276/4/006 Rev A Monitoring Point Location drawing of the plan incorporated into Table S1.2.) ### Or as otherwise agreed in writing by Natural Resources Wales | ### Quarterly | ### 30/03/07 |
| ### Groundwater ### Parameters as required by condition 3.6.1 | ### Groundwater monitoring points to be installed in accordance with Table S1.4, condition reference 3. | ### Quarterly | ### 30/03/07 |
| ### Hazardous Substances (formally List I substances) screening |  | ### Every two years |  |
| ### Landfill gas lateral migration ### Parameters as required by condition 3.6.1 | ### Existing in-waste monitoring borehole G1 and G2 (as indicated on Drawing LO6 of the existing Working Plan, Appendix 2, submitted with the PPC Application) | ### Quarterly | ### 30/03/07 |
|  | ### New landfill gas monitoring points to be installed beyond the site boundary in accordance with Table S1.3, condition reference 1.3.5 |  | ### After date of installation |
| ### Other Landfill gas monitoring ### Parameters are required by condition 3.6.1 | ### Existing in-waste monitoring borehole G3 (as indicated on Drawing LO6 of the existing Working Plan, Appendix 2, submitted with the PPC Application) | ### Quarterly | ### 30/03/07 |
|  | ### New landfill gas monitoring points to be installed beyond the site boundary in accordance with Table S1.3, condition reference 1.3.5 |  | ### After date of installation |
| ### Other surface water monitoring Parameters as required by condition 3.6.1 | ### Surface water monitoring points SW1, SW2a, SW2b, SW3, SW4 (as indicated on drawing 3276/4/006 of the plan incorporated into Table S1.2.) ### Or as otherwise agreed in writing by Natural Resources Wales | ### Quarterly | ### 30/03/07 |
| ### Hazardous Substances (formally List I substances) screening |  | ### Every two years |  |

| ### Table S4.2: Annual production/treatment |  |
| --- | --- |
| ### Parameter | ### Units |
| ### Surface water and/or groundwater: - Disposed of off site - Disposed of to any onsite effluent treatment plant | ### Cubic meters/year |

| ### Table S4.3 Reporting forms |  |  |
| --- | --- | --- |
| ### Media/parameter | ### Reporting format | ### Date of form |
| ### Controlled water | ### Form Water1 or other reporting format to be agreed in writing with Natural Resources Wales | ### 19/01/23 |
| ### Groundwater | ### Form Groundwater1 or other form as agreed in writing by Natural Resources Wales | ### 19/01/23 |
| ### Landfill gas | ### Form Air1 or other form as agreed in writing by Natural Resources Wales | ### 19/01/23 |
| ### Landfill topographical surveys and interpretation | ### Reporting format to be agreed in writing with Natural Resources Wales | ### n/a |

# Schedule 5 - Notification

### These pages outline the information that the operator must provide.

### Units of measurement used in information supplied under Part A and B requirements shall be appropriate to the circumstances of the emission. Where appropriate, a comparison should be made of actual emissions and authorised emission limits.

#### If any information is considered commercially confidential, it should be separated from non-confidential information, supplied on a separate sheet and accompanied by an application for commercial confidentiality under the provisions of the EP Regulations.

## Part A

| ## Permit Number |  |
| --- | --- |
| ## Name of operator |  |
| ## Location of Facility |  |
| ## Time and date of the detection |  |

| ## (a) Notification requirements for any malfunction, breakdown or failure of equipment or techniques, accident, or emission of a substance not controlled by an emission limit which has caused, is causing or may cause significant pollution |  |
| --- | --- |
| ## To be notified within 24 hours of detection |  |
| ## Date and time of the event |  |
| ## Reference or description of the location of the event |  |
| ## Description of where any release into the environment took place |  |
| ## Substances(s) potentially released |  |
| ## Best estimate of the quantity or rate of release of substances |  |
| ## Measures taken, or intended to be taken, to stop any emission |  |
| ## Description of the failure or accident. |  |

| ## (b) Notification requirements for the breach of a limit |  |
| --- | --- |
| ## To be notified within 24 hours of detection unless otherwise specified below |  |
| ## Emission point reference/ source |  |
| ## Parameter(s) |  |
| ## Limit |  |
| ## Measured value and uncertainty |  |
| ## Date and time of monitoring |  |
| ## Measures taken, or intended to be taken, to stop the emission |  |

| ### Time periods for notification following detection of a breach of a limit |  |
| --- | --- |
| ### Parameter | ### Notification period |

| ## (c) Notification requirements for the detection of any significant adverse environmental effect |  |
| --- | --- |
| ## To be notified within 24 hours of detection |  |
| ## Description of where the effect on the environment was detected |  |
| ## Substances(s) detected |  |
| ## Concentrations of substances detected |  |
| ## Date of monitoring/sampling |  |

## Part B - to be submitted as soon as practicable

| ### Any more accurate information on the matters for notification under Part A. |  |
| --- | --- |
| ### Measures taken, or intended to be taken, to prevent a recurrence of the incident |  |
| ### Measures taken, or intended to be taken, to rectify, limit or prevent any pollution of the environment which has been or may be caused by the emission |  |
| ### The dates of any unauthorised emissions from the facility in the preceding 24 months. |  |

| ### Name* |  |
| --- | --- |
| ### Post |  |
| ### Signature |  |
| ### Date |  |

### * authorised to sign on behalf of the operator

Schedule 6 - Interpretation

“accident” means an accident that may result in pollution.

“Annex I” means Annex I to Directive 2008/98/EC of the European Parliament and of the Council on waste.

“annually” means once every year.

“application” means the application for this permit, together with any additional information supplied by the operator as part of the application and any response to a notice served under Schedule 5 to the EP Regulations.

“authorised officer” means any person authorised by Natural Resources Wales under section 108(1) of The Environment Act 1995 to exercise, in accordance with the terms of any such authorisation, any power specified in section 108(4) of that Act.

“Construction Proposals”;

- for new cells, means written information at a level of detail appropriate to the complexity and pollution risk, on stability assessment (where relevant) and the construction quality assurance (CQA) programme in relation to the new cell.

- for landfill infrastructure, means the design, specifications of materials selected, stability assessment (where relevant) and the construction quality assurance (CQA) programme in relation to the landfill infrastructure.

“CQA Validation Report” means the final “as built” construction and engineering details of the new cell or of the Landfill Infrastructure. It must provide a comprehensive record of the construction and must include, where relevant:

- The results of all testing required by the CQA programme - this must include the records of any failed tests with a written explanation, details of the remedial action taken, referenced to the appropriate secondary testing;

- Plans showing the location of all tests;

- “As-built” plans and sections of the works;

- Copies of the site engineer’s daily records;

- Records of any problems or non-compliance and the solution applied;

- Any other site specific information considered relevant to proving the integrity of the new cell or Landfill Infrastructure;

- Validation by a qualified person that all of the construction has been carried out in accordance with the construction proposals.

“D” means a disposal operation provided for in Annex I to Directive 2008/98/EC of the European Parliament and of the Council on waste.

“emissions to land” includes emissions to groundwater.

“emissions of substances not controlled by emission limits” means emissions of substances to air, water or land from the activities, either from the emission points specified in schedule 3 or from other localised or diffuse sources, which are not controlled by an emission or background concentration limit.

“EP Regulations” means The Environmental Permitting (England and Wales) Regulations SI 2016 No.1154 and words and expressions used in this permit which are also used in the Regulations have the same meanings as in those Regulations.

“groundwater” means all water, which is below the surface of the ground in the saturation zone and in direct contact with the ground or subsoil.

“groundwater protection zones 1 and 2” have the meaning given in the document titled Groundwater Protection: Policy and Practice" published by the Environment Agency in 2006.

“hazardous property” has the meaning in Annex lll of the Waste Framework Directive.

“hazardous substance” means a substance classified as hazardous as a consequence of fulfilling the criteria laid down in parts 2 to 5 of Annex I to Regulation (EC) No 1272/2008.

“hazardous waste” has the meaning given in the Hazardous Waste (Wales) Regulations 2005 (as amended).

“impermeable surface” means a surface or pavement constructed and maintained to a standard sufficient to prevent the transmission of liquids beyond the surface.

“Landfill Infrastructure” means any specified element of the:

- surface water drainage systems;

- groundwater monitoring boreholes;

- landfill gas monitoring boreholes;

within the Site.

“Liquids” means any liquid other than leachate within the landfill.

“New Cell” means any new cell, part of a cell or other similar new area of the Site where waste deposit is to commence after issue of this permit and can comprise:

- groundwater under-drainage system;

- sub-grade;

- artificially established geological barriers;

- cell or area surface water drainage system;

- side wall subgrade and containment systems;

for the new cell.

“No impact” means that the change made to the construction process will not alter the agreed design criteria, specification or performance in a way that has a negative effect.

“quarter” means a calendar year quarter commencing on 1 January, 1 April, 1 July or 1 October.

“Waste code” means the six digit code referable to a type of waste in accordance with the list of wastes established by Commission Decision 2000/532/EC as amended from time to time (the ‘List of Wastes Decision’) and in relation to hazardous waste, includes the asterisk.

“Waste Framework Directive” or “WFD” means Waste Framework Directive 2008/98/EC of the European Parliament and of the Council on waste.

“year” means calendar year ending 31 December.

Where a minimum limit is set for any emission parameter, for example pH, reference to exceeding the limit shall mean that the parameter shall not be less then the limit.

Schedule 7 - Site plan

END OF PERMIT

Permit Number: RP3337SE

Operator: Hurt Plant Hire Ltd

Facility: Nant Newydd Quarry Landfill Site

Form Number: Water1 / 19/01/23

# Reporting of emissions to water (other than to sewer) and land for the period from DD/MM/YYYY to DD/MM/YYYY

| Emission Point | Substance/Parameter | Emission Limit Value | Reference Period | Result[1] | Test Method[2] | Sample Date and Times[3] | Uncertainty[4] |
| --- | --- | --- | --- | --- | --- | --- | --- |

- The result given is the maximum value (or the minimum value in the case of a limit that is expressed as a minimum) obtained during the reporting period, expressed in the same terms as the emission limit value. Where the emission limit value is expressed as a range, the result is given as the ‘minimum – maximum’ measured values.

- Where an internationally recognised standard test method is used the reference number is given. Where another method that has been formally agreed with Natural Resources Wales is used, then the appropriate identifier is given. In other cases the principal technique is stated, for example gas chromatography.

- For non-continuous measurements the date and time of the sample that produced the result is given. For continuous measurements the percentage of the process operating time covered by the result is given.

- The uncertainty associated with the quoted result at the 95% confidence interval, unless otherwise stated.

Signed …………………………………………………………….  		Date………………………..

(Authorised to sign as representative of Operator)

Permit Number: RP3337SE

Operator: Hurt Plant Hire Ltd

Facility: Nant Newydd Quarry Landfill Site

Form Number: Groundwater1 / 19/01/23

# Reporting of groundwater monitoring for the period from DD/MM/YYYY to DD/MM/YYYY

| Monitoring Point | Substance/Parameter | # Trigger level | Reference Period | Result[1] | Test Method[2] | Sample Date and Times[3] | Uncertainty[4] |
| --- | --- | --- | --- | --- | --- | --- | --- |

- The result given is the maximum value (or the minimum value in the case of a limit that is expressed as a minimum) obtained during the reporting period, expressed in the same terms as the emission limit value. Where the emission limit value is expressed as a range, the result is given as the ‘minimum – maximum’ measured values.

- Where an internationally recognised standard test method is used the reference number is given. Where another method that has been formally agreed with Natural Resources Wales is used, then the appropriate identifier is given. In other cases the principal technique is stated, for example gas chromatography.

- For non-continuous measurements the date and time of the sample that produced the result is given. For continuous measurements the percentage of the process operating time covered by the result is given.

- The uncertainty associated with the quoted result at the 95% confidence interval, unless otherwise stated.

Signed …………………………………………………………….  		Date………………………..

(Authorised to sign as representative of Operator)

Permit Number: RP3337SE

Operator: Hurt Plant Hire Ltd

Facility: Nant Newydd Quarry Landfill Site

Form Number: Air1 / 19/01/23

# Reporting of emissions to air for the period from DD/MM/YYYY to DD/MM/YYYY

| Emission Point | Substance/Parameter | Emission Limit Value | Reference Period | Result[1] | Test Method[2] | Sample Date and Times[3] | Uncertainty[4] |
| --- | --- | --- | --- | --- | --- | --- | --- |

- The result given is the maximum value (or the minimum value in the case of a limit that is expressed as a minimum) obtained during the reporting period, expressed in the same terms as the emission limit value. Where the emission limit value is expressed as a range, the result is given as the ‘minimum – maximum’ measured values.

- Where an internationally recognised standard test method is used the reference number is given. Where another method that has been formally agreed with Natural Resources Wales is used, then the appropriate identifier is given. In other cases the principal technique is stated, for example gas chromatography.

- For non-continuous measurements the date and time of the sample that produced the result is given. For continuous measurements the percentage of the process operating time covered by the result is given.

- The uncertainty associated with the quoted result at the 95% confidence interval, unless otherwise stated.

Signed …………………………………………………………….  		Date………………………..

(Authorised to sign as representative of Operator)

consisting of selected construction and demolition waste with low contents of other types of materials (like metals, plastic, organics, wood, rubber etc.), where the origin of the waste must be known.