Rest

Permit with introductory note

The Environmental Permitting (England & Wales) Regulations 2016

AWD (Group) Ltd

Byass Works

The Docks

Port Talbot

SA13 1RS

Permit number

EPR/AB3895CN

Byass Works

Permit number EPR/AB3895CN/V004

Introductory note

This introductory note does not form a part of the permit

The main features of the permit are as follows.

This is a Tier 3 bespoke permit for a hazardous and non-hazardous waste transfer station with treatment.

Treatment operations are limited to manual or mechanical:

- sorting and/or separation

- screening (inert soils and stones only)

- shredding (plastic waste only)

- crushing (excluding inert soils stones)

- compaction (excluding inert soils stones)

- granulating (plastic waste only)

- dewatering (grinding solids only)

- bulking up for onwards transfer.

All treatment can be for the purpose of recovery or disposal.

The maximum quantity of waste accepted and processed on site shall not exceed 24,999 tonnes per year.

Relevant storage and treatment capacity thresholds have been included in the permit to reflect the legal waste facility operational limits; above which, the site would be operating as an Industrial Emissions Directive installation. (See Table S1.1 for relevant limits.)

This permit does not allow any point source emission into surface waters or groundwater except:

- clean, rainfall dependant drainage from areas of the site not used in connection with the storage and/or treatment of waste, and

- integral discharge(s) from the permitted activity (external areas of the site used in connection with the storage and/or treatment of waste) in accordance with the limits set out in Table S1.1.

There are to be no emissions to surface and/or groundwater from the buildings on site.

All emissions from the activity which are not controlled via actual emission limits, are controlled through the ‘emission of substances not controlled by emission limits condition’ and any all relevant process requirements set out in Tables S1.1 and S1.2.

The burning of any wastes, either in the open, inside buildings or in any form of incinerator is not permitted.

The status log of the permit sets out the permitting history, including any changes to the permit reference number.

| Status log of the permit |  |  |
| --- | --- | --- |
| Description | Date | Comments |
| Permit determined EPR/AB3895CN/A001 | 06/04/18 | Standard Rules Permit SR2008 No3 issued to AWD (Group) Ltd |
| Variation determined EPR/AB3895CN/V002 | 18/09/18 | Variation to increase the site boundary issued. |
| Variation determined EPR/AB3895CN/V003 | 24/09/21 | Variation to increase the site boundary issued. |
| Variation application EPR/AB3895CN/V004 | Duly Made 13/03/22 | Application received to vary the permit to a bespoke permit. |
| Additional information received in response to Schedule 5 notice (dated: 02/09/22) | 04/10/22 | Consisting of: - Updated FPMP (Revision 3 – Sep 2022) - Updated Odour Management Plan (Version 2 – Oct 2022) |
|  | 30/10/22 | Consisting of: - Updated Environmental Risk Assessment (Version 3 – Oct 2022) - Updated Environmental Management System (Version 3 – Oct 2022) - Updated Environmental Accident Management Plan (Version 1 Oct 2022) - Updated Noise Management Plan including Noise Impact Assessment. (Version 2 – Oct 2022) |
|  | 14/12/22 | Consisting of: - Updated FPMP (Revision 4 – Dec 2022) |
| Additional information received in response to Schedule 5 notice (dated: 03/11/22) | 14/12/22 | Consisting of: - Updated FPMP (Revision 4 – Dec 2022) |
| Additional information received by email (dated: 21/12/22) | 21/12/22 | Consisting of: - Updated Environmental Management System (Version 3 – Oct 2022) - Updated Non Technical Summary (Nov 2021) - Updated Environmental Accident Management Plan (Version 1 - Oct 2022) - Updated Environmental Risk Assessment (Version 3 – Oct 2022) - Confirmation of minor change to annual throughput - Contact details for the Operator. |
| Additional information received by email (dated: 09/01/23) | 09/01/22 | Consisting of: - Updated Environmental Management System (Version 3 – Oct 2022) - Updated Non Technical Summary (Nov 2021) - Updated Environmental Risk Assessment (Version 3 – Oct 2022) |
| Additional information received by email 18/01/23 | 18/01/23 | Consisting of: - Updated Fire Prevention and Mitigation Plan (Revision 4 – Dec 2022) |
| Permit determined EPR/AB3895CN | 26/01/23 | Consolidated and varied permit issued to AWD (Group) Ltd. |

End of introductory note

Permit

The Environmental Permitting (England and Wales) Regulations 2016

Permit number

EPR/AB3895CN

The Natural Resources Body for Wales (“Natural Resources Wales”) authorises, under regulation 13 of the Environmental Permitting (England and Wales) Regulations 2016

AWD (Group) Ltd (“the operator”),

whose registered office is

1 Picton Lane

Swansea

City &County of Swansea

SA1 4AF

company registration number 09374296

to operate waste operations at

Byass Works

The Docks

Port Talbot

SA13 1RS

to the extent authorised by and subject to the conditions of this permit.

| Name | Date |
| --- | --- |
| Huw Davies | 26/01/2023 |

Authorised on behalf of Natural Resources Wales

Conditions

# Management

## General management

### The operator shall manage and operate the activities:

#### in accordance with a written management system that identifies and minimises risks of pollution, including those arising from operations, maintenance, accidents, incidents, non-conformances, closure and those drawn to the attention of the operator as a result of complaints; and

#### using sufficient competent persons and resources.

### Records demonstrating compliance with condition 1.1.1 shall be maintained.

### Any person having duties that are or may be affected by the matters set out in this permit shall have convenient access to a copy of it kept at or near the place where those duties are carried out.

### The operator shall comply with the requirements of an approved competence scheme.

## Avoidance, recovery and disposal of wastes produced by the activities

### The operator shall take appropriate measures to ensure that:

### the waste hierarchy referred to in Article 4 of the Waste Framework Directive is applied to the generation of waste by the activities; and

### any waste generated by the activities is treated in accordance with the waste hierarchy referred to in Article 4 of the Waste Framework Directive; and

### where disposal is necessary, this is undertaken in a manner which minimises its impact on the environment.

### The operator shall review and record at least every four years whether changes to those measures should be made and take any further appropriate measures identified by a review.

# 2	Operations

2.1	Permitted activities

### 2.1.1	The operator is only authorised to carry out the activities specified in schedule 1 table S1.1 (the “activities”).

## 2.2	The site

2.2.1	The activities shall not extend beyond the site, being the land shown edged in green on the site plan at schedule 7 to this permit.

## 2.3	Operating techniques

#### 2.3.1	(a)	The activities shall, subject to the conditions of this permit, be operated using the techniques and in the manner described in the documentation specified in schedule 1, table S1.2, unless otherwise agreed in writing by Natural Resources Wales.

#### (b)	If notified by Natural Resources Wales that the activities are giving rise to pollution, the operator shall submit to Natural Resources Wales for approval within the period specified, a revision of any plan or other documentation (“plan”) specified in schedule 1, table S1.2 or otherwise required under this permit which identifies and minimises the risks of pollution relevant to that plan, and shall implement the approved revised plan in place of the original from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

### 2.3.2	Waste shall only be accepted if:

### (a)	it is of a type and quantity listed in schedule 2 table(s) S2.1 [, S2.2 etc.]; and

### (b)	it conforms to the description in the documentation supplied by the producer and holder.

## 2.4	Improvement programme

### 2.4.1	The operator shall complete the improvements specified in schedule 1 table S1.3 by the date specified in that table unless otherwise agreed in writing by Natural Resources Wales.

### 2.4.2	Except in the case of an improvement which consists only of a submission to Natural Resources Wales, the operator shall notify Natural Resources Wales within 14 days of completion of each improvement.

2.5 	Hazardous waste storage and treatment

### 2.5.1	Hazardous waste shall not be mixed, either with a different category of hazardous waste or with other waste, substances or materials, unless it is authorised by schedule 1 table S1.1 and appropriate measures are taken.

# 3	Emissions and monitoring

## 3.1 	Emissions of substances not controlled by emission limits

### 3.1.1 	Emissions of substances not controlled by emission limits (excluding odour) shall not cause pollution. The operator shall not be taken to have breached this condition if appropriate measures, including, but not limited to, those specified in any approved emissions management plan, have been taken to prevent or where that is not practicable, to minimise, those emissions.

3.1.2	The operator shall:

#### (a)	if notified by Natural Resources Wales that the activities are giving rise to pollution, submit to Natural Resources Wales for approval within the period specified, an emissions management plan which identifies and minimises the risks of pollution from emissions of substances not controlled by emission limits;

#### (b)	implement the approved emissions management plan, from the date of approval, unless otherwise agreed in writing by Natural Resources Wales.

3.1.3	All liquids in containers, whose emission to water or land could cause pollution, shall be provided with secondary containment, unless the operator has used other appropriate measures to prevent or where that is not practicable, to minimise, leakage and spillage from the primary container.

## 3.2	Odour

#### 3.2.1	Emissions from the activities shall be free from odour at levels likely to cause pollution outside the site, as perceived by an authorised officer of Natural Resources Wales, unless the operator has used appropriate measures, including, but not limited to, those specified in any approved odour management plan, to prevent or where that is not practicable to minimise the odour.

## 3.3	Noise and vibration

#### 3.3.1	Emissions from the activities shall be free from noise and vibration at levels likely to cause pollution outside the site, as perceived by an authorised officer of Natural Resources Wales, unless the operator has used appropriate measures, including, but not limited to, those specified in any approved noise and vibration management plan to prevent or where that is not practicable to minimise the noise and vibration.

## 3.4	Pests

3.4.1	The activities shall not give rise to the presence of pests which are likely to cause pollution, hazard or annoyance outside the boundary of the site. The operator shall not be taken to have breached this condition if appropriate measures, including, but not limited to, those specified in any approved pests management plan, have been taken to prevent or where that is not practicable, to minimise the presence of pests on the site.

3.4.2	The operator shall:

(a)	if notified by Natural Resources Wales, submit to Natural Resources Wales for approval within the period specified, a pests management plan which identifies and minimises risks of pollution, hazard or annoyance from pests;

(b)	implement the pests management plan, from the date of approval, unless otherwise agreed in writing by Natural resources Wales.

## 3.5	Fire

3.5.1	The operator shall manage and operate the activities in accordance with a written fire prevention plan using the current, relevant fire prevention and mitigation plan guidance.

3.5.2	The operator shall:

(a)	if notified by Natural Resources Wales that the activities could cause a fire risk, submit to Natural Resources Wales a fire prevention and mitigation plan which identifies and minimises the risks of fire;

(b)	Operate the activity in accordance with the fire prevention and mitigation plan, from the date of submission, unless otherwise agreed in writing by Natural Resources Wales.

# 4	Information

## 4.1	Records

### All records required to be made by this permit shall:

### be legible;

### be made as soon as reasonably practicable;

### if amended, be amended in such a way that the original and any subsequent amendments remain legible, or are capable of retrieval; and

### be retained, unless otherwise agreed in writing by Natural Resources Wales, for at least 6 years from the date when the records were made, or in the case of the following records until permit surrender:

- off-site environmental effects; and

- matters which affect the condition of the land and groundwater.

4.1.2	The operator shall keep on site all records, plans and the management system required to be maintained by this permit, unless otherwise agreed in writing by Natural Resources Wales.

## 4.2	Reporting

4.2.1	The operator shall send all reports and notifications required by the permit to Natural Resources Wales using the contact details supplied in writing by Natural Resources Wales.

### 4.2.2	Within one month of the end of each quarter, the operator shall submit to Natural Resources Wales using the form made available for the purpose, the information specified on the form relating to the site and the waste accepted and removed from it during the previous quarter.

## 4.3	Notifications

### 4.3.1	Natural Resources Wales shall be notified without delay following the detection of:

### any malfunction, breakdown or failure of equipment or techniques, accident, or emission of a substance not controlled by an emission limit which has caused, is causing or may cause significant pollution;

### the breach of a limit specified in the permit; or

### any significant adverse environmental effects.

### 4.3.2	Any information provided under condition 4.3.1 shall be confirmed by sending the information listed in schedule 5 to this permit within the time period specified in that schedule.

### 4.3.3	Where Natural Resources Wales has requested in writing that it shall be notified when the operator is to undertake monitoring and/or spot sampling, the operator shall inform Natural Resources Wales when the relevant monitoring and/or spot sampling is to take place. The operator shall provide this information to Natural Resources Wales at least 14 days before the date the monitoring is to be undertaken.

### 4.3.4	Natural Resources Wales shall be notified within 14 days of the occurrence of the following matters, except where such disclosure is prohibited by Stock Exchange rules:

Where the operator is a registered company:

#### any change in the operator’s trading name, registered name or registered office address; and

#### (b)	any steps taken with a view to the operator going into administration, entering into a company voluntary arrangement or being wound up.

Where the operator is a corporate body other than a registered company:

#### any change in the operator’s name or address; and

#### any steps taken with a view to the dissolution of the operator.

#### In any other case:

#### the death of any of the named operators (where the operator consists of more than one named individual);

#### any change in the operator’s name(s) or address(es); and

#### any steps taken with a view to the operator, or any one of them, going into bankruptcy, entering into a composition or arrangement with creditors, or, in the case of them being in a partnership, dissolving the partnership.

### 4.3.5	Where the operator proposes to make a change in the nature or functioning, or an extension of the activities, which may have consequences for the environment and the change is not otherwise the subject of an application for approval under the Regulations or this permit:

### Natural Resources Wales shall be notified at least 14 days before making the change; and

### the notification shall contain a description of the proposed change in operation.

## 4.4	Interpretation

### 4.4.1	In this permit the expressions listed in schedule 6 shall have the meaning given in that schedule.

### 4.4.2	In this permit references to reports and notifications mean written reports and notifications, except where reference is made to notification being made “without delay”, in which case it may be provided by telephone.

Schedule 1 - Operations

| ### Table S1.1 activities |  |  |
| --- | --- | --- |
| ### Activity reference | ### Description of activities for waste operations | ### Limits of activities |
| ### Household, commercial and industrial waste transfer station with treatment | ### R3: Recycling/ reclamation of organic substances which are not used as solvents ### R4: Recycling/ reclamation of metals and metal compounds ### R5: Recycling/ reclamation of other inorganic compounds ### R13: Storage of waste pending any of the operations numbered R1 to R12 (excluding temporary storage, pending collection, on the site where it is produced) ### D9: Physico-chemical treatment not specified elsewhere in this Annex which results in final compounds or mixtures which are discarded by means of any of the operations numbered D1 to D12 (e.g. evaporation, drying, calcination, etc.) ### D14:Repackaging prior to submission to any of the operations numbered D1 to D13. ### D15: Storage pending any of the operations numbered D1 to D14 (excluding temporary storage pending collection on the site where it is produced). | All hazardous and non-hazardous waste must be stored and treated on an impermeable surface with sealed drainage. All inert waste must be stored and treated on hardstanding or on an impermeable surface with sealed drainage. The maximum quantity of hazardous waste that can be stored at the site in total for recovery or disposal, shall not exceed 50 tonnes at any one time. Treatment operations shall be limited to manual and/or mechanical:. - sorting and/or separation - screening (Inert soils and stones only) - shredding (plastic waste only) - crushing/compaction (excluding inert soils stones) - granulating (plastic waste only) - bulking up for onwards transfer - Dewatering (grinding solids only) of permitted waste for the purpose of recovery and/or disposal. No treatment of hazardous waste other than the dewatering of grinding solids and bulking up of hazardous waste for onward transfer. Treatment of hazardous waste shall not exceed 10 tonnes per day. No mixing of hazardous and non-hazardous waste. Waste types as specified in Table S2.1. |

| ### Table S1.2 Operating techniques |  |  |
| --- | --- | --- |
| ### Description | ### Parts | ### Date Received |
| ### Response to Schedule 5 Notice dated 02/09/22 | ### Odour Management Plan, Document Reference 2099R52d1022, Version 2, Oct 2022: - All Sections | ### 04/10/22 |
|  | ### Noise Management Plan, Document Reference 2099r7v2d1022, Version 2, Oct 2022. - All Sections | ### 30/10/22 |
| ### Response to email dated 20/12/22 | ### Environmental Accident Management Plan, Document Reference 2099r8v1d1022, Version 1, Oct 2022: - All Sections | ### 21/12/22 |
| ### Response to email dated 05/01/23 | ### Environmental Management System, Document Reference 2099r2v3d1022, Version 3, Oct 2022. ### Sections: - 3.3 Operational Hours - 4.2 Permitted Waste Types - 4.3 Waste Acceptance - 4.4 Hard plastic Processing - 4.5 UPVC Processing - 4.6 Window Company Skip Waste Processing - 4.7 Paint and Paint Tin Recovery - 4.8 Grinding solids - 4.9 Waste Handling and Storage - 5.3 Site Surfacing and Site Drainage - 7 Amenity Management ### 9 Planned Preventative Measures | ### 09/01/23 |
| ### Response to email dated 13/01/23 | ### Fire Prevention and Mitigation Plan, Document Reference 2099r4v4d1222, Revision 4, Dec 2022. - All Sections | ### 18/01/23 |
| ### Technical Guidance Document: ‘How to comply with your environmental permit’ | ### All relevant sections | ### N/A |
| ### Sector Guidance Note IPPC S5.06: Guidance for the Recovery and Disposal of Hazardous and Non Hazardous Waste | ### All relevant sections | ### N/A |
| ### Fire prevention and mitigation plan guidance - Waste Management | ### All relevant sections | ### N/A |

#### ‘

| ### Table S1.3 Improvement programme requirements |  |  |
| --- | --- | --- |
| ### Reference | ### Requirement | ### Date |
| ### IPC1 | ### The Operator shall undertake noise monitoring to assess the risk at the nearest sensitive receptors as identified in the Noise Management Plan, Document Reference 2099r7v2d1022, Version 2, Oct 2022 in Table S1.2 of this permit. ### This shall include: - A full noise monitoring survey and assessment meeting the BS4142:2014+A1:2019 standard. ### This noise monitoring survey must be undertaken in the summer months. Upon completion of the survey, a written report shall be submitted to Natural Resources Wales for approval. The report shall refer to the predictions in the report provided with the permit application. If rating levels likely to cause an impact at sensitive receptors are detected, the report shall include an assessment of the most suitable abatement techniques, an estimate of the cost and a proposed timetable for their installation on site. | ### 8 months after permit issue or otherwise agreed in writing with Natural Resources Wales |

# Schedule 2 - Waste types, raw materials and fuels

| Table S2.1 Permitted waste types and quantities for household, commercial and industrial waste transfer station with treatment |  |
| --- | --- |
| Maximum quantity | The maximum quantity of waste to be accepted on site shall not exceed 24,999 tonnes per year. |
| Exclusions | Notwithstanding the waste types set out in this table, wastes having any of the following characteristics shall not be accepted: - consisting solely or mainly of dusts, powders or loose fibres - odorous or odour producing |
| Waste code | Description |
| 02 | WASTES FROM AGRICULTURE, HORTICULTURE, AQUACULTURE, FORESTRY, HUNTING AND FISHING, FOOD PREPARATION AND PROCESSING |
| 02 01 | wastes from agriculture, horticulture, aquaculture, forestry, hunting and fishing |
| 02 01 04 | waste plastics (except packaging) |
| 03 | WASTES FROM WOOD PROCESSING AND THE PRODUCTION OF PANELS AND FURNITURE, PULP, PAPER AND CARDBOARD |
| 03 03 | wastes from pulp, paper and cardboard production and processing |
| 03 03 01 | waste bark and wood |
| 07 | WASTES FROM ORGANIC CHEMICAL PROCESSES |
| 07 02 | wastes from the MFSU of plastics, synthetic rubber and man-made fibres |
| 07 02 13 | waste plastic |
| 08 | WASTES FROM THE MANUFACTURE, FORMULATION, SUPPLY AND USE (MFSU) OF COATINGS (PAINTS, VARNISHES AND VITREOUS ENAMELS), ADHESIVES, SEALANTS AND PRINTING INKS |
| 08 01 | wastes from MFSU and removal of paint and varnish |
| 08 01 11* | waste paint and varnish containing organic solvents or other hazardous substances |
| 08 01 12 | waste paint and varnish other than those mentioned in 08 01 11 |
| 08 01 13* | sludges from paint or varnish containing organic solvents or other hazardous substances |
| 08 01 14 | sludges from paint or varnish other than those mentioned in 08 01 13 |
| 08 01 15* | aqueous sludges containing paint or varnish containing organic solvents or other hazardous substances |
| 08 01 16 | aqueous sludges containing paint or varnish other than those mentioned in 08 01 15 |
| 08 01 17* | wastes from paint or varnish removal containing organic solvents or other hazardous substances |
| 08 01 18 | wastes from paint or varnish removal other than those mentioned in 08 01 17 |
| 08 01 19* | aqueous suspensions containing paint or varnish containing organic solvents or other hazardous substances |
| 08 01 20 | aqueous suspensions containing paint or varnish other than those mentioned in 08 01 19 |
| 08 01 21* | waste paint or varnish remover |
| 10 | WASTES FROM THERMAL PROCESSES |
| 10 12 | wastes from manufacture of ceramic goods, bricks, tiles and construction products |
| 10 12 12 | wastes from glazing other than those mentioned in 10 12 11 |
| 10 13 | wastes from manufacture of cement, lime and plaster and articles and products made from them |
| 10 13 14 | waste concrete and concrete sludge |
| 12 | WASTES FROM SHAPING AND PHYSICAL AND MECHANICAL SURFACE TREATMENT OF METALS AND PLASTICS |
| 12 01 | wastes from shaping and physical and mechanical surface treatment of metals and plastics |
| 12 01 20* | spent grinding bodies and grinding materials containing hazardous substances |
| 15 | WASTE PACKAGING, ABSORBENTS, WIPING CLOTHS, FILTER MATERIALS AND PROTECTIVE CLOTHING NOT OTHERWISE SPECIFIED |
| 15 01 | packaging (including separately collected municipal packaging waste) |
| 15 01 02 | plastic packaging |
| 15 01 03 | wooden packaging |
| 15 01 04 | metallic packaging |
| 15 01 05 | composite packaging |
| 15 01 06 | mixed packaging |
| 15 01 07 | glass packaging |
| 16 | WASTES NOT OTHERWISE SPECIFIED IN THE LIST |
| 16 01 | end-of-life vehicles from different means of transport (including off-road machinery) and wastes from dismantling of end-of-life vehicles and vehicle maintenance (except 13, 14, 16 06 and 16 08) |
| 16 01 19 | plastic |
| 16 01 20 | glass |
| 17 | CONSTRUCTION AND DEMOLITION WASTES (INCLUDING EXCAVATED SOIL FROM CONTAMINATED SITES) |
| 17 01 | concrete, bricks, tiles and ceramics |
| 17 01 01 | concrete |
| 17 01 02 | bricks |
| 17 01 07 | mixtures of concrete, bricks, tiles and ceramics other than those mentioned in 17 01 06 |
| 17 02 | wood, glass and plastic |
| 17 02 01 | wood |
| 17 02 02 | glass |
| 17 02 03 | plastic |
| 17 04 | metals (including their alloys) |
| 17 04 01 | copper, bronze, brass |
| 17 04 02 | aluminium |
| 17 04 03 | lead |
| 17 04 04 | zinc |
| 17 04 05 | iron and steel |
| 17 04 06 | tin |
| 17 04 07 | mixed metals |
| 17 04 11 | cables other than those mentioned in 17 04 10 |
| 17 05 | soil (including excavated soil from contaminated sites), stones and dredging spoil |
| 17 05 04 | soil and stones other than those mentioned in 17 05 03 |
| 17 09 | other construction and demolition wastes |
| 17 09 04 | mixed construction and demolition wastes other than those mentioned in 17 09 01, 17 09 02 and 17 09 03 |
| 19 | WASTES FROM WASTE MANAGEMENT FACILITIES, OFF-SITE WASTE WATER TREATMENT PLANTS AND THE PREPARATION OF WATER INTENDED FOR HUMAN CONSUMPTION AND WATER FOR INDUSTRIAL USE |
| 19 12 | wastes from the mechanical treatment of waste (for example sorting, crushing, compacting, pelletising) not otherwise specified |
| 19 12 02 | ferrous metal |
| 19 12 03 | non-ferrous metal |
| 19 12 04 | plastic and rubber |
| 19 12 05 | glass |
| 19 12 07 | wood other than that mentioned in 19 12 06 |
| 19 12 09 | minerals (for example sand, stones) |
| 20 | MUNICIPAL WASTES (HOUSEHOLD WASTE AND SIMILAR COMMERCIAL, INDUSTRIAL AND INSTITUTIONAL WASTES) INCLUDING SEPARATELY COLLECTED FRACTIONS |
| 20 01 | separately collected fractions (except 15 01) |
| 20 01 02 | glass |
| 20 01 27* | paint, inks, adhesives and resins containing hazardous substances |
| 20 01 38 | wood other than that mentioned in 20 01 37 |
| 20 01 39 | plastics |
| 20 01 40 | metals |
| 20 02 | garden and park wastes (including cemetery waste) |
| 20 02 02 | soil and stones |
| 20 03 | other municipal wastes |
| 20 03 01 | mixed municipal waste comprising of construction and demolition waste, plastic, wood and metal (excluding biodegradable waste, batteries, WEEE) |

# Schedule 3 – Emissions and monitoring

#### There are no emission limits or associated monitoring requirements.

# Schedule 4 - Reporting

#### There is no reporting under this schedule.

# Schedule 5 - Notification

### These pages outline the information that the operator must provide.

### Units of measurement used in information supplied under Part A and B requirements shall be appropriate to the circumstances of the emission. Where appropriate, a comparison should be made of actual emissions and authorised emission limits.

#### If any information is considered commercially confidential, it should be separated from non-confidential information, supplied on a separate sheet and accompanied by an application for commercial confidentiality under the provisions of the EP Regulations.

## Part A

| ## Permit Number |  |
| --- | --- |
| ## Name of operator |  |
| ## Location of Facility |  |
| ## Time and date of the detection |  |

| ## (a) Notification requirements for any malfunction, breakdown or failure of equipment or techniques, accident, or emission of a substance not controlled by an emission limit which has caused, is causing or may cause significant pollution |  |
| --- | --- |
| ## To be notified within 24 hours of detection |  |
| ## Date and time of the event |  |
| ## Reference or description of the location of the event |  |
| ## Description of where any release into the environment took place |  |
| ## Substances(s) potentially released |  |
| ## Best estimate of the quantity or rate of release of substances |  |
| ## Measures taken, or intended to be taken, to stop any emission |  |
| ## Description of the failure or accident. |  |

| ## (b) Notification requirements for the breach of a limit |  |
| --- | --- |
| ## To be notified within 24 hours of detection unless otherwise specified below |  |
| ## Emission point reference/ source |  |
| ## Parameter(s) |  |
| ## Limit |  |
| ## Measured value and uncertainty |  |
| ## Date and time of monitoring |  |
| ## Measures taken, or intended to be taken, to stop the emission |  |

| ### Time periods for notification following detection of a breach of a limit |  |
| --- | --- |
| ### Parameter | ### Notification period |

| ## (c) Notification requirements for the detection of any significant adverse environmental effect |  |
| --- | --- |
| ## To be notified within 24 hours of detection |  |
| ## Description of where the effect on the environment was detected |  |
| ## Substances(s) detected |  |
| ## Concentrations of substances detected |  |
| ## Date of monitoring/sampling |  |

## Part B - to be submitted as soon as practicable

| ### Any more accurate information on the matters for notification under Part A. |  |
| --- | --- |
| ### Measures taken, or intended to be taken, to prevent a recurrence of the incident |  |
| ### Measures taken, or intended to be taken, to rectify, limit or prevent any pollution of the environment which has been or may be caused by the emission |  |
| ### The dates of any unauthorised emissions from the facility in the preceding 24 months. |  |

| ### Name* |  |
| --- | --- |
| ### Post |  |
| ### Signature |  |
| ### Date |  |

### * authorised to sign on behalf of the operator

Schedule 6 - Interpretation

- “accident” means an accident that may result in pollution.

“Annex I” means Annex I to Directive 2008/98/EC of the European Parliament and of the Council on waste.

“Annex II” means Annex II to Directive 2008/98/EC of the European Parliament and of the Council on waste.

- “application” means the application for this permit, together with any additional information supplied by the operator as part of the application and any response to a notice served under Schedule 5 to the EP Regulations.

“authorised officer” means any person authorised by Natural Resources Wales under section 108(1) of The Environment Act 1995 to exercise, in accordance with the terms of any such authorisation, any power specified in section 108(4) of that Act

“building” means a construction that has the objective of providing sheltering cover and minimising emissions of noise, particulate matter, odour and litter.

“D” means a disposal operation provided for in Annex I to Directive 2008/98/EC of the European Parliament and of the Council on waste.

“emissions of substances not controlled by emission limits” means emissions of substances to air, water or land from the activities, either from the emission points specified in schedule 3 or from other localised or diffuse sources, which are not controlled by an emission limit.

“EP Regulations” means The Environmental Permitting (England and Wales) Regulations SI 2016 No.1154 and words and expressions used in this permit which are also used in the Regulations have the same meanings as in those Regulations.

“groundwater” means all water, which is below the surface of the ground in the saturation zone and in direct contact with the ground or subsoil.

“hazardous property” has the meaning in Annex lll of the Waste Framework Directive.

“hazardous substance” means a substance classified as hazardous as a consequence of fulfilling the criteria laid down in parts 2 to 5 of Annex I to Regulation (EC) No 1272/2008.

“hazardous waste” has the meaning given in the Hazardous Waste (Wales) Regulations 2005 (as amended).

“impermeable surface” means a surface or pavement constructed and maintained to a standard sufficient to prevent the transmission of liquids beyond the surface.

“Pests” means Birds, Vermin and Insects.

“quarter” means a calendar year quarter commencing on 1 January, 1 April, 1 July or 1 October.

“R” means a recovery operation provided for in Annex II to Directive 2008/98/EC of the European Parliament and of the Council on waste.

“Waste code” means the six digit code referable to a type of waste in accordance with the list of wastes established by Commission Decision 2000/532/EC as amended from time to time (the ‘List of Wastes Decision’) and in relation to hazardous waste, includes the asterisk.

“Waste Framework Directive” or “WFD” means Waste Framework Directive 2008/98/EC of the European Parliament and of the Council on waste.

“WEEE” means waste electrical and electronic equipment.

“WEEE Directive” means Directive 2012/19/EU of the European Parliament and of the Council of 4th July 2012 on waste electrical and electronic equipment (WEEE).

“year” means calendar year ending 31 December.

Schedule 7 - Site plan

© Crown Copyright and database right 2023. Ordnance Survey licence number 100019741.

END OF PERMIT