| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-020020 |
| Permit Reference Number: | EPR-KP3594VE |
| Permit Holder: | Trade Effluent Services |
| Site Address: | Brookhouse Farm, Hanmer, SY13 3EQ |
| Grid Reference: | Farm location: SJ46858 39960 Fields: 1 5 SJ46691 40872 2 6/7 SJ46681 40685 3 8/9 SJ46689 40508 4 10/11 SJ46745 40148 5 14 SJ46797 39240 6 20 SJ46907 40156 7 21 SJ47197 39442 Storage: 1 SJ46603 40789 2 SJ47028 39376 3 SJ46828 39971 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: IndustryRegulation&Waste.NW@cyfoethnaturiolcymru.gov.uk Environment team: WrexhamFlintshireEnvironmentTeam@cyfoethnaturiolcymru.gov.uk NLO: Amy.Henderson@cyfoethnaturiolcymru.gov.uk |
| Date due: | 13/12/2022 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating B- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | Welsh-PaLS |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and Charging Scheme page 52 |
| Fee on MyNRW | Y | Amount: £1018.00 Date cleared on MyNRW: 02/11/22 Transaction number: 2108078 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | Y |  |
| Site Specific Risk Assessment | Y | Risk assessment provided is not site specific. |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | Y |  |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | Provided by applicant- CPH 56/241/0025 Mr Brian Jones This information was confirmed on mymap |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y |  |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y |  |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y |  |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | Y | PAN-015827 |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | High |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | Y | Risk assessment provided is not site specific and does not address all feasible pathways. |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | site‐specific risk assessment benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | N |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y |  |
| - About the waste |  |  |
| Has all parts of the waste types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable of non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y |  |
| Are the wastes listed in the deployment specifically listed in the permit? | Y | Maelor is an unknown waste code. |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | N | Penderyn, Beechdean, D.wise and Kellogs production site have not been specified in any documents. |
| - About the storage |  |  |
| Has all of waste storage table been completed? | Y | Storage tank is stored in a buffer zone. |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y | One storage tank is located in buffer zone this needs to be relocated. |
| If land was previously deployed on, and waste is stored in temporary field heaps; are the storage locations in the same or different locations? | n/a |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y |  |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | n/a |  |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | n/a |  |
| location of all storage sites for the waste to be spread | Y |  |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | Y | There are 2 springs in field 5/6 and 14 that have not been buffered in the application. |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | n/a |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | Y | Ramsar Midlands meres and mires phase 2 Llyn Bedydd SSSI |
| location of any public rights of way (shown on map) | Y |  |
| any buildings or houses within 250m (shown on map) | Y | Farm buildings and some marked as cottages |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Nitrate Vulnerable Zone (NVZ) - Groundwater Safeguard Zone - Hedges - Ditches | Y | 1 SAM Mount cop castle mound, no issue 23 Protected habitat, outside of spread area no issue 2 Ancient woodland areas, no issue 4 SINCs Coxton Marsh-400m north of field 5 Hanmer Hall woods- 320m west of field 6/7 Willis Gorse marsh 108m South of field 21 Arowry Moss- 132m west field14 |
| What is the slope of the fields? | 0-7 degrees (transparent)- fields: All fields 7 -12 degrees (yellow)- fields: Portions of fields 5, 6/7, 8/9, 10/11, 20 12 degrees plus (red)- fields: Portions of fields 5, 6/7, 8/9, 10/11, 20 |  |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y | Penderyn analysis is from 2021 and is therefore out of date. |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y | 020204- Conductivity, Fats/oils/grease Odour 02 03 05- odour 02 07 05- pH and soil type (calcareous soils) 02 05 01- odour, sodium 02 07 05- pH |
| - If yes; have they been tested for? | Y |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y | Fields are being spread at offtake. Though the fields are above target for phosphate. This is in line with RB209 guidance Waste stream Rowan Food is delivering too much phosphate and nitrogen |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y N | Maelor waste is uncoded |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y N n/a | Timings appear to span the majority of the year with no clear indication of actual spreading time and caveated that this may change. |
| - Amount of waste to be applied and application rate | Y | Inappropriate for rowan food |
| - Soil chemical properties (e.g. liming and pH) | Y |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | Y N n/a | As above |
| - Included the nutrient level in their environmental risk assessment | N |  |
| - Stated the source of information that they used to set the nutrient level. | Y |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y N |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | Y |  |
| - Is it only for operational purposes? | Y |  |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | Y |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y N | As above |
| - Have they stated when the waste will be spread? | Y N |  |
| - Have they stated how often the waste will be spread? | Y N |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients supplied match NRW calculations? | Y | Except for Rowan food |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y |  |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | Y |  |
| - The nutrients supplied by the material? | Y |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | N | Risk assessment is not site specific |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | Y |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | N | All sensitive receptors are not addressed |
| - Pathways | N | All pathways are not addressed |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | N |  |
| - Measures that will minimise or mitigate the impact will be put in place | N |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | Y |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | Y N n/a | App 4 not completed as application is returned, SSRA was not suitable to assess therefore an app 4 was not completed |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | Y |
| No FORM 1 completed: HRA not completed as applications site specific risk assessment was not suitable. This meant a proper HRA assessment was not possible and the application was returned. |  |

Comments

Consultation responses: Consultees noted several issues with the timing, mapping, waste analysis, and storage of the waste. Specific actions were raised regarding the soil chemical analysis in the agben, not agreeing with soil analysis provided, field 14 specifically was listed at phosphate indices 5. Discrepancy between the field sizing was also picked up as an issue that required addressing, agben states 51.5 ha LPD1 states 50, appears to be a rounding issue. These comments were incorporated into the return document.

MyMap, location and slope assessment: MyMap assessment: Mymap report is in draft on the WIP Location assessment: There is a track crossing field 5 that has not been addressed in the application. There are several residential builds near the spread area, an odour management plan has not been submitted. Slope assessment: Field number: 5, 6/7, 8/9, 10/11, 14, 20 Slope of the land: 7-12 degrees Soil type: medium loam Wastes types applied (solid, liquid, sludge etc.): sludge (possibly sludge cakes and solids also) Cropping type: wheat, grass and forage maize Method of application: deep leg injection. (SSRA states surface spread) Weather conditions before during and after spreading: Applicant has confirmed CoGAP will be followed Other risk factors not mentioned above: land is hydrologically linked to SSSI Llyn Bedydd and Ramsar Midland mires and meres phase 2 Conclusions (is run off risk high?): Moderate, injection of waste should prevent runoff, however SSRA states waste will be surface spread. Field number: 5, 6/7, 8/9, 10/11, 20 Slope of the land:12+ degrees Soil type: medium loam Wastes types applied (solid, liquid, sludge etc.): sludge (possibly sludge cakes and solids also) Cropping type: wheat, grass and forage maize Method of application: deep leg injection. (SSRA states surface spread) Weather conditions before during and after spreading: Applicant has confirmed CoGAP will be followed Other risk factors not mentioned above: land is hydrologically linked to SSSI Llyn Bedydd and Ramsar Midland mires and meres phase 2 Conclusions (is run off risk high?): High, injection of waste should prevent runoff, however SSRA states waste will be surface spread.

CPH details: Provided by applicant- CPH 56/241/0025 Mr Brian Jones This information was confirmed on mymap

Protection of habitats and ecosystems: Applicant provide satellite images of the spread area were provided showing the fields are established agricultural fields. The mapping report supports this.

Agricultural benefit calculations: Broadly calculations match those of the applicant apart from rounding errors, Rowan food however is not in line with our calculations and is applying too much nitrogen than is acceptable. Secanim is too high in FOGs to be spread. Secanim and penderyn are high RAN, this has been acknowledged in LPD1. Maelor foods, Waste regulation have informed us that the waste does not/may not meet its waste code 020204 as it has not been fully treated Rowan food is pH 3.82, Kelloggs is 4.02 and Beechdean is 3.76 but has been noted in agben 3.96. penderyn analysis is out of date. D.wise is overliming all fields.

Request for information: Return reason: 1. Unclear spread methods 2. Spread timings 3. Storage lagoon 4. Storage tank in buffer zone 5. Storage locations 6. Odour management plan 7. Spring buffers 8. Field slopes 9. Waste origin 10. Beechdean pH 11. Out of date analysis 12. Secanim FOGs 13. Rowan food Nutrient additions 14. Rowan food phosphate additions 15. D. wise liming 16. D. wise watse code 17. Encirc format 18. Risk assessment author 19. Risk assessment spread method 20. Risk assessment does not address pathways 21. Risk assessment field proximity.

Other information to note:

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes No |
| ## Issue date |  |
| ## Effective date |  |
| ## Permitting Officer Date | Ethan Banfield |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 09/02/2023 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 950 |
| Sign off | 10 |
| Issuing | 45 |
| Total | 1065 |