| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-020504 |
| Permit Reference Number: | EPR/JB3533RJ |
| Permit Holder: | Simon Jones |
| Site Address: | Land at Sarn Fadog, Llanerchymedd, LL71 8ER |
| Grid Reference: | Farm location: SH 42073 81013 Fields: 1 0528 SH 42060 81280 2 2928 SH 42280 81270 3 2952 SH 42310 81500 4 3171 SH 42300 81710 5 2949 SH 42500 81480 7 4726 SH 42470 81250 8 5111 SH 46500 91130 9 4300 SH 46420 91010 12 7291 SH 46710 90900 13 8570 SH 46860 90700 14 9685 SH 46960 90860 15 0909 SH 47080 91070 16 1794 SH 47150 90870 Storage: 1 SH 42070 81010 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: IndustryRegulation&Waste.NW@cyfoethnaturiolcymru.gov.uk Environment team: TimAmgylcheddMonacArfon@cyfoethnaturiolcymru.gov.uk NLO: TimAmgylcheddMonacArfon@cyfoethnaturiolcymru.gov.uk |
| Date due: | 03/02/2023 |
| Are fields on Anglesey? If yes, then send application. | Yes: ajxht@anglesey.gov.uk pem@anglesey.gov.uk |
| Permit compliance rating and multiplier | Rating A- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | EPR/JB3533RJ |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and deployment fee- Charging Scheme |
| Fee on MyNRW | Y | Amount:£798 Date cleared on MyNRW:23/01/23 Transaction number: 2111004 |
| Location map | Y |  |
| Benefit statement | Y | See RFI |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | Y |  |
| Site Specific Risk Assessment | n/a | High risk location only |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y | JB3533RJ |
| ## Has the permit holder name been provided? | Y | Simon Jones |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | CPH All fields 53/011/0026 M AND R PRITCHARD & SONS TY ISAF NANT PERIS CAERNARFON GWYNEDD LL55 4UN Differs from those provided although name and address are same. Two different landowner consent documents provide. 53/180/7500 53/166/0005 53/163/3003 RFI |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y |  |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y |  |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y |  |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | n/a |  |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | Medium (1) |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) of High risk deployment has a SSRA been provided? | n/a |  |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | n/a |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y |  |
| - About the waste |  |  |
| Have all parts of the waste types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable or non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y |  |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y |  |
| - About the storage |  |  |
| Has all of waste storage table been completed? | Y |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y |  |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | n/a | Nil See RFI |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | Y | See comments |
| location of all storage sites for the waste to be spread | Y | See RFI |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | Y | See comments |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | Y | Only those within 50m. none beyond |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | Y N n/a |  |
| location of any public rights of way (shown on map) | Y | Public right of way in field 4300. Shown by dotted line |
| any buildings or houses within 250m (shown on map) | Y |  |
| Within Groundwater Safeguard Zone | N |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Hedges - Ditches | Y |  |
| What is the slope of the fields? | All fields 0-7 apart from field 7291 which is 7-12 gradually sloping SE to NW. Minimal 12+ but low risk of run off. |  |

| Waste Storage | Y N n/a | Comments |
| --- | --- | --- |
| Is waste storage occurring at the place where waste is being deployed? | Y | Query on storage location for fields at Glen Rafon- See RFI |
| Is only waste as specified in this deployment being stored in the proposed storage locations? | Y |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in the same place one constructed within the last two years? or in different location? | n/a | Lagoon |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in a field: - liable to flooding or becoming waterlogged, or - within 50 metres of a borehole, spring or well or - within 10 metres of surface water or a land drain - within 30 meters of a watercourse on steeply sloping land | n/a |  |
| If the waste is non-stackable is the waste stored within either a vessel, a covered building, or on an impermeable surface | Y | Lagoon |
| If the waste is high RAN and it is being stored within 200 metres of a European Site (SAC/SPA), Ramsar Site or a Site of Special Scientific Interest (SSSI) is the waste being covered to prevent loss of ammonia? (condition 3.2.4 of the permit) | n/a | Not high RAN. |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | n/a |  |
| - If yes; have they been tested for? | n/a |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | Y |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| Timing of application- if any of the wastes are high RAN, are they being spread OUTSIDE of the closed periods | n/a |  |
| Groundwater Safeguard Zone for nitrate- if any of the wastes are high RAN, are the application rates less than 50 m3/ha, with a minimum 3 week gap between applications | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y | Odour Management Plan has been provided- see RFI |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need, as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If the cropping is any of the following, is the maximum nitrogen limit in line with that given in The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021: - 0 kg/ha- Field beans, Peas - 120 kg/ha- Sugar beet - 150 kg/ha- Asparagus, Carrots, Forage maize. Radish, Spring barley, Swedes - 180 kg/ha- Spring-sown wheat, Winter barley - 220 kg/ha- Autumn or early winter sown wheat | n/a |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | Y |  |
| - Is it only for operational purposes? | Y |  |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | Y |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients given by the applicant match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y | SNS for 4726 reported as 1 rather than low, medium, high. RFI |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y | FYM |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | N/A |  |
| - The nutrients supplied by the material? | N/A |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | n/a |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | n/a |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | n/a |  |
| - Pathways | n/a |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | n/a |  |
| - Measures that will minimise or mitigate the impact will be put in place | n/a |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | N |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | n/a |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | N |

Comments

Consultation responses: Responses noted

MyMap and location assessment: Storage location, Farm and field coordinates correct Drains and ponds within 1794 and well to SE of 5111. Watercourse running SE to NW through 7291 and 5111. Buffers in place on application maps. Well to SE of field 0528. Pond to E of field 2949. Pond south of field 2928. Drain flows N to S, buffer in place on application maps. Public right of way NE to south through field 4300 Slope Assessement All fields 0-7 apart from field 7291 which is mainly 7-12 gradually downward sloping SE to NW. Minor 12+, low risk of run off. Field number:7291 Slope of the land:7-12 gradually sloping SE to NW. Minor 12+ Soil type: Medium Wastes types applied (solid, liquid, sludge etc.): liquid, sludge Cropping type: grass Method of application: trailing shoe applicator Weather conditions before during and after spreading: in accordance with permit conditions and CoGAP Other risk factors not mentioned above: Nil

CPH details: See comments above

Protection of habitats and ecosystems

Agricultural benefit calculations: DAF Sludge shows minor decrease in in both phosphate and potash Heavy shows minor decrease in in both phosphate and potash Whey also shows minor decrease in in both phosphate and potash

Request for information: Sent 24/02/23. Response received 24/02/23 in blue below. About the land you want to treat. - You have stated that you want to treat agricultural land but have not provided the CPH number in the application form. You have provided two landowner confirmations in the application; these documents provide 3 differing CPH numbers for the fields in this deployment, two of which match the information given in your benefit statement section 2.1. However, none of these numbers match our records, and it is not clear what fields the CPH numbers you have provided apply to. ACTION: Confirm the CPH numbers for each field in this application. The form does not allow for multiple CPH numbers, hence I have listed them in section 2.1 of the ABS, with confirmation from the land occupier in appendix C. I am not aware as to why there is a discrepancy, would it be possible to supply additional information as to why the number you have differs from the one that the land occupier has? Land at Sarnfadog - 53/166/0005 Land at Glenrafon – 53/163/3003 About the storage - You have identified a storage lagoon in this application, but your benefit statement and odour management plan suggest that the wastes will be directly transferred to the deployment equipment. ACTION: Clarify if the storage is to be used. If this is Yes, then adjust your operational process in the benefit statement to reflect this. If No, then confirm removal of the storage from the application. The storage location is stated in appendix F with the NGR given in 4.3. - The proposed lagoon storage is 6.5 miles from the Land at Glen Rafon. The SR2010 No4 Table 2.1 requires, under R13: Storage of wastes pending the operation numbered R10, that the storage be at the place where it is to be used. ACTION: Confirm that the proposed storage will only be used for deployment on Land at Sarn Fadog. Also amend your benefit statement to describe how the wastes will be received and deployed on Land at Glen Rafon. The storage lagoon will only be used for Sarnfadog. ABS amended. - In your application you have provided Appendix E – nutrient calculator Sarn Fadog. This spreadsheet includes a tab for Washwater including an application rate, yet this material is not mentioned elsewhere in your deployment application. ACTION: Confirm that this material is not included in this deployment application and provide an amended nutrient calculator. Apologies, removed for Appendix E. Agricultural Benefit Statement - In your benefit statement, section 8.0 Potential sensitive receptors, you have noted that there are no Groundwater Source Protection Zones (SPZ) and no Sites of Special Scientific Interest within 500m of the notified area at Sarn Fadog, but you do not identify if there are any within 500m of the Land at Glen Rafon. ACTION: Ensure that section 8.0 of your benefit statement refers to all areas of land include within this deployment - Land at Glen Rafon AND Sarn Fadog. Clarification added. Odour Management Plan - Your odour management plan, section 11.2 states that the operational site is located to the north of Nebo. This is not correct as this deployment also includes the Land at Sarn Fadog which is to the southwest of Nebo. ACTION: Amend your odour management plan to take into account all of the land included in this application. Clarification added. - Your OMP, section 11.4, refers to the delivery of liquid waste by tanker but then states that effluent/sludges will be spread direct to land. It also suggests that the wastes will be transferred between the delivery vehicle and the tractor hauled tanker. ACTION: Clarify that the OMP covers the delivery of all wastes within the deployment and whether the proposed storage will be used. The latter should also include mitigation for potential odours from delivery to and storage of wastes in the lagoon. The OMP does cover the delivery/transfer by both tanker and that the tanker may spread directly to land. Both options possible, albeit less likely in the case of the tanker both transporting the liquid and spreading it. - Your OMP, section 11.5 potential control of odour and section 11.9 controls during receipt of waste, states that the transfer of waste/effluent is direct and does not mention controls for the proposed storage. ACTION: Confirm if this is direct to the tractor hauled tanker or to the proposed storage. If the proposed storage is to be used, then amend your OMP to include the controls for the proposed storage. Material will either be hauled to the field site and transferred directly to the application equipment, or it may be taken to the storage lagoon, depending on operational requirements at the time. - Your OMP, section 11.6 control measures refers to receipt and spreading but does not refer to the proposed storage. ACTION: Amend your OMP to include controls for the proposed storage. OMP amended to include storage - Your OMP section 11.13 actions contingencies and responsibilities table relates only to spreading and not receipt or storage. ACTION: Amend your OMP to take into account receipt and storage of the wastes. Amended - Your OMP section 11.13.1 odour control in the event of accidents and emergencies suggests that during very hot conditions the effluent will be injected to minimise the evaporation of odour from the ground surface. There are no other references to this method of delivery throughout your benefit statement. ACTION: Confirm the methodology and equipment for this injection and that the equipment is readily available. You should also amend your benefit statement, section 4.1 material application, to identify this and include benefits and risks of this method of delivery. Section 4.1 refers to how a trailing shoe adaptor works, for clarity I have amended the word rubber injection cone’ to ‘rubber application cone’. There is also a picture that shows the actual trailing shoe applicator used by the operator. Soil nitrogen supply - The SNS for field 4726 has been given as a numerical value. This should be given as low, moderate, or high, not a numerical value given the next crop on this field is to be Grass (Silage). ACTION: Review RB209 (Section 3, Table 3.6, page 13) and provide the correct SNS status for field 4726. Amended, however the soil status refers to the previous crop, not the subsequent one, as stated in RB209, Section 3, step 2, page 23. Low residual nitrogen vegetables (‘low N vegetables’) are crops such as carrots, onions, radish, swedes or turnips, in which the amount of crop residue is relatively small

Other information to note:

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 27/02/23 |
| ## Effective date | 27/02/23 |
| ## Permitting Officer Date | Bryan Burn 27/02/23 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 27/02/23 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 1240 |
| Sign off | 10 |
| Issuing | 45 |
| Total | 1355 |