# Natural Resources Wales permitting decisions

# Bespoke permit

We have decided to grant the permit for Trefenter operated by Mr Robert Guichard.

The application number is PAN-019061

The permit number is EPR-CB3799CJ.

The discharge is for a new surface water discharge at Troedyfoel, Trefenter, Aberystwyth, SY23 4HJ.  The discharge location NGR is SN 60579 68278.

The package treatment plant system will be serving 2x residential properties, 3 and 1 bedroom respectively.  The PTP conform to current British Standards BS 12566. The treated effluent is domestic in nature. The discharge is into an unnamed drain of the Nant Rhydroser.  A sewage effluent discharge of a low volume 1.2 m3/day has been estimated.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

- justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

# Annex 1: decision checklist

This checklist should be read in conjunction with the Duly Making checklist.

| ### Aspect | ### Justification / Detail | Criteria met |
| --- | --- | --- |
|  |  | Yes |
| Consultation |  |  |  |
| Scope of consultation | The consultation requirements were identified and implemented. The decision was taken in accordance with RGN 6 High Profile Sites, our Public Participation Statement and our Working Together Agreements. | ü |
| ### Responses to web publicising | The web publicising responses were taken into account in the decision. The decision was taken in accordance with our guidance. No responses were received. | ü |
| Operator |  |  |  |
| Control of the facility | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. | ü |
| European Directives |  |  |  |
| Applicable directives | All applicable European directives have been considered in the determination of the application. | ü |
| WFD | The discharge activity screens out from needing further assessment under WFD as it is not in a water body at high status or high status for morphology, and it is listed in Annex D as a low risk activity plus there is no in combination impact. WFD stage 1 screening has been completed and the activity/project have been ruled out as not requiring any further WFD Regulations 2017 assessment. Or, further assessment is not required because there is no conceivable impact pathway to any water body or Protected Area. | ü |
| The site |  |  |  |
| Extent of the site of the facility | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility including discharge points. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. | ü |
| Biodiversity, Heritage, Landscape and Nature Conservation and a designated site under the Wildlife and Countryside Act. | The application is within the relevant distance criteria of a site of heritage, landscape or nature conservation, and/or protected species or habitat . A full assessment of the application and its potential to affect the site has been carried out as part of the permitting process. We consider that the application will not affect the features of the site. Formal consultation has been carried out with Local Conservation Officer. The consultation response (Annex 2) was taken into account in the permitting decision. | ü |
| Habitat Regulation Assessment for a designated site under the EU Habitats Directive | No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site. An appropriate risk screening has been undertaken “Existing Sewage equal to or less than 2 m3/d to SW” screening to a radius of 500m from the proposed discharge point. The proposed discharge is deemed outside of any Natura 2000/Ramsar sites for this screening assessment. | ü |
| Environmental Risk Assessment and operating techniques |  |  |  |
| Environmental risk | We have carried out a risk assessment on behalf of the operator. The operator considers this risk assessment is satisfactory. The assessment shows that, applying the conservative criteria in our guidance on Environmental Risk Assessment all emissions may be categorised as environmentally insignificant. | ü |
| Operating techniques | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes according to TGN 7.01 . The proposed techniques / emission levels for priorities for control are in line with the benchmark levels contained in the TGN and we consider them to represent appropriate techniques for the facility. | ü |
| The permit conditions |  |  |  |
| Considerations of foul sewer | We agree with the operators justification for not connecting to foul sewer. | ü |
| Operator Competence |  |  |  |
| Environment Management System | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. | ü |
| Sustainable Management of Natural Resources (SMNR) |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources” | ü |
| Environment Wales Act 2016 |  |  |
| Biodiversity and resilience of ecosystems duty | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case, we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for a stand alone water discharge activity permit under EPR2016. | ü |

# Annex 2: Consultation

Response received from

Sophie Higgett – Conservation Officer

Brief summary of issues raised

I agree that the proposed permission is not likely to damage the special interests of the site, providing the applicant installs a better than British Standard treatment plant and reed bed system.

Summary of actions taken or show how this has been covered

No actions required.