# Natural Resources Wales permitting decisions

# Variation

We have decided to issue the variation for Gardden Clay Pit and Gardden Lodge operated by WRG (Midlands) Limited.

The variation number is BP3494FC/V009.

We consider in reaching that decision we have taken into account all relevant considerations and legal requirements and that the permit will ensure that the appropriate level of environmental protection is provided.

Purpose of this document

This decision document:

- explains how the application has been determined

- provides a record of the decision-making process

- shows how all relevant factors have been taken into account

justifies the specific conditions in the permit other than those in our generic permit template.

Unless the decision document specifies otherwise we have accepted the applicant’s proposals.

Structure of this document

- Annex 1 the decision checklist

## Annex 1: decision checklist

## This document should be read in conjunction with the Duly Making checklist, the application and supporting information and permit/ notice.

| ## Aspect considered |  | ## Justification / Detail | ## Criteria met |  |
| --- | --- | --- | --- | --- |
|  |  |  | Yes |  |
| Operator |  |  |  |  |
| Control of the facility |  | We are satisfied that the applicant (now the operator) is the person who will have control over the operation of the facility after the grant of the permit. The decision was taken in accordance with EPR RGN 1 Understanding the meaning of operator. |  |  |
| European Directives |  |  |  |  |
| Applicable directives |  | All applicable European directives have been considered in the determination of the application. |  |  |
| Sustainable Management of Natural Resources (SMNR) |  |  |  |
| Considerations of SMNR - Compliance with our General Purpose | We are satisfied that this decision is compatible with our general purpose of pursuing the sustainable management of natural resources in relation to Wales and applying the principles of sustainable management of natural resources. |  |  |
| Environment Wales Act 2016 – Biodiversity and resilience of ecosystems duty |  |  |  |
| Consideration of Section 6 | Section 6 of the Environment Wales Act 2016 requires that we seek to maintain and enhance biodiversity in the exercise of our functions, and in so doing promote the resilience of ecosystems, in a manner that is consistent with the proper exercise of our functions. NRW is satisfied that in this case, we have taken into account and had due regard to this duty in so far as it is consistent with the function of determining an application for an EPR permit. |  |  |
| The site |  |  |  |  |
| Extent of the site of the facility |  | The operator has provided a plan which we consider is satisfactory, showing the extent of the site of the facility. A plan is included in the permit and the operator is required to carry on the permitted activities within the site boundary. |  |  |
| Biodiversity, Heritage, Landscape and Nature Conservation |  | The application is within the relevant distance criteria of a site of heritage, landscape or nature conservation, and/or protected species or habitat. There is no increased risk from the changes made as a result of this variation to the Johnstown Newt Sites SAC and Stry Las a’r Hafod SSSI. |  |  |
| Environmental Risk Assessment and operating techniques |  |  |  |  |
| Environmental risk |  | We have reviewed the operator's assessment of the environmental risk from the facility. The operator’s risk assessment is satisfactory. |  |  |
| Operating techniques |  | We have reviewed the techniques used by the operator and compared these with the relevant guidance notes. The proposed techniques/ emission levels for priorities for control are in line with the benchmark levels contained in the TGN and we consider them to represent appropriate techniques for the facility. |  |  |
| The permit conditions |  |  |  |  |
| Updating permit conditions during consolidation. |  | We have updated previous permit conditions to those in the new generic permit template as part of permit consolidation. The new conditions have the same meaning as those in the previous permit(s). The operator has agreed that the new conditions are acceptable. |  |  |
| Improvement conditions |  | Based on the information on the application, we consider that we need to impose improvement conditions. We have imposed an improvement condition to ensure that the surface water management system and monitoring of surface waters is suitable and appropriate to the risk from the closed facility during its aftercare phase. |  |  |
| Incorporating the application |  | We have specified that the applicant must operate the permit in accordance with descriptions in the application, including additional information received as part of the determination process. These descriptions are specified in the Operating Techniques table in the permit. |  |  |
| Emission limits |  | We have decided that emission limits should be set for the parameters listed in the permit. The following substances have been identified as being emitted in significant quantities and ELVs or equivalent parameters or technical measures have been set for those substances. For the gas utilisation plant: Oxides of Nitrogen - 500 mg/m3 CO - 1400 mg/m3 Total VOCs - 1000 mg/m3 For the landfill gas flare: Oxides of Nitrogen - 150 mg/m3 CO - 50 mg/m3 Total VOCs - 10 mg/m3 It is considered that the ELVs/ equivalent parameters or technical measures described above will ensure that significant pollution of the environment is prevented and a high level of protection for the environment secured. These have been set in accordance with LFTGN08 and LFTGN05. It is considered that the numeric limits described below will prevent significant deterioration of receiving waters. We have imposed numeric limits because either a relevant environmental quality or operational standard requires this. Suspended Solids - 50 mg/l Ammoniacal Nitrogen - 1.3mg/l Chloride - 250 mg/l pH - >5 and <9 pH units Oil and grease - No visible oil and grease These limits are in accordance with water quality discharge permit CM0180301. |  |  |
| Monitoring |  | We have decided that monitoring should be carried out for the parameters listed in the permit, using the methods detailed and to the frequencies specified. We made these decisions in accordance with LFTGN02, LFTGN03, LFGN04, LFTGN05 and LFTGN08. |  |  |
| Reporting |  | We have specified reporting in the permit. The reporting frequencies are considered appropriate for the closed landfill in its aftercare phase. With reporting every 3 months for landfill gas lateral migration based on the risk posed by the facility, and for the leachate and groundwater levels based the facility being managed on the principal of hydraulic containment. We made these decisions in accordance with LFTGN02, LFTGN03, LFGN04, LFTGN05 and LFTGN08. |  |  |
| Operator Competence |  |  |  |  |
| Environment management system |  | There is no known reason to consider that the operator will not have the management systems to enable it to comply with the permit conditions. The decision was taken in accordance with RGN 5 on Operator Competence. |  |  |