| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-020944 |
| Permit Reference Number: | EPR-JB3533RJ |
| Permit Holder: | Simon Jones |
| Site Address: | Fferam Bailey, Trefdraeth, LL62 5ET |
| Grid Reference: | Farm location: SH 41019 70614 Fields: 8 SH 41060 71230 9 SH 40870 71020 10 SH 40960 70790 11 SH 41180 71080 12 SH 41310 70910 18 SH 41150 70390 19 SH 41300 70200 Storage: No storage details |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: IndustryRegulation&Waste.NW@cyfoethnaturiolcymru.gov.uk Environment team: TimAmgylcheddMonacArfon@cyfoethnaturiolcymru.gov.uk NLO: IndustryRegulation&Waste.NW@cyfoethnaturiolcymru.gov.uk |
| Date due: | 14/03/2023 |
| Are fields on Anglesey? If yes, then send application. | Yes: ajxht@anglesey.gov.uk pem@anglesey.gov.uk |
| Permit compliance rating and multiplier | Rating A- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | EPR/JB3533RJ |
| Correct application form | Y |  |
| Correct fee | N | See permit compliance multiplier above and deployment fee- Charging Scheme. Application has been assessed as High Risk not Medium 1 Returned |
| Fee on MyNRW | Y | Amount: £798 Date cleared on MyNRW: 23/01/23 Transaction number: 2111004 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | Y |  |
| Site Specific Risk Assessment | N | High risk location only. Application has been assessed as High Risk not Medium 1 Returned |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y | EPR/JB3533RJ |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | N | NGR provided is approx. 88m SW of the farm. Returned |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | Application CPH 53/127/0018 provided on landowner confirmation |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y | 50 |
| Is the land to be treated within a 10 miles radius of the main address? | Y | NGR for main address is incorrect. Returned |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | Y |  |
| ## Have details of the land owner or occupier been provided? | Y |  |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y |  |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | n/a |  |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | High Risk |  |
| Is the risk banding correct? | N | Application has been assessed as High Risk not Medium. Returned |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | N |  |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | n/a |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y |  |
| - About the waste |  |  |
| Have all parts of the waste types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable or non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y | Line 3 states ‘02 05 02 sludges from onsite effluent treatment’ but also ‘Liquid/nonstackable’. Is it either liquid or sludge. Returned |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y |  |
| - About the storage |  |  |
| Has all of waste storage table been completed? | n/a | Spread on delivery |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | n/a |  |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given. | N | No confirmation of authority to sign declaration. Returned |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y | On Behalf of Permit Holder however no confirmation to sign declaration. Returned |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y | NGR for main address incorrect. Returned |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | n/a | Spread on delivery |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y | Several ponds shown on MyMap not shown on application maps. Collects shown in field 12 flowing SE, application map does not clearly identify non spread buffers. Returned |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | Y |  |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | Y | Several wells, springs and issues within 50m not shown and no clear non spread buffers on application maps. Returned |
| location of all storage sites for the waste to be spread | n/a | No storage in application |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | Y | Several wells, springs and issues within 50m not shown and no clear non spread buffers on application maps. Returned |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | Y | Several wells, springs and issues within 250m not shown on application maps Returned |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | Y | SSSI S&SE of field 19 Returned |
| location of any public rights of way (shown on map) | Y |  |
| any buildings or houses within 250m (shown on map) | Y |  |
| Within Groundwater Safeguard Zone | N |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Hedges - Ditches | Y | See comments. Returned |
| What is the slope of the fields? | All fields 0-7 degrees |  |

| Waste Storage | Y N n/a | Comments |
| --- | --- | --- |
| Is waste storage occurring at the place where waste is being deployed? | N |  |
| Is only waste as specified in this deployment being stored in the proposed storage locations? | n/a |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in the same place one constructed within the last two years? or in different location? | n/a |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in a field: - liable to flooding or becoming waterlogged, or - within 50 metres of a borehole, spring or well or - within 10 metres of surface water or a land drain - within 30 meters of a watercourse on steeply sloping land | n/a |  |
| If the waste is non-stackable is the waste stored within either a vessel, a covered building, or on an impermeable surface | n/a |  |
| If the waste is high RAN and it is being stored within 200 metres of a European Site (SAC/SPA), Ramsar Site or a Site of Special Scientific Interest (SSSI) is the waste being covered to prevent loss of ammonia? (condition 3.2.4 of the permit) | n/a | Not stored |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y | Not assessed. Returned |
| Does the analysis include the following? |  |  |
| - pH | Y N |  |
| - Nitrogen (N) | Y N |  |
| - Phosphorus (P) | Y N |  |
| - Potassium (K) | Y N |  |
| - % dry matter | Y N n/a |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y N n/a |  |
| - Copper | Y N n/a |  |
| - Nickel | Y N n/a |  |
| - Lead | Y N n/a |  |
| - Cadmium | Y N n/a |  |
| - Mercury | Y N n/a |  |
| - Magnesium | Y N n/a |  |
| - Sodium | Y N n/a |  |
| - Sulphur | Y N n/a |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | N |  |
| - If yes; have they been tested for? | n/a |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y N n/a | Not assessed. Returned |
| Does the analysis include the following? |  |  |
| - pH | Y N |  |
| - Potassium (K) | Y N |  |
| - Phosphorus (P) | Y N |  |
| - Magnesium (Mg) | Y N |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | Y N n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y N |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y N n/a |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y N | Not assessed. Returned |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y N | Not assessed. Returned |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | n/a |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| Timing of application- if any of the wastes are high RAN, are they being spread OUTSIDE of the closed periods | n/a |  |
| Groundwater Safeguard Zone for nitrate- if any of the wastes are high RAN, are the application rates less than 50 m3/ha, with a minimum 3 week gap between applications | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | Y N n/a | Not assessed. Returned |
| - Included the nutrient level in their environmental risk assessment | Y N n/a | Not assessed. Returned |
| - Stated the source of information that they used to set the nutrient level. | Y N n/a | Not assessed. Returned |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need, as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y N | Not assessed. Returned |
| - Waste analysis | Y N | Not assessed. Returned |
| - Crop nutrient requirements | Y N | Not assessed. Returned |
| If the cropping is any of the following, is the maximum nitrogen limit in line with that given in The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021: - 0 kg/ha- Field beans, Peas - 120 kg/ha- Sugar beet - 150 kg/ha- Asparagus, Carrots, Forage maize. Radish, Spring barley, Swedes - 180 kg/ha- Spring-sown wheat, Winter barley - 220 kg/ha- Autumn or early winter sown wheat | n/a | grass |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | Y |  |
| - Is it only for operational purposes? | Y |  |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | Y |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients given by the applicant match NRW calculations? | Y N | Not assessed. Returned |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | n/a |  |
| - How much material was spread? | n/a |  |
| - At what application rate material was spread? | n/a |  |
| - The nutrients supplied by the material? | n/a |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | n/a |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | n/a |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | n/a |  |
| - Pathways | n/a |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | n/a |  |
| - Measures that will minimise or mitigate the impact will be put in place | n/a |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, then complete an Appendix 4 CroW assessment. | Y | Malltraeth Marsh / Cors Ddyga, a SSSI which is within 500metres South and Southeast of field 19. Not identified in application. Returned |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | Y N n/a | Not assessed. Returned |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | N |

Comments

Consultation responses: Checked and closed

MyMap assessment: Applicant has not identified the ponds in the south of field 10, to the east boundary of field 11, to the south of field 12, to the east boundary of field 10 or to the north east of field 19. Non spread buffers do not clearly shown on application maps. Well N of field 8, issue E and flowing E from the boundary of field 12, spring to SE of feild 12, drain to SE of field 10, well to W of field 9. None of these are identified on application maps and no clear non spreading buffers in place. Collects within and flowing SE to the boundary of field 12 does appear to potentially have a non spread buffer but again this is not clearly identified, nor is the distance of this buffer on the application map. Well SE of field 19 not identified on application map, drains NW/SW of fields 18 and 19 not clearly identified on application maps. Non spread buffers not clearly identified on application maps. Well W of field 10, SW of field 10, S of field 10, spring and pond SE of field 12, pond NW of field 8, drain E of field 12 drain W of field 9, drain S of field 10, Issue NE of field 12. None of these are identified on application maps. 3 wells W of field 18, spring and sink W of field 19, well SW of field 19, 2x drains SW of field 19, well E of field 19, drain and two ponds N of field 18. None of these are identified on application maps. Malltraeth Marsh / Cors Ddyga, a SSSI is min approx. 223m SE and max approx. 358m SE of field 19 also extends S of field 19 at approx. 240m. not identified on application maps. Plantation on ancient woodland and restored ancient woodland NW of field Coed Henblas LWS (SINCs) NE of field 12. Fferam Parc LWS (SINCs) immediately adjacent to SW of field 9. Purple moor grass and rush pastures, Lowland dry acid grassland and Lowland calcareous grassland to the S, SW, W, NW and N of fields 8/9/10. Purple moor grass and rush pastures and Lowland dry acid grassland and Lowland calcareous grassland S of field 19 and NW of field 18 and 19. Location assessment: NGR provided is approx. 88m from the main address Slope assessment: All fields 0-7 degrees.

CPH details: All fields CPH 53/127/0018 - FFERAM BAILEY LTD, FFERAM BAILEY, TREFDRAETH, BODORGAN, YNYS MÔN. LL62 5ET. CPH is as per application information provided.

Protection of habitats and ecosystems

Agricultural benefit calculations: Not completed as application returned

Request for information: Bryan Burn 23/03/23. Email note for returned application We stopped the determination of this application once we had completed the location assessment and identified that the risk banding is incorrect. About the land you want to treat - The national grid reference you have provided for the main address is incorrect, it is approximately 88 metres away. ACTION: If you choose to resubmit an application for this deployment location you will need to provide an accurate grid reference for the main address. Location risk - In your application form, you have chosen ‘Not within Source Protection Zone 2, and/or are over 500 meters from a European site, Ramsar, SSSI’. You have not considered Malltraeth Marsh / Cors Ddyga SSSI which is within 500 metres to the South and Southeast of field 19. As a result, you have incorrectly identified the risk band for this deployment as a Medium risk (1) deployment when it should be a High Risk deployment application. ACTION: If you choose to resubmit an application for this deployment location you will need to acknowledge Malltraeth Marsh / Cors Ddyga SSSI and select the correct answer. - As a result of point 2 above you have not paid the correct fee for this application. ACTION: If you choose to resubmit an application for this deployment you will need to identify and pay the relevant fee for a high risk deployment. - In your application form, you have chosen ‘I am not within an SPZ 2 and/or 500 m of a European site, Ramsar or SSSI’ but have addressed risks to other receptors in my benefit statement.’, You have not considered Malltraeth Marsh / Cors Ddyga SSSI which is within 500 metres to the South and Southeast of field 19. ACTION: If you choose to resubmit an application for this deployment location you will need to select the correct option and provide a site specific risk assessment. About the Waste. - In your application you have completed the table describing the wastes; however, there is not a clear or direct cross reference for the 02 05 01 wastes to the wastes in your submitted laboratory waste analysis in order for us to identify which waste is which. ACTION: If you choose to resubmit an application for this deployment location you will need to provide a clear cross reference for each of the wastes in your deployment application across the application form, the benefit statement and the waste analysis. - In the same table, the waste 02 05 02 is described as ‘sludges from onsite effluent treatment’ although you have described its physical form as ‘Liquid/nonstackable’. ACTION: If you choose to resubmit an application for this deployment location you will need to confirm if this waste is a sludge or a liquid. Declaration - You have signed the application declaration on behalf of Simon Jones but have not provided a letter of confirmation from Simon Jones giving you authority to do so. ACTION: If you choose to resubmit an application for this deployment location you will need to provide written confirmation from Simon Jones which clearly gives you authority to submit the application and sign the declaration on his behalf. Spreading maps - In your spreading maps you have not provided a key that can enable us to interpret the map to identify what is a non-spreading buffers and what is a field boundary. ACTION: If you choose to resubmit an application for this deployment location you will need to provide a key to your maps that will enable us to interpret your maps correctly. - In your spreading maps you have not identified, or indicated non spread buffers for, the ponds in the south of field 10, to the east boundary of field 11, to the south of field 12, to the east boundary of field 10 or within the Northeast boundary of field 19. Although possibly seasonal these still require a 10 metre non spread buffer. In addition, a watercourse collects and flows Southeast within and to the boundary of field 12, the red line on your application map does not clearly state if this is a non-spreading buffer or the distance of that buffer. ACTION: If you choose to resubmit an application for this deployment location you will need to adjust your spreading maps to identify the ponds and watercourse mentioned above. The resubmitted maps must also include any required non spreading buffers. This is in line with condition 2.1.7 of your SR2010 No 4 permit. - Your spreading map does not identify the following which are within 50 metres, a well North of field 8, an issue East and flowing East from the boundary of field 12, a spring Southeast of field 12, a drain Southeast of field 10 or a well West of field 9. Neither have you identified a well Southeast of field 19, drains Northwest and Southwest of fields 18 and 19. There are no clearly identifiable non spreading buffers in place on your application maps. ACTION: If you choose to resubmit an application for this deployment location you will need to adjust your spreading maps taking the above into account. The resubmitted maps must also include any required non spreading buffers. This is in line with condition 2.1.7 of your SR2010 No 4 permit and TGN 8.01. - In addition to those mentioned above within 50 metres, your spreading maps do not identify the following which are within 250 metres of your proposed location. Wells West of field 10, Southwest of field 10, and South of field 10, a spring and pond Southeast of field 12, a pond Northwest of field 8, drains East of field 12, West of field 9, South of field 10 or an issue Northeast of field 12. Neither have you identified three wells West of field 18, a spring and sink West of field 19, two wells SW and East of field 19, two drains SW of field 19 or a drain and two ponds N of field 18. ACTION: If you choose to resubmit an application for this deployment location you will need to adjust your spreading maps acknowledging the above. This is in line with TGN 8.01. - Your spreading maps and section 8.0 of your benefit statement do not identify Malltraeth Marsh / Cors Ddyga SSSI which is within 500metres to the South and Southeast of field 19. ACTION: If you choose to resubmit an application for this deployment location you will need to adjust your application taking the above into account. - Your spreading maps do not identify the following within 500m of your proposed location, ancient woodland to the Northwest of field 9, Coed Henblas Local Wildlife Site (SINCs) to the Northeast of field 12, You have not identified several habitats of principal importance for the purpose of maintaining and enhancing biodiversity in relation to Wales, as detailed in section 7 of the Environment (Wales) Act 2016, these are to the South, Southwest, West, Northwest, and North of fields 8/9/10, South of field 19 and Northwest of fields 18 and 19. Fferam Parc Local Wildlife Site (SINCs), which is immediately adjacent to the Southwest of field 9, is mentioned in section 8.0 of your benefit statement as a human receptor but you have not addressed the potentially negative impacts on the Local Wildlife Site from the deployment. ACTION: If you choose to resubmit an application for this deployment location you will need to adjust your application to acknowledge the above including any relevant control measures. Agricultural benefit statement - In section 5.0 of your benefit statement, you have stated that ‘The notified area is located outside a designated Nitrate Vulnerable Zone (NVZ).’. This is incorrect as the whole of Wales was designated as a NVZ on the 1st of January 2023 in accordance with The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021, closed periods came into effect on the 1st of January 2023. ACTION: If you choose to resubmit an application for this deployment location you will need to adjust your application to take the above into account including any relevant changes to your proposed deployment impacted by The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021. - In section 8.0 of your benefit statement (Potential Sensitive Receptors) you have stated that there are ‘no Sites of Special Scientific Interest (Maen Gwyn SSSI) within 500 m of the notified area.’. You have not identified Malltraeth Marsh / Cors Ddyga SSSI which is within 500 metres South and Southeast of field 19. ACTION: If you choose to resubmit an application for this deployment location you will need to adjust your application to take the above into account, also removing reference to Maen Gwyn SSSI

Other information to note:

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | No |
| ## Issue date | n/a |
| ## Effective date | n/a |
| ## Permitting Officer Date | Bryan Burn 23/03/23 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 23/03/23 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 1230 |
| Sign off | 20 |
| Issuing | Returned |
| Total | 1310 |