| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-020670 |
| Permit Reference Number: | EPR-GP3792SK |
| Permit Holder: | ByProduct Recovery Ltd. |
| Site Address: | Nantywenynen, Aberdare, Rhondda Cynon Taf, CF44 9JD |
| Grid Reference: | Farm location: SN 93760 14624 Fields: Nantywenynen 3 - SN 94182 16231 Nantywenynen 5 - SN 94156 15900 Nantywenynen 17 - SN 94060 14800 Beili Gwern 1 - SN 90581 24725 Beili Gwern 2 - SN 90418 24568 Beili Gwern 3 - SN 90606 24609 Beili Gwern 4 - SN 90573 24458 Beili Gwern 5 - SN 90433 24378 Penycae 1 - SN 84285 13515 Penderyn 1 - SN 94790 09370 Fields removed due to being on an SPZ1. Penderyn 2 - SN 94880 09430 Penderyn 3 - SN 94890 09291 Penderyn 4 - SN 94954 09371 Gorslwyd 1 - SN 93848 23645 Gorslwyd 2 - SN 93913 23434 Gorslwyd 3 - SN 93948 23299 Gorslwyd 4A - SN 93779 23368 Gorslwyd 4B - SN 93812 23286 Gorslwyd 5 - SN 93672 23386 Gorslwyd 9 - SN 93677 23578 Gorslwyd 12 - SN 93490 23571 Storage: SN 92972 24069 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: GwastraffaGorfodi.Canolbarth@cyfoethnaturiolcymru.gov.uk wasteregulation.southcentral@naturalresourceswales.gov.uk Environment team: environmentteam.southpowys@cyfoethnaturiolcymru.gov.uk cwmtaf&bridgendconsultations@cyfoethnaturiolcymru.gov.uk NLO: Ewan.Marshall-Harries@environment-agency.gov.uk |
| Date due: | 14/03/2023 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating B- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | English- EA public register, live, variation took place 2015 on DMS. |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and deployment fee- Charging Scheme |
| Fee on MyNRW | Y | Amount: £1,018 Date cleared on MyNRW: 21/09/2022 Transaction number: 2105706 : |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | Y |  |
| Site Specific Risk Assessment | Y | High risk location only |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | See comments at the bottom of the decision document |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y |  |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y |  |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y |  |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | Y | Previous deployment: PAN-014569 Expired: 20/08/2022 |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | High |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) of High risk deployment has a SSRA been provided? | Y |  |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | Site‐specific risk assessment |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | n/a |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y |  |
| - About the waste |  |  |
| Have all parts of the waste types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable or non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y |  |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y |  |
| - About the storage |  |  |
| Has all of waste storage table been completed? | Y |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y |  |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | Y | Fields P1-4 were removed from the deployment due to being located within an SPZ 1. |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | Y |  |
| location of all storage sites for the waste to be spread | Y |  |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | n/a |  |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | n/a |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | Y | Field BG 1 is located 463 meters from the River Usk SAC and the River Usk SSSI |
| location of any public rights of way (shown on map) | Y |  |
| any buildings or houses within 250m (shown on map) | Y |  |
| Within Groundwater Safeguard Zone | n/a |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Hedges - Ditches | N | See comments at the bottom of the decision document |
| What is the slope of the fields? | 0-7 degrees (transparent)-Fields: BG 1, G12 , G1, G3, G4B, G4B & Peny 1 7 -12 degrees (yellow)- Fields: BG3, G9, G2 (partially) BG 4, BG5, G5, N3 & N5 (mostly) 12 degrees plus (red) Fields: N17 (partially) N5 (mostly) See comments at the bottom of the decision document for more information. |  |

| Waste Storage | Y N n/a | Comments |
| --- | --- | --- |
| Is waste storage occurring at the place where waste is being deployed? | Y |  |
| Is only waste as specified in this deployment being stored in the proposed storage locations? | Y |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in the same place one constructed within the last two years? or in different location? | n/a |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in a field: - liable to flooding or becoming waterlogged, or - within 50 metres of a borehole, spring or well or - within 10 metres of surface water or a land drain - within 30 meters of a watercourse on steeply sloping land | n/a | The wastes are non-stackable sludges. |
| If the waste is non-stackable is the waste stored within either a vessel, a covered building, or on an impermeable surface | Y |  |
| If the waste is high RAN and it is being stored within 200 metres of a European Site (SAC/SPA), Ramsar Site or a Site of Special Scientific Interest (SSSI) is the waste being covered to prevent loss of ammonia? (condition 3.2.4 of the permit) | n/a | The waste does not have a high RAN content. |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | n/a |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y |  |
| - If yes; have they been tested for? | Y |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | Y |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y | Maintenance dressing only. |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | n/a |  |
| - Soil physical properties (e.g. organic matter and soil structure) | n/a |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| Timing of application- if any of the wastes are high RAN, are they being spread OUTSIDE of the closed periods | n/a |  |
| Groundwater Safeguard Zone for nitrate- if any of the wastes are high RAN, are the application rates less than 50 m3/ha, with a minimum 3 week gap between applications? | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need, as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If the cropping is any of the following, is the maximum nitrogen limit in line with that given in The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021: - 0 kg/ha- Field beans, Peas - 120 kg/ha- Sugar beet - 150 kg/ha- Asparagus, Carrots, Forage maize. Radish, Spring barley, Swedes - 180 kg/ha- Spring-sown wheat, Winter barley - 220 kg/ha- Autumn or early winter sown wheat | n/a |  |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | Y |  |
| - Is it only for operational purposes? | Y |  |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | Y |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients given by the applicant match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y |  |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | Y |  |
| - The nutrients supplied by the material? | N | See previous deployment: PAN-014569 |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | Y |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | Y |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | Y |  |
| - Pathways | Y |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | Y |  |
| - Measures that will minimise or mitigate the impact will be put in place | Y |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | Y |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | N |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | Y |
| No FORM 1 completed: HRA is not required because there is no conceivable impact pathway to any Natura 2000/Ramsar site for the following reasons: - Field BG 1 is the closest field to the River Usk SAC located 456 meters away. There is a 10m no spread buffer zone in field BG1 on the border of the Cwm Nant-Ddu stream to mitigate the risk of nutrients and/or PTEs entering the stream which could potentially lead to the River Usk SAC. This buffer zone complies with SRP. - Field BG 1 is located on flat to gently sloping land (0-7o) and does not pose a surface runoff risk as long as wastes are spread in line with the submitted documents. - Wastes will be spread via tractor and tanker using a low-trajectory splash plate. Which is an appropriate application method for the waste type (EWC 19 09 02). - All fields under this deployment have a greater overall offtake of phosphate than is being added by the spreading, so the overall levels of phosphate in the soils will decrease from their current level within the 12 month period of this deployment. There will be no overall phosphate increase to the land as a result of this application. Therefore it is unlikely excess phosphate will leachate into the Cwm Nant-Ddu stream as the soil will effectively uptake the phosphorous. - Water courses/springs have been appropriately buffered in accordance with the stand rules permit to prevent nutrients and/or PTEs from entering the water courses/ SAC. - High Aluminium sludges (DCWW Cefn Dryscoed) will not be spread on fields with a soil p.H with less than 6. - All wastes streams are non-stackable liquid sludges, so dust will not be generated during the activity. and as such, no dust from this activity should enter any protected water courses/SAC. - Wastes will be spread at appropriate times when the crop will benefit from the nutrients provided spring (late Feb – Apr), and after cutting of silage (2 cuts) through summer and autumn Applications will be carried out when ground conditions are suitable in line with COGAP. - A site-specific risk assessment has been provided by the operator to take into account any potential issues and has provided appropriate mitigation measures to protect the SAC and other sensitive sites from harm. - The maximum application rate for each field may be split into multiple applications and will not exceed 50t/ha in any one application to a field to reduce the risk of surface runoff and to insure nutrients are effectively up taken by the soils. - Sampling methods will be consistent with those set out in the RB209, and the analysis for PTEs are consistent with the code of agricultural practice. |  |

Comments

Consultation responses: - A comment was left to implement a no spread buffer zone around surface water in field N17. This was requested in the request for information 2. - It was requested a Spring in field Penderyn 3 is buffered by 50 meters. It was communicated to the commenting officer that these fields (Penderyn 1-4) were removed from the deployment due to being located on a source protection 1. - A comment was left to ensure that the applicant notifies NRW a minimum of 2 days prior to spreading. This was mentioned in the request for information 2 as a note. - Concerns were raised with Some field having a low p.H and spreading alum rich sludge. It was explained to the commenting officer that one field is below a pH of 6 and cannot receive the alum based sludge as per TGN 8.01. - A comment was left regarding the fields being not within 10 miles of the main farm address. It was explained to the commenting officer the fields are within a 10 mile radius of the main farm address where a map was used to confirm this. - Concerns were raised with spreading EWC 19 09 02 on fields located near the Cwm Cadlan SSSI/SAC. These fields have now been removed from the deployment due the fields being located on an source protect zone 1, which is not permitted in the standard rules permit. As a result the SSSI/SAC is not in screening distance to the other fields of this deployment. The commenting officer was made aware of this. - A comment was left stating fields Penderyn 1-4 are suitable to spread on. These fields were removed from the deployment as they are located on an source protection 1 zone.

MyMap assessment: Fields Penderyn 1, 2, 3 & 4 are located on a source protect zone 1. ‘Penderyn Bh source protect zone’. As a result these fields need to be removed from the deployment application as wastes cannot be stored or spread within an source protect zone 1 as stated in the SRP paragraph 2.1.7. The full Mymap assessment is available to view on the DMS. Slope assessment: Field number: N3, N5 & N17 Slope of the land: 7-12o (moderately sloping) & 12o+ (highly sloping) Soil type: Medium soils Wastes types applied: Non stackable sludges Cropping type: Grass Method of application: Low trajectory splash plate Weather conditions before during and after spreading: In compliance with the restrictions outlined in CoGAP. Conclusions (is run off risk high?): Areas of land that are highly sloping (12o+) have no spread buffer zones implemented to reduce the risk of surface runoff and/or leachate. The areas of land that are moderately sloping will still receive wastes where the wastes will be applied as followed: - The waste will be spread via an appropriate method for the waste type (low trajectory splash plate). The low trajectory method will help reduce the risk of nutrients running off the field. - No more than 50t/ha will be spread on a field in any 3-week period to allow the field to effectively uptake the wastes. - Non stackable sludges do not pose the highest risk of runoff in comparison to non-stackable liquids. The overall risk of surface runoff and/or leachate can be mitigated if the wastes are spread in line with the submitted documents I.e. Agricultural benefit statement. Field number: BG3, BG 4, BG5, G9, G2 & G5 Slope of the land: 0-7o (moderately sloping) Soil type: Medium soils Wastes types applied: Non stackable sludges Cropping type: Grass Method of application: Low trajectory splash plate Weather conditions before during and after spreading: In compliance with the restrictions outlined in CoGAP. Conclusions (is run off risk high?): See the above comments for moderately sloping land. For more information see the Mymap report on the DMS.

CPH details: CPH given LPD1: 52/071/0030, 52/068/0001, 52/031/0048, 58/569/0036, 52/035/0010 MyMap record - N3 - N17 = 52/071/0030 D T J DAVIES - BG1 - BG5 = 52/031/0048 HJM + B PRICE - PY 1 =52/068/001 BE TYLER - PD 1- 4 = 58/569/0036 DB + DR WILLAM - G1-G12= 52/035/0010 JK MORGAN The CPH numbers provided match the Mymap record.

Protection of habitats and ecosystems One field is located within a protected habitat: Lowland dry acid grassland. Aerial imagery and the submitted EIA evidence shows fields N3 is agriculturally improved fields. No action needed. Two fields are located within a protected habitat: Purple moor grass and rush pastures. Aerial imagery and the submitted EIA evidence shows fields BG 1 is agriculturally improved fields. No action needed.

Agricultural benefit calculations: Waste stream DCWW Cefn Dryscoed contains a high Al content. As a result fields with a pH of less than 6 will not receive this waste (Field Penycae 1).

Request for information: 10/03/2023 - Check the payment for this application has been paid. The applicant has confirmed the payment now has be paid. Information of this can be found at the top of the decision document. - Provide more details on the referenced that was used to make this payment including the date to which the payment was made. The applicant has confirmed when the payment was made 16/09/2022. - Update the LPD1 supplement to show how much waste was spread under the previous deployment. The applicant has updated the document. - Remove fields Penderyn 1, 2, 3 & 4 from this deployment application. The applicant has removed these fields from the deployment because they are on an source protection zone 1. - Update the site plan to show the ancient woodland site near field 4B. The applicant has updated the site plan. - Update the site plan to show the SINC site that borders field Penycae 1. The applicant has updated the site plan. - Update the site plan to show the SINC site that borders field Beili Gwern field 5. The applicant has updated the site plan. - Update the site plan so that acknowledges the fields in this deployment fall within the Brecon beacons national park. The applicant has updated the site plan. - Update the site plan to show the public footpath near field Nantywenynen 3 & Penycae 1. The applicant has updated the site plan. - Update the SSRA to include the following sensitive sites in the source receptor & pathway format. The applicant has updated the document and has risked assessed all the sensitive receptors with appropriate mitigation measures. - Confirm all wastes in this deployment are non-stackable sludges. The applicant this confirmed the wastes under this deployment are all non-stackable sludges. - If wastes are all non-stackable sludges remove the statement of solids wastes from the agricultural benefit statement. The applicant has removed the statement from the agricultural benefit statement. - Confirm only waste specified in this deployment (EWC 19 09 02) will be stored in the above ground storage tank. The applicant has confirmed this statement. Request for information 2 27/03/2023 - Update the site plan to show the surface water located in field N17. The applicant has updated the site plan. - Update the site plan to show a no spread buffer zone around the surface water in field N17. The applicant has update the site plan.

Other information to note:

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 29/03/2023 |
| ## Effective date | 29/03/2023 |
| ## Permitting Officer Date | Owen Clee 29/03/2023 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 29/03/2023 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 1,350 |
| Sign off | 10 |
| Issuing | 10 |
| Total | 1,430 |