PAN-021067  My Map Report:

Application maps

Deployment boundaries and storage

Detailed river network 10m:

Watercourse to SE of 13 flowing SW-NE, watercourse to S of 14 flowing W-E, pond NE of 12 with waterflowing NW-SE of 12 and 13. Buffers in place on application map.

Groundwater protection zones

None

Any well, spring (or issue) or borehole used / not used for the supply of water for human consumption including from a private water supply or supplies within 50m.

Issue S of 13 watercourse flowing NE, pond N of 12 watercourse flowing SE, NE and then to the S of 14 flowing E, covered reservoir NW of 1, buffers in place on application map.

Any well, spring (or issue) or borehole used / not used for the supply of water for human consumption including from a private water supply or supplies within 250m.

Spring W of 9, issue NW of 9, pond W of 1, identified on application map.

Spring and well N of 14, issue SE of 14, not identified on application maps due to scale RFI

European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m

None

Other protected sites within 500m:

- Protected Species-

- Protected Habitats- see below

- Scheduled Ancient Monument-

- Local Nature Reserve-

- National Nature Reserve-

- Ancient Woodlands- see below

- Local Wildlife Sites (SINC)-

- National Park-

- Nitrate Vulnerable Zone (NVZ)- All areas of wales NVZ as of 01/01/2023

- Groundwater Safeguard Zone-

- Hedges-

- Ditches-

Section 7 habitats

NW of 12, shown on application map but not identified as protected.

Adjacent W to 1 at covered reservoir – 50m no spread buffer on application map.

N of 14, not shown on application maps due to scale.

Ancient woodland

Adjacent E to 21, 22. E of 13, 14. Shown on application map but not identified as protected.

NW of 14, not shown on application maps due to scale

RFI

Public rights of way

Applicant has identified footpaths and marked 2m no spread zones

Distance from Main Farm Address

Slope Map

Fields 1, 2, 3, 12, 14 & 22 – 0-7 degrees

Fields 13 & 8 – 0-7 and 7-12 degrees

Fields 4, 5, 6, 7, 9 & 10 – 7-12 degrees

END