| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-021141 |
| Permit Reference Number: | EPR/SP3699VF |
| Permit Holder: | PAUL SWEENEY AGRONOMY LIMITED |
| Site Address: | HOME FARM, OAKENHOLT, FLINT, CH7 6DF - RFI |
| Grid Reference: | Farm location: SJ 26580 69482 Fields: 1 SJ2670 4020 SJ 2640070200 2 SJ2669 4055 SJ 2640069550 3 SJ2669 1769 SJ 2617069690 4 SJ2669 1745 SJ 2617069450 5 SJ2669 7444 SJ 2674069440 6 SJ2669 8631 SJ 2686069310 7 SJ2669 6414 SJ 2664069140 8 SJ2668 4378 SJ 2643068780 9 SJ2568 7388 SJ 2573068880 10 SJ2568 5488 SJ 2554068880 11 SJ2568 4967 SJ 2548968670 12 SJ2568 5975 SJ 2559068750 13 SJ2568 7677 SJ 2576068770 Storage: 1 SJ 2645 970353 - RFI 2 SJ 2645 268877 - RFI |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: WasteReg.NE@cyfoethnaturiolcymru.gov.uk Environment team: WrexhamFlintshireEnvironmentTeam@cyfoethnaturiolcymru.gov.uk NLO: sue.king@environment-agency.gov.uk |
| Date due: | 29/03/2023 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating A- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | EPR/SP3699VF |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and deployment fee- Charging Scheme |
| Fee on MyNRW | Y | Amount:798 Date cleared on MyNRW:19/01/23 Transaction number: 2110852 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | N | No lab analysis, only applicant interpretation The analysis of the waste must have been completed within 12 months of the deployment application date and should have been analysed in a laboratory with accredited methods. For all deployment applications the analysis must include:  the date when sampled  the sampling methodology and,  details of the laboratory that carried out the analysis The analysis should be supplied on laboratory data sheets RFI |
| Soil Analysis | N | No lab analysis, only applicant interpretation RFI |
| EIA Agri Regulations evidence | Y | FIELD SCHEDULE: ENVIRONMENTAL IMPACT ASSESSMENT, text only RFI |
| Site Specific Risk Assessment | n/a | High risk location only |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | Y | Withdrawn and returned x2 due to previous deployments expiration date- no technical assessment pre-application advice was given |
| ## Has the permit number been provided? | Y | EPR/SP3699VF - EA |
| ## Has the permit holder name been provided | Y | PAUL SWEENEY AGRONOMY LIMITED |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y | Farm name on MyMaps is Plas Bellin not Home Farm, visual map comparison. NGR is incorrect and shows as FRON BACH,OAKENHOLT LANE,NORTHOP,MOLD,CH7 6DF. Postcode is as per application but address is incorrect. RFI |
| ## Does the farm address match the grid reference? | N | Farm name on MyMaps is Plas Bellin not Home Farm, visual map comparison NGR is incorrect and shows as FRON BACH,OAKENHOLT LANE,NORTHOP,MOLD,CH7 6DF. Postcode is as per application but address is incorrect. RFI |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | Provided 56 / 232 / 0233 All fields 56/232/0233 C THOMAS FARMING LTD LEADRROOK HALL BARN. |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y |  |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y |  |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y |  |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | Y | Fields 1, 2, 9-13 only. PAN-017201 Expires 05/05/23 also see RFI comments |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | Medium (1) |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) of High risk deployment has a SSRA been provided? | n/a |  |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | n/a |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y | FIELD SCHEDULE: ENVIRONMENTAL IMPACT ASSESSMENT, text only RFI |
| - About the waste |  |  |
| Have all parts of the waste types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable or non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y | Waste 1 03 03 11 not identified as liquid, solid or sludge. 03 03 11 relates to sludge from on-site effluent treatment - RFI Waste 2 03 03 11 not identified as stackable or non stackable. Described as liquid. 03 03 11 relates to sludge from on-site effluent treatment - RFI |
| Are the wastes listed in the deployment specifically listed in the permit? | Y | 03 03 11 |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | Y |  |
| - About the storage |  |  |
| Has all of waste storage table been completed? | Y | Location 1 and 2 differ between list and map RFI |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y | 300 x 2 |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | n/a |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y | Field map page 1 shows field 1 at incorrect location when compared to the actual spread map for the field. Typo error. NGR is correct for all feilds |
| If there is storage; do the grid references match the site plan? | N | See comments |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y |  |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | Y | none |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | Y |  |
| location of all storage sites for the waste to be spread | Y | Incorrect NGR. See comments |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | Y |  |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | Y |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | Y |  |
| location of any public rights of way (shown on map) | Y |  |
| any buildings or houses within 250m (shown on map) | Y |  |
| Within Groundwater Safeguard Zone | Y |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Hedges - Ditches | Y |  |
| What is the slope of the fields? | All fields 0-7 degrees (transparent) |  |

| Waste Storage | Y N n/a | Comments |
| --- | --- | --- |
| Is waste storage occurring at the place where waste is being deployed? | Y |  |
| Is only waste as specified in this deployment being stored in the proposed storage locations? | Y |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in the same place one constructed within the last two years? or in different location? | Different | NGR provided in applicagtion indicate different locations however the spreading maps show storage in same location as previous PAN RFI |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in a field: - liable to flooding or becoming waterlogged, or - within 50 metres of a borehole, spring or well or - within 10 metres of surface water or a land drain - within 30 meters of a watercourse on steeply sloping land | Y | Location in field 8 is potentially within 50m of watercourse N and pond E. Location in field 1 is potentially within 50m of a watercourse and pond RFI |
| If the waste is non-stackable is the waste stored within either a vessel, a covered building, or on an impermeable surface | Y | Field heaps. Question the form of the wastes as one is stated as a liquid RFI |
| If the waste is high RAN and it is being stored within 200 metres of a European Site (SAC/SPA), Ramsar Site or a Site of Special Scientific Interest (SSSI) is the waste being covered to prevent loss of ammonia? (condition 3.2.4 of the permit) | n/a | Not high RAN, |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | N | No lab analysis, only applicant interpretation RFI |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | N |  |
| - If yes; have they been tested for? | n/a |  |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | N | No lab analysis, only applicant interpretation RFI |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | Y |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | Y |  |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | Y |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| Timing of application- if any of the wastes are high RAN, are they being spread OUTSIDE of the closed periods | n/a |  |
| Groundwater Safeguard Zone for nitrate- if any of the wastes are high RAN, are the application rates less than 50 m3/ha, with a minimum 3 week gap between applications | n/a | Not GSZ or high RAN |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | Y |  |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need, as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | Y |  |
| If the cropping is any of the following, is the maximum nitrogen limit in line with that given in The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021: - 0 kg/ha- Field beans, Peas - 120 kg/ha- Sugar beet - 150 kg/ha- Asparagus, Carrots, Forage maize. Radish, Spring barley, Swedes - 180 kg/ha- Spring-sown wheat, Winter barley - 220 kg/ha- Autumn or early winter sown wheat | Y | Spring barley, forage maize, , turnips, winter wheat, grass see RFI |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | n/a |  |
| - Is it only for operational purposes? | n/a |  |
| - Have individual application rates for each waste been given? | n/a |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | n/a |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y |  |
| - Have they stated when the waste will be spread? | Y |  |
| - Have they stated how often the waste will be spread? | Y |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients given by the applicant match NRW calculations? | Y |  |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y | See comments |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | N | Pevious PAN shows |
| - The nutrients supplied by the material? | N |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | n/a |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | N | See maps comments |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | n/a |  |
| - Pathways | n/a |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | n/a |  |
| - Measures that will minimise or mitigate the impact will be put in place | n/a |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | N |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | n/a |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | N |

Comments

Consultation responses: Comments raised by Agri officer regarding previous spreading of biosolids on Field 4 which is not identified in application RFI Also comments regarding buffer zones however 10m buffers are in place. Waste reg officer noted that Application states waste has been spread during the last 12 months but no record of any spreading notification being submitted RFI Consultation closed

MyMap assessment: Storage Storage in field 8 potentially within 50m of a watercourse N and pond E. RFI Storage in field 1 is potentially within 50m of a watercourse and pond N RFI Detailed river network 10m Field 1, ponds to N and SW, watercourse W flowing N. Buffers in place on application maps. Ponds within field 3, W/E/SE of 4, buffers in place on application maps. Ponds SW of 5, NE of 6, W of 7. Watercourse S/SE of 7 flowing N adjacent to 6, also watercourse flowing E/S between 5/6/7. Buffers in place on application maps. Pond E of 8, watercourse N flowing E, buffers in place on application maps. Ponds S of 11 and 13, watercourse N of 9 and 10 flowing E. Buffers in place on application maps Groundwater protection zones None Any well, spring (or issue) or borehole used / not used for the supply of water for human consumption including from a private water supply or supplies within 50m. Deregulated abstrasction E of 1, buffer in place. None at 2/3, drain E of 4, buffer in place Issues N of 5&6 and N of 7 not on application maps, require additional buffer RFI None at 8, 9-13 Any well, spring (or issue) or borehole used / not used for the supply of water for human consumption including from a private water supply or supplies within 250m. Issue SW of 1 not on application map needs to be acknowledged RFI Deregulated abstractions E of 1, acknowledged on application map. Deregulated abstractions E of 4, acknowledged on application map. Issue NW of 5 not acknowledged on application map due to scale. RFI Issue S of 6 not acknowledged but watercourse is shown. Issue S of 8 not acknowledged on application map due to scale. RFI None at 9-13 European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m None Other protected sites within 500m: Ancient woodland W, E and SE, local wildlife sites W and E of 1. Not identied on application maps RFI Ancient woodland NW, E, W, and SE, local wildlife sites W and E of 2-4. Not identified on application maps RFI Ancient woodland also immediately adjacent S of 2 and E of 4, shown on application map with buffer but not identified RFI Ancient woodland to all sides of 5-7 but scale of application maps insufficient to show RFI. Ancient woodland immediately adjacent to 6/7 is identified on application maps 20m buffers in place Local wildlife site immediately adjacent to 6/7 and E of 5 not identified on application map RFI Section 7 habitat S of 7 not identified on application map RFI Ancient woodland NW, N, NE, SE of 8, not shown on application map due to scale RFI. Section 7 habitat SE and SW of 8, not shown on application map due to scale RFI. Local wildlife sites (SINCs) NW and NE of 8, not shown on application map due to scale RFI Ancient woodland and local wildlife site (SINCs) NE of 9, identified on application map 10m buffer in place. Section 7 Habitats W, SW of 10/11, SE of 13, not shown due to scale of application map RFI Location assessment: Farm name on MyMaps is Plas Bellin not Home Farm, NGR is incorrect and shows as FRON BACH,OAKENHOLT LANE,NORTHOP,MOLD,CH7 6DF. Postcode is as per application but address is incorrect. RFI Field NGR correct NGR provided for both storage locations differs between list and map RFI Slope assessment: All fields 0-7degrees

CPH details: All fields 56/232/0233 C THOMAS FARMING LTD LEADRROOK HALL BARN.

Protection of habitats and ecosystems EIA evidence provided is text only, no physical evidence provided RFI

Agricultural benefit calculations: Essity 1 – decrease in both phosphate and potash for all fields. Nominal liming effect Essity 2 – minimal addition of phosphate and potash indicating decrease for fields 3 and 4.

Request for information: Bryan Burn: 14/04/2023 RFI sent by email 14/04/2023 Response received 17/04/2023. Reviewed 19/04/2023. All points answered satisfactorily.

Other information to note:

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | Yes |
| ## Issue date | 21/04/2023 |
| ## Effective date | 06/05/2023 |
| ## Permitting Officer Date | Bryan Burn 21/04/2023 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 21/04/2023 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 2100 |
| Sign off | 10 |
| Issuing | 30 |
| Total | 2200 |