| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-021069 |
| Permit Reference Number: | EPR-JP3094EF |
| Permit Holder: | John Willis |
| Site Address: | Bank Farm II, Deeside Lane, CH1 6BX |
| Grid Reference: | Farm location: SJ 36077 66960 Fields: SID2 SJ 37013 66196 SID3 SJ 36918 66070 WF11 SJ 36268 66559 WF12 SJ 36086 66663 WF30 SJ 35632 67659 WF31 SJ 35841 67507 WF7/8 SJ 36320 67036 Storage: SJ 36485 66670 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: WasteReg.NE@cyfoethnaturiolcymru.gov.uk Environment team: WrexhamFlintshireEnvironmentTeam@cyfoethnaturiolcymru.gov.uk NLO: None allocated |
| Date due: | 22/03/2023 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating B- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | English-EA public register |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and deployment fee- Charging Scheme |
| Fee on MyNRW | Y | Amount: £1018.00 Date cleared on MyNRW:17/03/2023 Transaction number: 2113002 |
| Location map | Y |  |
| Benefit statement | Y |  |
| Waste Analysis | Y |  |
| Soil Analysis | Y |  |
| EIA Agri Regulations evidence | Y |  |
| Site Specific Risk Assessment | N | High risk location only not site specific |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | Bespoke |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y |  |
| ## Has the nominated competent person details been provided? | Y |  |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | Applicant provided- 56/209/0093 no land owner/occupier name Found on GIS- 56/209/0089 P&RH Siddon 56/209/9000 WT Banks & co (Farming) LTD |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y |  |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | Y |  |
| ## Have details of the land owner or occupier been provided? | N | Appears to be two landowners/occupiers. |
| ## Has written consent of all owners(s) or occupiers(s) been given? | N |  |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | n/a | Fields have not been spread in the last 12 months |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | Bespoke |  |
| ## Is the risk banding correct? | Y |  |
| ## If Medium risk (2) of High risk deployment has a SSRA been provided? | Y | Bespoke risk assessment provided is not site specific |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | benefit statement bespoke permit- site‐specific risk assessment | Not site specific |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | N |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y |  |
| - About the waste |  |  |
| Have all parts of the waste types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable or non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y | Description of waste is a copy of EWC description. Producers have been provided but no permit numbers. |
| Are the wastes listed in the deployment specifically listed in the permit? | Y |  |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | N | Cannot verify as producers, EPR numbers and coding are not specific |
| - About the storage |  |  |
| Has all of waste storage table been completed? | Y | suitability of waste storage in question |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | Y |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y | Net silo |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y | Correctly buffered |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | Y | SPZ3 within 5000m of spreading, no additional actions required |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | n/a |  |
| location of all storage sites for the waste to be spread | Y |  |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | n/a |  |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | n/a |  |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | Y | SSSI- Afon Dyfrchwy/River Dee 27m from spread area SAC- River Dee and Bala Lake/Afon Dyfrdwy a lly n tegid 27m from spread area |
| location of any public rights of way (shown on map) | n/a |  |
| any buildings or houses within 250m (shown on map) | Y | Possible odour risks |
| Within Groundwater Safeguard Zone | n/a |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Hedges - Ditches | Y | 19 areas of protected species-no issue 10- areas of protected habitat-no issue 4 areas of ancient woodlands |
| What is the slope of the fields? | 0-7 degrees (transparent)- fields: All fields |  |

| Waste Storage | Y N n/a | Comments |
| --- | --- | --- |
| Is waste storage occurring at the place where waste is being deployed? | Y | Netsilo- cannot meet CoGAP requirements for waste storage. |
| Is only waste as specified in this deployment being stored in the proposed storage locations? | Y |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in the same place one constructed within the last two years? or in different location? | n/a | Wastes non-stackable |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in a field: - liable to flooding or becoming waterlogged, or - within 50 metres of a borehole, spring or well or - within 10 metres of surface water or a land drain - within 30 meters of a watercourse on steeply sloping land | n/a |  |
| If the waste is non-stackable is the waste stored within either a vessel, a covered building, or on an impermeable surface | N | Netsilo is uncovered and possibly not impermeable. There is also an odour risk. |
| If the waste is high RAN and it is being stored within 200 metres of a European Site (SAC/SPA), Ramsar Site or a Site of Special Scientific Interest (SSSI) is the waste being covered to prevent loss of ammonia? (condition 3.2.4 of the permit) | n/a | Waste is not high RAN |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y |  |
| - If yes; have they been tested for? | Y | 02 07 04- pH 04 02 20- bleaches 02 06 01- pH, must not contact surface or runoff water courses 02 03 04- Odour 02 02 04- FOGs, pH 19 09 02- Iron, aluminium, organic matter 02 01 03- dust, conductivity 02 06 01- leaf scorch, spreading on calcareous land, pH 02 03 05- Odour, crop smother |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a |  |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y |  |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y | Soils are on target waste are being applied at offtake. |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y |  |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | N | Wastes are being spread at maximum amount in most cases but with negligible additions of targeted nutrients |
| - Is the material suitable for its intended use? | Y |  |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y |  |
| - Timing of application | N | Spread times for maize have passed. Spread time for winter wheat is vague. multiple applications to the winter wheat by dribble bar. |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | Y |  |
| - Soil physical properties (e.g. organic matter and soil structure) | Y |  |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| Timing of application- if any of the wastes are high RAN, are they being spread OUTSIDE of the closed periods | n/a | Wastes are not high RAN |
| Groundwater Safeguard Zone for nitrate- if any of the wastes are high RAN, are the application rates less than 50 m3/ha, with a minimum 3 week gap between applications | n/a |  |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | N | Odour issues have not been addressed by the applicant. Risk assessment |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need, as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | Y |  |
| - Crop nutrient requirements | N | Waste are being spread to provide potash and phosphate to crops waste Nicepak, Sime darby oils, Westland, and cargills are being spread at maximum amounts permissible but will provide negligible amounts of phosphate and potash, this does not meet the definition of recovery. |
| If the cropping is any of the following, is the maximum nitrogen limit in line with that given in The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021: - 0 kg/ha- Field beans, Peas - 120 kg/ha- Sugar beet - 150 kg/ha- Asparagus, Carrots, Forage maize. Radish, Spring barley, Swedes - 180 kg/ha- Spring-sown wheat, Winter barley - 220 kg/ha- Autumn or early winter sown wheat | Y | Forage maize and winter wheat. |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | N | As above several wastes being spread provide negligible amounts of targeted nutrients but are being spread at the highest permissible volume. |
| - Is it only for operational purposes? | Y |  |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | Y | Mix rations have also been provided |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | Y | Spread timings have passed or are unclear. |
| - Have they stated when the waste will be spread? | Y | As above |
| - Have they stated how often the waste will be spread? | Y | As above |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients given by the applicant match NRW calculations? | Y | Applicant has provided a partially filled in copy of NRW nutrient calculator rather than a waste interpretation summary with totals. |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | n/a |  |
| - How much material was spread? | n/a |  |
| - At what application rate material was spread? | n/a |  |
| - The nutrients supplied by the material? | n/a |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | N | A generic risk assessment has been submitted but it does not address SSSI/SAC 27m from the spread zone. |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | N | A generic risk assessment has been submitted. |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | N | Storage may not be impermeable, filling it may cause odour issues |
| - Pathways | N | SSSI and SAC not noted |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | N | Odour and caravan park have not been noted. |
| - Measures that will minimise or mitigate the impact will be put in place | N | Site specific risks have not been addressed and therefore have not been mitigated. |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | Y | Afon Dyfrchwy/River Dee SSSI |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | n/a | Application returned. |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | Y |
| No FORM 1 completed: Application returned. A consulted HRA assessment likely needed if the applicant reapplies. SAC- River Dee and Bala Lake/Afon Dyfrdwy a lly n tegid 27m from spread area |  |

Comments

Consultation responses: Consultees highlighted the lack of a site specific risk assessment, the need for an odour management plan, the proximity of the fields to the river dee (SAC/SSSI). Also noted that soils were previously at index 3 for phosphate and are now at index 1 or 2. Actioned risk assessment, OMP and proximity issues into the return letter.

MyMap assessment: PAN-021069 Mapping report Location assessment: Portions of fields 7/8 and 12 appears to be badly torn up by tires and therefore may not be suitable for spreading under TGN 8.01 box 1.7. There are no tracks in the fields. Slope assessment: All fields are sloped between 0-7 degrees, no further assessment is required.

CPH details: Applicant provided- 56/209/0093 no land owner/occupier name Found on GIS- 56/209/0089 P&RH Siddon 56/209/9000 WT Banks & co (Farming) LTD

Protection of habitats and ecosystems: Applicant has provided images of each field. Fields are of established agriculture and do not contain habitat. Portions of fields 7/8 and 12 appears to be badly torn up by tires and therefore may not be suitable for spreading under TGN 8.01 box 1.7.

Agricultural benefit calculations: Applicant provided a copy of a filled in nutrient calculator which shows the analysis of the application of matches ours. Some of the waste streams- Westland, Valero, New Britain Oil, and AB InBev have a low pH, this has not been addressed by the applicant in the application documents. It is the applicants intention to spread the waste in a mix. The mix ratios have been provided. Some of the waste streams individually provide negligible amounts of target nutrients (potash and phosphate). These waste streams are cargils, Encirc, New Britain Oils, Nicepak, and westlands. These are being spread at the maximum permissible amount. This is against technical guidance and need to be removed or the agricultural benefit statement needs to be amended.

Request for information: Return 1. Parcels of land not completed correctly 2. Permit Holder is landowner 3. No landowner/occupier permissions 4. CPH numbers 5. Spread times Maize. 6. Spreading times for wheat 7. Spreading methods 8. Waste description 9. Waste pH 10. Waste with low nutrient benefit to targeted benefit. 11. FOGs 12. Storage 13. OMP 14. NVZ 15. SSSI/SAC 16. Field damage 17. Risk assessment

Other information to note:

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | No |
| ## Issue date |  |
| ## Effective date |  |
| ## Permitting Officer Date | Ethan Banfield 04/05/2023 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 04/05/2023 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 1120 |
| Sign off | 10 |
| Issuing |  |
| Total |  |