Part A

| Permit Number | PP3139GB |
| --- | --- |
| Name of Operator | Enovert North Limited |
| Location of Installation | Bangor Road, Johnstown, Wrexham, LL14 6ET |
| Time and date of the detection | When data entered onto database |
| b) Notification requirements for breach of a limit |  |
| To be notified within 24 hours of detection unless otherwise specified |  |
| Emission point reference/source | Leachate Head Limit: >2m Sample Point; Date; Leachate Head (m). HF MP1b; 28/07/2022; 31.90. HAF LMP2AR; 21/07/2022; 1.81mtr (Transducer)+ offset. HAF LC3; 21/07/2022; 1.87mtr (Transducer). HAF LMP4b; 28/07/2022; 17.27. HAF LC4; 21/07/2022; 2.59mtr (Transducer). HAF LC5; 28/07/2022; 1.36. HAF LMP5a; 28/07/2022; 3.46. HAF LMP5b; 28/07/2022; 3.14. |
| Parameter(s) |  |
| Limit |  |
| Measured value and uncertainty |  |
| Date and time of monitoring |  |
| Measures taken, or intended to be taken, to stop the emission | The leachate remains contained and no emission from the site is occurring. |

Part B

| Any more accurate information on the matters for notification under Part A. | In July 1038m3 of leachate was tankered off site for treatment. Due to the very dry weather experienced over the month, there is significantly less leachate being produced at the site. We have been unable to install a reliable pump in MP1b. Every time we install electric pumps they only last a matter of days. Alternative pumps have been trialled i.e. pneumatic and eductor pump, but without much success due to blockages. We have had a specialist pumping contractor out to look at the wells and they are in the process of designing a pumping system that can cope with the harsh conditions in the wells. We were unable to obtain a manual dip in LMP2AR due to the dip tape catching on the pump cable, pipework and transducer cable. The pump and transducer are fitted in LMP2AR to a depth of 40m (base 47.4m). The liquid has been pumped down and the transducer is showing the level at 1.5 - 2.5m above the base of the pump. The level above base is now 9-10m. We have had our pumping contractors out to extend the electric cable on this pump but once the pump was lower in the chamber it clogged up. We moved it back up a few meters and it then began pumping again. We will look to get this well desilted in the next few weeks. Unfortunately, we were unable to obtain a manual dip for LC3 due as the probe fouled at the top of the chamber. However, the transducer attached to the pump gives a reading of between 0.8m and 1.9m throughout July. A CCTV survey will be undertaken at the same time as when we do LC4 (see below). We are now able to safely access LC4, MP4a and MP4b. The transducer dip in LC4 is 2.59m. We will now arrange to pull the pump out of LC4 and conduct a CCTV survey to see if there is any perched leachate also getting into this chamber. As stated in the previous S5’s the manual dips in LMP4a and LMP4b are thought to be due to perched levels. We have been unable to obtain manual dips for LMP4a, but transducers have been installed in both MP4a and MP4b. The transducer readings today are 11.37m for MP4a and 11.40m for MP4b. LMP5A and LMP5B are both showing similar leachate levels, however LC5 is within compliance. |
| --- | --- |
| Measures taken, or intended to be taken, to prevent a recurrence of the incident | As above. |
| Measures taken, or intended to be taken, to rectify, limit or prevent any pollution of the environment or harm which has been or may be caused by the emission | Not applicable as no pollution has been caused because no emission is occurring. |
| The dates of any unauthorised emissions from the installation in the preceding 24 months. | Please see previous notifications. |
| Name* | Elizabeth Forrest |
| Post | Environmental Data Analyst |
| Date | 01.08.2022 |

* authorised to sign on behalf of Enovert North Ltd