Permit Number: 	EPR/BW9999IG			Operator: 		Kronospan Limited

Facility:		Chirk Particleboard Factory		Form Number: 	 Air1 / 16/05/2023

# Reporting of emissions to air for the period from DD/MM/YYYY to DD/MM/YYYY

| Emission Point | Substance / Parameter | Emission # Limit Value | Reference Period | Result [1] | Test Method [2] | Sample Date and Times [3] | Uncertainty [4] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | Formaldehyde | 5 mg/m3 | Average value of three consecutive measurements of at least 30 minutes each |  | CEN / TS 13649:2014 until 30th June 2023 and thereafter CEN / TS 17638:2021 |  |  |
|  | Total volatile organic compounds | 30 mg/m3 | Average value of three consecutive measurements of at least 30 minutes each |  | EN 12619 |  |  |
| A5 | Formaldehyde | 5 mg/m3 | Average value of three consecutive measurements of at least 30 minutes each |  | CEN / TS 13649:2014 until 30th June 2023 and thereafter CEN / TS 17638:2021 |  |  |
|  | Total Volatile Organic Compounds | 30 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each |  | EN 12619 |  |  |
|  | Particulates | 20 mg/m3 | Average value of three consecutive measurements of at least 30 minutes each |  | BS EN 13284-1 |  |  |
| A6 | Formaldehyde | 5 mg/m3 | Average value of three consecutive measurements of at least 30 minutes each |  | CEN / TS 13649:2014 until 30th June 2023 and thereafter CEN / TS 17638:2021 |  |  |
| A6 | Total Volatile Organic Compounds | 30 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each |  | EN 12619 |  |  |
|  | Particulates | 20 mg/m3 | Average value of three consecutive measurements of at least 30 minutes each |  | BS EN 13284-1 |  |  |
| A16 | Oxides of nitrogen (NO and NO2 expressed as NO2) | 200 mgN/m3 | Periodic (average value of three consecutive measurements of at least 30 minutes each) |  | BS EN 14792 |  |  |
|  | Carbon Monoxide | No Limit set |  |  | BS EN 15058 |  |  |
| A17 | Oxides of nitrogen (NO and NO2 expressed as NO2) | 200 mgN/m3 | Periodic (average value of three consecutive measurements of at least 30 minutes each) |  | BS EN 14792 |  |  |
|  | Carbon Monoxide | No Limit set |  |  | BS EN 15058 |  |  |
| A18 | Oxides of nitrogen (NO and NO2 expressed as NO2) | 200 mgN/m3 | Periodic (average value of three consecutive measurements of at least 30 minutes each) |  | BS EN 14792 |  |  |
|  | Carbon Monoxide | No Limit set |  |  | BS EN 15058 |  |  |
| A19 | Oxides of nitrogen (NO and NO2 expressed as NO2) | 390 mgN/m3 | Periodic (average value of three consecutive measurements of at least 30 minutes each) |  | BS EN 14792 |  |  |
|  | Carbon Monoxide | No Limit set |  |  | BS EN 15058 |  |  |
| A20 | Oxides of nitrogen (NO and NO2 expressed as NO2) | 390 mgN/m3 | Periodic (average value of three consecutive measurements of at least 30 minutes each) |  | BS EN 14792 |  |  |
|  | Carbon Monoxide | No Limit set |  |  | BS EN 15058 |  |  |
| A21 | Oxides of nitrogen (NO and NO2 expressed as NO2) | 280 mgN/m3 | Periodic (average value of three consecutive measurements of at least 30 minutes each) |  | BS EN 14792 |  |  |
|  | Carbon Monoxide | No Limit set |  |  | BS EN 15058 |  |  |
| A22 | Oxides of nitrogen (NO and NO2 expressed as NO2) | 280 mgN/m3 | Periodic (average value of three consecutive measurements of at least 30 minutes each) |  | BS EN 14792 |  |  |
|  | Carbon Monoxide | No Limit set |  |  | BS EN 15058 |  |  |
| A23 | Oxides of nitrogen (NO and NO2 expressed as NO2) | 280 mgN/m3 | Periodic (average value of three consecutive measurements of at least 30 minutes each) |  | BS EN 14792 |  |  |
|  | Carbon Monoxide | No Limit set |  |  | BS EN 15058 |  |  |
| A24 | Oxides of nitrogen (NO and NO2 expressed as NO2) | 280 mgN/m3 | Periodic (average value of three consecutive measurements of at least 30 minutes each) |  | BS EN 14792 |  |  |
|  | Carbon Monoxide | No Limit set |  |  | BS EN 15058 |  |  |
| A25 | Oxides of nitrogen (NO and NO2 expressed as NO2) | 280 mgN/m3 | Periodic (average value of three consecutive measurements of at least 30 minutes each) |  | BS EN 14792 |  |  |
|  | Carbon Monoxide | No Limit set |  |  | BS EN 15058 |  |  |
| A26 | Oxides of nitrogen (NO and NO2 expressed as NO2) | 250 mg/m3 | Periodic (average value of three consecutive measurements of at least 30 minutes each) |  | BS EN 14792 |  |  |
|  | Carbon Monoxide | 150 mg/m3 | Periodic (average value of three consecutive measurements of at least 30 minutes each) |  | BS EN 15058 |  |  |
|  | Particulate Matter | 50 mg/m3 | Periodic (average value of three consecutive measurements of at least 30 minutes each) |  | BS EN 13284-1 |  |  |
|  | Sulphur Dioxide | 200 mg/m3 | Periodic (average value of three consecutive measurements of at least 30 minutes each) |  | BS EN 14791 |  |  |
| A27 | Hydrogen Fluoride | 3 mg/m3 | Periodic over minimum 1-hour period |  | CEN TS 17340:2020 |  |  |
|  | Cadmium & thallium and their compounds (total) | 0.05 mg/m3 | Periodic over minimum 30-minute, maximum 8 hour period |  | BS EN 14385 |  |  |
|  | Mercury and its compounds | 0.5 mg/m3 | Periodic over minimum 30-minute, maximum 8 hour period |  | BS EN 13211 |  |  |
|  | Antimony, Arsenic, Lead, Chromium, Cobalt, Copper, Manganese, Nickel & Vanadium and their compounds (total) | 0.5 mg/m3 | Periodic over minimum 30-minute, maximum 8 hour period |  | BS EN 14385 |  |  |
|  | Dioxins and Furans (I-TEQ) | 0.1 ng/m3 | Periodic over minimum 6 hours, maximum 8 hour period |  | BS EN 1948 Parts 1, 2 and 3 |  |  |
| A28 | Particulate Matter | 15 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each |  | BS EN 13284-1 |  |  |
|  | Total Volatile Organic Compounds | 100 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each |  | BS EN 12619 |  |  |
|  | Formaldehyde | 15 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each |  | CEN / TS 13649:2014 until 30th June 2023 and thereafter CEN / TS 17638:2021 |  |  |
| A29 | Particulate Matter | 20 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each |  | BS EN 13284-1 |  |  |
|  | Total Volatile Organic Compounds | 120 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each |  | BS EN 12619 |  |  |
|  | Formaldehyde | 15 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each |  | CEN / TS 13649:2014 until 30th June 2023 and thereafter CEN / TS 17638:2021 |  |  |
|  | Oxides of nitrogen (NO & NO2 expressed as NO2) | 100 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each |  | BS EN 14792 |  |  |
|  | Carbon Monoxide | 200 mg/m3 | Average value of three consecutive measurements of at least 30 minutes each |  | BS EN 15058 |  |  |
|  | Hydrogen chloride | 30 mg/m3 | Average value of three consecutive measurements of at least 30 minutes each |  | BS EN 1911 |  |  |
|  | Hydrogen fluoride | 1 mg/m3 | Average value of three consecutive measurements of at least 30 minutes each |  | CEN TS 17340:2020 |  |  |
|  | Mercury | No limit Set | Average value of three consecutive measurements of at least 30 minutes each |  | EN 13211 |  |  |
|  | Metals (Antimony, Arsenic, Cadmium, Cobalt, Chromium, Copper, Manganese, Nickel, Lead, Thallium & Vanadium) | No limit Set | Average value of three consecutive measurements of at least 30 minutes each |  | EN 14385 |  |  |
|  | Polychlorinated dibenzo-dioxins and -furans | No limit Set | Average value of three consecutive measurements of at least 30 minutes each |  | EN 1948 parts 1, 2 and 3 |  |  |
|  | Ammonia | No limit Set | Average value of three consecutive measurements of at least 30 minutes each |  | EN ISO 21877 |  |  |
| A30 | Particulate Matter | 20 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each |  | BS EN 13284-1 |  |  |
|  | Total Volatile Organic Compounds | 120 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each |  | BS EN 12619 |  |  |
|  | Formaldehyde | 15 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each |  | CEN / TS 13649:2014 until 30th June 2023 and thereafter CEN / TS 17638:2021 |  |  |
|  | Carbon Monoxide | 200 mg/m3 | Average value of three consecutive measurements of at least 30 minutes each |  | BS EN 15058 |  |  |
|  | Oxides of nitrogen (NO & NO2 expressed as NO2) | 100 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each |  | BS EN 14792 |  |  |
|  | Hydrogen Chloride | No limit set | Average value of three consecutive measurements of at least 30 minutes each |  | BS EN 1911 |  |  |
|  | Hydrogen Fluoride | No limit set | Average value of three consecutive measurements of at least 30 minutes each |  | CEN TS 17340:2020 |  |  |
|  | Mercury | No limit set | Average value of three consecutive measurements of at least 30 minutes each |  | EN 13211 |  |  |
|  | Metals (Antimony, Arsenic, Cadmium, Cobalt, Chromium, Copper, Manganese, Nickel, Lead, Thallium & Vanadium) | No limit set | Average value of three consecutive measurements of at least 30 minutes each |  | EN14385 |  |  |
|  | Polychlorinated dibenzo-dioxins and -furans | No limit set | Average value of three consecutive measurements of at least 30 minutes each |  | EN 1948 parts 1, 2 and 3 |  |  |
|  | Ammonia | No limit set | Average value of three consecutive measurements of at least 30 minutes each |  | EN ISO 21877 |  |  |
| A31 | Particulate Matter | 15 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each |  | BS EN 13284-1 |  |  |
|  | Total Volatile Organic Compounds | 100 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each |  | BS EN 12619 |  |  |
|  | Formaldehyde | 5 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each |  | CEN / TS 13649:2014 until 30th June 2023 and thereafter CEN / TS 17638:2021 |  |  |
| A32 A32 | Particulate Matter | 20 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each |  | BS EN 13284-1 |  |  |
|  | Total Volatile Organic Compounds | 200 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each |  | BS EN 12619 |  |  |
|  | Formaldehyde | 10 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each |  | CEN / TS 13649:2014 until 30th June 2023 and thereafter CEN / TS 17638:2021 |  |  |
|  | Oxides of nitrogen (NO & NO2 expressed as NO2) | 100 mg/Nm3 | Average value of three consecutive measurements of at least 30 minutes each |  | BS EN 14792 |  |  |
|  | Carbon Monoxide | No limit set | Average value of three consecutive measurements of at least 30 minutes each |  | BS EN 15058 |  |  |
|  | Hydrogen Chloride | No limit set | Average value of three consecutive measurements of at least 30 minutes each |  | BS EN 1911 |  |  |
|  | Hydrogen Fluoride | No limit set | Average value of three consecutive measurements of at least 30 minutes each |  | CEN TS 17340:2020 |  |  |
|  | Mercury | No limit set | Average value of three consecutive measurements of at least 30 minutes each |  | EN13211 |  |  |
|  | Metals (Antimony, Arsenic, Cadmium, Cobalt, Chromium, Copper, Manganese, Nickel, Lead, Thallium & Vanadium) | No limit set | Average value of three consecutive measurements of at least 30 minutes each |  | EN 14385 |  |  |
|  | Polychlorinated dibenzo-dioxins and -furans | No limit set | Average value of three consecutive measurements of at least 30 minutes each |  | EN 1948 parts 1, 2 and 3 |  |  |
| B01 - B31 | Particulate Matter | 5 mg/Nm3 to be determined following outcome of IC49 | Average value of three consecutive measurements of at least 30 minutes each |  | BS EN 13284-1 |  |  |

- The result given is the maximum value (or the minimum value in the case of a limit that is expressed as a minimum) obtained during the reporting period, expressed in the same terms as the emission limit value. Where the emission limit value is expressed as a range, the result is given as the ‘minimum – maximum’ measured values.

- Where an internationally recognised standard test method is used the reference number is given. Where another method that has been formally agreed with Natural Resources Wales is used, then the appropriate identifier is given. In other cases the principal technique is stated, for example gas chromatography.

- For non-continuous measurements the date and time of the sample that produced the result is given. For continuous measurements the percentage of the process operating time covered by the result is given.

- The uncertainty associated with the quoted result at the 95% confidence interval, unless otherwise stated.

Signed …………………………………………………………….  		Date………………………..

(Authorised to sign as representative of Operator)