| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-021626 |
| Permit Reference Number: | EPR-KP3594VE |
| Permit Holder: | Trade Effluent Services Limited |
| Site Address: | Gwaenynog Bach Farm, Pentrefoelas Road, Bryn Ynys, Denbigh, Y Groes, Denbighshire, LL16 5RP mapping system shows LL16 5RW |
| Grid Reference: | Farm location: SJ 02451 65690 Fields: 5905 SJ0258466056 4279 SJ0244665796 7605 SJ0273966074 3851 SJ0240065556- soil analysis has this as 3581 6418 SJ0263166192 1843 SJ0208565411 5974 SJ0259065744 5490 SJ0254665910 2565 SJ0225765644 7887 SJ0278165868 2946 SJ0229465501 2881 SJ0228065834 3398 SJ0230165977 5694 SJ0157364976 2991 SJ0124064962 3799 SJ0136964983 5813 SJ0163265153 1390 SJ0213965902 9596 SJ0195465966- Map shows this as 9256- LPD1 and ag ben as 9596. 0716 SJ0206366161 2211 SJ0221866110 Storage: Nurse tank in 3851 SJ0236465598- doesn’t match map Nurse tank in 5813 SJ0159865192 Lagoon SJ0245165690 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: WasteReg.NE@cyfoethnaturiolcymru.gov.uk IndustryRegulation&Waste.NW@cyfoethnaturiolcymru.gov.uk Environment team: DenbighshireEnvironmentTeam@cyfoethnaturiolcymru.gov.uk ConwyEnvironmentTeam@cyfoethnaturiolcymru.gov.uk NLO:Amy.Henderson@cyfoethnaturiolcymru.gov.uk |
| Date due: | 20/04/2023 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating B- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | Welsh permit No variations |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and deployment fee- Charging Scheme |
| Fee on MyNRW | Y | Transaction ref: 2113225 Date: 27/03/2023 Reference: BS/270323/22 TRADE EFFLUENT LTD EPDEPTRADE0058 Amount:798.00 |
| Location map | Y | in “agben” |
| Benefit statement | Y | “agben” |
| Waste Analysis | Y | “Waste analysis” |
| Soil Analysis | Y | “Soils” |
| EIA Agri Regulations evidence | Y | in “agben” |
| Site Specific Risk Assessment | n/a | Medium risk so SSRA is not required |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y | “KP3594VE” |
| ## Has the permit holder name been provided? | Y | “Trade Effluent Services” Permit is for Trade Effluent Services Limited |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y |  |
| ## Has the technically competent manager details been provided? | Y | Given as Richard Street, however Mr Street no longer works at TES as of 06/05/23- Need updated TCM info |
| ## Has the nominated competent person details been provided? | Y | Given as Richard Street, however Mr Street no longer works at TES as of 06/05/23- Need updated NCP info |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | N | Post code is incorrect. Given as LL16 5RP; mapping system shows LL16 5RW |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | On LPD1 form- 56/40/117 On MyMap: all fields: 56/040/0117 E W HUGHES & PARTNERS GWAENYNOG BACH |
| ## Has all of parcels of land table been completed? | Y | On separate document “Field Description”. Table includes all required information Queries re field number discrepancies |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y | 48.6 ha- not on form but confirmed in “agben” document |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y | Single landowner/occupier: Mr Euros Hughes. This is confirmed by CPH data |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y | “The file we have for written consent is too large to be attached in this section. I have included it in the end in additional documents and also attached it to the agben document.” Have presumed const document is the document called “Farmer Oder”? Will need to confirm this. Document “Farmer Oder” is not dated and does not include storage locations- see full assessment at the end of this document. |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | Y | All fields except 1390, 9596, 0716, 2211, and 2565 were applied for under PAN-019963 but the application was returned 06/02/2023. Document “Previous Wastes” states all fields had 10m3 of FYM and no waste. No information on slurry or fertilisers. |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | Medium (1) |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) of High risk deployment has a SSRA been provided? | n/a |  |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | n/a |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y | Page 17 of “agben” shows land in agricultural use |
| - About the waste |  |  |
| Have all parts of the waste types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable or non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y | Physical form of waste missing- liquid, sludge or solid. Benefit statement refers to wastes as sludges. |
| Are the wastes listed in the deployment specifically listed in the permit? | Y | Codes yes, see below for further information |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | N | Descriptions: Need more information on the Penderyn Distillery and Kelloggs wastes- not enough information. Beechdean Ice cream, Penderyn Distillery and Kelloggs has no EPR numbers. Nor any information on where the waste has come from i.e. an address. See information from consultation response |
| - About the storage |  |  |
| Has all of waste storage table been completed? | Y | 2 nurse tanks and 1 lagoon “The lagoon will be free from any other material prior to use and only waste from the agreed deployment will be stored until the lagoon is empty” Benefit Statement Additional queries re waste storage- see consultation response section |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y |  |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | n/a |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y | Area 1- none present Area 2- all water course buffered |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | n/a | None present |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | Y | Area 1- none present Area 2- all wells/spring (or issue)/borehole are correctly buffered |
| location of all storage sites for the waste to be spread | Y |  |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | Y | Area 1- none present Area 2- all wells/spring (or issue)/borehole are correctly shown on map |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | Y | Area 1- none present Area 2- all wells/spring (or issue)/borehole are correctly shown on map |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | n/a | None present |
| location of any public rights of way (shown on map) | Y | Area 1- along western side of field 2991- no action needed. Area 2- PRoW though centre of field block- shown on map |
| any buildings or houses within 250m (shown on map) | Y |  |
| Within Groundwater Safeguard Zone | n/a | None present |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species- none present - Protected Habitats- none present - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Hedges - Ditches | n/a | None present |
| What is the slope of the fields? | 0-7 degrees (transparent)- all fields 7 -12 degrees (yellow)- fields 9256, 5905, 7605 and 7887 all buffered/no spread zones 12 degrees plus (red)- no fields |  |

| Waste Storage | Y N n/a | Comments |
| --- | --- | --- |
| Is waste storage occurring at the place where waste is being deployed? | Y |  |
| Is only waste as specified in this deployment being stored in the proposed storage locations? | Y |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in the same place one constructed within the last two years? or in different location? | n/a | Not stored in temporary field heaps |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in a field: - liable to flooding or becoming waterlogged, or - within 50 metres of a borehole, spring or well or - within 10 metres of surface water or a land drain - within 30 meters of a watercourse on steeply sloping land | n/a | Not stored in temporary field heaps |
| If the waste is non-stackable is the waste stored within either a vessel, a covered building, or on an impermeable surface | Y | Nurse tanks and lagoon |
| If the waste is high RAN and it is being stored within 200 metres of a European Site (SAC/SPA), Ramsar Site or a Site of Special Scientific Interest (SSSI) is the waste being covered to prevent loss of ammonia? (condition 3.2.4 of the permit) | n/a | Not high RAN wastes |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | N | ENCIRC- dated 17 March 2022- i.e. sample was taken more than 12 months before application was made |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y | 02 05 01- Fats/oils/grease value |
| - If yes; have they been tested for? | N | Beachdeen Ice cream not tested for FOG |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a | Not in a groundwater safeguard zone |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y | All dated October 2022 |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y | Richard Street- while they are no longer with the company we can accept a benefit statement written by them. |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | N | Not fully demonstrated- see queries re claimed benefits, time of application, nutrient additions, waste analysis, odour control, sources of waste |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y | Claimed benefit: Nitrogen. Phosphorus, Potash Wastes supply these- pending Encirc waste analysis |
| - Timing of application | N | Grass only: Time of the application of wastes has been given as May to October 2023 and from Feb 2024 to the end of deployment, during the growing season, to give crops the nutrients prior to grazing or harvest or after cuts silage or grazing. These time periods are too vague and must be more specific. |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | N | Claimed benefit: pH- this is not a benefit and should be in the negative effects section. 3 of the wastes have pH below 4.02 |
| - Soil physical properties (e.g. organic matter and soil structure) | Y | Claimed benefit: Organic matter- wastes include small amount |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| Timing of application- if any of the wastes are high RAN, are they being spread OUTSIDE of the closed periods | n/a | Wastes are not high RAN |
| Groundwater Safeguard Zone for nitrate- if any of the wastes are high RAN, are the application rates less than 50 m3/ha, with a minimum 3 week gap between applications | n/a | Spreading not occurring on/in Groundwater Safeguard Zone |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | N | pH- 3 of the wastes have pH lower than 4.02. information does not adequately address the risks- only talks of soil type not pH levels of the fields odour- no information provided when waste is delivered to/transferred between the tanks/lagoon |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need, as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | N | Croda Goole sodium is too high |
| - Crop nutrient requirements | Y |  |
| If the cropping is any of the following, is the maximum nitrogen limit in line with that given in The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021: - 0 kg/ha- Field beans, Peas - 120 kg/ha- Sugar beet - 150 kg/ha- Asparagus, Carrots, Forage maize. Radish, Spring barley, Swedes - 180 kg/ha- Spring-sown wheat, Winter barley - 220 kg/ha- Autumn or early winter sown wheat | n/a | Cropping is grass |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | N | Cannot confirm due to out of date analysis |
| - Is it only for operational purposes? | Y |  |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | Y |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y | Shallow injector |
| - Is the timing of application appropriate? | N | Grass only: Time of the application of wastes has been given as May to October 2023 and from Feb 2024 to the end of deployment, during the growing season, to give crops the nutrients prior to grazing or harvest or after cuts silage or grazing. These time periods are too vague and must be more specific. |
| - Have they stated when the waste will be spread? | Y | See above |
| - Have they stated how often the waste will be spread? | N |  |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients given by the applicant match NRW calculations? | N | Croda Widnes doesn’t match our calculations. ENCIRC analysis is out of date, therefore this cannot be verified |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y | FYM. Query re slurry and fertilisers |
| - How much material was spread? |  |  |
| - At what application rate material was spread? |  | 10 m3/ha |
| - The nutrients supplied by the material? |  | Standard figures from RB209 |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | n/a |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | n/a |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | n/a |  |
| - Pathways | n/a |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | n/a |  |
| - Measures that will minimise or mitigate the impact will be put in place | n/a |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | N |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | n/a |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | N |

Comments

Consultation responses: Information provided that requires addressing: - only FYM has ben included in previous spreading information. No information provided on any fertilisers used - waste from “Kelloggs” has been from multiple sources. No information has been included in this application to show where the waste is sourced from - farm has a slurry lagoon which the application states is empty of all other material- where was slurry spread? - planning permission for the lagoon states it will be used for slurry storage for the cattle farm- is this no longer the case? - location of [some] waste producers not provided - spreading times cover a wide time period- needs to be more specific - application mentions a management system under odour but this has not been included - odour information only addresses storage not spreading. No buffer zones are in place around residential properties - site hazards section in section 8 of the benefit statement refers to fields not within the deployment- is this section site specific?

MyMap, location and slope assessment MyMap/location assessment: - land in deployment is clear of any designations - any water courses, springs/issues or wells have been appropriately buffered. - see below for slope assessment Slope assessment: - fields in deployment are mostly 0-7 degree in slope. - No fields have a slope of over 12 degrees. - Several fields have small areas of 7-12 degrees slope; however these are all excluded from the spreading areas. - No further assessment is required.

CPH details: - On LPD1 form- 56/40/117 - On MyMap: all fields: 56/040/0117 E W HUGHES & PARTNERS GWAENYNOG BACH

Protection of habitats and ecosystems - aerial images provided as evidence

Agricultural benefit calculations: Secanim - Nutrient offtake is higher that waste additions on all fields Croda Goole - Nutrient offtake is higher that waste additions on all fields - Sodium is too high based on max sodium levels (372.11 kg/ha) at application rate of 132 t/ha Croda Widnes - Nutrient offtake is higher that waste additions on all fields Maelor Foods - Nutrient offtake is higher that waste additions on all fields Encirc - waste analysis is out of date- assessment is incomplete Beechdean Ice cream - pH of 3.93 - no FOG value - Nutrient offtake is higher that waste additions on all fields PENDERYN DISTILLERY - pH of 3.74 - Nutrient offtake is higher that waste additions on all fields Kelloggs - pH of 4.02 - Nutrient offtake is higher that waste additions on all fields

Other information to note: Waste specific information from TGN 8.01 02 02 04- sludges from on-site effluent treatment plant from abattoirs, poultry preparation plants, rendering plants or fish preparation plants only - odour - waste must be injected 02 03 05- sludges from on-site effluent treatment - none 02 05 01- biodegradable materials unsuitable for consumption or processing - Fats/oils/grease value - spread over several applications - if growing crop- waste should be spread on the soil surface not onto the crop - high application rates may coat soil particles- creating waterproof barrier 02 07 05- sludges from on-site effluent treatment non-alcoholic drinks only - acidic- should not be applied to spoils with pH less than 5.5 - high BOD - difficult to justify untreated waste - odours 07 07 12- sludges from on-site biological effluent treatment plant at chemical manufacturing sites other than those mentioned in 07 01 11 only - none Landowner/occupier consent We will require a document signed and dated by the landowner/occupier that includes: - the farm name(s) - field reference(s) - waste(s) to be spread on each field - waste storage locations “Farmer Oder” document is signed, has farm name(s), field reference(s), wastes to be spread on each field, but is not dated and does not include storage locations.

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | No |
| ## Issue date | n/a |
| ## Effective date | n/a |
| ## Permitting Officer Date | Louise Bailey 30/05/23 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 30/05/2023 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 755 |
| Sign off | 10 |
| Issuing | 10 |
| Total | 835 |