| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-021636 |
| Permit Reference Number: | EPR-KP3594VE |
| Permit Holder: | Trade Effluent Services Limited |
| Site Address: | Kilford farm, Whitchurch Road, Denbighshire, LL16 4ER |
| Grid Reference: | Farm location: SJ 07660 66146 Fields: DE1 SJ0790267122 DE2 SJ0794867333 DE3 SJ0831767566 DE4 SJ0821667342 DE5 SJ0812267088 DE6 SJ0830967030 DE9 SJ0818366720 DE9a SJ0806166488 DE10 SJ0802366264 Storage: Nurse tank in DE10 SJ0797766136 Nurse tank in DE2 SJ0805467135 Nurse tank in DE4 SJ0809767295 Nurse tank in DE5 SJ0802866934 Lagoon SJ0779566483 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: WasteReg.NE@cyfoethnaturiolcymru.gov.uk Environment team: DenbighshireEnvironmentTeam@cyfoethnaturiolcymru.gov.uk NLO: Amy.Henderson@cyfoethnaturiolcymru.gov.uk |
| Date due: | 20/04/2023 |
| Are fields on Anglesey? If yes, then send application. | No: N/A |
| Permit compliance rating and multiplier | Rating B- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided? (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | DMS/PaLS |
| Correct application form | Y |  |
| Correct fee | Y | See permit compliance multiplier above and deployment fee- Charging Scheme |
| Fee on MyNRW | Y | Transaction ref: 2113226 Date: 27/03/2023 Reference: BS/270323/23 TRADE EFFLUENT EPDEPTRADE0060 Amount: 798.00 |
| Location map | Y | In “agben” |
| Benefit statement | Y | Agben odour management plan Storage Risk assessment Farmer Order |
| Waste Analysis | Y | Waste analysis |
| Soil Analysis | Y | F990_38356_PDF (1) |
| EIA Agri Regulations evidence | Y | In “agben” |
| Site Specific Risk Assessment | n/a | Not a high risk deployment |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y | “KP3594VE” |
| ## Has the permit holder name been provided? | Y | “Trade Effluent Services” Permit is for Trade Effluent Services Limited |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y | Given as Richard Street, however Mr Street no longer works at TES as of 06/05/23- Need updated contact info |
| ## Has the technically competent manager details been provided? | Y | Given as Richard Street, however Mr Street no longer works at TES as of 06/05/23- Need updated TCM info |
| ## Has the nominated competent person details been provided? | Y | Given as Richard Street, however Mr Street no longer works at TES as of 06/05/23- Need updated NCP info |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | Y |  |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | CPH number given in LPD1 form: 56/44/260 All fields: 56/044/0260 registered to RD & RE EVANS & SON of KILFORD List the CPH numbers for each field, who registered them and the farm name in the comments section |
| ## Has all of parcels of land table been completed? | Y |  |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y | 48.79 ha |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y | Single landowner/occupier: Mr Dilwyn Evans of Kilford Farm. This is confirmed by CPH data |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y | Word and PDF documents “Farmer Order”. “Farmer Order” is not dated and does not include storage locations- see full assessment at the end of this document. |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | Y | FYS- farmyard slurry. PAN-019962 was returned. No mention of fertilisers |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | Medium (1) |  |
| Is the risk banding correct? | Y |  |
| ## If Medium risk (2) of High risk deployment has a SSRA been provided? | n/a |  |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | benefit statement | Odour management plan and Storage Risk assessment also included |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | n/a |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y | Page 13 of “agben” shows land in agricultural use |
| - About the waste |  |  |
| Have all parts of the waste types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable or non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y | Physical form of waste missing- liquid, sludge or solid. Benefit statement refers to wastes as sludges |
| Are the wastes listed in the deployment specifically listed in the permit? | Y | Codes yes, see below for further information |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | N | Descriptions: Need more information on the PENDERYN DISTILLERY and kelloggs wastes- not enough information. Beechdean Ice cream, PENDERYN DISTILLERY and Kelloggs has no EPR numbers. Nor any information on where the waste has come from i.e. an address. Also see information from consultation response |
| - About the storage |  |  |
| Has all of waste storage table been completed? | Y | 4 nurse tanks and 1 lagoon Additional queries re waste storage- see consultation response section |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y | “A lagoon is available for use located at SJ 07795 66483, the lagoon is used for farm slurry during the year. If any waste under this deployment is stored in the lagoon, then the lagoon will be free from any other material prior to use and only waste from the agreed deployment will be stored until the lagoon is empty.” Query re where slurry is to be stored when it is in use by the deployment and where slurry is to be spread prior to use |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | n/a |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | Y | Watercourse in DE2 not buffered All other required buffer zones are in place |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | n/a | None found |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | N | None on mapping system within 50 meters of spreading/storage Kilford farm, The Croft and Tan-y-dderwen do not appear to be on mains drinking water- borehole or spring may be in place that would need to be buffered. |
| location of all storage sites for the waste to be spread | Y |  |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | N | None on mapping system within 50 meters of spreading/storage Kilford farm, The Croft and Tan-y-dderwen do not appear to be on mains drinking water- borehole or spring may be in place that would need to be shown on map. |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | N | None on mapping system within 50 meters of spreading/storage Kilford farm, The Croft and Tan-y-dderwen do not appear to be on mains drinking water- borehole or spring may be in place that would need to be shown on map. |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | n/a | None within 500 meters |
| location of any public rights of way (shown on map) | N | PRoW on road between DE1/2 and DE4/5. It does not run through the fields, but it is not shown on the map- needs to be identified |
| any buildings or houses within 250m (shown on map) | Y |  |
| Within Groundwater Safeguard Zone | n/a | None found |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Hedges - Ditches | n/a | None found other than the expected hedges and ditches |
| What is the slope of the fields? | 0-7 degrees (transparent)- all field. No areas of 7 -12 degrees (yellow) or 12 degrees plus (red) found |  |

| Waste Storage | Y N n/a | Comments |
| --- | --- | --- |
| Is waste storage occurring at the place where waste is being deployed? | Y |  |
| Is only waste as specified in this deployment being stored in the proposed storage locations? | Y |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in the same place one constructed within the last two years? or in different location? | n/a | No temporary field heaps |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in a field: - liable to flooding or becoming waterlogged, or - within 50 metres of a borehole, spring or well or - within 10 metres of surface water or a land drain - within 30 meters of a watercourse on steeply sloping land | n/a | No temporary field heaps |
| If the waste is non-stackable is the waste stored within either a vessel, a covered building, or on an impermeable surface | Y | Nurse tanks and lagoon |
| If the waste is high RAN and it is being stored within 200 metres of a European Site (SAC/SPA), Ramsar Site or a Site of Special Scientific Interest (SSSI) is the waste being covered to prevent loss of ammonia? (condition 3.2.4 of the permit) | n/a | Not high RAN |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the waste analysis report less than 12 months old at time of application? | N | ENCIRC- dated 17 March 2022- i.e. sample was taken more than 12 months before application was made |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y | 02 05 01- Fats/oils/grease value |
| - If yes; have they been tested for? | N | Beachdeen Ice cream not tested for FOG |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a |  |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a | Not in a groundwater safeguard zone |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a |  |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y | All dated October 2022 |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | Y |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | Y | Richard Street- while they are no longer with the company we can accept a benefit statement written by them. |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | N | Not fully demonstrated- see queries re claimed benefits, time of application, nutrient additions, waste analysis, odour control, sources of waste |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y | Claimed benefit: Nitrogen. Phosphorus, Potash Wastes supply these- pending Encirc waste analysis |
| - Timing of application | N | Grass fields April to October 2023 before the growing season, after cuts silage or before grazing arable fields before seedbed preparation maize fields before seedbed preparation from February to April 2024 winter wheat fields from August 2023 to October 2023 depending on when the crop has been sown These time periods are too vague and must be more specific. |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | N | Claimed benefit: pH- this is not a benefit and should be in the negative effects section. 3 of the wastes have pH below 4.02 |
| - Soil physical properties (e.g. organic matter and soil structure) | Y | Claimed benefit: Organic matter- wastes include small amount |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| Timing of application- if any of the wastes are high RAN, are they being spread OUTSIDE of the closed periods | n/a | None of the wastes are High RAN |
| Groundwater Safeguard Zone for nitrate- if any of the wastes are high RAN, are the application rates less than 50 m3/ha, with a minimum 3 week gap between applications | n/a | Spreading not occurring on/in Groundwater Safeguard Zone |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | N | pH- 3 of the wastes have pH lower than 4.02. information does not adequately address the risks- only talks of soil type not pH levels of the fields |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need, as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | N | Croda Goole sodium is too high |
| - Crop nutrient requirements | Y |  |
| If the cropping is any of the following, is the maximum nitrogen limit in line with that given in The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021: - 0 kg/ha- Field beans, Peas - 120 kg/ha- Sugar beet - 150 kg/ha- Asparagus, Carrots, Forage maize. Radish, Spring barley, Swedes - 180 kg/ha- Spring-sown wheat, Winter barley - 220 kg/ha- Autumn or early winter sown wheat | N | Forage maize on fields DE4 DE5- too high on Croda Widnes Winter wheat on fields DE1, DE2, DE3, DE6 all under 220 Other fields (DE9, 9a and 10) are grassland- |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | N | Cannot confirm due to out of date analysis |
| - Is it only for operational purposes? | Y |  |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | Y |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y | sub-soil injection/deep-leg injector |
| - Is the timing of application appropriate? | N | Grass fields April to October 2023 before the growing season, after cuts silage or before grazing arable fields before seedbed preparation maize fields before seedbed preparation from February to April 2024 winter wheat fields from August 2023 to October 2023 depending on when the crop has been sown These time periods are too vague and must be more specific. |
| - Have they stated when the waste will be spread? | Y | See above |
| - Have they stated how often the waste will be spread? | N |  |
| Have the total nutrients supplied by the waste been given? | Y | Table 5 is incorrect. Croda Goole and Croda Widnes pH are mixed up. |
| Does the total nutrients given by the applicant match NRW calculations? | N | Croda Widnes additions do not match. they appear to have been calculated using the Crode Goole analysis ENCIRC analysis is out of date, therefore this cannot be verified |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y | FYS Query re fertilisers |
| - How much material was spread? | Y | Can be calculated from held date |
| - At what application rate material was spread? | Y | 10 m3/ha |
| - The nutrients supplied by the material? | Y | Standard figures from RB209 |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | n/a |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | Y | Storage Risk assessment odour management plan |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | n/a |  |
| - Pathways | Y |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | Y | See query on storage risk assessment |
| - Measures that will minimise or mitigate the impact will be put in place | Y |  |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | N |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only | n/a |  |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | N |

Comments

Consultation responses: Information provided that requires addressing: - concerns re consent document - Water course in DE2 not buffered - Kilford farm, The Croft and Tan-y-dderwen do not appear to be on mains drinking water- borehole or spring may need to be shown on map - Spreading times cover a large time period - Site location for producers is not specified - storage location/odour risk assessment incorrect re distance to residential properties

MyMap, location and slope assessment MyMap/location assessment: - land in deployment is clear of any designations - any water courses, springs/issues or wells have been appropriately buffered apart from DE2. Slope assessment - all fields between 0 and 7 degrees in slope- assessment not required

CPH details: CPH number given in LPD1 form: 56/44/260 All fields: 56/044/0260 registered to RD & RE EVANS & SON of KILFORD List the CPH numbers for each field, who registered them and the farm name in the comments section

Protection of habitats and ecosystems - aerial images provided as evidence

Agricultural benefit calculations: Secanim - increase in phosphate across all fields- 27kg/ha on WW, 9 kg/ha on maize and 18 kg/ha on grass. - All fields are under target index for P - Potash offtake is higher that waste additions on all fields Croda Goole - increase in phosphate across WW and maize fields- 7kg/ha on WW, 10 kg/ha on maize. - All fields are under target index for P - P offtake is higher than additions for grass fields - Potash offtake is higher that waste additions on all fields - Sodium is too high based on max sodium levels- 372.11 kg/ha at application rate of 132 t/ha and 391.84 kg/ha at application rate of 139 t/ha Croda Widnes - increase in phosphate across WW and grass fields- 10 kg/ha on WW, 1.55 kg/ha on grass. - All fields are under target index for P - P offtake is higher than additions for maize fields - Potash offtake is higher that waste additions on all fields - Nitrogen limit for forage maize on fields DE4 & DE5 are too high Maelor Foods - increase in phosphate across WW and grass fields- 25 kg/ha on WW, 17 kg/ha on grass. - All fields are under target index for P - P offtake is higher than additions for maize fields - Potash offtake is higher that waste additions on all fields Encirc - waste analysis is out of date- assessment is incomplete Beechdean Ice cream - pH of 3.93 - no FOG value - Nutrient offtake is higher that waste additions on all fields PENDERYN DISTILLERY - pH of 3.74 - increase in phosphate across WW and maize fields- 8.41 kg/ha on WW and maize. - All fields are under target index for P - P offtake is higher than additions for grass fields - increase in K on WW fields (1.73 kg/ha) - Potash offtake is higher that waste additions on maize and grass fields Kelloggs - pH of 4.02 - increase in phosphate across WW and grass fields- 25 kg/ha on WW, 17 kg/ha on grass. - All fields are under target index for P - P offtake is higher than additions for maize fields - Potash offtake is higher that waste additions on all fields

Other information to note: Landowner/occupier consent: We will require a document signed and dated by the landowner/occupier that includes: - the farm name(s) - field reference(s) - waste(s) to be spread on each field - waste storage locations “Farmer Order” includes farm name, field references, and wastes, but is not dated and doesn’t include storage information. Storage Risk assessment - The risk assessment states it is for the tank location in DE10, however neither of the 2 residential properties to the east and west of the tank location are 155 meters away. The tank in field DE10 is approx. 68 meters away from The Croft and 171 meters from Tan-y-dderwen.

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | No |
| ## Issue date | n/a |
| ## Effective date | n/a |
| ## Permitting Officer Date | Louise Bailey 30/05/23 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 30/05/2023 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 795 |
| Sign off | 10 |
| Issuing | 10 |
| Total | 875 |