| Details |  |
| --- | --- |
| Deployment Reference Number: | PAN-022093 |
| Permit Reference Number: | FB3606GC |
| Permit Holder: | Agrispread Ltd |
| Site Address: | On LPD1 form: Prysau Farm Ffordd Babell Pantile Brynford Babell, CH8 8PY On Agricultural Benefit Statement: Allt Y Chwiler, Pantlle, Brynford, Flintshire, Wales, CH8 8LR, United Kingdom (56/204/0136) Bryn-YCOSYN Farm, Lixwm, Holywell, CH8 8AE (56/204/0066) |
| Grid Reference: | Farm location: SJ 16692 73411 Fields: BW1 SJ1661772802 BW2 SJ1670172692 BW3 SJ1681772588 BW4 SJ1703672590 BW6a SJ1701172727 BW6b SJ1705272844 BW7 SJ1720672896 BYC1 SJ1810572753 BYC2 SJ1798972664 BYC3 SJ1794072593 BYC4 SJ1804772538 BYC5 SJ1813472497 BYC6 SJ1795372429 BYC7 SJ1788972434 Storage: Nurse tank in BW6a: SJ1716872719 Nurse tank in BYC3: SJ1791872521 Nurse tank in BYC1: SJ1812672782 |

| Administrative Checks- by permit support officer |  |
| --- | --- |
|  | Comments |
| Consultation | Waste reg team: WasteReg.NE@cyfoethnaturiolcymru.gov.uk Environment team: WrexhamFlintshireEnvironmentTeam@cyfoethnaturiolcymru.gov.uk NLO: GMMCLandandWater@environment-agency.gov.uk |
| Date due: | 01/06/2023 |
| Are fields on Anglesey? If yes, then send application. | N/A |
| Permit compliance rating and multiplier | Rating A- 100% |

| Administrative Checks- by determining permitting officer |  |  |
| --- | --- | --- |
| Item | Action completed, or document provided (Yes, No or n/a) | Comments |
| Is the base permit still live? Has there been any variations we need to be aware of? | Y | EA public register |
| Correct application form | Y |  |
| Correct fee | N | See below |
| Fee on MyNRW | Y N | Application has been made as high risk with a stated fee of £1,018 under reference EPDEPTRADE0057. No payment of that amount with that reference is on MyNRW. However the following payments were made on the same date: - Transaction number: 2113229 Date: 27/03/2023 Ref: BS/270323/31 TRADE EFFLUENT EPDEPTRADE0057 Amount: 798.00 - Transaction number: 21132160 Date: 27/03/2023 Ref: BS/270323/13 TRADE EFFLUENT EPDEPTRADE0056 Amount: 1,018.00 Payment 2 has already been attributed to PAN-020784 |
| Location map | Y | In “agben” |
| Benefit statement | Y | “agben” |
| Waste Analysis | Y | “Waste Analysis” - missing analysis for Kelloggs waste |
| Soil Analysis | Y | F990_13236_PDF F990_38355_PDF (2) |
| EIA Agri Regulations evidence | Y | In “agben” |
| Site Specific Risk Assessment | Y | Bryn Y Cosin Risk Assessment |
| Other documents | Y | field information Odour Management Plan previous wastes Scan 17 May 23 12·49·19 (consent) |

| ## Application Form Word versions of online forms - NRW View (sharepoint.com) | Y N n/a | Comments |
| --- | --- | --- |
| - About the Permit |  |  |
| ## Has there been any pre-application discussions? | N |  |
| ## Has the permit number been provided? | Y |  |
| ## Has the permit holder name been provided? | Y |  |
| ## What permit is being deployed under? | SR2010 No4 |  |
| - Contact details |  |  |
| ## Has contact information been provided? | Y | Mr Ryan Griffiths-Patel |
| ## Has the technically competent manager details been provided? | Y | Mr Rob Piggot |
| ## Has the nominated competent person details been provided? | Y | Mr Rob Piggot |
| - About the land you want to treat |  |  |
| ## Has the address and grid reference of the main farm address been given? | Y |  |
| ## Does the farm address match the grid reference? | N | NGR is for DYFFRYN CALCOED, CAELCOED LANE, BRYNFORD, HOLYWELL, CH8 8LE Main address given as Prysau Farm- Ffordd Babell Pantile Brynford Babell CH8 8PY Additionally none of the spreading appears to be occurring at Prysau Farm. Two addresses given in the agben are: - land at Allt Y Chwiler, Pantlle, Brynford, Flintshire, Wales, CH8 8LR, United Kingdom (56/204/0136) - Bryn-Y-Cosyn Farm, Lixwm, Holywell, CH8 8AE (56/204/0066) |
| ## What type of land do they want to treat? | Agricultural land |  |
| If it is agricultural land; has the holding number been given? | Y | On LPD1 form: 46/204/0136 In “agben” and field information”: - Allt Y Chwiler- 56/204/0136 - Bryn-YCOSYN Farm- 56/204/0066 MyMap: - “BW” fields: 56/204/0136- MESSRS B AND V WILLIAMS- PRYSAU FARM - “BYC” fields: 56/204/0066- MRS GLENYS ANWEN HUGHES- BRYN Y COSYN FARM CPH numbers match |
| ## Has all of parcels of land table been completed? | Y | See “field information” document- which is a copy of the first 2 “agben” pages- Table 1 Area given in “agben” under Table 2 |
| ## Is the total area of land to be treated less than 50 ha, or 100 hectares for a single continuously managed area of land? | Y | 30.5 ha |
| Is the land to be treated within a 10 miles radius of the main address? | Y |  |
| ## Is this deployment one of a batch (multiple deployments) for the same area of land? | N |  |
| ## Is the permit holder the owner or occupier of the land you want to spread on/treat? | N |  |
| ## Have details of the land owner or occupier been provided? | Y | Bryn Williams of Prysau Farm Mr Hughes of Bryn Y Cosin Farm |
| ## Has written consent of all owners(s) or occupiers(s) been given? | Y | Consent document “Scan 17 May 23 12·49·19” Consent requirements have been met Consent for “BW” fields gives the name and address as PRYSAU FARM and not land Allt Y Chwiler as in other parts of the application. |
| - Previous land treatment |  |  |
| Has all of the previous land treatment table been completed? | Y | See previous wastes document - FYM to all fields at Prysau Farm in 2022. Should this not be “land at Allt Y Chwiler”? - commercial waste to all fields at Bryn Y Cosin Farm in 2022- PAN-015919 issued 17/01/2022 |
| - Which risk band does the activity fall within? - SR2010No4 List A wastes (Lower risk) - Location risk - Low risk deployment - Medium risk (1) deployment - Medium risk (2) deployment - High risk deployment |  |  |
| ## Which risk band does the activity fall within? | High |  |
| Is the risk banding correct? | Y N | “BW” fields more than 500 meters from SAC and SSSI “BYC” fields- within 500 meters of: - Comin Helygain a Glaswelltiroedd Treffynnon / Halkyn Common and Holywell Grasslands SSSI - Parc Linden, Lixwm SSSI - Halkyn Mountain / Mynydd Helygain SAC |
| ## If Medium risk (2) or High risk deployment has a SSRA been provided? | Y | Bryn Y Cosin Risk Assessment |
| - Additional information on sensitive receptors |  |  |
| ## What type of risk assessment has been submitted? | site‐specific risk assessment and benefit statement |  |
| Does the land onto which the specified waste is to be spread as part of this deployment fall within scope of the Environmental Impact Assessment (Agriculture) (Wales) Regulations 2017 as semi-natural (habitat)? | N |  |
| If yes has the screening opinion provided by Welsh Government been provided? | N/A |  |
| If no have details of the assessment evidence to support this conclusion been provided? | Y | Satellite images of the land included in “agben” |
| - About the waste |  |  |
| Have all parts of the waste-types table been completed? - List of waste code (6 digit) - Description of waste (this must be an accurate description specific to this waste that refers to the List of Waste written description) - Physical form of waste/Is it stackable or non-stackable - Is the waste high in readily available nitrogen? - Producer of waste & permit number (if applicable) - Total amount being spread/used (tonnes) | Y |  |
| Are the wastes listed in the deployment specifically listed in the permit? | Y | Codes yes, see below for further information |
| Can you verify the waste producer (source of waste) is correct in terms of production, coding and description? | N | Description of waste: Need more information on the Penderyn Distillery and Kelloggs wastes- not enough information. Producer of waste: Beechdean Ice cream, Penderyn Distillery and Kelloggs has no EPR numbers. Nor any information on where the waste has come from i.e. an address. |
| - About the storage |  |  |
| Has all of waste storage table been completed? | Y |  |
| Is the storage given no more than 1250 tonnes shall be non-stackable waste, and no more than 3000 tonnes of all other wastes? | Y | 3X NURSE TANKS holding 120 tonnes of waste each at any one time |
| 18. Supporting documents |  |  |
| Has checklist been completed? | Y |  |
| 20. Confidentiality and national security |  |  |
| Has confidentially been claimed? | N |  |
| Has national security been claimed? | N |  |
| 21. Declaration |  |  |
| If signing on behalf of a relevant person, has written confirmation from a relevant person been given? | n/a |  |
| Has the operator confirmed they will comply with the conditions of the rule set? | Y |  |
| Has the operator confirmed they will comply with the NRW pre-notification requirements. | Y |  |
| Has a relevant person or approved person signed the declaration? | Y |  |

| Location | Y N n/a | Comments |
| --- | --- | --- |
| Has determining officer completed MyMap screening report? | Y |  |
| Do the grid references match the site plan? | Y |  |
| If there is storage; do the grid references match the site plan? | Y |  |
| The map must include the following details in relation to both storage and spreading (TGN 8.01, box 3.1) |  |  |
| surface water courses (permit condition: no storage or spreading within 10 meters) (buffer zones needed) | n/a | None within 10 meters that require buffering |
| groundwater source protection zones (permit condition: no storage or spreading in SPZ1) (high risk deployment if in SPZ2) | n/a | None present |
| any well, spring (or issue) or borehole used for the supply of water for human consumption including from a private water supply or supplies. (permit condition: no storage or spreading within 50 meters) | Y | “BW” fields- well to the NE of fields- shown on map and buffered “BYC” fields- none |
| location of all storage sites for the waste to be spread | Y |  |
| any spring (or issue), well or borehole not used to supply water for domestic or food production purposes within 50m (shown on map) | Y | “BW” fields- well to the NE of fields- shown on map and buffered “BYC” fields- none |
| any spring (or issue), well or borehole used to supply water for domestic or food production purposes within 250m (shown on map) | Y | “BW” fields- well to the NE of fields- shown on map and buffered “BYC” fields- none |
| any European designated sites- Natura 2000 (SPA, pSPA, SAC or cSAC) or Ramsar- or SSSI within 500m (shown on map) | N | “BW” fields- none “BYC” fields 5x SAC and 1xSSSI not shown on site plan 1xSSSI is shown on site plan, but only a small portion of it |
| location of any public rights of way (shown on map) | Y | “BW” fields- along southern edge of BW3 and BW4, in BW1 and BW2 Only one of the BY1 paths is shown on map- not included in OMP “BYC” fields- in BYC6 and BYC7. Shown on site plan- not included in OMP |
| any buildings or houses within 250m (shown on map) | Y |  |
| Within Groundwater Safeguard Zone | n/a |  |
| the location of any other designated environmentally sensitive sites within 500m. Including but not limited to: - Protected Species - Protected Habitats - Scheduled Ancient Monument - Local Nature Reserve - National Nature Reserve - Ancient Woodlands - Local Wildlife Sites (SINC) - National Park - Hedges - Ditches | Y | “BW” fields Ancient woodland/local wildlife site to east of field BW1 Local wildlife site to south of field BW2 “BYC” fields Lowland calcareous grassland- BY4, BY6, BY7 Bryn y Cosyn Round Barrows x3 Local wildlife site to the south of BCY4 & BCY5, and east of BYC6 |
| What is the slope of the fields? | 0-7 degrees (transparent)- all fields 7 -12 degrees (yellow)- fields BY1, BY2 and BY3 12 degrees plus (red)- no fields See comments section for assessment |  |

| Waste Storage | Y N n/a | Comments |
| --- | --- | --- |
| Is waste storage occurring at the place where waste is being deployed? | Y |  |
| Is only waste as specified in this deployment being stored in the proposed storage locations? | Y |  |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in the same place one constructed within the last two years? or in different location? | n/a | Not stored in temporary field heaps |
| If the waste is stackable and stored in temporary field heaps; are the storage locations in a field: - liable to flooding or becoming waterlogged, or - within 50 metres of a borehole, spring or well or - within 10 metres of surface water or a land drain - within 30 meters of a watercourse on steeply sloping land | n/a | Not stored in temporary field heaps |
| If the waste is non-stackable is the waste stored within either a vessel, a covered building, or on an impermeable surface | n/a | Not stored in temporary field heaps |
| If the waste is high RAN and it is being stored within 200 metres of a European Site (SAC/SPA), Ramsar Site or a Site of Special Scientific Interest (SSSI) is the waste being covered to prevent loss of ammonia? (condition 3.2.4 of the permit) | n/a | Wastes not high RAN |

| Waste Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Waste analysis document is missing analysis for Kelloggs waste- subsequent answers only apply to the non-Kelloggs waste analysis |  |  |
| Is the waste analysis report less than 12 months old at time of application? | Y | All wastes tested Jan/Feb/Mar 2023 |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Nitrogen (N) | Y |  |
| - Phosphorus (P) | Y |  |
| - Potassium (K) | Y |  |
| - % dry matter | Y |  |
| Does the analysis include the following? If it is a required parameter for the waste (see section below) and has not been tested for; has adequate justification been provided? |  |  |
| - Zinc | Y |  |
| - Copper | Y |  |
| - Nickel | Y |  |
| - Lead | Y |  |
| - Cadmium | Y |  |
| - Mercury | Y |  |
| - Magnesium | Y |  |
| - Sodium | Y |  |
| - Sulphur | Y |  |
| Check Appendix A of TGN 8.01 for the wastes types applied for. |  |  |
| - Are there any special testing requirements? | Y | 02 05 01- Fats/oils/grease value |
| - If yes; have they been tested for? | N | Beechdeen Ice cream not tested for FOG- ABS states it has been, but this was not included in the WA |
| If spreading waste soil, has the analysis been carried out at an MCERTS Lab? | n/a | Not spreading waste soil |
| If the spreading is in a groundwater safeguard zone (permit condition 2.1.8); are any wastes “high readily available nitrogen wastes”? i.e. organic wastes in which more than 30% of the total nitrogen content is available to the crop at the time of spreading. | n/a | Not in groundwater safeguard zone |
| - If yes, have they stated condition 2.1.8 of the permit will be complied with? | n/a | Not in groundwater safeguard zone |

| Soil Analysis | Y N n/a | Comments |
| --- | --- | --- |
| Is the soil analysis report less than 4 years old at time of application? | Y | BW analysis dated Oct 2022 BYC analysis dated July 2020 All SA within date ABS states all soils were sampled in Oct 2022- needs to be clarified. Table 6 in ABS matches SA |
| Does the analysis include the following? |  |  |
| - pH | Y |  |
| - Potassium (K) | Y |  |
| - Phosphorus (P) | Y |  |
| - Magnesium (Mg) | Y |  |
| If the waste analysis shows high PTE’s; have these been tested for in the soil? | n/a |  |
| Have the soil indices for P, K and Mg on each field been given? | Y |  |
| Does analysis show that the soils are deficient in the nutrients or properties required by the proposed cropping? | Y |  |

| Benefit Statement | Y N n/a | Comments |
| --- | --- | --- |
| Does the writer of the benefit statement have appropriate technical expertise? (TGN 8.01 6.1) | N | Section 1 of ABS states it has been “prepared for Agri spread LTd by Trade Effluent Services”, and that it has been checked by a FACTS qualified advisor and gives a list of experience and qualifications, however no information is given as to who that FACTS qualified advisor is |
| Does the activity meet the definition of recovery? (TGN 8.01 6.2) |  |  |
| - Is there a clear benefit from the activity? | Y |  |
| - Is the material suitable for its intended use? | N | Not fully demonstrated- see queries re claimed benefits, time of application, nutrient additions (sodium), waste analysis, odour control, sources of waste |
| - Is the minimum amount of waste being used to achieve the intended benefit? | Y |  |
| - Is the waste being used as a substitute for a non-waste material? | Y |  |
| - Will the proposal be completed to an appropriate standard? NOTE: For this activity, the standard is the deployment application that has been assessed and approved. Any deviation from this which has not been agreed is likely to be viewed as disposal. | Y |  |
| Benefit should be assessed against the following criteria. If the chosen benefit, has information been given on? (TGN 8.01 6.3) |  |  |
| - Plant nutrient requirements | Y | Section 6 of ABS: nutrients, specifically nitrogen, phosphorous, potash, organic matter Wastes supply these- pending Kelloggs waste analysis |
| - Timing of application | N | Section 3 of ABS: Grass fields may be spread before the growing season or after cuts of silage (May-September) where there is a significant nutrient requirement. These time periods are too vague and must be more specific. When is “before growing season”? |
| - Amount of waste to be applied and application rate | Y |  |
| - Soil chemical properties (e.g. liming and pH) | N | Claimed benefit: pH- this is not a benefit and should be in the negative effects section. 3 of the wastes have pH below 4.02 |
| - Soil physical properties (e.g. organic matter and soil structure) | Y | Section 6 of ABS: organic matter- wastes include small amount |
| - Soil water content | n/a |  |
| - Levelling the land | n/a |  |
| Timing of application- if any of the wastes are high RAN, are they being spread OUTSIDE of the closed periods | n/a | Wastes are not high RAN |
| Groundwater Safeguard Zone for nitrate- if any of the wastes are high RAN, are the application rates less than 50 m3/ha, with a minimum 3 week gap between applications | n/a | Spreading not occurring on/in Groundwater Safeguard Zone |
| If an operator wants to apply more nutrients than is recommended in RB209 have they? (TGN 8.01 6.3) |  |  |
| - Justified the nutrient level with appropriate technical expertise | n/a |  |
| - Included the nutrient level in their environmental risk assessment | n/a |  |
| - Stated the source of information that they used to set the nutrient level. | n/a |  |
| Have any potential negative impacts of the activity been addressed? (TGN 8.01 6.5) | N | pH referenced in section 7 of ABS; reasoning given in section 6 Odour issues- OMP provided- see comments section Storage tanks (section 7) states “locations may vary slightly due to unforeseen operational requirements.” this would be an operational change and would therefore need to be approved. |
| Is the application rate appropriate (i.e. not be applied in excess of crop or soil need, as this may not meet the principles of waste recovery) in terms of? (TGN 8.01 6.7) |  |  |
| - Waste type | Y |  |
| - Waste analysis | N | Unable to assess Kelloggs as WA not provided Croda Goole sodium is too high (377.75 kg/ha at application rate of 134t/ha) |
| - Crop nutrient requirements | Y |  |
| If the cropping is any of the following, is the maximum nitrogen limit in line with that given in The Water Resources (Control of Agricultural Pollution) (Wales) Regulations 2021: - 0 kg/ha- Field beans, Peas - 120 kg/ha- Sugar beet - 150 kg/ha- Asparagus, Carrots, Forage maize. Radish, Spring barley, Swedes - 180 kg/ha- Spring-sown wheat, Winter barley - 220 kg/ha- Autumn or early winter sown wheat | n/a | Cropping is grass |
| If multiple wastes are being mixed: (TGN 8.01 6.8) |  |  |
| - Does each waste confer benefit in its own right? | N | Cannot confirm due to missing Kelloggs analysis |
| - Is it only for operational purposes? | Y |  |
| - Have individual application rates for each waste been given? | Y |  |
| - If mixing ratios have not been given, is there a statement saying that the highest limiting parameter will be used as the maximum application rate allowed to be spread, i.e. the waste will be spread at the lowest application rate. | Y |  |
| Application of waste (TGN 8.01 6.10) |  |  |
| - Is method of application appropriate for the waste type and cropping? | Y |  |
| - Is the timing of application appropriate? | N | See above comments on timing of application |
| - Have they stated when the waste will be spread? | Y | See above comments on timing of application |
| - Have they stated how often the waste will be spread? | N | “Where more than one application (split) occurs through the season the application rates across the deployment will not be exceeded.” Above does not confirm how often waste will be spread. |
| Have the total nutrients supplied by the waste been given? | Y |  |
| Does the total nutrients given by the applicant match NRW calculations? | Y | Unable to verify Kelloggs |
| Has total nutrients supplied been given in oxide format? | Y |  |
| Has the SNS for each field been provided? Or enough information provided to calculate the SNS? | Y |  |
| If the operator has stated (or intelligence from local teams have told us) that other sources of nutrients have been applied in the last 12 months; have they stated: |  |  |
| - What material was spread? | Y | FYM to “all fields at Prysau Farm in 2022“ should this be “land at Allt Y Chwiler?” Waste spread to “all fields at Bryn Y Cosin Farm in 2022” |
| - How much material was spread? | Y |  |
| - At what application rate material was spread? | Y |  |
| - The nutrients supplied by the material? | Y |  |

| Site specific risk assessment (TGN 8.01 2.2) | Y N n/a | Comments |
| --- | --- | --- |
| If the deployment is in a high-risk location as per LPD1 has a site-specific risk assessment been submitted? | Y |  |
| If there are other designated environmentally sensitive sites within 500 meters has a site-specific risk assessment been submitted? | n/a |  |
| Does the site-specific risk assessment identify all potential? |  |  |
| - Sources of pollution or hazards. Potential hazards include but are not limited to dust, odours, run-off, leachate etc. | Y |  |
| - Pathways | Y |  |
| - Receptors that can be affected by that source of pollution. Receptors include but are not limited to environmentally sensitive sites, watercourse, local residents, public rights of way, housing, schools, workplaces and recreational facilities etc. | Y | Receptors lists are: - Halkyn Mountain / Mynydd Helygain SAC - Comin Helygain a Glaswelltiroedd Treffynnon / Halkyn Common and Holywell Grasslands SSSI - Parc Linden, Lixwm SSSI |
| - Measures that will minimise or mitigate the impact will be put in place | Y | Spreading method doesn’t match Incorrectly states SSSI(s) are shown on maps. Only small part of 1 SSSI is shown. SSRA doesn’t include any distances between spreading/storage and designates sites. |

| Appendix 4 assessment | Y N n/a | Comments |
| --- | --- | --- |
| Is the site within 500 meters of a SSSI? - If yes, the complete an Appendix 4 CroW assessment. | Y |  |
| Has a likely significant effect been identified? - If yes, consult with the relevant NRM team. - If no, save the Appendix 4 to the DMS as information only |  | Not completed due to errors in risk assessment |

| Habitat Regulation Assessment | Comments |
| --- | --- |
| Is the site within 500 meters of a Natura 2000 (SPA, pSPA, SAC or cSAC) or a Ramsar site? | Y |
| No FORM 1 completed: Not completed due to errors in risk assessment |  |

Comments

Consultation responses: Information provided that requires addressing: - PAN-015919 deployment for Bryn y Cosyn received numerous odour complaints due to surface spreading in hot conditions. This is a highly sensitive area. Storage location BYC1 it is likely that this location may have added to the odour nuisance and storage location BYC1 should not be sited here again. - query re timing of spreading- “before growing season” - Site addresses for the waste producers are not specified. - FOG for Beechdean not included - No analysis submitted for Kelloggs - Following site visit- fields BYC7&6 are very scrubby/overgrown/unimproved and are not suitable for landspreading of waste - they should be removed.

MyMap, location and slope assessments MyMap/location assessment: - watercourses, wells and burrows all correctly buffered. - SAC and SSSI are not shown on site plans - public rights of way are not correctly shown on and around fields BW1, BW2, BW3, BW4, BYC6 and BYC7 - Local wildlife site to south of field BW2 - Local wildlife site to the south of BCY4 & BCY5, and east of BYC6 - Google earth shows fields BYC6 and BYC7 may not be suitable for spreading. This was also highlighted in a consultation response and in the aerial view provided on page 4 of the ABS Slope assessment All fields have areas of no colour (0-7 degrees), no fields have any areas of red (12+ degrees) Field number: BY1, BY2 and BY3 Slope of the land: yellow- 7 -12 degrees Soil type: Clay Loam Wastes types applied (solid, liquid, sludge etc.): sludges Cropping type: grass- silage and grazing Method of application: subsoil injection- deep-leg injector Weather conditions before during and after spreading: will follow permit conditions and CoGAP Other risk factors not mentioned above: none Conclusions (is run off risk high?): no- no further action required

CPH details: On LPD1 form: 46/204/0136 In “agben” and field information”: - Allt Y Chwiler- 56/204/0136 - Bryn-YCOSYN Farm- 56/204/0066 MyMap: - “BW” fields: 56/204/0136- MESSRS B AND V WILLIAMS- PRYSAU FARM - “BYC” fields: 56/204/0066- MRS GLENYS ANWEN HUGHES- BRYN Y COSYN FARM CPH numbers match

Protection of habitats and ecosystems - aerial images provided as evidence

Agricultural benefit calculations: Secanim - Nutrient offtake is higher that waste additions on all fields - Fats and Oils- 12750 mg/kg- well within permissible range of 40,000 mg/kg (4%) as set out in TGN8.01 Croda Goole - Nutrient offtake is higher that waste additions on all fields - Sodium is too high based on max sodium levels (377.75 kg/ha) at application rate of 134 t/ha Croda Widnes - Nutrient offtake is higher that waste additions on all fields Maelor Foods - Nutrient offtake is higher that waste additions on all fields Encirc - Nutrient offtake is higher that waste additions on all fields Beechdean Ice cream - pH of 3.93 - no FOG value- ABS states it was included - Nutrient offtake is higher that waste additions on all fields PENDERYN DISTILLERY - pH of 3.74 - Nutrient offtake is higher that waste additions on all fields Kelloggs - no analysis is provided

Other information to note: Waste specific information from TGN 8.01 02 02 04- sludges from on-site effluent treatment plant from abattoirs, poultry preparation plants, rendering plants or fish preparation plants only - odour - waste must be injected 02 03 05- sludges from on-site effluent treatment - none 02 05 01- biodegradable materials unsuitable for consumption or processing - Fats/oils/grease value - spread over several applications - if growing crop- waste should be spread on the soil surface not onto the crop - high application rates may coat soil particles- creating waterproof barrier 02 07 05- sludges from on-site effluent treatment - non-alcoholic drinks only - acidic- should not be applied to spoils with pH less than 5.5 - high BOD - difficult to justify untreated waste - odours 07 07 12- sludges from on-site biological effluent treatment plant at chemical manufacturing sites other than those mentioned in 07 01 11 only - none Agricultural Benefit Statement States Tank 1 at NGR SJ 17168 72719 is at Prysau Farm- should this not be “land at Allt Y Chwiler”? Odour Management Plan - not site specific as doesn’t include reference to any specific fields, storage locations or receptors (public via rights of way/use of the area and residential uses) nor the distances between them. It only references the general field names/areas: “land at Allt Y Chwiler” and “Bryn-Y-Cosyn Farm” - makes reference to waste stored in lagoons, however this is not proposed under this deployment - doesn’t include weather conditions or wind directions - doesn’t include specific information on waste types H4 Odour Management guidance sets out what information should be in an odour management plan. This guidance has not been followed Sections of the ABS refer to odour storage plans for the storage location in BYC1, however this has not been supplied, and the odour management plan does not include site specific information. Odour and Noise Control section of ABS gives a distance from storage to dwellings (384m at Prysau Farm) but it is not clear which of the 3 storage locations are being referenced. PAN-015919 deployment for Bryn y Cosyn received numerous odour complaints due to surface spreading in hot conditions. This is a highly sensitive area. Storage location BYC1- it is likely that this location may have added to the odour nuisance and storage location BYC1 should not be sited here again.

| Decision |  |
| --- | --- |
| ## Can the deployment application be approved? | No |
| ## Issue date | n/a |
| ## Effective date | n/a |
| ## Permitting Officer Date | Louise Bailey 13/06/2023 |
| ## Team leader or Transfer of Powers (ToP) approval Date | Huw Davies 13/06/2023 |

| Time recording |  |
| --- | --- |
| Admin/consultations | 60 |
| Determination | 735 |
| Sign off | 10 |
| Issuing | 10 |
| Total | 815 minutes |